3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2103426

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	6.1.3.1

Source:	Ericsson

Title:	Remaining corrections for Intra-UE prioritization

Work Item:	NR_IIOT-Core

Document for:	Discussion, Decision

# 1	Introduction

This paper discusses the RAN1 LS [1] on overlapped data and SR of equal L1 priority. In the LS, RAN1 asks RAN2 a couple of MAC specification understandings.

# 2	Discussion

## 2.1 Case 2-1 and case 4

Case 2: other UCI(s) i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority and the SR overlaps with the PUSCH of an equal L1 priority - Case 2-1: the final PUCCH resource after UCI multiplexing among different PUCCHs carrying HARQ-ACK/CSI and SR does not overlap with the PUSCH Case 2-1: the final PUCCH resource after UCI multiplexing does not overlap with PUSCH For case 2-1, if there are other UCI(s) i.e., HARQ-ACK/CSI of the equal L1 priority overlapping with SR, and the final PUCCH resource after UCI multiplexing among different PUCCHs does not overlap with the PUSCH and does not overlap with any other PUSCH if any, RAN1 has the following two understandings: - Understanding 1: MAC is not aware of the UCI multiplexing in PHY, MAC does not know whether the final PUCCH overlaps with the PUSCH or not, MAC only knows configured PUCCH resource for SR. Therefore, MAC can decide to deliver SR or PUSCH. - Understanding 2: MAC is aware of the UCI multiplexing in PHY based on UL skipping agreement (as in LS R1-2009772). If MAC is aware that the final PUCCH resource does not overlap with the PUSCH, and does not overlap with any other PUSCH, then for case 2-1, MAC can send both SR and PUSCH to PHY. Case 4: other UCI(s), i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority For case 4, if there is no resource overlapping between SR and PUSCH of an equal L1 priority, and the final PUCCH resource after UCI multiplexing among different PUCCHs overlap with the PUSCH, RAN1 has the following two understandings: - Understanding 1: MAC is not aware of the UCI multiplexing in PHY, MAC does not know whether the final PUCCH overlaps with the PUSCH or not, MAC only knows configured PUCCH resource for SR. Therefore, MAC can send both SR and PUSCH to PHY, based on current RAN1 specification TS 38.213, PHY will multiplex other UCI(s) i.e., HARQ-ACK/CSI in the PUSCH and does not transmit SR. - Understanding 2: MAC is aware of the UCI multiplexing in PHY, If MAC is aware that the final PUCCH resource overlaps with the PUSCH, then MAC can decide to deliver SR or PUSCH.

These cases are not unique in the Rel-16 IIoT discussion but also apply for the Rel-15 MAC specification. In the MAC specification, the wording is that “the PUCCH resource for the SR transmission occasion”. See below highlights from subclause 5.4.4, MAC 38.321 v15

1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a UL-SCH resource:

In the MAC 38.321 v16, the same wording as the v15 is used.

3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specfied in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

In Rel-15, the impact of UCI multiplexing was not explicitly discussed in RAN2 and so “the SR transmission occasion” is understood as the SR transmission occasion configured by the RRC, not the “real” SR transmission occasion after the UCI multiplexing procedure in PHY layer specs. Since the same wording is used in Rel-16, the same understanding applies here for Rel-16.

For understanding 2, we would like to point out that the LS R1-2009772 [4] refers to a specific case, i.e., the MAC consider the UCI multiplexing outcome during the UL skipping check, and the correction CR [5] does not modify the text in the scheduling request sub-clause (5.4.4). One should not over-interpret the consequences and conclude that MAC should be aware of the UCI multiplexing in all cases. Whether it is needed in the cases of SR and PUSCH overlapping should rather be based on the discussion of the technical needs, the business needs, and/or the implementation complexity. If there is no such discussion, one should assume the legacy behavior which is that the MAC is not aware of the UCI multiplexing.

RAN2 to reply that: “for case 2-1 and case-4, as in legacy Rel-15, MAC is not aware of the UCI multiplexing in PHY. “

## 2.2 Case 2-2 and case 3

- Case 2: other UCI(s) i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority and the SR overlaps with the PUSCH of an equal L1 priority - Case 2-2: the final PUCCH resource after UCI multiplexing among different PUCCHs carrying HARQ-ACK/CSI and SR overlaps with the PUSCH Case 2-2: the final PUCCH resource after UCI multiplexing overlaps with PUSCH - Case 3: other UCI(s) i.e., HARQ-ACK/CSI overlaps with a PUSCH of an equal L1 priority, SR overlaps with the PUSCH of equal L1 priority, but other UCI(s) do not overlap with the SR Case 3: other UCI(s) overlaps with a PUSCH, SR overlaps with the PUSCH, SR does not overlap with other UCI(s) For case 2-2 and case 3, RAN1 has the following two different understandings: - Understanding 1: the UL skipping-related check is prioritized over the LCH based prioritization check in MAC. Therefore, if the PUSCH in the LS is expected to have UCI multiplexing, MAC does not prioritize SR over PUSCH, and send a MAC PDU to PUSCH instead. - Understanding 2: the LCH based prioritization check is prioritized over the UL skipping-related check in MAC. Therefore, the SR in the LS is prioritized in MAC and is delivered and MAC shall not deliver the MAC PDU for the PUSCH.

The existing MAC spec has a specific order in which the LCH based prioritization check comes before the UL skipping-related check. One can argue that if this procedure is to be followed, the LCH based prioritization check is prioritized over the UL skipping-related check (UCI-related). However, there is no limitation (specification-wise) that MAC spec cannot achieve the other order, e.g., it can be clarified the grant that is to have UCI multiplexed is assigned with the highest LCH priority. In this way, the UL skipping-related check is prioritized over the LCH based prioritization check. One example TP is to add the below underlined sentence in subclause 5.4.1.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR. The priority of an uplink grant for which there is UCI to be multiplexed is higher than the priority of any uplink grant which does not have UCI to be multiplexed.

The MAC spec can support that the UL skipping-related check is prioritized over the LCH based prioritization check.

Since the MAC spec can support either priority order, we believe that the discussion should focus on which order is the most useful one. If a UCI-to-be-multiplexed grant can be skipped, then gNB must blind-decode two hypotheses in each serving cell, i.e., one hypothesis is that UCI is multiplexed on PUSCH and another hypothesis is that the UCI is not multiplexed on PUSCH and then subsequently might be multiplexed on another PUSCH in another cell. This latter multiplexing procedure to another cell is the legacy PHY procedure. Blinding-decoding PUSCH is considerably more challenging than PUCCH and this is further complicated if there are multiple serving cells for a UE. See more details in the paper in the last meeting [3].

If both supported, the UL skipping-related check should be prioritized over the LCH based prioritization check, in light of UE and gNB implementation complexity.

The email discussion “[AT113-e][019][NR16 IIOT] UL Skipping” [2] has discussed a similar issue. Since the technical aspects are originated and discussed in RAN1, it is not clear how RAN2 can discuss and even conclude on this issue. As a compromise, a working assumption is agreed simply by a count of votes.

[019] Working assumption: The MAC entity does not generate a MAC PDU for a deprioritized uplink grant even when its associated PUSCH is overlapping with PUCCH. This working assumption is not agreed until confirmed by RAN1.

For the understanding that the LCH based prioritization check is prioritized over the UL skipping-related check, the argument for it is that it has limited MAC specification impacts. However, this understanding has further complications in the PHY/MAC layer interactions and has impacts on the PHY processing timeline. In a nutshell, it has been assumed that the PUSCH resource is ALWAYS present in the UCI multiplexing procedure. In the understanding 2, a MAC PDU may not be delivered for a PUSCH by MAC, and so the PHY layer cannot perform UCI multiplexing until the PHY layer is certain this PUSCH may/may not be part of the UCI multiplexing procedure, or, the UE has assumed the presence of the PUSCH and later needs to re-perform another UCI multiplexing procedure without this PUSCH. Extensive PHY processing timeline discussions are expected.

The understanding that “the LCH based prioritization check is prioritized over the UL skipping-related check” has large impacts on PHY processing timeline.

In light of the discussions in this paper and the conclusion in the last RAN2#113 meeting, one compromise is to dis-allow a simultaneous configuration of the LCH-based prioritization and the UL-skipping. This option is also mentioned in the RAN1 email discussion [6] (see the option 1 in the last question of Third Round in the section 3.2.1).  We believe this is a good way forward.

Third Round Q-E: if RAN1 cannot make consensus on whether to extend the UL skipping agreement to the case where LCH prioritization is configured, and there is resource overlapping for PUCCH and PUSCH(s) of a single PHY priority, any suggestions on how to move forward? - Option 1: no need to configure the two features UL skipping and LCH based prioritization simultaneously, RAN1 continue discussing how to handle the UCI transmission, UCI is based on the hypothetical PUSCH or actual PUSCH. - Option 2: consult with RAN2, asking RAN2 views on how they implement UL skipping and LCH prioritization, for example, whether UL skipping rule can be prioritized over LCH prioritization? - Option 3: other options, any suggestions?

To sum up, in this paper, we propose the below compromise for this discussion.

RAN2 to reply that: “for case 2-2 and case-3, RAN2 has not concluded on the intended MAC layer behavior and the MAC specification can support (with simple CRs) both understandings. A third understanding, that the UL skipping feature and the LCH-based prioritization feature are not configured simultaneously, is also acceptable for RAN2.“

# 3. Conclusion

In the previous sections we made the following observations:

Observation 1	The MAC spec can support that the UL skipping-related check is prioritized over the LCH based prioritization check.

Observation 2	If both supported, the UL skipping-related check should be prioritized over the LCH based prioritization check, in light of UE and gNB implementation complexity.

Observation 3	The understanding that “the LCH based prioritization check is prioritized over the UL skipping-related check” has large impacts on PHY processing timeline.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to reply that: “for case 2-1 and case-4, as in legacy Rel-15, MAC is not aware of the UCI multiplexing in PHY. “

Proposal 2	RAN2 to reply that: “for case 2-2 and case-3, RAN2 has not concluded on the intended MAC layer behavior and the MAC specification can support (with simple CRs) both understandings. A third understanding, that the UL skipping feature and the LCH-based prioritization feature are not configured simultaneously, is also acceptable for RAN2.“

# 4 References

R1-2102244, Reply LS on overlapped data and SR are of equal L1 priority

R2-2102458, Report of [AT113-e][019][NR16 IIOT] UL Skipping, vivo

R2-2100341, UL PUSCH skipping with Intra-UE prioritization, Ericsson

R1-2009772, LS on PUSCH skipping with UCI in Rel-16

R2-2102459, Correction to PUSCH skipping with UCI without LCH-based prioritization

R1-2102151, Email discussion summary on [104-e-NR-L1enh-URLLC-06] for intra-UE prioritization

# 5 Text Proposal for 38.321

### 5.4.1	UL Grant reception

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this deprioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. The priority of an uplink grant for which there is UCI to be multiplexed is higher than the priority of any uplink grant which does not have UCI to be multiplexed.