3GPP TSG-RAN WG2 Meeting #113bis e	R2-2103545

Online, April 12 – 20, 2021

Agenda item:	8.3.3

Source:	Sony

Title:	Discussion on Busy Indication, and leaving in Multi-SIM

Document for:	Discussion

# Introduction

In previous RAN2#113e meeting the RRC based busy indication was discussed but no conclusions were agreed [1]. The pros and cons of the RRC based busy indication needs to be discussed further.

The Leaving from using one SIM to the other one was also discussed with a few agreements during last RAN2 meeting, also here the discussion of pros and cons of RAN based Leaving indication.

In SA2, the normative work for the NAS based busy indication and leaving. However SA2 is still awaiting RAN feedback on the progress in RAN2 in order to coordinate the efforts, see LS from SA2 [2][3].

# Busy Indication

In SA2 they are discussing how to implement the NAS based busy indication in the normative specifications. The busy indication is used by a UE to inform the NW that the UE cannot set up a connection based on a received paging. When receiving the busy indication the AMF or MME in the core network can stop the page to the UE and thereby also stop the paging escalation and save paging resources. Furthermore, the networks has knowledge about the position and reachibility of the UE when receiving the Busy Indication.

Due to enhanced security aspects in 5GS, the core network shall provide a new temporary ID after receiving a service request due to paging the UE. This means that a new 5G-GUTI will be provided from core network every time the UE responds with a NAS based busy indication. The UE must therefore, when a busy indication is sent to NAS, continue to be active while waiting for the response from Core Network containing the new 5G-GUTI.

For EPS, the network does not assign a new temporary ID after receiving a service request due to paging the UE. For EPS this means that there is an opportunity to optimize the NAS based busy indication response, since the UE does not necessarily need to wait for a response from the MME.

In 5GS, and for both NR and LTE, a UE has two sleep states RRC_IDLE and RRC_INACTIVE. The AMF is responsible for paging when the UE is in RRC_IDLE, and RAN is responsible when the UE is in RRC_INACTIVE  state. In case of RRC_INACTIVE, when RAN is responsible for sending the paging, then it would be relevant for the UE in RRC_INACTIVE state to send a busy indication to RRC as a new response to RAN paging instead of sending the busy indication to the AMF which then would forward it to RAN. The busy indication should as a proposal be sent in the RRCResumeRequest to RRC, this message is protected and the network can respond with the RRC Release message as specified in 3GPP TS 38.331.

Observation 1: In RRC_INACTIVE state in 5GS/NR, and in LTE, an agreements is needed  by RAN plenary to make changes in 36.331, a RAN-based busy indication would optimize the time the UE is away from the other network. The latency would change from the estimated time of up to 100ms for the NAS based to a about 10 ms for the RAN based proposal [3]

Proposal 1: Specify a RAN based busy indication as a response to RAN paging when in RRC_ Inactive state.

# Leaving

In EPC/LTE, the leaving is proposed to be handled in Core Network to minimize the impact on RAN. The leaving is not very time sensitive since the UE is leaving one PLMN and have not started the communication on the other PLMN. Therefore it is proposed to have same solution in 5GC as in EPC. There is no reason to have different implementations.

Observation 2: There is no reason to have different solution for the MuSIM leaving procedures in 5GC compared to what is decided for EPC, where it is decided to be NAS based,. This signalling for leaving is not time critical.

Proposal 2: To use the NAS-based leaving procedure for a MuSIM device when switching from one SIM to another SIM.

# Conclusion

Observation 1: In RRC_INACTIVE state in 5GS/NR, and in LTE, if it is agreed to make changes in 36.331, a RAN-based busy indication would optimize the time the UE is away from the other network. The latency would change from the estimated time of up to 100ms for the NAS based to a about 10 ms for the RAN based proposal [2]

Observation 2: There is no reason to have different solution for the MuSIM leaving procedures in 5GC compared to what is decided for EPC, where it is decided to be NAS based,. This signalling for leaving is not time critical.

Proposal 1: Specify a RAN based busy indication as a response to RAN paging when in RRC_ Inactive state.

Proposal 2: To use the NAS-based leaving procedure for a MuSIM device when switching from one SIM to another SIM.

# References

- Draft Report of 3GPP TSG RAN2#113-e meeting

- R2-2008754, LS on System support for Multi-USIM devices, RAN2#112-e, November 2-12, 2020.

- R2-2011241, Reply LS on System support for Multi-USIM devices, RAN2#112-e, November 2-12, 2020.