3GPP TSG RAN WG2 Meeting #113bis-e                                                        R2-2103649

e-Meeting, 12th – 20th April, 2021

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on MRB

Document for:	Discussion and Decision

1	Introduction

In RAN2#113-e, RAN2 agreed one option of MRB structure as follows:

For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast).

This document discusses other remaining issues on MBS radio bearer (MRB).

2	Discussion

In RAN1#104-e, RAN1 agreed to combine PTM and PTP for a HARQ transmission and retransmission as follows:

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

This RAN1 agreement implies that PTP MAC PDU and PTM MAC PDU will be for the same logical channel and the same RLC entity. However, RAN2 has assumed that an RLC entity is either PTM RLC or PTP RLC. Thus, we need to define a different type of RLC entity associated with both PTM HARQ transmission by group common RNTI (e.g. G-RNTI) and PTP HARQ transmission by UE specific RNTI (e.g. C-RNTI or new PTP RNTI). In our view, PTM RLC entity associated with a group RNTI supporting PTM transmission should be considered as a PTM RLC bearer. An RLC entity associated with no PTP transmission can be considered as a special case of the PTM RLC bearer.

Proposal 1. PTM RLC bearer is associated with one group RNTI (e.g. G-RNTI) and optionally one unicast RNTI (e.g. C-RNTI or new RNTI).

Proposal 2. PTP RLC bearer is associated with one unicast RNTI (e.g. C-RNTI or new RNTI) and no group RNTI (e.g. G-RNTI).

With the definition of RLC bearer above, RAN2 can proceed the discussion on MRB type and switching mechanism.

RAN2 is currently assuming a split-like bearer with a common PDCP. The split-like bearer is configured with one PTM RLC entity shared by multiple UEs and one PTP RLC dedicated to a UE. In the split bearer of unicast, PDCP is the anchor entity and PDCP reordering function supports in-sequence delivery and duplicate detection. For MRB, the same reordering function can be applied.

Regarding the split-like MRB, we could have two switching options between PTM RLC and PTP RLC:

-  Option A (Implicit Switching): Two RLCs are always active. gNB can choose one switching option among 1) delivery via PTM RLC 2) delivery via PTP RLC 3) delivery via PTM RLC and PTP RLC (i.e., duplication) by gNB implementation. The selected switching option is not indicated to UE. Since PDCP has duplicate detection and discard function, the duplicate reception will not be a problem.

Figure 1. Option A: switching by gNB implementation

-  Option B (Explicit Switching): Active RLC (PTM or PTP) is switched by NW signalling, e.g., MAC/PDCP/RRC/etc. UE only needs to monitor G-RNTI or C-RNTI according to the indicated data path.

Figure 2. Option B: switching by NW signaling

In uplink of unicast, data path, i.e., primary RLC, split threshold, duplication, is configured by gNB and a UE just follows the configuration. In contrast, there is no such configuration for downlink data path selection which is all up to NW implementation. MBS is mainly for downlink transmission. Considering the complexity of UE operation, Option A, similar to downlink split bearer, seems better.

Proposal 3. Assuming split-like MRB, dynamic switching is up to gNB. No additional switching mechanism (e.g. switching indication by MAC/PDCP/RRC) is necessary.

Proposal 4. RLC switching and MAC switching are not additionally supported.

Figure 3. PTM-only MRB

For MBS, PTP RLC is not always necessary. If NW wants not to configure PTP RLC, an MBS bearer associated with only one PTM RLC should be possible as shown in Figure 3. To sum up, an MRB can be configured with one PDCP, one PTM RLC and optionally one PTP RLC. A radio bearer without PTM RLC is the same as the unicast radio bearer (DRB/SRB). We do not need to define an MRB without PTM RLC.

Proposal 5. For MBS RB, RRC signalling configures one PDCP, one PTM RLC and optionally one PTP RLC, i.e. two bearer types are supported:

- Split MRB configured with one PTM RLC and one PTP RLC

- MRB configured with one PTM RLC.

We may need to consider MBS bearer re-configuration, for instance, PTP RLC is released or newly added. This reconfiguration may occur according to NW’s policy, UE’s mobility and so on. Also, bearer type change from MBS RB to unicast DRB can be considered.

Proposal 6. Reconfiguration of MRB RB from MRB to unicast DRB is supported.

Proposal 7. MRB type change is supported.

When the MRB type is changed, data loss may occur. The data loss needs to be recovered by PDCP retransmission based on status report as in unicast transmission. It should be noted that PDCP status report is supported only for RLC AM and transmitted via the primary RLC entity. Hence, we can reuse this principle.

Proposal 8. PDCP Status Report can be transmitted for MRB type change.

Proposal 9. Status report is transmitted via AM RLC entity.

3	Conclusion

The below proposals are made:

Proposal 1. PTM RLC bearer is associated with one group RNTI (e.g. G-RNTI) and optionally one unicast RNTI (e.g. C-RNTI or new RNTI).

Proposal 2. PTP RLC bearer is associated with one unicast RNTI (e.g. C-RNTI or new RNTI) and no group RNTI (e.g. G-RNTI).

Proposal 3. Assuming split-like MRB, dynamic switching is up to gNB. No additional switching mechanism (e.g. switching indication by MAC/PDCP/RRC) is necessary.

Proposal 4. RLC switching and MAC switching are not supported.

Proposal 5. For MBS RB, RRC signalling configures one PDCP, one PTM RLC and optionally one PTP RLC, i.e. two bearer types are supported:

- Split MRB configured with one PTM RLC and one PTP RLC

- MRB configured with one PTM RLC.

Proposal 6. Reconfiguration of MRB RB from MRB to unicast DRB is supported.

Proposal 7. MRB type change is supported.

Proposal 8. PDCP Status Report can be transmitted for MRB type change.

Proposal 9. Status report is transmitted via AM RLC entity.