3GPP TSG-RAN WG2 #113bis-e	R2-2103661

Electronic meeting, 12th – 20th April 2021

Agenda Item:	6.1.4.1.4

Source:	Ericsson

Title:	Introducing the UE config release in INM

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2 meeting, it was discussed whether to introduce a UE config release to mimic was has been done in LTE. However, no conclusion was reached during the meeting and the topic was postponed:

R2-2101023	Introducing UE Config Release for NR	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.1	2378	-	B	TEI16

-	[014] Rap: Some companies see some benefit (“field exist in LTE”), but since no support by other network vendors, the rapporteur proposes to not agree the CR. As one company remarked, this is a duplication of information.

-	[014] was later requested to be postponed.

[014] Postponed

R2-2101024	Improving description of ue-ConfigRelease	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.3.0	4561	-	F	TEI16

[014] Postponed

In this contribution, we would like to clarify what is out point of view about the introduction of this new feature and why it does not make sense to have it for NR.

# 2	Discussion

The ue-ConfigRelease is a field that has been introduced during Rel-9 in LTE. The main motivation for introducing this field in LTE was for the target node to decide whether full configuration should be applied in case of handover.

HandoverPreparationInformation-v920-IEs	::= SEQUENCE {

ue-ConfigRelease-r9					ENUMERATED {

rel9, rel10, rel11, rel12, v10j0, v11e0,

v1280, rel13, ..., rel14, rel15, rel16}		OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v9d0-IEs		OPTIONAL

}

HandoverPreparationInformation field descriptions

as-Config The radio resource configuration. Applicable in case of intra-E-UTRA handover. If the target receives an incomplete MeasConfig and/or RadioResourceConfigDedicated in the as-Config, the target eNB may decide to apply the full configuration option based on the ue-ConfigRelease.

as-Context Local E-UTRAN context required by the target eNB.

makeBeforeBreakReq To request the target eNB to add the makeBeforeBreak indication in the mobilityControlInfo in case of intra-frequency handover.

rrm-Config Local E-UTRAN context used depending on the target node's implementation, which is mainly used for the RRM purpose. May also be provided at inter-RAT handover from NR.

sourceRB-ConfigIntra5GC NR radio bearer config used at intra5GC handover, as defined by RadioBearerConfig IE in TS 38.331 [82].

ue-ConfigRelease Indicates the RRC protocol release or version applicable for the current UE configuration. This could be used by target eNB to decide if the full configuration approach should be used. If this field is not present, the target assumes that the current UE configuration is based on the release 8 version of RRC protocol. NOTE 1.

ue-RadioAccessCapabilityInfo For E-UTRA radio access capabilities, it is up to E-UTRA how the backward compatibility among supportedBandCombinationReduced, supportedBandCombination and supportedBandCombinationAdd is ensured. If supportedBandCombinationReduced and supportedBandCombination/supportedBandCombinationAdd are included into ueCapabilityRAT-Container, it can be assumed that the value of fields, requestedBands, reducedIntNonContCombRequested and requestedCCsXL are consistend with all supported band combination fields. NOTE 2

ue-SupportedEARFCN Includes UE supported EARFCN of the handover target E-UTRA cell if the target E-UTRA cell belongs to multiple frequency bands.

NOTE 1:	The source typically sets the ue-ConfigRelease to the release corresponding with the current dedicated radio configuration. The source may however also consider the common radio resource configuration e.g. in case interoperability problems would appear if the UE temporary continues extensions of this part of the configuration in a target PCell not supporting them.

At the time, introducing this indication was mainly because there were network implementation not using any ASN.1 decoder at the terget eNB and it was then difficult to detect and skip the non-comprehended parts. Please note that these non-comprehended parts refer basically to the case on when the source and target nodes implement two different 3GPP releases or two different versions of the same 3GPP release.

The main motivation in LTE for introducing the ue-ConfigRelease was mainly because there were network implementation not using any ASN.1 decoder at the target eNB and it was then difficult to detect and skip the non-comprehended parts.

However, when it comes to NR, the expectation is that pretty much all the network implementations use the ASN.1 decoder to “softly” detect that the received bitstream contains (or not) non-comprehended parts. It is said “softly” because the target node does not need a full decoding of the entire message from the source node.

On top on this, during the time that the ue-ConfigRelease is used (and maintained) in LTE, there are different examples in which this field it turned out to be anyway insufficient. One of those is when the source node uses the Rel-16 ASN.1 but does not use any rel-16 extensions in the UE configuration.

Therefore, an issue that comes with the implementation of the ue-ConfigRelease is also how to maintain the values of this field. There is not only the need to include the release, but we need to make sure each time to include also the version of a given release. Given that each release may have 10+ versions, this on the long term will become quite unpractical. On top on this, since the main scenario the ue-ConfigRelease wants to address is the inter-vendor one, RAN2 would need to be very careful on how such field should be set and a clear behaviour on how the source node should set the values of this field would be needed.

The target node can use the ASN.1 decoder to detect that the received bitstream contains (or not) non-comprehended parts.

It is not clear how the ue-ConfigRelease should be maintained, which value to assign, and how the source node should set this field. This requires additional standardization effort.

All in all, it is clear that the introduction on the ue-ConfigRelease field in NR does not bring any benefits are the target node has already other means to understand whether the received bitstream contains (or not) non-comprehended parts and thus trigger full configuration. According to this, and with the target to reduce the standardization efforts to introduce this field, our proposal is to not support the ue-ConfigRelease field in NR.

The ue-ConfigRelease field is not introduced in NR.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The main motivation in LTE for introducing the ue-ConfigRelease was mainly because there were network implementation not using any ASN.1 decoder at the target eNB and it was then difficult to detect and skip the non-comprehended parts.

Observation 2	The target node can use the ASN.1 decoder to detect that the received bitstream contains (or not) non-comprehended parts.

Observation 3	It is not clear how the ue-ConfigRelease should be maintained, which value to assign, and how the source node should set this field. This requires additional standardization effort.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The ue-ConfigRelease field is not introduced in NR.

# References