3GPP TSG-RAN WG2 #113b-e	R2-2103675

Electronic meeting, 12th – 20th Apr, 2021

Agenda Item:	8.16.2

Source:	Ericsson

Title:	SNPN access using subscription from external Credentials Holder (CH)

Document for:	Discussion

# 1	Introduction

During RAN2#113e, the following was agreed related to broadcast:

- A new indicator that "access using credentials from a separate entity is supported" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenarios.

- RAN2 assumes that the new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1.

- The supported Group IDs are broadcasted

- A new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenario.

- RAN2 assumes that the new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1.

Moreover, it was agreed that in the UE, AS reports to NAS the following broadcasted parameters:

- Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN

- Supported Group IDs

- Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN.

Open issues to SA2 were asked to SA2 in the RAN2 LS, R2-2102489 [1], and SA2 replied in S2-2101076 [2] as follows:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

[SA2 answer] Yes, These parameters should be set uniformly per SNPN.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

[SA2 answer] Yes, It is assumed that that the Group IDs will be broadcast per SNPN.

According to the email discussion summary in R2-2102413 [3], the following issues need further discussion:

- the maximum number of Group IDs, and the SIB to be used for broadcasting GIDs.

- whether there is a need to broadcast Human Readable Network Names for GIDs.

- whether supporting SNPN with subscription or credentials by a separate entity has impacts on cell selection or cell reselection based on feedback received from SA2

- whether supporting SNPN with subscription or credentials by a separate entity has impacts on connected mode mobility; inputs from other groups are needed.

- whether the indication "access using credentials from a separate entity is supported" needs to be set if the SNPN only supports access via credentials from a separate entity indicated by GIDs.

It should be noted that SA2 agreed to refer to

- the separate entity, or Service Provider (SP), as Credentials Holder (CH);

- the GID as Group ID for Network selection (GIN).

The following is specified in TS 23.501, clause 5.30.2.2 [6], regarding SIB:

- An indication per SNPN of whether access using credentials from a Credentials Holder is supported;

- List of supported Group IDs for Network Selection (GINs) per SNPN. GIN reuses the NID encoding in TS 23.003 [15] and can be self-managed or globally unique;

- An indication per SNPN of whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e. UEs that do not have any PLMN ID and NID nor GIN broadcast by the SNPN in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs.

In this contribution, we will address the above open issues.

To be aligned with SA2 terminology, we will use “GIN” as abbreviation instead of “GID”.

# 2	Discussion

## 2.1	 Broadcasting details

### 2.1.1	How to set the new broadcast parameters

At RAN2#113e, RAN2 agreed on a new indicator that "access using credentials from a separate entity is supported" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenarios.

The indication was mainly introduced for the “SNPN ID solution”, i.e., for UEs that use the SNPN ID in the pre-configured UE NAS list to compare it with the SNPN ID broadcast in SIB1, for the SNPN selection. Thus, one bit will be introduced, typically the optional “ENUMERATED {supported}” bit, which will be set to ‘0’ to indicate absence of the “supported bit”, and to ‘1’ to indicate presence of the “supported bit”.

For the “GIN solution”, the UE compares the GIN as pre-configured in the UE NAS list with the supported GIN broadcast in SIB.

In R2-2102413 [3], some companies proposed that this indication does not need to be set if the SNPN only supports access using 3rd party credentials indicated by GINs. Even if the presence of a GIN in the SIB implicitly indicates that 3rd party credentials are supported, the explicit bit indicator will anyway be broadcast. In this regard, introducing a clarification that the bit does not need to be set to ‘1’ for specific cases unnecessarily reduces the readability of the specification. Therefore, the bit should simply be set in accordance with its meaning, i.e., whenever 3rd party credentials are supported by the SNPN.

The indicator "access using credentials from a separate entity is supported" is set independent of the GIN broadcast.

### 2.1.2	GIN broadcasting details

As captured in TS 23.501, clause 5.30.2.2 [6], the GIN reuses the encoding of SNPN IDs, i.e., PLMN ID + NID. The PLMM ID is composed by the MCC and MNC, where the MCC consists of 3 (optional) digits, plus 1 bit to indicate its absence/presence and the MNC consists of 2-3 digits. A PLMN ID is thus represented by 9-25 bits, as shown in the corresponding ASN.1 code below. On the other hand, the NID comprises 44 bits. This means that one GIN requires the broadcast of 53-69 bits.

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC     OPTIONAL,   -- Cond MCC     -- 1 bit or 1+12 bits

mnc                                 MNC									-- 8-12 bits

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

#### 2.1.2.1	Providing the GINs per SNPN

At RAN2#113e, RAN2 agreed that the “supported Group IDs are broadcasted”. However, one company expressed some doubt whether the GINs should be broadcast per SNPN or per cell. Therefore, SA2 was asked to clarify and later confirmed in the reply LS, S2-2101076 [2], that:

It is assumed that that the Group IDs will be broadcast per SNPN.

In network sharing scenarios, multiple SNPNs may support the same GIN. Given that the GIN is represented by a non-neglectable number of bits, using a simple approach to broadcast GINs per SNPN may unnecessarily burden the broadcast.

Broadcasting the same GIN multiple times (i.e. once for each SNPN) in a shared network unnecessarily burdens the broadcast.

In this regard, it is worth noting that to address the above issue, it is alternatively possible to provide an association between SNPNs and supported GINs.

Thus, instead of introducing one list of GINs per SNPN, a new “GIN list” (to which we will refer herein as “GIN-InfoList”) could be introduced, where each list element associates a GIN with one or more SNPNs, i.e. each list element consists of:

- the GIN

- the SNPNs supporting access for the indicated GIN.

These SNPNs could be a subset of the SNPNs broadcast in the npn-IdentityInfoList which support access for a certain GIN. The supporting SNPNs can be indicated by means of a simple bitmap pointing to the elements listed in the npn-IdentityInfoList as follows:

GIN-Info-r17 ::=         SEQUENCE {

gin-r17						GIN-r17,

supportingSNPNs-r17			BIT STRING (SIZE (maxNPN-r16)) 	      OPTIONAL,		 -- Need R

...

}

If one GIN is supported by 2 SNPNs in a network sharing scenario, at least 2x 53 bits = 106 bits would be broadcast using the GIN per SNPN broadcasting approach.

If the bitmap approach is used, as shown in the ASN.1 example above, the GIN would be broadcast only once, i.e., GIN (53 bits) + the bitmap (12 bits) = 65 bits. Thus, saving 41 bits to convey the same information.

Such optimizations can be discussed at a later stage.

Introduce a new IE to allow mapping SNPNs and their supported GINs. Further optimizations are FFS.

#### 2.1.2.2	In which SIB to broadcast the GIN info

The GIN may identify one or multiple CHs and is used for SNPN selection during initial registration. Once registered at the SNPN, the GIN is no longer needed, i.e., it is neither needed for cell (re)selection, nor is it needed for cell access. Consequently, it is not considered an “essential” parameter and thus, it is used for a delay-tolerant procedure.

In addition, SIB1 has a limited number of available free bits. Thus, the GIN-InfoList should be broadcast in a SIB different than SIB1, such that the GIN-InfoList can be broadcast less frequently.

Broadcast the GIN-InfoList in a SIB different than SIB1.

#### 2.1.2.3	On the GIN encoding

As described above, SA2 concluded to reuse the SNPN encoding for the GIN, i.e. GIN = PLMN ID + NID.

In addition, TR 23.700-07 [4] explains that the Group ID (GIN) can identify a multi-national operator or an interconnection provider. An interconnection provider could also have partner networks from different countries.

For an association of (multi-national) partners and given that the NID itself can be globally unique or self-managed, it could be questioned whether the PLMN ID would always need to be included in the GIN.

If the PLMN ID can be either completely removed from the GIN encoding or, at least, be optional for the self-managed case, this would also help minimize the broadcasting overhead.

Ask SA2 if the PLMN ID in the GIN encoding needs to be broadcast or whether it can be at least optional in the GIN.

### 2.1.3	Manual selection: Human readable name for the GIN

R2-2101717 [5] proposed to add a human readable (network) name for the GIN. However, TR 23.007-07 [4] clause 8.1.4 states the following:

- For manual SNPN selection the UE presents all available SNPNs, which broadcast the "access using credentials from a separate entity is supported" indication.

The TR does not mention the separate entity nor Group ID for manual selection. Thus, the UE would present the existing HRNN of the SNPN(s) to the end user for manual selection, and a human readable name for the GIN is not needed.

For manual SNPN selection, the UE uses the existing HRNN for the SNPN.

If a human readable network name for GINs is considered beneficial, this should be addressed by SA2 first.

RAN2 assumes that human readable names for GINs are not needed, unless SA2 indicates otherwise.

## 2.2	Mobility procedures

As clarified by SA2 in [2], the new parameters related to authentication by a separate entity should be homogeneous across the network. Furthermore, TR 23.700-07 [4], clause 8.1.2, has been updated accordingly:

- Support of authentication using credentials from an external entity is homogenous throughout a SNPN i.e., the SIB information in clause 8.1.4 should be set uniformly and no changes in mobility handling are needed to address inhomogeneous support of the feature.

### 2.2.1	Cell (re)selection impact

Several companies stated that there would be impacts on the suitable criterion if the parameters related to access using 3rd party credentials were set differently within the same SNPN. As concluded in R2-2102413 [3], the decision should be based on feedback received from SA2. Since the parameters are assumed to be homogeneous throughout the SNPN, they can be transparently passed from the AS to the NAS layer, and after the SNPN selection has been performed by the NAS layer, the parameters do not need to be taken into account for cell (re)selection. Thus, legacy cell (re)selection procedures apply.

The cell (re)selection procedure is not impacted by introducing the new parameters to support authentication using credentials from an external entity.

### 2.2.2	Connected mode mobility support

Based on R2-2102413 [3], RAN2 can assume that there is no impact on connected mode mobility. Potential impact can be addressed by RAN3.

Wait for RAN3 input for potential impact on connected mode mobility.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Broadcasting the same GIN multiple times (i.e. once for each SNPN) in a shared network unnecessarily burdens the broadcast.

Observation 2	For manual SNPN selection, the UE uses the existing HRNN for the SNPN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The indicator "access using credentials from a separate entity is supported" is set independent of the GIN broadcast.

Proposal 2	Introduce a new IE to allow mapping SNPNs and their supported GINs. Further optimizations are FFS.

Proposal 3	Broadcast the GIN-InfoList in a SIB different than SIB1.

Proposal 4	Ask SA2 if the PLMN ID in the GIN encoding needs to be broadcast or whether it can be at least optional in the GIN.

Proposal 5	RAN2 assumes that human readable names for GINs are not needed, unless SA2 indicates otherwise.

Proposal 6	The cell (re)selection procedure is not impacted by introducing the new parameters to support authentication using credentials from an external entity.

Proposal 7	Wait for RAN3 input for potential impact on connected mode mobility.

# 4	References

R2-2102489, “Clarification request for eNPN features”, RAN2, RAN2#113e, 2021-02

S2-2101076, “Reply LS on clarification request for eNPN features”, SA2, SA2#143e, 2021-03

R2-2102413, “Summary of [AT113-e][031][eNPN] SNPN with subscription or credentials by a separate entity”, Nokia, RAN2#113e, 2021-02

TR 23.700-07, “Study on enhanced support of non-public networks”, TSG SA, V2.0.0, 2021-03

R2-2101717, “Support SNPN along with credentials owned by an entity separate from the SNPN”, CMCC, RAN2#113e, 2021-02

TS 23.501, “System architecture for the 5G System (5GS); Stage 2”, V17.0.0, 2021-03