3GPP TSG-RAN WG2 # 113bis electronic                                                  R2-2103690

Online, April 12 – April 20, 2021

Agenda Item:	8.16.3

Source: 	CMCC

Title:  	Discussion the issues to support UE on-boarding and remote provisioning

WID/SID:          NG_RAN_PRN_enh

Document for:	Discussion and Decision

Introduction

In last SA2#143-e meeting, SA2 sent a reply LS to RAN in S2-2101076 LS on clarification request for eNPN features [1] to provide the answers on the exact questions from that RAN2. And the answers to questions related to SNPNs with subscription or credentials by a separate entity are as follows:

- Question related to support UE onboarding and provisioning for NPN: - Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?) - [SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection. - Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

In the RAN2 last meeting #113e, the following agreements were achieved to support UE on-boarding and provisioning for NPN:

Broadcast a 1-bit indication for onboarding per O-SNPN. R2 assumes that the 1-bit indication for onboarding is in SIB1. The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection). Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network.

The further study for enhancement of NPN is captured in SA2 TR 23700-07 [1]. In this contribution, we would concentrate on the left issues listed in TR 23700-07 and SA2’s feedback in the LS [2] to support UE on-boarding and provisioning in idle mode and connected mode for SNPN and PNI-NPN.

Discussion

In the SA2 TR 23700-07 [1], key issue #4 aims to study the architecture and solutions to support UE on-boarding and remote provisioning. Two components listed in this key issue #4 include UE on-boarding issue and remote provisioning for SNPN or PNI-NPN credentials. Since the remote provisioning for SNPN credential or PNI-NPN credentials mainly impacts the 5GC, only the UE on-boarding issue is discussed below.

  - SNPN

The SA2 TR 23700-07 [1] describes the network architecture to support UE on-boarding and provisioning:

The UE performs the registration procedure using a default UE credential for the Default Credential Server (DCS) authentication in the on-boarding network (ON), which could possibly be a PLMN or a SNPN network (O-SNPN).  The UE gets access to the on-boarding network and can be provisioned with UE’s desired SNPN subscriptions credentials or other configurations from Subscription Owner SNPN (SO-SNPN), which provides the service to UE. Indeed, the O-SNPN and SO-SNPN may possible be different SNPNs or one SNPN takes both roles. The Figure 1 illustrates the network architecture to support the UE on-boarding and provisioning for SNPN.

Figure 1. The network architecture to support the UE on-boarding and provisioning for SNPN

    - System Information

In the last meeting, the indication for on-boarding enabled has been decided to be broadcasted on per O-SNPN basis in the SIB1. As suggested in SA2 TR 23700-07 [1] as below, the group IDs or O-SNPN network identifiers per on-boarding network could be broadcasted optionally to assist the on-boarding procedure for UEs.

- The NG-RAN of the Onboarding network includes an indication for Onboarding enabled in the SIB (per O-SNPN, considering that the NG-RAN can be shared) so that the UE can discover and select an appropriate O-SNPN. The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers or Group ID(s)). The O-SNPN network selection information can assist the UE such that the UE either preferably or exclusively select an O-SNPN corresponding to the O-SNPN network identifiers or Group ID(s). NOTE 2: The format of the pre-configured information assisting the UE for O-SNPN selection is not specified. NOTE 3: The Group ID(s) in the SIB that UE can use for selecting an O-SNPN are the same as the Group ID(s) in the SIB that the UE uses for SNPN selection as part of KI#1. NOTE 4: Whether the indication for Onboarding is sufficient or more SIB information is needed can be further discussed in the normative phase.

As noted in the TR 23700-07, the UE may take account of the O-SNPN network identifiers list or group ID list per on-boarding network to camp on the suitable network or cell. The group IDs in KI#4 for selecting an O-SNPN are same as the group IDs which is defined in KI#1 as a specific case of SNPN ID reusing SNPN ID encoding in TS 23.003[1].  Therefore, the supported SO-SNPN list or group ID list per on-boarding network is needed to be broadcasted optionally in SIB1 to assist the on-boarding procedure and cell selection or reselection procedure for UE. The indication for on-boarding enabled, group ID list or O-SNPN network identifiers could be extended in the PLMN-IdentityInfo and the NPN-Identity as below.

PLMN-IdentityInfo ::=               SEQUENCE {

plmn-Identity                               PLMN-Identity,

cellReservedForOperatorUse          ENUMERATED {reserved, notReserved}

onboarding-Indication-r17	           ENUMERATED {true},                         OPTIONAL   -- Need R

group-IdentityList-r17                   SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16  OPTIONAL   -- Need R

}

NID-r17 ::=         SEQUENCE {

NID-r16                                                  BIT STRING (SIZE (44)),

onboarding-Indication-r17				ENUMERATED {true},                         OPTIONAL   -- Need R

group-IdentityList-r17                               SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16 OPTIONAL   -- Need R

}

In Rel-16, the maximum number of networks is set to 12 in the RRC signalling. In order to re-use the same RRC signalling, the same restriction on the number was applied to the number of group ID list or O-SNPN network identifiers. Similarly, it is proposed that it is up to 12 different SO-SNPN IDs or group IDs can be broadcasted in a cell.

Proposal 1: it is proposed that the supported SO-SNPN list or group ID list per on-boarding network is needed to be broadcasted optionally in SIB1 to assist the on-boarding procedure and cell selection or reselection procedure for UE.

Proposal 2: it is proposed that it is up to 12 different SO-SNPN IDs or group IDs can be broadcasted in a cell.

    - Connected mode

Based on the conclusion from SA2 TR 23700-07 [1] listed below, The UE provides an indication explicitly or information implicitly at RRC level and NAS level to inform the gNB and AMF whether the on-boarding procedure is performed. The indication or information assists the gNB or AMF to select appropriate AMF or SMF since the usage restriction for the on-boarding service is controlled by certain AMF and SMF.

In order to inform the gNB the purpose of connection, the UE reports the indication per ON in message RRCSetupComplete to indicate the RRC connections for on-boarding procedure. Furthermore, the UE should optionally provide the selected SO-SNPN lists associated with ON in message RRCSetupComplete to gNB when it accesses the ON. The gNB could select AMF based on the information or indication provided in RRCSetupComplete to restrict the usage only to the on-boarding service. Therefore, the extension to RRCSetupComplete is to include boarding procedure indication per ON and optionally the selected SO-SNPN lists.

Proposal 3: In order to inform the gNB the purpose of connection, the UE reports the indication per ON in message RRCSetupComplete to indicate the RRC connections for on-boarding procedure. Furthermore, the UE should optionally provide the selected SO-SNPN lists associated with ON in message RRCSetupComplete to gNB when it accesses the ON.

    - Network selection:

Similar as the legacy network selection mechanism for PLMN and SNPN, the UE could use an autonomous search function to find any cells per PLMN or SNPN. The UE will compare the list of PLMN ID or NID broadcasted in cells with the subscription stored in UE as pre-configuration to find the suitable cells to camp on.

The network selection is supported in on-boarding procedure, and its impact on RAN is similar to that of SNPN or PLMN selection [3] except that the indication for on-boarding enabled per SNPN or PLMN should also be considered in the network selection procedure, as follows:

- When the PLMN acts as the on-boarding network:

If the UE can read one or several PLMN identities and the enabled on-boarding indications of ON in the strongest cell, each found PLMN and the enabled on-boarding indications of ON shall be reported to the NAS. For manual selection, UE shall upon request by NAS report available PLMN identifiers with the enabled on-boarding indications of ON to the NAS and the search for available PLMNs may be stopped on request of the NAS.

- When the SNPN acts as the on-boarding network:

If the UE can read one or several SNPN identities and the enabled on-boarding indications of ON in the strongest cell, each found SNPN and the enabled on-boarding indications of ON shall be reported to the NAS. For manual selection, UE shall upon request by NAS reports available SNPN identifiers together with their HRNN (if broadcast) and the enabled on-boarding indications of ON to the NAS and the search for available SNPNs may be stopped on request of the NAS.

Proposal 4: The indication for on-boarding enabled per on-boarding network should also be reported from AS to NAS and be considered in the network selection procedure.

    - Cell access and congestion control

In the LS from SA2 [2], it is suggested that the indication for on-boarding enabled could be used to avoid the load from on-boarding UEs. However, it treads all the UE blocked and the UAC value may bring more flexibility for the operator to control the congestion level. Therefore an operator defined UAC value or new normative UAC value could be used for RAN-level congestion control handling to partially block the UEs access.

Proposal 5: it is proposed that an operator defined UAC value or new normative UAC value could be used for RAN-level congestion control handling to partially block the UEs access.

  - PNI-NPN

    - The network architecture and concept to support UE on-boarding and provisioning

The UE performs the registration procedure using a UE default PLMN credential in the PLMN as an on-boarding network (ON). The UE gets access to the on-boarding network and can be provisioned with UE’s desired PNI-NPN credential or other configurations, such CAG information. The Figure 2 illustrates the network architecture to support the UE on-boarding and provisioning for PNI-NPN.

Figure 2. the network architecture to support the UE on-boarding and provisioning for PNI-NPN

The UE with the default PLMN credential could access the PLMN which PNI-NPN relies on and acquire the UE PNI-NPN credential or other information via the PLMN. After the UE successfully completes the remote provisioning for PNI-NPN credential or other information, UE triggers the deregistration process and use the PNI-NPN credential or CAG information to access PNI-NPN network hosted by PLMN for PNI NPN service.

Since the default PLMN credential could be used by UE for UE on-boarding and network selection and reselection, no enhancement is needed for UE on-boarding and remote provisioning for PNI-NPN.

Observation 1: The UE with the default PLMN credential could access the PLMN which PNI-NPN relies on and acquire the UE desired PNI-NPN credential or other information via the PLMN. And UE uses the obtained PNI-NPN credential or CAG information to access PNI-NPN network hosted by PLMN for PNI NPN service.

Proposal 6: it is proposed that no enhancement is need for UE on-boarding and remote provisioning for PNI-NPN.

Conclusion

Proposal 1: it is proposed that the supported SO-SNPN list or group ID list per on-boarding network is needed to be broadcasted optionally in SIB1 to assist the on-boarding procedure and cell selection or reselection procedure for UE.

Proposal 2: it is proposed that it is up to 12 different SO-SNPN IDs or group IDs can be broadcasted in a cell.

Proposal 3: In order to inform the gNB the purpose of connection, the UE reports the indication per ON in message RRCSetupComplete to indicate the RRC connections for on-boarding procedure. Furthermore, the UE should optionally provide the selected SO-SNPN lists associated with ON in message RRCSetupComplete to gNB when it accesses the ON.

Proposal 4: The indication for on-boarding enabled per on-boarding network should also be reported from AS to NAS and be considered in the network selection procedure.

Proposal 5: An operator defined UAC value or new normative UAC value could be used for RAN-level congestion control handling to partially block the UEs access.

Observation 1: The UE with the default PLMN credential could access the PLMN which PNI-NPN relies on and acquire the UE desired PNI-NPN credential or other information via the PLMN. And UE uses the obtained PNI-NPN credential or CAG information to access PNI-NPN network hosted by PLMN for PNI NPN service.

Proposal 6: it is proposed that no enhancement is need for UE on-boarding and remote provisioning for PNI-NPN.

Reference

- 3GPP TS 23.700-07	 v2.0.0 Study on enhanced support of non-public networks; (Release 17)

- S2-2101076 Reply LS on clarification request for eNPN features.

- 3GPP TS 36.304 v16.2.0 User Equipment (UE) procedures in idle mode

- 3GPP TS 23.501 v16.1.0 System Architecture for the 5G System; stage2.