3GPP TSG-RAN WG2 #113bis electronic                                    R2-2103700

Online: April 12 – 20, 2021

Agenda Item:	8.10.3.3

Source:	CMCC

Title:	Discussion on SMTC/Gap enhancements for NTN

Document for:	Discussion, Decision

Introduction

In the RAN112e meeting, there were some discussions on the measurement aspects, and some agreements were reached as follows:

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

5.	Legacy SSB periods (as in TN) shall be supported in NTN

Agreements via email - offline 106:

- RAN2 understanding that UE shall not be forced to detect the SSB burst outside the corresponding configured SMTC window in NTN, just like the principle in TN.

Agreements:

- SMTC and gap configuration in NTN are configured based on the timing of PCell

- RAN2 can first identify the scenarios and discuss how serious the impact is before addressing any enhancement for SMTC configuration in NTN.

- RAN2 can’t assume that the network will always have UE accurate location info for SMTC window configuration in NTN

- UE along with the network in NTN should also have the same understanding of the timing, including the timing for measurement gap, to avoid any un-synchronized scheduling between UE and the network, just like the way we have in TN

And in the [POST113-e][108][NTN] SMTC and measurement gaps (Intel), some issues about measurement framework, SMTC and measurement gaps are discussed among companies, in this contribution, we would like to provide some further discussion on some FFS issues about measurement enhancements including SMTC and measurement gaps in the email, for example, specific measurement configuration solution, UE assistance information for NW and the issue that the effectiveness of the measurement configuration which is not discussed in the email but also important.

Discussion

For NTN scenario, as shown in Fig.1,the transmission distance difference between serving and neighbor satellites may be too long(e.g. about 0~serval hundreds of kilometres for LEO and 0~serval thousands of kilometres for GEO) leading to quite large propagation delay difference. Different from the existing measurement mechanism of the terrestrial network, if the propagation delay difference is not considered in NTN, the UE will miss the SSB/CSI-RS measurement window of neighbour satellites. Therefore, in order to perform the measurement correctly in NTN system, the measurement configuration containing both SMTC and measurement gaps needs to consider the propagation delay difference information.

Fig.1 Distance difference between two satellites

Proposal 1: In NTN, both SMTC and measurement gaps configuration need to consider the propagation delay difference information.

First of all, the measurement configuration solution may be considered from two categories: NW-based decision and UE-based decision.

For NW-based decision solution, the final SMTC/measurement gap configuration is provided by NW. Similar to the traditional procedure of UE requesting something from the NW, UE reports the calculated propagation delay information between target cell and serving cell. Then, the serving cell provided proper measurement configuration to the UE according the reported propagation delay information.

Considering the RTT delay, the reporting granularity of the propagation delay could be a specific delay or a step range (e.g. 100ms as a step). For the step range reporting granularity, the NW configured same measurement configuration to UEs that report the propagation delay difference information belonging to the same delay step range (for example, corresponding to the maximum value of the propagation delay difference reported by the UEs).

Proposal 2: RAN2 can regard NW-based decision solution as baseline, i.e., the serving cell provided proper measurement configuration to the UE according the reported propagation delay information by the UE.

Proposal 3: Considering the RTT delay, the reporting granularity of the propagation delay could be a specific delay or a step range (e.g. 100ms as a step).

For UE-based decision solution, in general, the final SMTC/measurement gap configuration is determined by UE. For example, the NW configures the UE with multiple SMTC/measurement gap configurations corresponding to the delay difference between different target cell and the serving cell. The UE calculates the propagation delay difference between at least one target cell and the serving cell according to its own location information and satellite ephemeris information, selecting the most matching measurement configuration according to the UE-calculated propagation delay difference. As a prerequisite, the mapping relationship between propagation delay difference and measurement configuration should be known by both UE and NW.

In order for the UE and the NW to have a consistent understanding, the UE needs to report the selected SMTC/measurement gap configuration to the NW. Then, the NW and the UE have an alignment at any time and reduce the report signaling overhead by the UE.

Proposal 4: We suggest RAN2 consider UE-based decision solution, the NW configures a UE with multiple SMTC/measurement gap configurations corresponding to different propagation delay information, the UE select a appropriate measurement configuration matching the UE-calculated propagation delay difference.

Proposal 5: it is proposed that in the UE-based decision approach, the UE needs to report the selected SMTC/measurement gap configuration to the NW to guarantee an alignment between the NW and the UE.

In order to improve measurement robustness, the effectiveness of the measurement configuration needs to be considered due to the long delay and the high-speed movement of the satellite. For instance, timer-based or threshold-based solution could be discussed. For timer-based solution, the criteria could be that once the timer expires, the measurement configuration needs to be updated. And the value range of the timer can be determined according to remaining serving time of the current cell. For threshold-based solution, for example, if the distance between the UE and the satellite or the distance between the UE and the cell center exceeds a given threshold, the measurement configuration needs to be updated.

Proposal 6: In order to improve measurement robustness, the effectiveness of the measurement configuration needs to be considered due to the long delay and the high-speed movement of the satellite. And timer-based or threshold-based solution could be discussed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some further discussions on the measurement enhancements for NTN and have the following proposals:

Proposal 1: In NTN, both SMTC and measurement gaps configuration need to consider the propagation delay difference information.

Proposal 2: RAN2 can regard NW-based decision solution as baseline, i.e., the serving cell provided proper measurement configuration to the UE according the reported propagation delay information by the UE.

Proposal 3: Considering the RTT delay, the reporting granularity of the propagation delay could be a specific delay or a step range (e.g. 100ms as a step).

Proposal 4: We suggest RAN2 consider UE-based decision solution, the NW configures a UE with multiple SMTC/measurement gap configurations corresponding to different propagation delay information, the UE select a appropriate measurement configuration matching the UE-calculated propagation delay difference.

Proposal 5: it is proposed that in the UE-based decision approach, the UE needs to report the selected SMTC/measurement gap configuration to the NW to guarantee an alignment between the NW and the UE.

Proposal 6: In order to improve measurement robustness, the effectiveness of the measurement configuration needs to be considered due to the long delay and the high-speed movement of the satellite. And timer-based or threshold-based solution could be discussed.

Reference

[1] RAN2-113-e - R16 eMIMO-CLI-PRN-RACS - R17 NTN-REDCAP (Sergio)_2021_02_05_EOM

[2] [POST113-e][108][NTN] SMTC and measurement gaps (Intel)