3GPP TSG RAN WG2 #113b-e	R2-2103727

Electronic Meeting, 12th Apr – 20th Apr 2021

Agenda Item:	9.2.3

Source:	CMCC

Title:	RLF-based NB-IoT mobility in NTN

Document for:	Discussion

Introduction

In previous RAN2 #112-e and RAN2 #113-e meeting, NB-IoT and eMTC support for NTN was discussed and the following agreements related to Mobility were made [1][2]:

providing satellite ephemeris data and other information using System Information (SI) message for eMTC/NB-IoT NTN. [035] 16: RAN2 agrees to use Rel-16 RLF-based NB-IoT mobility as a baseline for mobility in NB-IoT over NTN. [035] 17: RAN2 will wait until agreements regarding handover, including Conditional Handover, solutions are made in the NR-NTN WI, discuss if it would be beneficial for eMTC over NTN, if adopted. [035] 18: RAN2 should wait for RAN1’s input on supporting multiple beams per cell for eMTC/NB-IoT over NTN.

For eMTC in NTN - CHO can be used for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline. - (i) The existing measurement framework for CHO (e.g. measurement configuration, execution) is the baseline. - (ii) The existing eMTC measurement criteria and event can be used in NTN. Support for new measurement would need justification, but is not precluded, e.g. for enh coverage. - (iii) Time or timer based and Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, can be introduced for both moving cell and fixed cell scenarios. Support for new triggering events is not precluded. - (note that LTE CHO isn’t supported for 5GC, and same assumptions as LTE applies). [036] Rel-17 RLF enhancements in NB-IoT can be considered in NB-IOT NTN, if applicable. Further enhancements on RLF-based mobility can be considered, e.g. by using satellite assistance (ephemeris) information.

In this contribution, we discuss some aspects of connected RLF-based for NB-IoT.

Discussion

To reduce the cost and considering the NB-IoT UEs are always stationary or in low mobility, NB-IoT UEs do not support handover, mobility is only based on RLF and RRC connection re-establishment procedure. Most of the issues identified in NR NTN and corresponding enhancements are purely related to the handover procedure which is not applicable to NB-IoT UEs.

However, as in NR NTN, the satellites in non-GEO orbits move with high speed relative to a fixed position on earth, leading to frequent and unavoidable handover or cell reselection even for stationary UEs. As mentioned in TR 38.821, “Neglecting UE movement, a UE served by an NTN LEO cell of diameter 50 km and 1000 km may remained connected for a maximum of 6.61 seconds and 132.38 seconds respectively due to satellite movement.” Additionally, the larger propagation delay in NTN will introduce additional latency to RLF-based mobility procedures, while may further increase the frequency of RRC re-establishment, although the UEs are stationary.

In RAN2#112 e-meeting, RAN2 has agreed to use Rel-16 RLF-based NB-IoT mobility as a baseline for mobility in NB-IoT over NTN. And in RAN2#113 e-meeting, RAN2 further agreed Rel-17 RLF enhancements in NB-IoT can be considered in NB-IOT NTN, if applicable. Further enhancements on RLF-based mobility can be considered, e.g. by using satellite assistance (ephemeris) information.

In the special NTN scenarios, the usage of satellite assistance (ephemeris) information is obviously beneficial to address the above issues. However, RAN2 need further study how to use the information.

As RAN2 agreed that for eMTC in NTN, CHO can be used for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline.

(i) The existing measurement framework for CHO (e.g. measurement configuration, execution) is the baseline.

(ii) The existing eMTC measurement criteria and event can be used in NTN. Support for new measurement would need justification, but is not precluded, e.g. for enh coverage.

(iii) Time or timer based and Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, can be introduced for both moving cell and fixed cell scenarios. Support for new triggering events is not precluded.

Similarly, in our understanding, the conditional RRC connection re-establishment procedure which configure the procedure triggering condition and target cells (instead of traditional cell selection after RLF) to the UE beforehand, based on the pre-configured satellite tracking information per the ephemeris information and UE’s location, can also been introduced to address the extremely frequent RLF and re-establishment procedure in NTN network, which can be clarified as the following conditions to trigger the RRC connection re-establishment procedure as for CHO:

- Time, timer or time range based C-Re-establishment triggering event

- Location based C-Re-establishment triggering event

Otherwise, NB-IoT UEs select target cell for RRC connection re-establishment just based on existing cell selection procedure upon frequent RLF, NB-IoT UEs need to scan and measurement all neighbor cells or cell list in the history information to find a suitable cell to camp on, which increases UE's power consumption and network access delay. As we know, power consumption is a critical KPI for NB-IoT devices.

Observation 1: The mobility challenges for NR NTN including long transmission latency, frequent signal strength variation and cell change due to the fast satellite switching are impact on the RLF-based mobility for NB-IoT in NTN.

Proposal 1: it is proposed to introduce the conditional RRC connection re-establishment procedure which configures the procedure triggering condition and target cells to the UE beforehand.

Proposal 2: the conditional RRC connection re-establishment condition setting and execution, can base on the ephemeris information and UE’s location, which can be clarified as the following conditions to trigger the RRC connection re-establishment procedure as for CHO:

- Time, timer or time range based C-Re-establishment triggering event

- Location based C-Re-establishment triggering event

On the other hand, frequent cell switching could happen for a large amount of UEs in a short period due to satellite movement. Thus, the group Re-establishment mechanism as for group HO is required to study for NB-IoT UEs as well. For example, some signalling and information that are common to all the UEs can be notified to the UEs through broadcast as studied in SI to avoid signalling storm.

Proposal 3: the group Re-establishment mechanism as for group HO is required to study for NB-IoT UEs as well.

Conclusions

Observation 1: The mobility challenges for NR NTN including long transmission latency, frequent signal strength variation and cell change due to the fast satellite switching are impact on the RLF-based mobility for NB-IoT in NTN.

Proposal 1: it is proposed to introduce the conditional RRC connection re-establishment procedure which configures the procedure triggering condition and target cells to the UE beforehand.

Proposal 2: the conditional RRC connection re-establishment condition setting and execution, can base on the ephemeris information and UE’s location, which can be clarified as the following conditions to trigger the RRC connection re-establishment procedure as for CHO:

- Time, timer or time range based C-Re-establishment triggering event

- Location based C-Re-establishment triggering event

Proposal 3: the group Re-establishment mechanism as for group HO is required to study for NB-IoT UEs as well.

References

- RAN2 112-e Chairman Notes

- RAN2 113-e Chairman Notes