3GPP TSG RAN WG2 Meeting #113bis-e	      					R2-2103735

e-Meeting, 12 April – 20 April 2021

Agenda item:	8.5.4

Source:		Intel Corporation

Title: 		RAN2 Enhancements for Support of QoS Parameters

Document for: 	Discussion

# Introduction

In the last two meetings in RAN2 #112e and #113e, survival time and burst spread have been discussed to identify possible RAN enhancements which may be required to support these new QoS related parameters decided in SA2. The QoS parameter of burst spread is still under discussion in SA2 therefore no significant discussion has taken place in RAN2 in this regard. However, for survival time, RAN2 discussed, among other issues, the LS received from SA2 [1] regarding the definition and format of the survival time, and if it is sufficient to meet the performance requirement for CSA set by SA1. In this contribution we put forth the open issues in RAN2 regarding the monitoring of survival time state and methods for avoiding consecutive transmission failures. We also briefly discuss the current status regarding burst spread.

# Discussion

In RAN2 #113e, the agreements made in relevance to survival time and burst spread are as given below. It has been agreed that only periodic traffic is considered for survival time work in Rel-17 for which knowledge of the TSN traffic pattern, such as QoS parameters of survival time and burst spread, can allow the gNB to efficiently schedule resources (CG/SPS or dynamic grants) for reliable communication.

Agreements

-	Communication service availability (CSA) is not needed on top of survival time.  Send a reply LS to SA2 to notify such confirmation

-	RAN2 confirms that specification enhancement for survival time support may only needed for uplink.  Downlink is addressed by implementation and no specification impacts.

-	Support for survival time in UCE is up to network configuration.

-	Continue discussing whether burst spread and burst ending time is beneficial from RAN2 perspective, but trigger the discussion after SA2 progress in February

-	Communication service reliability (CSR) is not needed on top of survival time

-	Only periodic traffic is considered for survival time work in Rel-17

-	RAN2 assumes one application message is conveyed by one PDCP SDU, and may further consider the cases where one application message is conveyed by varying number of PDCP SDUs depending on the progress

Survival Time

In SA2 #142e, following principles, among others, haven been agreed for normative work and are captured in TR 23.700-20 [3] under conclusions for Key issue # 5 Use of Survival Time for Deterministic Applications in 5GS.

- Survival Time is transferred as part of the TSCAI parameter but the TSCAI may not always comprise of Survival time.

- Survival Time information is specified by the AF in units of "time" with respect to burst periodicity or as the maximum number of consecutive message transmission failures (i.e. whose loss can be tolerated).

NOTE 1: There is a single message per burst periodicity and the burst contains the application message. It is conveyed together with TSCAI Periodicity parameter (the time between periodic TSC bursts) and burst size (e.g. MDBV).

When survival time information is provided for a TSC stream, then it should not be aggregated with other TSC streams into a single QoS flow, or if they are aggregated, then the survival time parameter shall not be provided.

Survival time specified in unit of "time" will be supported over NGAP.

The following definition of survival time was agreed in RAN2 112e [6] and communicated to SA2.

Time period during which “message loss” can be tolerated is adopted as the preferred format for Survival time.  FFS how this will be achieved and what message loss means in RAN2

However, details regarding monitoring of survival time state, including discussion on introduction of new QoS parameter of “Burst End Time (BET)” was left FFS in RAN2 #113e [2].

Burst end time has not been discussed in SA2 and it is worth noting that such a method of measuring survival time using BET would require a change up to the CNC to include the new QoS parameter of BET at SA level which may not be possible in Release 17 during WI phase.

Observation 1: SA2 has not discussed introduction of new QoS parameter of Burst End Time (BET).

In TS 23.501 [4], Burst Arrival Time (BAT) is included in the TSCAI as shown in Table 1, and is defined as the latest possible time when the first packet of the data burst arrives. As highlighted above in the excerpt from TR 23.700-20 and also as indicated in the LS from SA2 to RAN2 [1], SA2 has made clear that there is a single application message per burst periodicity and TSN streams are not aggregated into a single QoS flow when survival time is included in the TSCAI.

Table 1: TSC Assistance Information (Table 5.27.2-1 in TS.23.501)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

Observation 2: It is assumed that there is no aggregation of TSN streams carrying messages from multiple applications when survival time is included in the TSCAI i.e. a single application message per burst periodicity.

For the use-cases discussed in TS 22.104, the message size for IIoT applications is usually in the range of 20 bytes to 1024 bytes. For few applications, e.g mobile robots and asset monitoring, the message size is up to 250 kbytes, however in such cases the transfer interval is between 100 ms to 1 s with data rates up to 2Mbits/s. Therefore, in light of the performance requirements laid in TS 22.104, and the SA2 requirement that the survival time is indicated to 5GS when the burst carries a single application’s message, it can be assumed that the difference between the start and end of the burst, that is between BAT and BET will be negligible. Also, as noted in TR 23.700-20, information of burst size and BAT which is known, may be useful towards determining the burst end time if and when required, though from the current requirements as laid by TS 22.104, no such need can be established.

Observation 3: Based on SA2 requirement that the survival time is indicated to 5GS when the burst carries a single application’s message, and considering the small message size that a burst typically carries, the difference between the start and end of the burst, i.e. between BAT and BET, will be negligible.

Proposal 1 The time difference between burst arrival time and burst end time is not expected to be large enough to be taken into account by the gNB for scheduling. Introduction of new QoS parameter of Burst End Time (BET) is therefore not required.

Monitoring of Survival Time and Consecutive Transmission Failure Avoidance

In RAN2 113e, it has been agreed: RAN2 confirms that specification enhancement for survival time support may only needed for uplink.  However, it was discussed without any conclusion whether UE enhancements are needed or if gNB implementation can be enough. Several companies pointed out that stringent use-cases such as those of motion control (Table 5.2-1 in TS 22.104), with a survival time requirement of 500us require further evaluation if some enhancements are required at the UE side. But there are other use-cases with somewhat relaxed survival time requirement of 10ms – 50ms (Table 5.2-1 in TS 22.104) which also need to be considered. It is therefore practical to evaluate methods for monitoring of survival time and enhancing reliability to avoid consecutive transmission failures, in accordance with the demands of particular use-cases. Note that the gNB is cognizant of the transfer interval and survival time as included in the TSCAI, and also of the packet delay budget (PDB) as a 5G QoS parameter and separately of the Core Network PDB (5G-AN-PDB) which is indicated for GBR QoS flows such as those for TSN applications [7]. With this information available to the gNB, it is feasible to implement different schemes/solutions in accordance with the performance requirements of a given use-case.

We propose here three schemes as a solution to meet the CSA requirement in accordance with the knowledge of survival time. Scheme 1 is proposed as a solution for stringent use-cases with a latency requirement of 500us, while Scheme 2 and Scheme 3 are proposed to cater for use cases with relaxed latency requirement. We also summarize the required enhancements and impacts for these schemes.

- Scheme 1: CG Type-2 Reconfiguration for Stringent Use-Cases

Survival time is defined in TS 22.104 as the time that an application consuming a communication service may continue without an anticipated message [5] and indicates the time available to the communication service to recover from a failure or lost transmission. The requirements provided in TS 22.104 [5], the duration of survival time depends on the application, and can range from zero to multiples of the transfer interval. For non-zero survival time, consecutive impairments and/or delays are ignored until the respective timer has expired. For stringent use-cases with e2e latency and survival time requirement of 500us, HARQ retransmission may not be beneficial due to the tight latency requirement. This is because for the considered use-case for motion control applications, both transfer interval and survival time is 500us and the e2e latency which is <= transfer interval is very small to accommodate retransmission. Therefore in such cases it is imperative to increase the reliability for the subsequent transmissions in case a transmission failure occurs to avoid communication service unavailability.

Observation 4: For stringent use-cases with e2e latency and survival time requirement of 500us, HARQ retransmission may not be beneficial due to the tight latency requirement, rather it is imperative to increase the reliability for the subsequent transmissions in case a transmission failure occurs to avoid communication service unavailability.

For an uplink transmission, the gNB is aware in case of a transmission failure, and can send DCI to reconfigure CG type-2 with a more robust MCS level, hence enhancing reliability as opposed to the usual principle of setting MCS level according to the QoS requirements, e.g PER. Increasing reliability for subsequent burst(s) after a lost packet can avoid consecutive transmission failures. In this scheme, the UE is not required to track the survival time and gNB implementation is enough.

Proposal 2: For use-cases with stringent latency requirement, the gNB may reconfigure CG type-2 with a more robust MCS level to avoid consecutive transmission failures and meet the CSA performance requirement.

Proposal 3: For use-cases with stringent latency requirement, gNB implementation is enough, and there is no need to specify additional UE behavior.

Scheme 2: L1/MAC reconfiguration for Relaxed Use-Cases

For use-cases with relaxed latency constraints, such that the e2e latency and survival time > 500us, there is time provision allowing for both retransmission in case of a packet loss, and also to enhance reliability for subsequent bursts in such a case to avoid future lost packets. In this method, a PDCCH-based option is considered where a gNB can schedule a dynamic retransmission in the case of packet loss. The gNB can then either send DCI to reconfigure CG type-2 with a more robust MCS level to enhance reliability or the MAC CE can be used to activate PDCP duplication of DRB for subsequent transmissions to avoid message loss in the future. Since the latency constraint is relaxed for the considered use-cases for this scheme, the additional latency due to MAC CE reconfiguration can be tolerated.

Note that this scheme can also be applicable to use-cases where survival time is 0, such as in process automation (Table 5.2-1 in TS 22.104) since the transfer interval and e2e latency in such use-cases is 10 ms which allows the gNB sufficient time to schedule both a dynamic retransmission as well as a CG type-2 reconfiguration, even for numerology of 15 kHz.

Proposal 4: For use-cases with e2e latency > 500us, the gNB can schedule a dynamic retransmission followed by either a CG type-2 reconfiguration or MAC CE reconfiguration to activate PDCP duplication. This can be handled by gNB implementation.

- Scheme 3: TX Side Timer and PDCP Duplication for Relaxed Use-Cases

In this scheme, the survival time state is monitored at the UE side for uplink transmissions using a TX-side timer. For use-cases with relaxed latency constraint and survival time > 500 us, it is sufficient for the UE to rely on HARQ feedback. Note that for GBR QoS Flows using the Delay-critical resource type such as those for TSN traffic, a packet delayed more than PDB is counted as lost [23.501]. Referring to Figure 1 below, a HARQ retransmission can therefore only be useful if successful retransmission for Burst n completes within the duration of 5G-AN-PDB. It may be required to introduce some other feedback mechanism from the gNB indicating a negative acknowledgement in case a message failure occurs. The UE can then decide based on the 5G-AN-PDB and survival time, whether it is possible to have a successful re-transmission in such a case, or if it should be counted as a burst failure hence shifting the focus to boosting reliability for the next transmission burst. With this understanding, the timer for survival time, can be started after a duration x after Burst Arrival Time (BAT). Here x equal the time when the UE receives a negative acknowledgement from the gNB and decides that no retransmission is necessary. The duration x is at most equal to  1 x 5G-AN-PDB.

Observation 5: For a failed transmission of Burst n, HARQ retransmission is only useful if successful retransmission for Burst n completes within the duration of 5G-AN-PDB.

Proposal 5: A failed transmission occurs if a message is not delivered within the time equal to Burst Arrival Time + PDB. The timer for survival time is started at Burst Arrival Time + PDB at the latest, and may be triggered earlier depending on feedback received from the gNB about failed transmission.

The survival time indicates to the communication service the time available to recover from failure. Based on the performance requirements by SA1 in Table 5.2-1 in TS 22.104 Communication Service Availability (CSA) is related to survival time and reliability (related to dynamic 5QI parameter of packet error rate (PER)). Therefore, if the timer for survival time is started, it implies a transmission failure has occurred and to avoid communication service unavailability it will be necessary to avoid consecutive transmission failures. To this end, PDCP duplication, autonomously activated by UE, can be adopted to enhance L2 reliability in the event of a failed transmission. For UE-autonomous reliability enhancement, the gNB may pre-configure a criterion, and a UE can activate/deactivate PDCP duplication if the trigger condition is satisfied. If UE-autonomous PDCP duplication is introduced in Release-17, delays incurred using network-controlled duplication, e.g transmission delay for CSI/measurement report may be avoided.

Proposal 6: UE autonomous PDCP duplication can be adopted to enhance L2 reliability.

Figure 1: Monitoring of Survival Time State

Return to Normal Configuration

To avoid consecutive transmission failures, it is assumed that upon such trigger of transmission failure, the subsequent transmission(s) are scheduled with enhanced reliability. Among the solutions for reliability enhancement as discussed in RAN2 113e, e.g PDCP duplication and/or more reliable L1/L2 configuration, the underlying assumption is that there is correlation between a past packet error and future packet transmissions, e.g. radio channel coherence. It is worth noting that issues in radio channels which may cause packet transmission failures may not be resolved within the duration of a single message transmission or survival time, therefore it may not be enough to consider successful transmission of a single packet as a condition for the UE to return to normal configuration. To ascertain the validity of the assumption of correlation between past transmission errors and future successful transmission and to identify the condition for the UE to fallback to normal configuration for future transmissions requires further analysis in RAN2 which may be based on statistical evaluation. Considering Scheme 3, if the timer for survival time is triggered for Burst n, then UE activates PDCP duplication for the transmission of Burst n+1. However if the survival time timer is not triggered for Burst n+1, assuming successful transmission, it doesn’t necessarily imply that the need for PDCP duplication or enhanced reliability is eliminated. Rather the channel response needs to be evaluated over a number of failed and successful transmissions to ascertain whether it is sufficient for the UE to return to normal configuration if the survival timer for Burst n expires, and such timer is not triggered for Burst n+1.

Proposal 7: RAN2 to further discuss and evaluate the condition for the UE to fallback to normal configuration after reliability enhancement is implemented in response to survival time triggering.

While the actual condition for returning to normal configuration needs further evaluation, we can nonetheless compare the impact/requirements for reconfiguration for each of the schemes proposed above. In Scheme 1 and Scheme 2 (using L1 reconfiguration), DCI can be used to reconfigure CG type-2 setting the MCS level based on QoS requirements e.g PER. For Scheme 2 (using MAC reconfiguration), MAC CE can be used to deactivate PDCP duplication. For UE-autonomous PDCP duplication in Scheme 3, the UE can release the radio-resource for duplication in a timely manner when the return-to-normal condition is satisfied.

In Table 2 below we summarize a comparison of the three schemes for the relaxed latency constraint use-cases.

Table 2: Comparison for Proposed Schemes

| Scheme | Survival time monitoring | gNB implemen-tation enough? | Packet recovery using retrains-mission | Impact for reliability enhancement | Impact for return to normal configuration | Latency Impact | Signalling Overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | No | Yes | No | DCI to enhance MCS level | DCI to reconfigure MCS level | Lowest | DCI, L1 reconfiguration |
| 2 w/ L1 reconfiguration | No | Yes | Yes | DCI to enhance MCS level | DCI to reconfigure MCS level | Low | DCI, L1 reconfiguration. |
| 2 w/ MAC reconfiguration | No | Yes | Yes | MAC CE activation | MAC CE deactivation | Low | MAC reconfiguration |
| 3 w/ UE autonomous duplication | Yes | No | Yes | UE autonomous activation based on preconfigured condition | UE autonomous deactivation based on preconfigured condition | Lower | Negative acknowledgement from gNB |

Proposal 8: For stringent use-cases gNB implementation is enough. For relaxed use-cases, both UE and gNB implementation can be supported whereby the network can decide which scheme is used, hence offering flexibility.

Burst Spread

Burst spread is defined in TR 23.700-20 [3] as “the variation of burst arrival time for DL traffic resulting from jitter on N6”, where burst arrival time is defined in TS 23.501 as “the latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction)”. It is discussed in Solution #5: Deterministic QoS for Native 5GS in TR 23.700-20 [3] that if AF provides burst spread, the 5GS will forward the burst spread as part of TSCAI to the NG-RAN. The UE and UPF impacts of this SA2 solution are also FFS and as per the conclusion to the relevant Key Issue #3A: Exposure of deterministic QoS, whether the burst spread should be included is FFS.

A discussion on how the UPF may detect burst spread has been included in SA2 Solution #22: Detect the Burst spread at UPF in TR 23.700-20 [3], however the mechanism for whether the UPF/NW-TT detects the burst spread is FFS. Based on the discussion in SA2, and by definition, since burst spread is the variation in burst arrival time introduced by the jitter on N6, no information on burst spread is available before the application starts to send the data to the UE. This implies that unless the DL data reaches N6 interface, the variation in arrival time aka burst spread cannot be determined. However, the procedure on how the AF can obtain this burst spread once the UE starts receiving application data in the downlink is still pending further discussion in SA2. It is therefore appropriate for RAN2 to wait for further progress in SA2 WG before analysing the RAN impacts (if any) on introducing this new QoS parameter of burst spread since SA2 has not made any interim agreements related to burst spread yet.

However, similar to existing QoS parameters, while the new burst spread parameter (if introduced) may be useful to the gNB scheduler, its utilization is expected to be handled by the gNB implementation without any explicit RAN2 specification impact.

Proposal 9: RAN2 anticipates no explicit specification impact from introducing the new QoS parameter of burst spread, nonetheless RAN2 shall wait for further discussion and conclusions in SA2 regarding burst spread.

# Conclusions

In this contribution we discuss the RAN enhancements (if any) required to support newly introduced QoS parameters of survival time and burst spread and make the following observations and proposals.

Observation 1: SA2 has not discussed introduction of new QoS parameter of Burst End Time (BET).

Observation 2: It is assumed that there is no aggregation of TSN streams carrying messages from multiple applications when survival time is included in the TSCAI i.e. a single application message per burst periodicity.

Observation 3: Based on SA2 requirement that the survival time is indicated to 5GS when the burst carries a single application’s message, and considering the small message size that a burst typically carries, the difference between the start and end of the burst, i.e. between BAT and BET, will be negligible.

Proposal 1 The time difference between burst arrival time and burst end time is not expected to be large enough to be taken into account by the gNB for scheduling. Introduction of new QoS parameter of Burst End Time (BET) is therefore not required.

Observation 4: For stringent use-cases with e2e latency and survival time requirement of 500us, HARQ retransmission may not be beneficial due to the tight latency requirement, rather it is imperative to increase the reliability for the subsequent transmissions in case a transmission failure occurs to avoid communication service unavailability.

Proposal 2: For use-cases with stringent latency requirement, the gNB may reconfigure CG type-2 with a more robust MCS level to avoid consecutive transmission failures and meet the CSA performance requirement.

Proposal 3: For use-cases with stringent latency requirement, gNB implementation is enough, and there is no need to specify additional UE behavior.

Proposal 4: For use-cases with e2e latency > 500us, the gNB can schedule a dynamic retransmission followed by either a CG type-2 reconfiguration or MAC CE reconfiguration to activate PDCP duplication. This can be handled by gNB implementation.

Observation 5: For a failed transmission of Burst n, HARQ retransmission is only useful if successful retransmission for Burst n completes within the duration of 5G-AN-PDB.

Proposal 5: A failed transmission occurs if a message is not delivered within the time equal to Burst Arrival Time + PDB. The timer for survival time is started at Burst Arrival Time + PDB at the latest, and may be triggered earlier depending on feedback received from the gNB about failed transmission.

Proposal 6: UE autonomous PDCP duplication can be adopted to enhance L2 reliability.

Proposal 7: RAN2 to further discuss and evaluate the condition for the UE to fallback to normal configuration after reliability enhancement is implemented in response to survival time triggering.

Proposal 8: For stringent use-cases gNB implementation is enough. For relaxed use-cases, both UE and gNB implementation can be supported whereby the network can decide which scheme is used, hence offering flexibility.

Proposal 9: RAN2 anticipates no explicit specification impact from introducing the new QoS parameter of burst spread, nonetheless RAN2 shall wait for further discussion and conclusions in SA2 regarding burst spread.

# References

[1] R2-2010692 LS on Use of Survival Time for Deterministic Applications in 5GS (S2-2007880; contact: Nokia)

[2] R2-2101954 Report for Rel-17 Small data and URLLC/IIoT and Rel-16 NR-U, Power Savings, and 2step RACH, 3GPP TSG-RAN WG2 Meeting #113 electronic

[3] TR 23.700-20 Study on enhanced support of Industrial Internet of Things (IIoT) in the 5G System (5GS)

[4] TS 23.501 System architecture for the 5G System (5GS); Stage 2

[5] TS 22.104 Service requirements for cyber-physical control applications in vertical domains

[6] R2-2010704 Report for Rel-16 (NR-U, Power Savings and 2-step RACH) and IIoT and Small Data

[7] [TS 38.413] TS 38.413 NG-RAN; NG Application Protocol (NGAP)