3GPP TSG RAN WG2 Meeting #113bis-e	                                        R2- 2103736

e-Meeting, 12 – 20 April 2021

Agenda item:	8.15.3

Source:		Intel Corporation

Title: 	Resource Allocation Enhancements

Document for: 	Discussion

# Introduction

The Release 17 Sidelink Enhancements WI has been discussed in RAN2 112e and 113e. However, the meetings were focused on the discussion on Discontinuous Reception (DRX) and resource allocation enhancements have not yet been treated. The WID on NR Sidelink Enhancements for Release 17 [1] includes resource allocation enhancements and Discontinuous Reception (DRX). The main motivation for NR sidelink enhancements in Release 17 is to perform sidelink operations in a power efficient manner while also developing solutions that can enhance reliability and reduce latency.

Power saving has captured significant interest to enable UEs with limited battery reserves to perform sidelink communication. The objective in the WID, as captured below, is to specify resource allocation to reduce power consumption of the UEs and also to introduce enhancements for improved reliability in consideration of both PRR and PIR. Previously, Release 16 sidelink has mostly focused on UEs installed in vehicles with ample battery capacity, whereby such UEs have been assumed to be “always-on” when operating in sidelink. However, Rel 17 V2X shall support Vehicle-to-Pedestrian (V2P), Vulnerable Road Users (VRUs), and other use-cases (public safety and commercial) where there is a need to minimize power consumption, which necessitates the development of power saving mechanisms and solutions. It is also expected that solutions be devised for Rel 17 that can overcome the limitation of Rel-16 in achieving high reliability and low latency in certain communication conditions such as when the wireless channel is relatively busy and has high offered load. Mode-2 enhancements such as the use of inter-UE coordination, as captured from the WID below, can allow the support of URLLC-type sidelink use-cases in wider operation scenarios.

This contribution discusses the concepts of partial sensing and random resource selection as potential means for power saving sidelink enhancements in NR Rel 17. We study possible interaction of resource allocation with sidelink DRX operation and discuss congestion control based on Channel Busy Ratio (CBR) and Channel occupancy Ratio (CR) for resource allocation schemes in NR Rel 17. We also consider the current status in RAN WGs for the support of inter-UE coordination, and put forth the aspects which require further study in RAN2.

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any. - Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89. 3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the correspnding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

# Random Resource Selection and Partial Sensing

## Discussion

In RAN1 103e, it was agreed to adopt partial sensing and random resource selection as power saving resource allocation schemes, and some applicability conditions have been agreed in RAN1 104e. Still many details which can help ascertain required enhancements are FFS.

PSCCH and PSSCH resources are defined within resource pools for the respective channels. This notion of resource pools (while applicable for both mode 1 and mode 2), is particularly relevant in resource allocation mode 2, where a UE autonomously makes resource selections from the resource pool based on sensing. The sensing procedure as defined in Rel 16 NR sidelink is not particularly power efficient, in which case random resource selection, where a UE randomly selects resources from the resource pool without performing sensing can provide significant power savings.

Observation 1: Random resource selection can increase power efficiency.

For use-cases where the UE does not support sidelink reception, it may select transmission resources randomly, without performing the sensing operations. In such scenarios, random resource selection is the only option to support sidelink transmissions. In RAN1 104e, it has been agreed that random resource selection is applicable to both aperiodic and periodic transmissions.

Observation 2: Random resource selection is the only option that can be used by UEs that do not support sidelink reception.

Observation 3: Random resource selection is applicable to both periodic and aperiodic transmissions, however, conditions for random selection are FFS in RAN1.

While random resource selection can be power efficient, it comes at the expense of decreased reliability due to increased congestion and interference. Currently, for Rel 16 NR sidelink, the UE is only temporarily allowed to perform random resource selection for sidelink transmission in mode 2 based on the configuration of the exceptional transmission resource pool indicated by sl-TxPoolExceptional [2]. However, since NR sidelink communication is purposed to support a variety of use-cases, ranging from best-effort communication to mission-critical and public safety services, it is imperative to inculcate flexibility in the random resource selection mechanism, such that it may be enabled or disabled per resource pool, per BWP, according to the different sidelink transmission priority levels or per different power saving requirements/states. For Rel 14 LTE V2X this was done by including a parameter randomSelection in resourceSelectionConfigP2X in the SL-CommResourcePool [3]. Similar signalling for NR sidelink operation could be included by adding random selection configuration within the SL-ResourcePool information element.

Proposal 1: NR should support flexible design such that specific resource pools can be configured for random resource selection, e.g. by including random selection (optional) configuration in the SL-ResourcePool information element.

For transmission over NR sidelink, a UE performs sensing continuously over a configurable sensing window implying continuous power consumption for the ongoing sensing procedure. In Rel 14 LTE V2X sidelink, a pedestrian UE (P-UE) with presumably limited battery supply could be (pre-) configured to perform partial sensing where only a subset of the subframes in the typical sensing window are required to be monitored as shown in Figure 1 [4]. It is up to UE implementation to choose the number of subframes to monitor, as long as this number is larger than the (pre-)configured minimum number required to be monitored. In doing so, the UE can perform a trade-off between transmission reliability and power savings. Using Rel 14 LTE V2X sidelink as baseline, similar slot-based partial sensing principle can be adopted in Rel 17 NR sidelink as a power saving technique as opposed to the contiguous sensing of Rel 16 NR sidelink. Similar to random selection, partial sensing can also be configured in the SL-ResourcePool information element in NR sidelink. A resource selection configuration from the network can indicate the allowed resource selection mechanism(s), i.e. partial sensing and/or random selection for each transmission pool. If both partial sensing and random selection are configured, it may be left upto UE implementation to select a specific resource selection mechanism.

Proposal 2: Similar to LTE V2X sidelink, for mode 2 operation, slot-based partial sensing in lieu of contiguous sensing of Rel 16 NR sidelink can be configured for SL resource pools by including partial sensing configuration in the SL-ResourcePool information element.

Figure 1: Partial Sensing Operation

It is worth noting that partial sensing in LTE V2X sidelink has been primarily designed for periodic traffic with fixed set of sensing/no-sensing sub-intervals. In Release-17, the terminology of “periodic-based partial sensing” has been used which is based on the “partial sensing” used in LTE-V2X. This is because unlike LTE-V2X, advanced V2X and public safety use-cases supported in NR-sidelink commonly rely on aperiodic sidelink communication. Only periodic-based partial sensing has been considered in RAN-1 for Rel 17 thus far, however, if in future aperiodic-based partial sensing is also considered for this release, it may require evaluation in RAN2 to identify possible specification impact.

Proposal 3: Periodic-based partial sensing in Rel-17 is based on LTE-V2X partial sensing. If aperiodic-based partial sensing is considered for NR sidelink, possible specification impact in RAN2 needs to be evaluated accordingly.

Different from the legacy broadcast communication for LTE-V2X sidelink, NR sidelink supports additional cast types of unicast and groupcast communication. Some procedural enhancements to the baseline Rel 14 LTE V2X sidelink partial sensing operation may be adopted in NR sidelink, e.g HARQ feedback mechanism may be utilized for groupcast communication to adapt the partial sensing window according to the resource utilization, such that under high utilization, the sensing window may be lengthened to increase the probability of successful retransmission in the case of receiving HARQ NACK after initial transmission with short sensing window as discussed in [5]

Observation 4: Procedural enhancements to the baseline Rel 14 LTE V2X sidelink partial sensing operation may be added based on NR sidelink features for unicast and groupcast communication.

# Congestion Control based on CBR and CR

For NR Sidelink, following in the footsteps of LTE sidelink design, a physical measurement of the Channel Busy Ratio (CBR) is defined in each subframe which measures the portion of the resources in a resource pool with high received signal energy (S-RSSI) in the last 100 subframes. Accordingly, the Channel Occupancy Ratio (CR) measurement counts the number of subchannels used by the UE to transmit data in, during a window of 1000 ms including the current subframe.

To control channel utilization and allow congestion control, based on the Channel Busy Ratio (CBR) and Channel Occupancy Ratio (CR), the network can indicate how the UE adapts its transmission parameters for each transmission pool. A UE chooses its transmission parameters from within the range corresponding to the prevailing CBR and associated CR limit. A UE may perform CBR measurement of all configured transmission pools including exceptional pool, for both the PSSCH and PSCCH resources. In Rel-16 NR Sidelink, CBR measurement reporting mechanism of LTE V2X sidelink communication is re-used.

In Rel-14 LTE V2X sidelink, in the interest of power saving, pedestrian UEs do not perform CBR measurement. In such cases, P-UEs using power saving resource allocation schemes such as random selection and partial sensing adjust their transmission parameters based on the default transmission parameter configurations which can be provided by the network via RRC signaling. For Rel-17, in RAN1 # 103e, it has been agreed, as captured below, that further study is required on congestion control based on CBR for power saving resource allocation schemes [6].

Agreements: - Further study congestion control based on CBR and CR for power saving RA schemes - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

Since the support of random resource selection and partial sensing can cause congestion and interference, congestion control based on CBR may be further explored. For Rel-16 NR sidelink, cbr-ReportSidelink indicates whether UE can report CBR measurement to gNB and can adjust its radio parameters based on CBR measurement and CRlimit [2][7]. The IE cbr-CR-TimeLimitSidelink indicates the time within which UE can process CBR and CR. However, based on RAN1 agreement, it needs to be identified what necessary changes may be required from Rel-16 CBR/CR (if any) before adapting it to the power saving resource allocation schemes (which was not provisioned in Rel-14 LTE V2X sidelink). RAN2 needs to discuss, what changes may be required to the current congestionControlSidelink-r16 IE which indicates whether UE supports sidelink congestion control for NR sidelink. However, RAN2 impact can be analysed better once more progress has been made in RAN1.

Observation 5: Different from Rel-14 LTE, RAN1 is under discussion to apply CBR/CR to power saving RA schemes. However, it is still FFS how transmission and transmission parameters can be adjusted.

Proposal 6: RAN2 to wait for further progress in RAN1 regarding congestion control based on CBR before considering RAN2 impact and required support.

# Inter-UE Coordination

## RAN1 Status

The topic of inter-UE coordination was put forth in the RAN 91e and it was agreed with a majority to keep the current objective in the WID on the scope of inter-UE coordination. In the last RAN1 meeting # 104e, it was concluded that inter-UE coordination in Mode 2 is feasible and beneficial compared to Rel-16 Mode 2 RA, and thus RAN1 recommended specification of the feature. Detailed observations made by RAN1 based on simulation and evaluation results provided by different companies were shared in an LS to RAN plenary [8].

Multiple schemes for inter-UE coordination have been proposed by different companies in RAN1, however, no consensus or baseline scheme has been down-selected so far. The design principles for the inter-UE coordination schemes proposed by different companies have much variation. For instance, one design direction is towards solutions pertaining to post-collision indication where an assisting UE can detect collision between two transmitters and indicate to the transmitting UEs via feedback channel the need to update their set of resources accordingly. Another solution is to introduce a resource controller or scheduling UE, where such scheduling UE is elected as a resource coordinator (multiple scheduling UEs elected in a distributive manner) which assigns resource pools to different UEs for transmission. This issue was put forth in RAN 91e, whether it is necessary for RAN to down-select or prioritize some inter-UE coordination types. Most companies responded that it is not practical for RAN to down-select or prioritize some inter-UE coordination types with the information provided in the current RAN1 conclusion. Therefore the decision has been to go forward with no WID update and no down-selection is made for inter-UE coordination type.

Observation 6: No down-selection or prioritization of inter-UE coordination types has been made. RAN WGs shall proceed working on inter-UE coordination without WID update.

## RAN2 aspects

While no scheme for inter-UE coordination has been down-selection, however, some key areas of consideration can be identified which require further study for inter-UE coordination in RAN WGs.

- Identification of Coordinating UEs

It needs to be determined how two UEs are selected/configured for inter-UE coordination, that is, “How UE-A and UE-B are determined” as per RAN1 conclusion. Based on further discussion in RAN1 and the criteria which will eventually be identified in RAN1 regarding the identification of coordinating UEs, the authorization and capability information which may be required to support such identification needs to be discussed in RAN2. It has also been discussed by some companies whether the coordinating or assisting UE-A is actively participating in communication with UE-B, or whether it is a third-party UE, which has unicast links established with the transmitting UEs, with the sole purpose of providing inter-UE coordination information to UE-B without any involvement in the data communication [9].

- Triggering Condition for Inter-UE coordination

The inter-UE coordination signalling can be triggered in a number of ways.

- It can be a periodic update where the granularity for the signalling could be pre-configured at UE-A (assisting UE).

- It can be requested by UE-B from UE-A, to forward the set of resources which may be preferred for UE-B’s transmission

- It can be triggered based on a condition or locally available information which may prompt UE-A to send coordination signalling to UE-B.

The triggering condition requires further study in RAN1 and RAN2, as has also been mentioned in the RAN1 agreement that “When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it” needs to be further discussed. A common requirement, however, for all three types of triggers is that the usefulness for inter-UE coordination would depend on how current the coordination information is, that is, it is imperative for inter-UE coordination information to be delivered with low latency and reliably, such that the set of resources forwarded by UE-A to UE-B are not outdated/stale.

- Signalling aspects and message format

As mentioned in 2) above, it is imperative that the inter-UE coordination information is transmitted reliably and also with reduced latency in order to avail its benefits. To this end, it needs to be discussed in RAN WGs, how this information is signalled, and also the format for the message or container used to carry this information. This signalling could be at the physical layer or higher layers, where the size of the coordination information may be a basic determinant regarding the choice of signalling and message formats. Short coordination messages with small size could utilize SCI signalling format and communicated through L1 signalling. In other cases, PC-5 RRC signalling or MAC CE may be supported. It has also been proposed to discuss a new SL MAC CE which could be assigned a higher priority than normal SL data [10].

- Supported Cast Types

It needs further discussion in RAN1 to conclude which cast types, unicast, groupcast or broadcast, will be supported for inter-UE coordination. Note that in the case if PC5-RRC signalling is used for transmitting inter-UE coordination information as discussed in 3) above, it may inherently be implied that inter-UE coordination is supported over the unicast link. Whether all three, or a subset of the cast types is supported, it needs to be also discussed whether a common signalling method needs to be adopted for all cast types, or different signalling aspects need to be considered for the different cast types.

Based on the key areas identified above, the following proposals can be made.

Proposal 7: For inter-UE coordination, several key aspects require further discussion in RAN WGs, including but not limited to

- identification of coordinating UEs

- triggering condition for inter-UE coordination

- signalling aspects and message format

- supported cast types

# Conclusions

In this contribution we discuss resource allocation enhancements in Rel 17 NR sidelink and make the following observations and proposals

Observation 1: Random resource selection can increase power efficiency.

Observation 2: Random resource selection is the only option that can be used by UEs that do not support sidelink reception.

Observation 3: Random resource selection is applicable to both periodic and aperiodic transmissions, however, conditions for random selection are FFS in RAN1.

Proposal 1: NR should support flexible design such that specific resource pools can be configured for random resource selection, e.g. by including random selection (optional) configuration in the SL-ResourcePool information element.

Proposal 2: Similar to LTE V2X sidelink, for mode 2 operation, slot-based partial sensing in lieu of contiguous sensing of Rel 16 NR sidelink can be configured for SL resource pools by including partial sensing configuration in the SL-ResourcePool information element.

Proposal 3: Periodic-based partial sensing in Rel-17 is based on LTE-V2X partial sensing. If aperiodic-based partial sensing is considered for NR sidelink, possible specification impact in RAN2 needs to be evaluated accordingly.

Observation 4: Procedural enhancements to the baseline Rel 14 LTE V2X sidelink partial sensing operation may be added based on NR sidelink features for unicast and groupcast communication.

Observation 5: Different from Rel-14 LTE, RAN1 is under discussion to apply CBR/CR to power saving RA schemes. However, it is still FFS how transmission and transmission parameters can be adjusted.

Proposal 6: RAN2 to wait for further progress in RAN1 regarding congestion control based on CBR before considering RAN2 impact and required support.

Observation 6: No down-selection or prioritization of inter-UE coordination types has been made. RAN WGs shall proceed working on inter-UE coordination without WID update.

Proposal 7: For inter-UE coordination, several key aspects require further discussion in RAN WGs, including but not limited to

- identification of coordinating UEs

- triggering condition for inter-UE coordination

- signalling aspects and message format

- supported cast types

# References

[1] RP-202846 WID revision: NR sidelink enhancement, 3GPP TSG RAN Meeting #90e,  December 2020

[2] TS 38.331 V16.2.0 (2020-09) Radio Resource Control (RRC) protocol specification

[3] TS 36.331 V14.15.0 (2020-09) Radio Resource Control (RRC) protocol specification

[4] TS 36.300 V14.12.0 (2020-03) E-UTRA E-UTRAN Overall Description Stage 2

[5] R1-2100687, Resource allocation mechanisms for power saving, Ericsson, RAN1 104e

[6] R1-2100001 Final Report of 3GPP TSG RAN WG1 #103-e v1.0.0

[7] TS 38.306 NR; User Equipment (UE) radio access capabilities

[8] R1-2102168, LS on Mode 2 enhancements in NR sidelink, RAN WG1 #104e

[9] R2-2009028 Discussion on sidelink inter-UE coordination,  ZTE Corporation, Sanechips

[10] R2-2101647	Transmission of assistance information for Mode 2 enhancement, MediaTek Inc., RAN2 #113e