3GPP TSG-RAN2 Meeting #113bis-e	R2-2103773

eMeeting, 12th – 20th April, 2021

Agenda Item:	8.9.2 Idle/inactive-mode UE power saving

Source: 	Ericsson

Title:  	Group info signaled via Paging PDCCH

Document for:	Discussion and Decision

# Introduction

In this contribution the use of grouping info in paging PDCCH to avoid false paging alarms is discussed.

# Discussion

Introduction

RAN2 has discussed introduction of DL scheduling slot offset capabilities in the Paging message [1-4]. Introduction of these capabilities in the Paging message enables the NW to use K0 > 0 in Paging PDCCH when only UEs that support K0 > 0 are paged in the PO. But it does not enable UEs that support K0 > 0 to avoid trying to receive Paging message on PDSCH in the following slot, i.e. only when the UE knows in advance that K0 = 0 is not used for Paging in the PO the UE can prevent receiving Paging PDSCH in the slot, i.e. the UE needs processing time to determine whether it needs to receive the Paging PDSCH in the following slot. But K0 = 0 is used for legacy UEs that do not support K0 > 0. Thus UEs supporting K0 > 0 can only save power in receiving the Paging PDSCH, when the grouping information indicates that they are not paged, when they are paged in a dedicated PO where legacy UEs are not paged:

Observation 1: The UE can only save power with grouping information in Paging PDCCH when the UE supports K0 > 0 and the UE is paged in a dedicated PO where legacy UEs are not paged.

Discussion

RAN2 discussed grouping info to reduce false paging alarms and considered that both signalling via PEI and Paging DCI are within scope:

RAN2 have discussed and considered “paging indication for UE subgroups using paging DCI”, “paging early indication or wake-up signal (WUS) for UE subgroups”, “cross-slot scheduling of paging for UE subgroups”.

To enable grouping information via Paging DCI it is proposed:

Proposal 1: Network can configure additional PO(s) dedicated to UEs that support K0 > 0.

Solution

There are different ways to configure additional paging capacity, i.e. POs for UEs supporting K0 > 0. But to minimize the impact it is preferred to re-use the paging framework and paging formula. For example a simple solution is to configure additional slots via a second nAndPagingFrameOffset and firstPDCCH-MonitoringOccasionOfPO for UEs supporting K0>0:

Proposal 2: Network can configure dedicated POs for UEs supporting K0>0 via a second nAndPagingFrameOffset and/or firstPDCCH-MonitoringOccasionOfPO parameter.

# Summary

RAN2 is kindly asked to discuss group information using paging PDCCH and Cross-Slot Scheduling:

Proposal 1: Network can configure additional PO(s) dedicated to UEs that support K0 > 0.

Proposal 2: Network can configure dedicated POs for UEs supporting K0>0 via a second nAndPagingFrameOffset and/or firstPDCCH-MonitoringOccasionOfPO parameter.

# References

- R2-2103768, Summary of [Post113-e][051][NR15] DL scheduling slot offset, Ericsson, Report, RAN2#113bis-e

- R2-2103769, Open issues K0 configuration and use, Ericsson, DISC, RAN2#113bis-e

- R2-2103770, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericssion, CR 38.331, Rel-15, RAN2#113bis-e

- R2-2103771, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericssion, CR 38.331, Rel-16, RAN2#113bis-e