3GPP TSG-RAN2 Meeting #113bis-e	R2-2103774

eMeeting, 12th – 20th April, 2021

Agenda Item:	8.9.3 Other aspects RAN2 impacts

Source: 	Ericsson

Title:  	TRS exposure to UEs in idle and inactive

Document for:	Discussion and Decision

# Introduction

In this contribution exposure of the configured TRS for connected mode UEs to UEs in idle and inactive mode is discussed further.

# Background

RAN2#113-e

RAN2 discussed how to convey the connected mode TRS/CSI-RS configuration to the UE in idle/inactive in offline #041 during RAN2#113-e [1]. RAN2 excludes SIB1 but keeps existing or new SIB FFS. Other methods (e.g. dedicated signalling) are not excluded (yet). RAN2 did not discuss availability signalling, i.e. signalling indicating whether the TRS, indicated by the TRS configuration in System Information, is transmitted. RAN2 reached the following agreements:

[041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s):

- SIB signalling is the baseline;

- Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone.

[041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s):

- Option 2: Existing SIB, other than SIB1;

- Option 3: New SIB type, e.g. SIB-x;

RAN1#104-e

RAN1 reached the following agreements for TRS/CSI-RS exposure to idle/inactive mode:

Agreements:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least:

- powerControlOffsetSS,

- scramblingID

- firstOFDMSymbolInTimeDomain,

- startingRB.

- nrofRBs,

- FFS other parameters

- FFS applicable values

Agreements:

The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b:

- Alt1: same as initial BWP

- Alt2: configurable parameter

Agreements:

Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details (including whether or not to restrict the RS to be TRS only)

Update on 1/31:

Agreements:

For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication.

- FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.)

- There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

Conclusion

From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs.

Agreements:

The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e:

- Alt-1: within initial DL BWP

- Alt-2: is not restricted by initial BWP

  - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

Agreements:

To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives:

- Alt-1: from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS

- Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO

- FFS details

- Other alternatives are not precluded

Conclusion:

Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

RAN#91-e

A company contribution was submitted to RAN plenary proposing not to consider serving cell RRM measurement relaxation [2].

# Discussion

TRS/CSI-RS configuration info

RAN1 has agreed on a set of TRS configuration parameters, but other parameters and value ranges are FSS. It is assumed that RAN1 will provide the TRS configuration information in an RRC parameter list to RAN2 for implementation in 38.331.

RAN1 will decide on whether to support periodic CSI-RS in RAN1#104bis-e, for which (additional) configuration info may be required:

Proposal 1: RAN2 to wait for RAN1 to provide the TRS/CSI-RS configuration info.

SIB for TRS/CSI-RS configuration

TRS/CSI-RS configuration info specifies where to find the TRS/CSI-RS occasions (e.g. BWP, slot periodicity and offset, band info) and how the reference signal is configured. The information size is expected to be more than a few bits, given by the set of agreed parameters already, but also because there may be a different configuration for each beam. Furthermore there is a need to configure the PEI, and there may be a need to broadcast information for other UE power saving features (e.g. RedCap). This type of information does not functionally fit in existing SIBs. Therefore it is proposed to introduce a new SIB for UE power saving information:

Proposal 2: Introduce a new SIB for UE Power Saving (UPS) information to convey TRS/CSI-RS configuration and other UE power saving info.

On-demand SIB

On-demand system information enables the network to optionally not broadcast a SIB and save system resources, but still provide the SIB service by enabling the UE to request the SIB when needed. On-demand system information may be a suitable choice for the power saving SIB, because the UE power savings features are expected to be optional and not supported by all UEs:

Proposal 3: The UPS SIB can be configured on demand for UEs in idle and inactive mode.

Area specific SIB

The TRS/CSI-RS configuration may be configured differently per beam, and neighbouring cells may have different beam configurations. It is therefore not clear if it is useful to have the UPS SIB area specific:

Proposal 4: RAN2 to discuss if the UPS SIB can be configured area specific.

Availability signalling

The connected mode TRS configuration is exposed to the UEs in idle/inactive. When there is no UE in connected mode the network may switch the TRS and the network is not required to transmit TRS. This understanding is reflected in the agreed RAN1 notes below:

-  Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share).

-  Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted.

-  Note: Always-on TRS/CSI-RS transmission by gNodeB is not required

-  FFS: Whether UE blind detection is required or not.

RAN1 decided that the UE is not required to perform blind detection, and that the UE assumes that the TRS is not transmitted when the network does not indicate that the TRS is available.

When the TRS configuration is present in the UPS SIB, this does not indicate that the TRS is available, i.e. that the TRS is transmitted. RAN1 has discussed the need for explicit (e.g. in PEI) or implicit (coupling with paging) signalling of the TRS configuration availability, i.e. signalling that TRS is actually transmitted.

In case TRS availability would be signalled via UPS SIB presence (or change of the UPS SIB content) then there could be a strong impact on the signalling via Paging for SI change:

Observation 1: In case TRS availability would be signalled via SI change procedure, then this may have strong impact on the Paging signalling and prohibit the network from signalling TRS availability.

During periods when the last UE leaves connected mode and the TRS is switched off, and the first UE enters connected mode again and the TRS is switched on, there could be frequent Paging due to SI change. The NW needs to page the UEs in all POs of the DRX cycle at least one time, but perhaps a second time to reach all UEs reliably. This would impact the power consumption of UEs not supporting TRS in idle/inactive. Furthermore the content of system information does not change often (at all) in legacy, except for ETWS/CMAS warning messages when there is an emergency (which typically and luckily is not all the time). Signalling impact for TRS availability does not have the same level of urgency and is not as well motivated, compared to warning messages which can be life threatening.

To protect the network against excessive paging load during periods when the TRS is frequently switched on and off, the network may prohibit availability signalling to the UE. RAN1 will continue to discuss the details how to explicitly indicate availability [3]. RAN2 should therefore not discuss TRS/CSI-RS availability:

Proposal 5: TRS/CSI-RS availability signalling is discussed in RAN1 only until RAN1 has concluded on this topic.

TRS configuration change

The connected mode TRS configuration is assumed to change infrequent, if at all. The network is not assumed to change TRS configuration for a (new or existing) UE that often  Thus the paging load due to TRS configuration change is assumed to be (very) low:

Proposal 6: When the TRS configuration (not availability) changes this is indicated via the normal SI modification procedure.

NOTE: the connected mode TRS configuration can change at any point in time, but it may take some time for the UEs in idle/inactive mode to have received the change (e.g. up to a modification period).

Dedicated signalling

The TRS/CSI-RS configuration is common information, i.e. the same for all UEs in the cell. There is does not seem to be a motivation for a UE specific TRS/CSI-RS configuration. But RAN2 can evaluate more if there are use cases where dedicated signalling for the TRS/CSI-RS exposure is needed:

Proposal 7: RAN2 to discuss if there are uses cases that require dedicated signalling for TRS/CSI-RS exposure.

# Summary

RAN2 is kindly asked to discuss TRS exposure:

Proposal 1: RAN2 to wait for RAN1 to provide the TRS/CSI-RS configuration info.

Proposal 2: Introduce a new SIB for UE Power Saving (UPS) information to convey TRS/CSI-RS configuration and other UE power saving info.

Proposal 3: The UPS SIB can be configured on demand for UEs in idle and inactive mode.

Proposal 4: RAN2 to discuss if the UPS SIB can be configured area specific.

Proposal 5: TRS/CSI-RS availability signalling is discussed in RAN1 only until RAN1 has concluded on this topic.

Proposal 6: When the TRS configuration (not availability) changes this is indicated via the normal SI modification procedure.

Proposal 7: RAN2 to discuss if there are uses cases that require dedicated signalling for TRS/CSI-RS exposure.

# References

- R2-2102469, Summary of [041][ePowSav] TRS/CSI-RS for IDLE INACTIVE, Xiaomi, Report, RAN2#113-e

- RP-210329, Clarification of the UE power saving enhancements WI scope; Nokia, DISC, RAN#91

- R1-21xxxxx, Provisioning of TRS occasions to Idle/Inactive UEs, Ericsson, DISC, RAN1#104bis-e