3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103782

12th April – 20th April 2021

Agenda item:	8.16.2

Source:	MediaTek Inc.

Title:	RAN2 impact to support SNPN with credentials by a separate entity

WID/SID:	NG_RAN_PRN_enh - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses RAN2 support of SNPN with subscription/credentials owned by a separate entity following the agreement in RAN2#113e [1]:

In the UE, AS reports to NAS about the following broadcasted new parameters:

- Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN

- Supported Group IDs

- Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN.

# 2	Discussion

## 2.1	Broadcasting enhancements

npn-IdentityInfoList

The npn-IdentityInfoList is used to configure a set of NPN-IdentityInfo elements. Each of those elements contains a list of one or more NPN Identities and additional information associated with those NPNs. The total number of PLMNs (identified by a PLMN identity in plmn -IdentityList), PNI-NPNs (identified by a PLMN identity and a CAG-ID), and SNPNs (identified by a PLMN identity and a NID) together in the PLMN-IdentityInfoList and NPN-IdentityInfoList does not exceed 12, except for the NPN-only cells. In case of NPN-only cells the PLMN-IdentityList contains a single element that does not count to the limit of 12. The NPN index is defined as B+c1+c2+…+c(n-1)+d1+d2+…+d(m-1)+e(i) for the NPN identity included in the n-th entry of NPN-IdentityInfoList and in the m-th entry of NPN-Identitylist within that npn-IdentityInfoList entry, and the i-th entry of its corresponding NPN-Identity, where

- B is the index used for the last PLMN in the PLMN-IdentittyInfoList; in NPN-only cells B is considered 0;

- c(j) is the number of NPN index values used in the j-th NPN-IdentityInfoList entry;

- d(k) is the number of NPN index values used in the k-th npn-IdentityList entry within the n-th NPN-IdentityInfoList entry;

- e(i) is

- i if the n-th entry of NPN-IdentityInfoList entry is for SNPN(s);

- 1 if the n-th entry of NPN-IdentityInfoList entry is for PNI-NPN(s).

In TS 38.331 [2] the total number of NPNs that can be signalled does not exceed 12 (including NPN-only cells):

As defined above, an SNPN is identified by a PLMN identity and a NID. The example structure below outlines how the Rel-16 SNPN information is signaled to the UE in SIB1:

List[j] of NPN-IdentityInfo                                                                                                   c (j)

|----> …

|----> List[k] of NPN-Identity                                                                                     d (k)

|                |----> pni-npn

|                |                | ----> …

|                |----> snpn

|                |                |----> PLMN-Identity

|                |                |----> List[i] of NID                                                                  e (i)

The reply from SA2 [3] below, clarifies that the new indicators (using credentials from a separate entity; registration attempts from other UEs allowed) as well as the group IDs are to be signaled per SNPN:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

[SA2 answer] Yes, These parameters should be set uniformly per SNPN.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

[SA2 answer] Yes, It is assumed that that the Group IDs will be broadcast per SNPN.

With this clarification, the new parameters are expected to be signaled as shown in the example below:

List[j] of NPN-IdentityInfo                                                                                                   c (j)

|----> …

|----> List[k] of NPN-Identity                                                                                     d (k)

|                |----> pni-npn

|                |                | ----> …

|                |----> snpn

|                |                |----> PLMN-Identity

|                |                |----> List[i] of NID                                                                  e (i)

|                |                |                |----> separateEntityCredentials                               new optional field

|                |                |                |----> ExtRegistrationAllowed                                 new optional field

|                |                |                |----> List[l] of groupID                                           new optional list f(l)

As parameters separateEntityCredentials and ExtRegistrationAllowed are only indicators, they can be signaled as simple flags. With regards to signaling group information, we note the following from TR 23.700-07, section 8.1.4 [4]:

The following enhancements will be progressed in the normative phase:

- Group ID as a specific case of SNPN ID reusing SNPN ID encoding in TS 23.003 [15], where

…

NOTE 2:	The number of supported Group IDs that can be broadcast will be determined by RAN2.

From the text above, we assume that a groupID would be signaled similar to the NID for SNPNs today, i.e. as 44bit value. This assumption can be revised as the normative phase in SA2 progresses.

Therefore our assumptions for the rest of this section are:

separateEntityCredentials and ExtRegistrationAllowed are 1-bit flags

groupID reuses the same encoding as a NID (i.e. 44 bit value)

We note that SA2 expects RAN2 to determine the number of group IDs that can be signaled. In relation to this, we have the following restriction on the size of a SIB in NR:

- TS 38.331 section 5.2.1 [2]:

NOTE: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

- The corresponding physical layer limit is captured in TS 38.214 section 5.1.3.2 [6]:

The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.

Each SNPN can use up to 80 bits (plmn-Identity and NID) to be signaled in SIB1. Therefore to accommodate 12 SNPNs in SIB1, we could use up to 960bits in SIB1. This accounts for ~32% of the maximum size available for SIB1. As SIB1 is also used to provide the downlink and uplink configuration of the cell, it is important that we do not further increase the overhead associated with NPN signaling beyond what exists today, even when considering the new parameters to be signaled.

Observation 1: Signaling of SNPN lists alone take up to 32% of the maximum size of the SIB.

Proposal 1: The total number of PLMNs, PNI-NPNs, SNPNs and groupIDs in a cell does not exceed 12.

## 2.2	Idle Mode Mobility

In response to RAN2’s earlier question on whether the new indicators and group IDs are uniform across a network or registration area, SA2 have responded [3] that the information is set uniformly across the SNPN.

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

[SA2 answer] Yes, These parameters should be set uniformly per SNPN.

In RAN2, we already check whether the candidate cell for reselection belongs to the same SNPN as the registered or selected SNPN (see excerpt from [5] below). Given that these new parameters are uniformly set across all cells in the SNPN, following initial cell selection, there is no need for a further check at the time of reselection.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to one or more of the following reasons:

…

- this cell does not belong to a SNPN that is equal to the registered or selected SNPN of the UE in SNPN access mode,

the UE shall not consider this cell and, for operation in licensed spectrum, other cells on the same frequency as candidates for reselection for a maximum of 300 seconds.

Proposal 2: There are no impacts to the cell reselection procedure due to the new parameters defined in Rel-17 for SNPNs.

# 3	Conclusion

This document notes the following observation and makes the following proposals:

Observation 1: Signaling of SNPN lists alone take up to 32% of the maximum size of the SIB.

Proposal 1: The total number of PLMNs, PNI-NPNs, SNPNs and groupIDs in a cell does not exceed 12.

Proposal 2: There are no impacts to the cell reselection procedure due to the new parameters defined in Rel-17 for SNPNs.

# 4	References

[1] Report of 3GPP TSG RAN WG2 meeting #113e – Online 25 January – 5 February, 2021

[2] Radio Resource Control (RRC) Protocol Specification (3GPP TS 38.331 Release 16)

[3] Reply LS on clarification request for eNPN features (R2-2102658 / S2-2101076)

[4] Study on enhances support for non-public networks (3GPP TR 23.700-07 Release 17)

[5] User Equipment procedures in idle mode and in RRC Inactive state (3GPP TS 38.304 Release 16)

[6] Physical layer protocol (3GPP TS 38.214 Release 16)