3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103784

Online, April 12 – April 20, 2021

Source: 	MediaTek Inc.

Title: 	On RRM relaxation for RedCap devices

Agenda items: 	8.12.3.2

Document for: 	Discussion and Decision

# Introduction

RRM relaxations for RedCap devices were discussed extensively in RP#91e, but a clear WI objective could not be defined as there are no recommendations in the TR regarding this topic. Therefore it was decided in the plenary to task RAN2 to downselect from the various options listed in the TR as highlighted below [1]:

- RRM relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected, considering the alternatives identified in the RedCap SI:

  - Study until RAN#92e, and, if agreed, specify RRM measurement relaxation criteria (where, for RRC_Idle/Inactive the Rel-16 mechanism is the baseline, and for RRC_Connected the mechanism reuses the Rel-16 RRM relaxation criteria from RRC_Idle/Inactive so as to maximize the commonality with Idle/Inactive UEs) [RAN2]

    - Enabling/disabling of RRM relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM relaxation.

  - After RAN#92e, if agreed in RAN2, specify RRM measurement relaxation [RAN4]

  - No RRM relaxations are specified for the serving cell.

It is therefore imperative that RAN2 provides some clear recommendations on the way forward with respect to RRM relaxation by RP#92e. In order to have a clear WI objective that can be completed in the Rel-17 timeframe, we provide our recommendations in this document.

# Discussion

To start with, it is useful to point out the reason for introducing RRM relaxations in RedCap. The related SI objective [2] is copied below for reference. This feature is meant to address optimisations for stationary devices, and this can be used as a guidance to reach a recommendation, i.e. does the recommended solution meet this objective or not.

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]:

•	RRM relaxation for stationary devices [RAN2]

While there are several options listed in the TR [3] as solutions to trigger RRM relaxations, they can be largely classified into 3 groups:

1. Extend the Rel-16 baseline with additional settings

It is important to note that in Rel-16, RRM relaxations for UEs in Idle/Inactive state with low mobility have already been defined. The trigger for RRM relaxation in Rel-16 is based on the variance of the downlink signal strength, and if the UE is at cell-edge or not. Enhancements 1, 2 and 5 discussed in section 8.4.2 in the TR [3] take this approach further by defining additional thresholds of ‘variance’ to have a two-step approach for further RRM relaxations.

However, as noted in the TR, these options cannot reliably distinguish between truly stationary UEs and UEs with low mobility. Considering this downside associated with this option, these options to trigger RRM relaxation cannot be considered suitable to meet the SI objective. Therefore we propose to disregard this option for the WI.

Observation 1: RRM relaxation triggers that introduce additional thresholds for speed evaluation do not reliably distinguish between stationary UEs and UEs with limited mobility.

2. Use number of beam changes

This option relies on the number of beam changes to determine UE mobility. However, this option once again is fuzzy at differentiating between truly stationary devices and UEs with limited mobility. Furthermore, beam level measurements fluctuate significantly more than cell level measurements, and as such is a worse option than what we have in Rel-16. Therefore we propose to disregard this option for the WI as well.

Observation 2: RRM relaxation triggers based on beam changes also do not reliably distinguish between stationary UEs and UEs with limited mobility, and suffer from further disadvantages due to high levels of fluctuation in beam-level measurements.

3. Preconfigured stationary property

Enhancements 4 and 6 in section 8.4.2 of the TR [2] depend on a preconfigured stationary property. Rather than duplicating the Rel-16 approach (which are anyways available for RedCap operation), these options trigger RRM relaxations using information that’s configured at deployment (e.g. using subscription information) to enable further relaxations without relying on downlink measurements.

For situations corresponding to RedCap use-cases, such as in Industrial scenarios or surveillance systems, this ‘stationary’ property can be pre-configured in the device, or be configured at deployment depending on deployment. These options can be reliably used to distinguish between ‘truly stationary’ devices and devices with ‘limited mobility’, and therefore meet the RedCap SI objective.

Observation 3: A stationary property that’s part of UE subscription information can reliably distinguish between stationary UEs and UEs with limited mobility.

Furthermore, Rel-16 based measurement relaxation triggers can easily distinguish between whether a UE is at cell-edge or not. On combining the preconfigured ‘stationary’ property with Rel-16 triggers, the disadvantage noted in the TR with regards to UEs at cell edge can be easily avoided.

Observation 4: When combined with Rel-16 triggers, RRM relaxation based on a ‘stationary’ property can easily distinguish between cell-centre and cell-edge UE deployment.

Considering these arguments, we propose to go forward with this option in the RedCap WI, as it is the only option that reliably addresses the SI objective.

Proposal 1: RedCap devices use subscription information (e.g. in USIM) to trigger RRM relaxations.

# Conclusion

In this contribution, we look at means to reduce the scope of RRM relaxation discussions to have a clear WI direction. We note the following observations.

Observation 1: RRM relaxation triggers that introduce additional thresholds for speed evaluation do not reliably distinguish between stationary UEs and UEs with limited mobility.

Observation 2: RRM relaxation triggers based on beam changes also do not reliably distinguish between stationary UEs and UEs with limited mobility, and suffer from further disadvantages due to high levels of fluctuation in beam-level measurements.

Observation 3: A stationary property that’s part of UE subscription information can reliably distinguish between stationary UEs and UEs with limited mobility.

Observation 4: When combined with Rel-16 triggers, RRM relaxation based on a ‘stationary’ property can easily distinguish between cell-centre and cell-edge UE deployment.

Based on the observations above, we have the following proposal.

Proposal 1: RedCap devices use subscription information (e.g. in USIM) to trigger RRM relaxations.

# References

- RP-210918 “Revised WID on support of reduced capability NR devices”, Nokia, Ericsson, March 2021

- RP-201386 “Revised SID on Study on support of reduced capability NR devices”, Ericsson, RAN#88e, July 2020

- “TR 38.875 - Study on support of reduced capability NR devices”