3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103821

# Electronic Meeting, 12th – 20th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0014 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On corrections to packet loss rate measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As part of the objective of some of the individual delay measurements’, it is explicitly stated that the measurement can be used for the OAM performance observability or for QoS verification of MDT. However, the packet loss rate measurements introduced in the layer-2 specification mention only OAM performance observability as the objective and not the QoS verification of MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoS verification of MDT is added as an objective for the ‘Packet Uu Loss Rate in the DL per DRB per UE’ measurement. It is also clarified that the F1-U packet loss measurement specified in TS 28.552 is performed per DRB per UE level granularity to handle the MDT related UE specific measurement. Impact Analysis Impacted functionality Immediate MDT Inter-operability analysis There are no inter-operability issue is foreseen associated to the issue captured in this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear as to whether the ‘Packet Uu Loss Rate in the DL per DRB per UE’ measurement defined in this spec is applicable for MDT or not. It is not clear to the OAM node whether the received M7 measurement associated to the F1-U packet loss rate, is measured at a per UE level granularity or across multiple UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.5.1, 4.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.2.1.5	Packet Loss Rate

##### 4.2.1.5.1	Packet Uu Loss Rate in the DL per DRB per UE

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability or for QoS verification of MDT.

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

NOTE 1:	Packet loss is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501 [4]) of the DRB which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

#### 4.2.1.6	Other measurements defined in TS 28.552 [2]

The granularity for PDCP SDU Data Volume measurement defined in TS 28.552 [2] is per DRB per UE.

The granularity for Average UE throughput measurement defined in TS 28.552 [2] is per UE and per DRB per UE.

The granularity for UL F1-U Packet Loss Rate and DL F1-U Packet Loss Rate measurement defined in TS 28.552 [2] is per DRB per UE.

PRB usage measurements are defined in TS 28.552 [2], i.e. DL/UL Total PRB Usage, Distribution of DL/UL Total PRB Usage. M(T), M1(T), P(T) are measured per cell. P(T) is the total available PRBs for this cell. M1(T) is the PRBs used for traffic transmission in this cell. Counting unit for PRB usage measurement is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier).

END OF CHANGE