3GPP TSG-RAN WG2 Meeting # 113 bis Electronic	R2-2103832

Online, April 12 – April 20, 2021

Agenda Item:	[8.3.3] [UE notification on network switching for multi-SIM]

Source: 	Apple Inc

Title:  	Aspects of MUSIM RRC Band Conflict and Processing Delay Requirements

Document for:	Discussion and Decision

# Introduction

Multi-SIM devices have proven to be very popular in certain cellular markets owing to the flexibility it provides to the user to manage multiple SIM subscriptions in the same physical device. Different flavours of Multi-SIM implementation are commercially available, based on the Radio Access Technology combinations that each SIM instance can support, or based on the number of concurrent Transmit and/or Receive chains that are possible to be handled.

Until 3GPP R17, commercially available Multi-SIM solutions are primarily based of UE proprietary implementations, leading to varied UE side behaviour and inconsistent user experience. A new work item on standardizing Multi-SIM behaviour was approved [1] and this work item covers one of the important aspects of Multi-SIM which deals with network switching of Multi-SIM devices.

As indicated before there can be multiple flavours of Multi-SIM implementation. Specifically based on the number of concurrent Transmit and/or Receive chains that are possible, the following combinations are most prevalent UE architecture for Multi-SIM. Note for the case of generality the paper refers to Multi-SIM, but commercial UE implementations typically have two SIM instances (Dual-SIM).

- Single-Rx / Single-Tx

  - One single Rx/Tx that is shared across the two SIM instances, allowing the MUSIM UE to receive traffic from only one network at a time and transmit traffic to only one network at a time.

- Dual-Rx / Single-Tx

  - Each SIM instance has its own dedicated Rx, and One single Tx chain that is shared across the two SIM instances, thereby allowing the MUSIM UE to receive traffic from both networks at a time and transmit traffic to only one network at a time.

- Dual-Rx / Dual-Tx

  - Each SIM instance has its own dedicated Rx and Tx, thereby allowing the MUSIM UE to transmit and receive traffic to their respective networks simultaneously.

In this contribution, we would like to highlight certain issues related to Band Conflict and RRC Processing requirements in the context of MUSIM.

# Discussion

## Band Conflict scenarios in MUSIM

A MUSIM UE having two SIM instances will camp independently on two bands (B_X and B_Y). Depending on whether the SIM instances correspond to the same or different operators and based on other aspects like Network coverage, deployment the bands can be same or different. In certain band combinations, it could be possible that B_X and B_Y are harmonics of each other.

For example, consider a MUSIM device operating in Dual-Rx/Single-Tx mode. In this case, if SIM instance 1 is camped on Band B1(2100 MHz) in RRC IDLE mode and SIM instance 2 is camped on Band B12 (700 MHz) in RRC IDLE mode, then simultaneous DL reception on the two receivers will result in RF band conflict, resulting in poor page reception, SIB decode and or serving and/or neighbour cell measurements.

As another example, if SIM instance 1 is in RRC CONNECTED mode on Band B1, and SIM instance 2 is in RRC IDLE mode on Band B2, SIM instance 1 transmission on B1 has direct interference with the DL reception on SIM instance 2.

The level of interference is increased, if the MUSIM device operates in Dual-Rx/Dual-Rx mode, as there would be simultaneous transmission from each SIM instance, along with simultaneous reception. The problem gets compounded if more than 2 number of SIM instances are configured in the UE.

In summary, both Dual-Rx/Single-Tx and Dual-Rx/Dual-Rx mode of operation, band conflicts scenarios are encountered that impairs the optimal functionality of MUSIM UE.

The following table lists out some of the scenarios in which typical MUSIM Band conflict is observed.

| RRC State of SIM Instance 1 / RRC State of SIM instance 2 | Scenario |
| --- | --- |
| RRC IDLE / RRC IDLE or RRC IDLE / RRC INACTIVE or RRC INACTIVE / RRC IDLE or RRC INACTIVE / RRC INACTIVE | - Upon Cell Selection, serving cell bands to two SIMs in their respective RRC states conflict with each other - Upon Cell Reselection of SIM instance 1 from a non-conflicting band to a conflicting band, based on the current camped band of SIM instance 2. - Same scenario with SIM instance 2 reselecting into a conflicting band with SIM instance 1 |
| RRC CONNECTED / RRC IDLE or RRC IDLE / RRC CONNECTED or RRC CONNECTED / RRC INACTIVE or RRC INACTIVE / RRC CONNECTED or RRC CONNECTED / RRC CONNECTED* (*only for Dual-Rx/Dual-Tx) | - Measurement on a given band for one SIM instance that conflicts with the serving cell frequency of the other SIM instance - The SIM Instance 1 in RRC CONNECTED mode whose UL transmission conflicts with the DL RRC IDLE/INACTIVE mode reception on the other SIM instance 2 - The SIM instance 1 in RRC CONNECTED mode gets redirected on connection release to a conflicting band with the other RRC IDLE/INACTIVE SIM instance 2 - The SIM instance 1 in RRC CONNECTED mode gets handed over to a new band which is conflicting with the other RRC IDLE/INACTIVE SIM instance 2 - The SIM instance 1 in RRC CONNECTED mode is configured with a new component carrier / DC or MR-DC leg that is on a new band which is conflicting with the other RRC IDLE/INACTIVE SIM instance 2 - The SIM instance 2 in RRC IDLE/INACTIVE reselects to a new band which is conflicting with the other RRC CONNECTED SIM instance 2 - For the case of Dual-Rx/Dual-Tx, the UL and/or DL of one SIM instance in RRC CONNECTED mode conflicting with the UL and/or DL of the other SIM instance in RRC CONNECTED mode Note : Scenarios 3 – 9 are applicable with SIM instance 1 and SIM instance 2 swapped as well. |

Observation 1 : Based on UL and DL bands in which the MUSIM UE operates in RRC IDLE/INACTIVE/CONNECTED modes, there are scenarios in which both Dual-Rx/Single-Tx and Dual-Rx/Dual-Rx mode of operation are impaired due to RF band conflict.

Upon detecting such Band conflicts there are ways by which this can be mitigated

- Option 1 : Autonomous mitigation methods wherein the MUSIM UE autonomously without any NW support, tries to steer one of the SIM instances to a non-conflicting band (if available)

- Option 2 : Switch from Dual-Rx/Dual-Tx or Dual-Rx/Single-Tx to Single-Rx/Single-Tx mode of operation.

- Option 3 : UE reports to the NW, a band conflict scenario and the NW helps the UE to mitigate the band conflict situation through signalling.

Option 1 will result in non-standard UE behaviour, while Option 2 would result in sub-optimal UE performance as the UE has to downgrade to Single-Rx/Single-Tx. Hence there is a need for Option 3, with a graceful specification defined signalling based solution intended to mitigate such band conflicts.

Observation 2 : Autonomous MUSIM UE based solution to mitigate band conflict would result in sub-optimal and non-standard behaviour.

Proposal 1 : RAN2 to consider such Band conflict scenarios for MUSIM to arrive at a graceful specification based solution intended to mitigate such conflicts.

## MUSIM RRC Processing Delay Requirements

Current LTE and NR RRC specifications [1][2], provide an upper limit on the RRC Processing Delay as a means to specify UE performance requirements for RRC procedures. These processing delay requirements are intended for the UE doing only one RRC procedure at a time.

Observation 3 : Current LTE and NR RRC Processing Delay Requirements are meant for single SIM cases, wherein only one RRC procedure is expected to run at any given instance of time.

In a MUSIM UE scenario, there can be multiple UE RRC and each RRC can independently be in RRC CONNECTED or RRC IDLE or RRC INACTIVE modes, depending on the current use case. In such cases, it is possible that a given RRC procedure on a given SIM instance can be pre-empted due to a higher priority RRC procedure on the other SIM instance (e.g.) Network Switching for instance.

In these circumstances it is not clear whether the MUSIM UE should consider that,

- The current RRC Processing Delay Requirement mentioned in [1] [2] is applicable for MUSIM cases as well or

- The RRC Processing Delay Requirement for MUSIM UEs to be adapted accordingly based on the number of concurrent SIM instances, and if so by how much.

Observation 4 : MUSIM UEs can have an ongoing RRC procedure pre-empted due to a concurrent MUSIM use case, and this might result in very tight RRC processing deadline.

Proposal 2 : Clarify in LTE and RRC specifications for Release-17 that the existing RRC Processing Delay requirements is applicable only for UE operating in Single-SIM mode and is NOT applicable for RRC procedures for UE’s operating in MUSIM mode of operation.

Proposal 3 : RAN2 to further study the RRC Processing Delay Requirements for MUSIM UEs based on the solutions agreed for the other MUSIM WI objective (Paging Collision, Network Switching, Busy Indication etc.)

# Conclusion

Observation 1 : Based on UL and DL bands in which the MUSIM UE operates in RRC IDLE/INACTIVE/CONNECTED modes, there are scenarios in which both Dual-Rx/Single-Tx and Dual-Rx/Dual-Rx mode of operation are impaired due to RF band conflict.

Observation 2 : Autonomous MUSIM UE based solution to mitigate band conflict would result in sub-optimal and non-standard behaviour.

Proposal 1 : RAN2 to consider such Band conflict scenarios for MUSIM to arrive at a graceful specification based solution intended to mitigate such conflicts.

Observation 3 : Current LTE and NR RRC Processing Delay Requirements are meant for single SIM cases, wherein only one RRC procedure is expected to run at any given instance of time.

Observation 4 : MUSIM UEs can have an ongoing RRC procedure pre-empted due to a concurrent MUSIM use case, and this might result in very tight RRC processing deadline.

Proposal 2 : Clarify in LTE and RRC specifications for Release-17 that the existing RRC Processing Delay requirements is applicable only for UE operating in Single-SIM mode and is NOT applicable for RRC procedures for UE’s operating in MUSIM mode of operation.

Proposal 3 : RAN2 to further study the RRC Processing Delay Requirements for MUSIM UEs based on the solutions agreed for the other MUSIM WI objective (Paging Collision, Network Switching, Busy Indication etc.)

# Reference

[1] 3GPP TS 36.331 V16.3.0 (2021-01) – EUTRA : RRC Protocol Specification Release 16

[2] 3GPP TS 38.331 V16.3.1 (2021-01) – NR : RRC Protocol Specification Release 16