3GPP TSG RAN WG2 #113bis-e			R2-2103839

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.10.2.2

Source:	Apple

Title:	Considerations for RA Type and TA Timer MAC Enhancements in Non Terrestrial Networks

Document for:	Discussion/Decision

# Introduction

RAN2 made the agreement in RAN2#112 to further discuss the corresponding enhancement:

- At least the following are FFS in Rel-17 NTN: • Report UE-calculated TA in e.g. msg3/msg5/msgA • Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH • LCP impact caused by disabling HARQ UL retransmission

In RAN2 [POST113-e][106][NTN] MAC aspects (Huawei) [11], of the NTN WID [1], [2] for UE Idle mode behaviors is two important discussions related to RA Type selection and TA Timer reporting from UE have been discussed. In this contribution, we provide our views on this topic on why the shortlisted solution approaches will not completely work and will create further issues that will need to be re-discussed in RAN2.

# Discussion

## RA Type selection

In this context, three shortlisted options have been identified by the rapporteur and the options seem to hold considerable majority over different groups of companies. Based on the summary of [11], new criteria for RA Type selection could be based on -

| # | Option |
| --- | --- |
| Option 1 | Based on “UE specific UE-satellite RTT” or “distance between UE and satellite" |
| Option 2 | Based on delay QoS requirement of logic channel |
| Option 3 | Only use RSRP as in legacy |

With option 1, while “UE specific UE-satellite RTT” or “distance between UE and satellite" are good options, the UE being able to calculate the distance on its own to the satellite is dependent on the UE knowing the location of the satellite and the satellite velocity. While RAN1 is continuing to discuss what parameters will be available and if in fact the full ephemeris can itself be sent to the UE, RAN2 cannot just conclude this information on its own. On the contrary, if the network wants to identify the location of the satellite, it has not been yet agreed if and how the UE will send its location even when ignoring the quite obvious and blatant privacy constraints of the UE. Also, discussions related to this topic are currently happening in SA3. Given these constraints, a solution approach that is heavily dependent on the ephemeris and location topics’ progress in other groups is not the best or obvious approach.

Observation 1: Any location based or ephemeris based selection of RA Types to be postponed until after other 3GPP working groups can reach a conclusion.

With option 2, where the RA type selection is “based on delay QoS requirement of logic channel”, the only possibility here would be for scenarios requiring QoS and will defeat the overall purpose of 2-step RACH to provide faster access to UEs to this long propagation delay satellite network. Any solution here would just make it difficult to differentiate between different logical channels. Further most logical channels are not established during RACH and often done after the Best Effort bearer has been established making this mechanism severely limited in current deployments.

Observation 2: QoS based RA Type selection is limited in its usefulness in solving the high propagation delay issues in NTN communications.

A simple third alternative as in Option 3 is to leave the existing RSRP criteria as in legacy behavior. While this option might be suitable for LEO scenarios in lower orbit configurations, for high altitude LEOs and GEOs where the RSRP variation is negligible from the UE measurements perspective would make deployments difficult. This solution can would also be contrary to the WI objective of reducing the impact of propagation delay. Further, this option also depends on the UE knowing which criteria it can use in case of mixed LEO-GEO configurations which reverts the discussion back to the ephemeris information that will be available on the UE.

Observation 3: Legacy RSRP based RA Type selection mechanism are unsuitable for GEO and high altitude scenarios as discussed in 38.821[3].

Other alternatives though interesting but not shortlisted include variants of the UE-satellite RTT (in terms of elevation angle or distance both of which depend on ephemeris data at UE), applying 2-step RA procedure to near cell UEs (which will further delay the already delayed procedures for the worse affected UEs in far cell conditions) or leave them completely up to network implementation (which is not always in the UEs best interest especially in regard to QoS based services). From this perspective, we prefer a method that can still achieve the objective of the WI (which is reducing the impacts of the long propagation delays in satellite networks) while still being useful in all scenarios. We can easily achieve the goal by making 2-step RACH the default RA type without any unnecessary UE or network implementation impacts and revert to 4-step RACH in case of failure as in legacy scenarios. This would ensure that UEs will at least reduce the overall setup delay (by reducing at least 2 steps of the overall RACH procedure which is very useful in GEO and high altitude LEO configurations) and have a fall back mechanism in case the RA fails.

The main concern for this approach has been dimensioning the network to ensure enough 2 step resources are present. This is a familiar task for the networks and the task is left up to the networks and operators to decide this based on the traffic seen, cell loading and other typical criteria that they use. Given this analysis, we sincerely request RAN2 to consider the following proposal.

Proposal 1: For Non Terrestrial Networks, RAN2 to consider 2-step RACH as the baseline mechanism without unnecessary overhead to UE and network implementations. Upon failure, the 2-step RACH mechanism will revert to 4-step RACH procedure as in legacy methods.

With this proposal, RAN2 will not need to have additional discussions on other offshoot topics of “RA Type selection for handover scenarios” or if a particular “RA Type needs to be limited to one particular Logical Channel Type”. Additionally, other propagation delay enhancements such as those needed to indicate UE BSRs in initial access will also be taken care of without major specification impacts.

## Report UE Calculated TA

Similar to the proposals for RA Type selection, the post email discussion [11] introduces another set of proposal in terms of triggering TA report from the UE using the following three mechanisms:

| # | Option |
| --- | --- |
| Option 1 | TA report can be triggered when RACH is initiated, and whether TA report is included in MSG3/MSG5 or MSGA/next UL Grant following MSGB depends on existing LCP procedure |
| Option 2 | TA report can be triggered by some event or rule, e.g. If the difference between the current TA used by the UE and the TA value known to gNB (=the value last reported by the UE) exceeds a threshold; or by a threshold/hysteresis in the UE (the threshold can be wrt the last reported TA + common drift rate) |
| Option 3 | Whether UE reports UE-calculated TA to NW and in which message the report should be included should only be controlled by NW |

The biggest concern with Option 1 is that the network sends an accurate TA to the UE in MSG2 and based on the current 3GPP specifications, the UE just applies this new value making its calculated value from TA1 redundant at MSG2. With this existing specification, the request that UE needs to send in a historical value that might be inaccurate or at most coarsely accurate seems to be an unnecessary burden and overhead on the RACH procedure in a network configuration where the propagation delay is already large. Not only will RAN2 have to design a protocol to fit in the new TA Report into MSG3 or MSG5 for 4-step RACH and MSGB for 2-step RACH, additional scenarios on improving its reliability need to be created and evaluated thus consuming valuable 3GPP TU allocation time.

Similar concerns on why this value is useful from the UE exist even for options 2 and 3 as well.

The only potential use for the reporting UE calculated TA is to ensure its use for the KOffset parameter used by the network to improve scheduling efficiencies. This topic however, is RAN1 specific and is still being debated.

Observation 4: KOffset design is a RAN1 specific deliverable.

However, this discussion is not yet complete in RAN1 and thus RAN2 doesn’t have to take up this discussion until the RAN1 decisions are complete. In this regards, we have the following proposal.

Proposal 2: RAN2 to postpone the discussion of reporting UE calculated TA to the network until RAN1 discussion on KOffset is completed.

# Conclusion

In this contribution, we provided our views on the MAC enhancements for RA Type and Reporting of UE calculate TA. We hope RAN2 will consider the following observations and proposals in this regards.

Observation 1: Any location based or ephemeris based selection of RA Types to be postponed until after other 3GPP working groups can reach a conclusion.

Observation 2: QoS based RA Type selection is limited in its usefulness in solving the high propagation delay issues in NTN communications.

Observation 3: Legacy RSRP based RA Type selection mechanism are unsuitable for GEO and high altitude scenarios as discussed in 38.821[3].

Proposal 1: For Non Terrestrial Networks, RAN2 to consider 2-step RACH as the baseline mechanism without unnecessary overhead to UE and network implementations. Upon failure, the 2-step RACH mechanism will revert to 4-step RACH procedure as in legacy methods.

Observation 4: KOffset design is a RAN1 specific deliverable.

Proposal 2: RAN2 to postpone the discussion of reporting UE calculated TA to the network until RAN1 discussion on KOffset is completed.

# References

- RP-193234, Solutions for NR to support non-terrestrial networks (NTN), Sitges, Spain, Dec. 2019.

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Jun. 2020.

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.

- 3GPP TR 38.321, Medium Access Control (MAC) Protocol Specification, v. 16.1.0, Jul. 2020.

- RP-182894, New work item: 2-step RACH for NR, ZTE, RAN#82

- 3GPP TR 38.304, User Equipment (UE) procedures in idle mode and in RRC Inactive state, v. 16.2.0, Oct. 2020.

- 3GPP TR 38.133, Requirements for support of radio resource management, v. 16.5.0, Oct. 2020.

- 3GPP RAN2#111-e, Chairman’s Notes, Sept. 2020.

- 3GPP RAN2#112-e, Chairman’s Notes, Nov. 2020.

- 3GPP RAN3#110-e, Chairman’s Notes, Nov. 2020.

- 3GPP Email Discussions, [POST113-e][106][NTN] MAC aspects (Huawei), March 2021.