3GPP TSG-RAN WG2 Meeting #113bis-e                                 	                       R2-2103882

Electronic, April 12 - April 20, 2021

Agenda item:	8.8.3

Source:	Apple

Title:	Discussion on slice based RACH

Document for:	Discussion

# 1	Introduction

RAN slicing WID [1] is agreed in RAN#91 meeting, with one objective shown below.

- Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2]

- Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group

- Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group

- Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs).

This paper discusses the slice based RACH configuration related issues.

# 2   Discussion

Topic 1: Slice group definition

When talking about slice based RACH configuration, as mentioned in the WID, slice group should be enabled in order to reduce the RACH resource fragmentation when slice number is too many.

Then the question becomes as how the slice group is represented. One possible approach is to use the access category defined in [2]. Note that previously in both SA2 and RAN2, one common sense is the operator defined access categories can serve the purpose of slice based UAC configuration, which justifies using access categories to represent the slice group.

Another straightforward approach is to directly use a set of S-NSSAI to define the slice group.

Proposal 1: RAN2 to discuss whether to use S-NSSAI set or access categories to represent slice group.

Topic 2: Flexible slice specific RACH resource configuration

Present RACH resources comprise of separate resources for 2-step RACH and 4-step RACH. UE determines between two resources based on a configured RSRP threshold. When exploring the separated RACH configuration for slice group, we believe it’s desirable to enable sufficiently flexible configuration. In details, it should be possible to enable both or one of 2-step/4-step RACH resources for each slice group. And it should be possible to provide a common set of 2-step RACH and 4-step RACH resources for legacy UE(s) and slices which do not belong to any slice groups with dedicated RACH resources. Furthermore, we should also allow flexible RSRP threshold for each slice group.

Proposal 2: Enable flexible configurations to provide both or only one of 2-step/4-step RACH resources for each slice group.

Topic 3: RACH type selection and RACH fallback

The existing scheme of RACH type selection is shown as below. UE first performs carrier type selection between NUL and SUL, then performs RA type selection and beam selection for RO selection.

Before going to details, it’s helpful to clarify in the context of RAN slicing, whether to consider the complicated case that NUL and SUL support different slices. This may not be that controversial but we see the potential because SA2 has an objective to support the radio spectrum attribute for slice. In our opinion, SUL is only to complement the coverage in case NUL is on a higher frequency. We don’t see the necessities and benefits to support different slices on SUL and NUL.

Proposal 3: RAN2 to agree that the same slices should be supported on NUL and SUL in a cell.

If this understanding could be confirmed, the carrier type selection would follow the conventional way in RAN slicing context without checking the slice availability on each UL carrier. And the procedure would look like the figure below.

The only difference between conventional fallback to 4-step RACH would be the determination of 4-step RACH resource availability for the traffic initiating slice group.

Topic 4: Collision with MPS/MCS RACH configuration

From [2], MPS/MCS are two special access identities which are handling emergency related services. Thus, our thinking is in case collision happens between two configurations when MPS/MCS device are initiating some traffic for a certain slice, we can consider to always use the MPS/MCS config, or to use the more optimal one.

Proposal 4: To address the collision with MPS/MCS configuration, suggest to always use the MPS/MCS config or to use the more optimal one.

# 3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: RAN2 to discuss whether to use S-NSSAI set or access categories to represent slice group.

Proposal 2: Enable flexible configurations to provide both or only one of 2-step/4-step RACH resources for each slice group.

Proposal 3: RAN2 to agree that the same slices should be supported on NUL and SUL in a cell.

Proposal 4: To address the collision with MPS/MCS configuration, suggest to always use the MPS/MCS config or to use the more optimal one.

# 4	References

- RP-210872  New WID on enhancement of RAN Slicing for NR (CMCC)

- TS 23.501 System architecture for the 5G System (5GS)