3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2103890

Online, Apr 12th – Apr 20th, 2021

Agenda item:	8.2.2.1

Source:	Qualcomm Incorporated

Title:	Deactivation of SCG

WID/SID:	LTE_NR_DC_enh2-Core – Release 17

Document for:	Discussion and Decision

# Introduction

Agreements:

9 NW-triggered SCG deactivation can be indicated to the UE via the MCG. FFS via SCG.

11 It is FFS whether the UE can provide some assistance information for deactivation of the SCG (but there is no proposal so far).

1b SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

3 RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

On SCG deactivation signalling discussions took place in the previous RAN2 #113-e meeting, both in the online meeting [1] and in the email discussion [2]. Good progress was made and some basic agreements were reached. It was agreed that MN or SN may request SCG deactivation and other details regarding the procedure were left FFS. The following are the agreements reached in the previous RAN2 #113-e meeting [1].

In this contribution paper, we discuss some of the items left as FFS from the previous meetings and some of the details of the SCG deactivation procedure.

# MN and SN requested SCG deactivation

MN requested SCG deactivation

MN may send a request for deactivation of the SCG to the SN if, e.g., MN determines for MN terminated bearers that use SCG resources that there is no data to transmit over the SCG. SN may reject the deactivation request if it determines that for SN terminated bearers that use SCG resources, there is data to transmit over the SCG (MN wouldn’t know the traffic throughput profile or data buffer status in the SN for such bearers). It is better to request RAN3 to confirm and decide on this issue.

Proposal 1. Whether SN can reject an MN requested SCG deactivation should be left up to RAN3 to decide.

SN requested SCG deactivation

SN may send a request for SCG deactivation to the MN if, e.g., SN determines that there is no SCG data activity or if SN determines for SN terminated bearers that use SCG resources there is no data to transmit over the SCG.

In normal circumstances, MN should not be rejecting the request since SN has full knowledge about data activity in the SCG. MN may reject the request however if it determines that there is data to be transmitted over MN terminated bearers that use SCG resources. We think it is better to request RAN3 to decide on this issue also.

Proposal 2. Whether MN can reject an SN requested SCG deactivation should be left up to RAN3 to decide.

An issue that was left FFS in the previous meeting is whether network triggered SCG deactivation can be indicated to the UE via the SCG.

It was agreed in RAN2 #113-e [1] that RRC signalling can be used for SCG deactivation signalling to UE. If the RRC message carrying SCG deactivation is transmitted via SCG then SRB3 will be used. Since coordination with MN is required for SCG deactivation, sending RRC message (this will likely be RRC reconfiguration) carrying SCG deactivation over SRB3 violates R15 principles. Also, not all implementations support SRB3.

Observation 1. Since coordination with MN is required for SCG deactivation, sending RRC message (this will likely be RRC reconfiguration) carrying SCG deactivation over SRB3 violates Rel-15 principles.

Even if it is agreed that lower layer signalling can be used for SCG deactivation, we do not see a good reason or scenario where it should be transmitted over SCG.

For all the above reasons, we think SCG deactivation signalling should be sent over the MCG only.

Proposal 3. Network triggered SCG deactivation should not be indicated to the UE via the SCG.

# UE requested SCG deactivation

UE may request the network for SCG deactivation, e.g., if there is no traffic or no data pending transmission in UE buffer for SCG RLC bearers. UE may also request SCG deactivation in case of overheating.

Proposal 4. UE may request the network for SCG deactivation.

We think RRC UEAssistanceInformation message can be used for UE requested SCG deactivation. A simple indication can be included in the message for this purpose. The message can be sent to the MN over SRB1.

We prefer that the UE be allowed to just send a request to the network for SCG deactivation without providing additional specific information. It can be complex to specify the information and difficult for companies to arrive at a consensus on what information to include.

Proposal 5. RRC UEAssistanceInformation message can be used for SCG deactivation request by UE, including a simple SCG deactivation indication. The message can be sent to the MN over SRB1.

# 4 Conclusion

Based on the above discussions, we recommend that RAN2 discuss the following observations and proposals.

MN and SN requested SCG deactivation

Proposal 1. Whether SN can reject an MN requested SCG deactivation should be left up to RAN3 to decide.

Proposal 2. Whether MN can reject an SN requested SCG deactivation should be left up to RAN3 to decide.

Observation 1. Since coordination with MN is required for SCG deactivation, sending RRC message (this will likely be RRC reconfiguration) carrying SCG deactivation over SRB3 violates Rel-15 principles.

Proposal 3. Network triggered SCG deactivation should not be indicated to the UE via the SCG.

UE requested SCG deactivation

Proposal 4. UE may request the network for SCG deactivation.

Proposal 5. RRC UEAssistanceInformation message can be used for SCG deactivation request by UE, including a simple SCG deactivation indication. The message can be sent to the MN over SRB1.

# References

[1] RAN2 Chair notes, RAN2 #113-e meeting.

[2] R2-2101969	Email discussion, [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation.