3GPP TSG-RAN WG2 Meeting #113-bis	R2-2103896

Electronic Meeting, April 12 – April 20, 2021

Agenda item:		8.5.4

Source:		Futurewei

Title:	Discussion on entering and exiting survival time state

Document for:		Discussion and Decision

# Introduction

This contribution discusses various issues regarding entering and exiting survival time state.

# Discussions

## Applicability of survival time state to gNB and UE

It is our view that, for both DL and UL periodic traffics, only the gNB may enter a survival time state as an operational state (but not an RRC state), in which the gNB may take actions to boost the reliability of the DL or UL transmission through dynamic change of scheduling parameters and/or dynamic activation of reliability enhancement features (such as PDCP duplication), while the UE merely follows these dynamic signaling without a need to be in a survival time state in its operation.

Proposal 1. For both DL and UL periodic traffics, only the gNB may enter a survival time state as an operational state, in which the gNB may take actions to boost the reliability of the DL or UL transmission through dynamic change of scheduling parameters and/or dynamic activation of reliability enhancement features. The UE merely follows these dynamic signaling without a need to be in a survival time state in its operation.

## Triggers for gNB to enter survival time state

During offline e-mail discussion [506] at RAN2 meeting #113 [1], 11 options of triggers to enter survival time state were proposed.

It is our view that the triggers for the gNB to enter a survival time state (for DL or UL) should be timely, with the reliability of the trigger being a secondary concern, meaning that some false alarms may be tolerable.

For example, for DL traffics, a PDB timer may be started by the gNB when the gNB receives a message from the CN for a transmission to the UE. The PDB timer will be stopped and reset if the message is transmitted successfully. When the PDB timer expires without the message being successfully transmitted, the gNB enters the survival time state for the DL. When the message transfer interval and the survival time are short (e.g., less than 10 msec), the gNB may use HARQ ACK/NACK feedback to determine whether the message has been transmitted successfully. On the other hand, when the message transfer interval and the survival time are sufficiently long (e.g., on the order of 10s msec for certain applications [2]), the gNB may solicit RLC Status feedback from the UE in order to determine whether the message has been transmitted successfully. The RLC Status feedback would be more reliable than the HARQ ACK/NACK feedback but is feasible only when the message transfer interval and the survival time are sufficiently long. Given the TSCAI (more specifically, the periodicity and survival time) information would be available at the gNB, the gNB can choose between these two alternative feedbacks based on implementation.

For UL periodic traffics, the gNB knows the 5G-AN PDB and when to expect a next message based on the periodicity information in the TSCAI. Hence, the gNB can use a second PDB timer to determine when to enter the survival time state for the UL. For example, the gNB may start a second PDB timer when a next message is expected to be received, and when the second PDB timer expires without the expected message being received, the gNB enters the survival time state for the UL.

Proposal 2: The triggers for the gNB to enter the survival time state (for DL or UL) should be timely, with the reliability of the trigger being a secondary concern.

## Triggers for gNB to exit survival time state

It is our view that the triggers for the gNB to exit the survival time state (for DL or UL) should be reliable, with the timeliness being a secondary concern, meaning that some delay may be tolerable in determining the exiting condition being met.

After the gNB enters a survival time state (for DL or UL) and before the survival time expires, as soon as the gNB determines that the message has been transmitted or received successfully, the gNB exits the survival time state and return to a normal state.

For DL traffics, because of the nonnegligible error rate on the HARQ ACK/NACK feedback, using a HARQ ACK received from the UE to determine that the message has been transmitted successfully may present a risk for the gNB to prematurely exit the survival time state for the DL, and more importantly, prematurely remove reliability boosting for DL transmission, which may result in a violation of the survival time requirement. Therefore, the gNB may solicit an RLC Status feedback from the UE in order to determine if the message has been transmitted successfully before the survival time expires. The latency associated with soliciting the RLC Status feedback may cause the gNB to stay in the survival time state longer than necessary. However, that is not as critical as otherwise potentially violating the survival time requirement.

For UL periodic traffics, the gNB is the recipient of the message. Hence, there is no reliability issue in determining whether the message has been received successfully by the gNB or not.

Proposal 3: The triggers for the gNB to exit the survival time state (for DL or UL) should be reliable, with the timeliness being a secondary concern.

# Conclusions

The followings are proposed:

Proposal 1. For both DL and UL periodic traffics, only the gNB may enter a survival time state as an operational state, in which the gNB may take actions to boost the reliability of the DL or UL transmission through dynamic change of scheduling parameters and/or dynamic activation of reliability enhancement features. The UE merely follows these dynamic signaling without a need to be in a survival time state in its operation.

Proposal 2: The triggers for the gNB to enter the survival time state (for DL or UL) should be timely, with the reliability of the trigger being a secondary concern.

Proposal 3: The triggers for the gNB to exit the survival time state (for DL or UL) should be reliable, with the timeliness being a secondary concern.

# References

R2-2102074 Offline on RAN enhancements QoS [AT113-e][506].

TS 22.104 v17.4.0.