3GPP TSG-RAN WG2 #113bis-e	R2-2103904

E-meeting, 12th – 20th April, 2021

Agenda item:	8.6.3

Source:	Huawei, HiSilicon

Title:	Control plane common aspects for SDT

Document for:	Discussion

# Introduction

In this paper, we further discuss the control plane common aspects for small data transmission (SDT) between random access based schemes and CG based scheme.

# Discussion

## 2.1 SDT triggering

Provided that SDT resources are configured and available upon uplink data arrival at the UE side, one general issue is that the UE has to decide whether to initiate SDT transmission procedure or the legacy RRC Connection Resume procedure (non-SDT transmission), i.e., transit to RRC_CONNECTED state before performing data transmission. RAN2 agreed the SDT is configured per DRB. We think there are two possible cases for data arrival that should be considered for deciding whether to initiate SDT procedure or legacy resume procedure.

- Case1: only SDT data arrives;

- Case2: both SDT data and non-SDT data are arrived simultaneously.

For the first case, we have already agreed that the UE shall determine whether to initiate SDT procedure based on the data volume threshold or other criteria (e.g. RSRP threshold). For the second case, we think that the UE shall initiate the legacy RRC resume procedure to request the network to resume the RRC connection. Afterwards, the network sends the RRCResume message to move the UE to RRC_CONNECTED state where the UE transmits both SDT and non-SDT data.

Proposal 1: If uplink data arrives at the UE for DRBs not configured for SDT, the UE shall initiate the legacy RRC resume procedure (i.e. the UE does not trigger SDT transmission even if SDT data is also available).

In POST RAN2#113 email discussion, the issue whether the data volume threshold is common or separate for CG-SDT and RA-SDT is covered. Based on the replies, most companies would like to have a common threshold. Before deciding this, we think it is necessary and meaningful to first discuss the SDT use cases, in particular whether or not the CG-SDT and RA-SDT could target for different services for a UE, since it is more reasonable to have separate data volume thresholds if they are indicated to target for different services. From our point of view, both RA-SDT and CG-SDT were agreed to be included in the WID, exactly due to the fact they are targeting different services. For example, CG-SDT is more suitable for ensuring services with lower latency and higher reliability requirement due to the fact that the CG-SDT resource can be dedicated to a specific UE and can, e.g. support repetitions (note RAN1 is considering the repetition for association between CG resource and SSB) while the RA resource has to be shared and does not allow for repetitions.

Observation 1: The reason to specify both CG-SDT and RA-SDT is that they are targeting different kinds of services.

Based on this observation, we think it does not really make sense to require CG-SDT and RA-SDT to use different data volume thresholds. Considering that the additional specifications complexity for supporting separate data volume thresholds would be very low, we propose the following:

Proposal 2: It should be possible to configure different SDT data volume threshold for CG-SDT and for RA-SDT.

Another aspect covered in POST RAN2#113 email discussion is the issue on applicability of RSRP-based UL carrier selection for CG-SDT and based on the replies, it seems companies would like to introduce such mechanism for CG-SDT. At the same time, the argument used by many companies is that this is how SUL/UL selection is done “as per legacy”. However, in existing specifications, RSRP-based SUL/UL selections is made at MAC layer and only for RA procedure:

## 5.1 Random Access procedure Xxx 1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the signalled carrier. 1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

There is currently no uplink selection for Configured Grants, i.e. according to current specifications, in case CG is configured on both carriers, the UEs uses both of them and the network needs to ensure they do not overlap in time, as per section 10.3 of TS 38.300:

When SUL is configured, the network should ensure that an active configured uplink grant on SUL does not overlap in time with another active configured uplink grant on the other UL configuration.

This was agreed in Rel-16 and was confirmed during the last RAN2 meeting by approving the CR to TS 38.331 in [2]. For SDT, RAN2 also assumed that the behavior should be aligned with that in the previous release, which was expressed by the following agreement:

CG-PUSCH resources can be separately configured for NUL and SUL.  FFS if we allow them at the same time.  This depends on the alignments CRs for Rel-16.

Based on the agreement, the same should apply to CG-SDT, so there is no need for UL carrier selection for CG-SDT. CG-SDT will anyway be checked against minimum SS-RSRP and RSRP range as agreed in RAN2#113 meeting, so additional RSRP for SUL/NUL selection would be redundant. Therefore, we propose the following:

Proposal 3: There is no need for uplink carrier selection for CG-SDT, i.e. the UE can use CG-SDT configurations from both SUL and UL simultaneously, if configured and valid (according to CG-SDT validity criteria).

Based on the above, RSRP threshold in SDT should only be applicable to RA-SDT. In this case, the current uplink carrier selection procedure at MAC layer (as quoted above) can be reused.

Proposal 4: For RA-SDT, an existing uplink carrier selection threshold/procedure can be reused.

## 2.2 Fallback to non-SDT transmission

When SDT is triggered based on the SDT selection criteria, a PDU including both uplink RRC message and user data will be constructed and transmitted by a UE. However, there may be cases when the network will not be able to decode such a big PDU successfully due to adverse wireless environment conditions. The network should be able to send an indication to fallback to non-SDT transmission if it finds SDT is not suitable for the UE, similar to the fallback in PUR and EDT in LTE. This was in theory discussed in the “[POST113-e][501][SDT] Selection criteria and overall Procedure” discussion, but it should be noted companies interpreted the question differently there, which affected the replies. In our understanding the fallback is mainly about the handling of initial SDT transmission failures, in particular in case the network was not able to decode the initial SDT message. In such situation, the network is not able to reply with the RRCResume message. Of course, we agree that the network can send RRCResume message at any point during the SDT process to terminate the SDT procedure, but this is should not be understood as a fallback to the legacy non-SDT RA procedure. Therefore, we think the fallback is needed in case the network is able to decode at least preamble (for RA-SDT) or DMRS (for CG-SDT).

After receiving fallback indication from the network, the UE should perform legacy RRC Connection Resume procedure. The fallback indication may be different for RA-based scheme and CG-based scheme:

- For 2-step RA based SDT, the fallback can be realized with fallbackRAR indicating the UE to switch to non-SDT 4-step RA

- For 4-step RA based SDT, the fallback can be realized by gNB providing a smaller grant in RAR and optionally indicating that the UE should only include RRC message (no user data) in msg3

- For CG-based SDT, we propose to include the indication in DCI.

Proposal 5: Fallback to non-SDT RRC Connection Resume procedure can be explicitly indicated by network.

- For 2-step RA based SDT, the fallback is indicated by fallbackRAR. UE falls back to non-SDT 4-step RA

- For 4-step RA based SDT, the fallback can be realized by gNB providing a smaller grant in RAR and optionally indicating that the UE should only include RRC message (no user data) in msg3

- For CG-based SDT, fallback indication is included in DCI.

## 2.3 RRC message for RRC-based SDT

In previous meetings, we agreed the RRC-based SDT is used as a baseline. In this section, we are going to discuss the uplink RRC message and downlink RRC message for SDT.

### 2.3.1 Uplink RRC message for SDT

When initiating an SDT transmission, the UE will include RRC message in the initial PUSCH transmission. The main purpose of the RRC message is to allow the network to identify and authenticate the UE in RRC_INACTIVE state. This is achieved by including I-RNTI and resumeMAC-I in the RRCResumeRequest or RRCResumeRequest1 message. These two fields are also required in the initial RRC message sent by the UE for SDT.

Observation 2: I-RNTI and resumeMAC-I have to be included in the initial RRC message sent by the UE for SDT for the network to identify and authenticate the UE.

The third element contained in RRCResumeRequest and RRCResumeRequest1 messages is ResumeCause. In LTE UP-EDT, uplink user data is multiplexed with RRCConnectionResumeRequest message containing the resume identity, shortResumeMAC-I and resumecause, and the following cause values are supported by taking all the resume procedures into account.

ResumeCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1

}

However, for LTE CP-EDT where the uplink user data are piggybacked into the RRCEarlyDataRequest messge, the number of establishment causes was limited to two for the purpose of CP-EDT only, which is different from that in the RRCConnectionResume.

establishmentCause-r15           ENUMERATED {   mo-Data-r15, delayTolerantAccess-r15},

When it comes to NR, the number of resume cases has been extended to up to 11 values as shown below, which would always consume 4bits in each RRC resume procedure (due to additional spare values). Even though for the small data transmission procedure in Rel-17, it seems that only a few resume causes are necessary in the context of uplink data transmission only, e.g. mo-Data and highPriorityAccess, which is similar to CP-EDT causes, in practice we could save 2-3 bits of overhead compared with legacy resume causes in NR.

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

Observation 3: Not all resume causes are applicable to SDT, but the potential gains of limiting the number of causes in SDT UL CCCH message are very limited (2-3 bits).

In the previous meetings, it was agreed to further study introducing a new cause for SDT. We think the legacy resume cause can be reused and it seems there is no good motivation to introduce such cause.

Observation 4: There is no good motivation to introduce a new resume cause for SDT.

Based on the above observations, we note that the contents of the first UL RRC message sent by the UE would be the same as those of current RRCResumeRequest. Based on this, the following is proposed:

Proposal 6: RRCResumeRequest message is reused as the first UL RRC message sent by the UE for RRC-based SDT.

### 2.3.2 Downlink RRC message for SDT

After small data transmission in the uplink (and optionally in downlink), the following cases are possible for the network to terminate SDT procedure:

- If the network wants to send the UE to RRC_CONECTED and resume the connection, network shall send RRCResume.

- If the network fails to retrieve the UE context (e.g. due to failed security check or no response from the anchor gNB), RRCSetup is sent for falling back to RRC connection establishment.

- If the network wants to keep the UE in RRC_INACTIVE and terminate the transmission, RRCRelease with suspend configuration is sent.

- If the network wants to send the UE to IDLE and terminate the small data transmission, RRCRelease without suspend configuration is sent and the UE goes to RRC_IDLE.

- If the network experiences high traffic load and does not provide access to the UE, RRCReject can be sent and UE discards stored security keys and suspends SRB1.

We think all the above scenarios are possible for small data transmission procedure, i.e., after the uplink RRC message is transmitted, the network can respond to the UE with RRCResume or RRCSetup or RRCRelease with suspend configuration or RRCRelease without suspend configuration or RRCReject message as the downlink RRC response depending on the network situation.

Proposal 7: The downlink RRC message in response to uplink SDT RRC message can be RRCResume, RRCSetup, RRCRelease with suspendConfig, RRCRelease without suspendConfig or RRCReject message (i.e. the same as in the legacy procedures).

### 2.3.3 New Downlink RRC message for SDT

In the previous meetings we had agreed that the first UL RRC message, similar to RRCResumeRequest, will be used for initiating the SDT procedure and RRCRelease message will be used for terminating the SDT procedure. We believe that these two RRC messages will be enough for initiating and terminating a single shot SDT procedure where one UL data packet and optionally one DL data packet is exchanged between the UE and the network as shown in the example in Figure 1 for both 4 Step RACH and 2 Step RACH

Figure 1 Single Shot SDT procedure for 4 Step and 2 Step RACH

If we think of extending the basic single shot SDT procedure to a multi-shot SDT procedure we observe that any subsequent SDT data that needs to be exchanged between the UE and the network during a multi-shot SDT session must be exchanged between the first UL RRC message/RRCResumeRequest and RRCRelease message that terminates the SDT procedure.

Observation 5: Any subsequent SDT data that needs to be exchanged between the UE and the network during a multi-shot SDT session must be exchanged between the first UL RRC message/ RRCResumeRequest and RRCRelease message that terminates the SDT procedure.

For a multi-shot SDT procedure, we believe it will be beneficial for the network if the UE provides the traffic pattern information in terms of number of packets to be transmitted and the periodicity of the packets in MSG 3 or MSG A as this may be known at the UE. Based on traffic pattern information, the network may want to allocate CG resources for the subsequent data transmissions if the traffic pattern indicated by the UE is periodic in nature. For the network to be able to provide CG Configuration, a DL RRC message, other than RRCRelease message, will be needed which can signal such configurations during the SDT procedure but will not terminate the SDT procedure. The periodic traffic data will then be transmitted in multiple CG resource occasions.

Figure 2 shows multi shot SDT procedure using 4 Step RACH and 2 Step RACH with subsequent data being transmitted over multiple CG resource occasions which is configured using the new DL RRC SDT message

Figure 2 Multi Shot SDT procedure with CG Resource configured with DL RRC SDT message

In addition to providing CG resource configuration as depicted above in Figure 2, the new DL RRC message   can serve as a means of network verification from UE point of view as this will be received on DCCH before continuing with subsequent data transmission and it can also be used for providing other configurations such as PUCCH/SR configuration or RLM/BM configuration to the UE if these are considered necessary during SDT.

Proposal 8: A new downlink RRC message that does not terminate the SDT procedure, should be defined. This DL RRC message can be used for sending configuration parameters for CG resources for subsequent data transmissions or for providing PUCCH/SR configuration or RLM/BM configuration to the UE during the SDT procedure

## 2.4 Data loss avoidance during SDT

When it comes to SDT, the data loss at the UE can happen when the cell reselection occurs during small data transmission procedure.

In Rel-15 NR, the UE goes to RRC_IDLE when cell re-selection occurs during RRC resume procedure, which is similar to case 1 above. After going to RRC_IDLE, the UE discards Inactive AS context and releases the suspendConfig. This is shown in the following text from RRC and PDCP spec.

TS 38.331 #### 5.3.13.6 Cell re-selection or cell selection while T390, T319 or T302 is running (UE in RRC_INACTIVE) The UE shall: 1> if cell reselection occurs while T319 or T302 is running: 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'; ====OMITTED====== ### 5.3.11 UE actions upon going to RRC_IDLE The UE shall: ====OMITTED====== 1> discard the UE Inactive AS context, if any; - release the suspendConfig, if configured; ====OMITTED====== 1> release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs; TS 38.323 ### 5.1.3 PDCP entity release When upper layers request a PDCP entity release for a radio bearer for Uu or PC5 interface, the UE shall: - discard all stored PDCP SDUs and PDCP PDUs in the transmitting PDCP entity;

For small data, the duration of the RRC resume procedure may be much longer than in the previous releases, since the maximum duration is no longer limited by the current T319 (the length of timer T319 in specification ranges from 100ms to 2s) due to the subsequent uplink transmissions. In the e-mail discussion, some companies argued that cell reselection during SDT procedure is a corner case, not worth an optimization. However, considering that SDT procedure durations which were mentioned previously by the companies, can be as long as 10s, the probability of the cell reselection is rather high. Table 1 presents some estimation of the magnitude of the problem.

Table 1: Cell reselection probability during SDT procedure

| UE velocity [km/h] | SDT duration [s] | Distance covered by the UE during SDT procedure [m] | Cell reselection probability when the ISD ranges from 500 m to 100 m |
| --- | --- | --- | --- |
| 3 | 5 | 4.166666667 | 0.8~4.2% |
| 3 | 10 | 8.333333333 | 1.7~8.3% |
| 30 | 5 | 41.66666667 | 8~42% |
| 30 | 10 | 83.33333333 | 17~83% |
| 60 | 5 | 83.33333333 | 17~83% |
| 60 | 10 | 166.6666667 | 33~100% |
| 120 | 5 | 166.666667 | 33~100% |
| 120 | 10 | 333.3333333 | 67~100% |

For this simple exercise, we assume that the UEs are moving along the straight line. Considering that the typical cell radius in urban scenario will be 100-500 meters and the maximum duration of SDT is 10 seconds, according to the last column of Table 1, we can identify the possibility of cell reselection during SDT procedure. For example, if the velocity of the UE is 30km/h, the duration of the SDT is 10s, and the cell radius is 500 meters, then the probability of cell reselection is 17%. Further, if the speed of the UE is 60km/h, the duration of the SDT is 10s, and the probability of cell reselection occurring is up to 33% and 17% if SDT lasts 5 second. For some cases, e.g. UE velocity of 60 km/h and SDT procedure lasting 10 seconds, there is actually a high probability that the UE will perform more than one cell reselection during a single SDT procedure.

Of course, there are cases where cell reselection probability is low, e.g. if the speed of the UE is 3km/h, the SDT duration is 10s, and the cell radius is 500 m, the probability of cell reselection is 1.7%. However, we think SDT procedure applies to multiple application scenarios including smartphone applications in mobile scenarios where the UE is on a bus or on a train. Therefore, the SDT should be comprehensively considered to cover scenarios where the UE moves fast as well.

Hence, based on the above analysis, we believe the possibility of cell reselection during SDT cannot be seen as a corner case. Cell reselection will happen much more often than previously during the RRC resume procedure. Each time this happens there would be data loss due to discarding of all PDCP SDUs and PCDP PDUs according to current procedures as mentioned above. In order to achieve lossless transmission of user data, a different approach is needed.

Proposal 9: Lossless data transmission should be addressed when cell reselection occurs during small data transmission.

To solve the data loss problem described above, two main approaches were proposed in [1]:

- The UE stays in RRC_INACTIVE after a cell reselection and performs RRC Re-establishment procedure.

- The UE stays in RRC_INACTIVE after a cell reselection and performs RRC Resume (either SDT or non-SDT) in the new cell.

In both of these cases, the DRBs are suspended and, in consequence, the previously transmitted PDCP SDU can be still kept in the PDCP buffer. PDCP SDU retransmission can be triggered when PDCP entity resumes again and there is no data loss. The drawbacks of RRC Re-establishment procedure which were mentioned is that this procedure is targeted at the UEs in RRC Connected and it requires the UE to be provided with C-RNTI while it is unclear at the moment whether the UE will have C-RNTI in SDT. Since option 2 reuses RRC Resume procedure it is more appropriate for UEs in RRC INACTIVE state, but the potential issue which was raised is whether the same NCC can be reused in the new cell by the UE. It should be noted that there is already a number of cases where the NCC is reused (e.g. RRC Reject), so we do not believe this is an issue. However, it would if there are doubts, this could be verified with SA3 together with informing SA3 about other potential security impacts of SDT.

Proposal 10: To avoid data loss when cell reselection is performed by the UE during SDT procedure, the UE stays in RRC_INACTIVE after a cell reselection, suspends SDT DRBs and PDCP entities and performs SDT/RRC Resume procedure in the reselected cell. If needed, RAN2 may check with SA3 whether there is a security concern with reusing the same NCC in the new cell in this case.

## 2.5 UE assistance information for SDT

Based on the WID, i.e. “Transmission of small data in UL, subsequent transmission of small data in UL and DL and the state transition decisions should be under network control”, we need to consider the subsequent transmission for small data transmission. With respect to this objective, during RAN2#111-e the following was agreed:

9	UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

10	When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

There are two main cases where subsequent transmissions are needed:

- Case 1: There are multiple messages (including both UL message and DL message), each message is small but they arrive at different times. These messages are to be transmitted by different transmissions.

- Case 2: One message (DL or UL) is split into multiple transmissions due to its large size which cannot be accommodated by a single MAC PDU.

For both options, as mentioned above, both the UE and the network need to transmit the message(s) by multiple transmissions within one SDT procedure. The information about subsequent DL and UL transmissions is usually known at the UE’s application layer. From the network perspective, it would be useful to receive such information from the UE. Based on the assistance information, the network can determine when to send the RRCRelease message to terminate the SDT procedure, e.g. the network transmits the RRCRelease message immediately if it is aware there is no subsequent transmission expected, or whether the RRCRelease message should be multiplexed with the DL user data in the subsequent DL transmission. Additionally, it may be useful to help the anchor gNB to make the decision about whether or not to relocate the UE’s context during SDT procedure, as agreed by RAN3:

WA3: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of such information are pending to future discussion in RAN3 and/or RAN2 inputs.

In LTE, Release Assistance Indication (RAI) has been defined and is used to inform the network about expected transmissions. Based on the RAI, the network can determine when RRCRelease message should be sent. The AS RAI MAC CE in LTE has the following information.

Table 6.1.3.19-1: Values for AS RAI Codepoint/Index; Value. 00; No RAI information. 01; No subsequent DL and UL data transmission is expected. 10; A single subsequent DL transmission is expected. 11; Reserved.

We think the benefits for LTE hold also for NR and similar mechanism can be used for NR SDT as a baseline. However, there may be some differences. In LTE EDT, only one-shot transmission is supported while multiple-shot transmissions are to be supported in NR SDT. In that case more information should be included in NR RAI, because additional use cases of subsequent transmission need to be considered, e.g., whether subsequent uplink transmission is expected, whether there is no subsequent DL and UL data transmission, whether a single subsequent DL transmission is expected, or whether multiple subsequent DL transmissions are expected etc. From the network perspective, it would be beneficial to receive such information in the first UL message of SDT. Based on the NR RAI, the network can decide when to send the RRCRelease message to terminate the SDT procedure.

The RAI can be applicable to both case 1 and case 2 of subsequent transmission as mentioned above. For case 1, the UE is expected to be provided with information about subsequent transmissions from the application layer of the UE. For case 2, the data is already buffered in 3GPP layers, the 3GPP layers of UE can generate assistance information directly.

Proposal 11: The UE can send a MAC CE with assistance information to the network indicating the expected subsequent UL and DL transmissions, similar to the one used for RAI in LTE.

## 2.6 Non-SDT data arrival indication

In POST RAN2#113-e email discussion for CP issues, one issue was covered to discuss how the UE indicates to the network about the data arrival for non-SDT RBs during SDT procedure and three options were provided in the email discussion:

- Option 1: Another CCCH message is sent

- Option 2: A new MAC level indication is triggered

- Option 3: A DCCH message is triggered

One inconsistency in the options descriptions that has to be clarified is the NAS/AS interactions for triggering a new RRC Resume procedure. In the e-mail discussion, it is claimed by the rapporteur that this aspect has to be clarified solely for option 1. However, it should be noted that NAS specifications are rather clear that this issue depends only on whether the non-SDT DRBs are resumed or not, which is a separate issue, relating equally to all the options mentioned above. According to TS 24.501:

Upon: a) a trigger of a procedure which requires sending of a NAS message different from a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed"; or b) an uplink user data packet to be sent for a PDU session with suspended user-plane resources; the UE in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access shall request the lower layers to transition to RRC_CONNECTED state (see 3GPP TS 38.300 [27]).

Therefore, it can be seen that when non-SDT arrives, NAS will trigger another Resume procedure according to current specifications regardless of which option is selected to indicate the non-SDT data arrival.

Observation 6: According to current NAS specifications, NAS will trigger another Resume procedure according regardless of which option is selected to indicate the non-SDT data arrival.

Therefore, NAS and AS interaction issue should be considered for all three options discussed in the email discussion.

Nevertheless, based on the replies, it seems most companies would like to follow option 3. However, in the discussion which followed after the rapporteur provided the summary, one issue that has been raised was that CCCH based solution would anyways be required to cover a scenario where non-SDT data arrives before contention resolution took place for the in SDT-RACH procedure. This can be seen as the corner case, but on the other hand, the only advantage of having a new DCCH message instead of reusing an existing CCCH message and procedure, is that security issue of repeating the same shortMAC-I can be avoided. However, as we mentioned in the e-mail discussion, there are already cases where the UE repeats the same security material, e.g. after receiving RRC Reject and reattempting a connection in the same cell. Hence, whether there is a real security issue is unclear and cannot be decided. Before jumping to conclusions, we believe SA3 should first confirm whether the issue really exists.

Proposal 12: SA3 should be consulted to verify whether repeating the same security material in the second RRC Resume Request message is a security issue.

Regardless of SA3 feedback, it should be noted that the only difference in terms of security between using CCCH and DCCH for non-SDT data indication, is that for DCCH the UE uses newly generated KRRCint key while for calculating shortMAC-I for the CCCH message, the UE uses its stored KRRCint which is still its previous/old KRRCint.

Observation 7: The main difference between using DCCH and using CCCH message for non-SDT data arrival is that for DCCH a new KRRCint is used while, according to current specifications, for shortMAC-I in CCCH, the UE would use its old KRRCint.

Other than that, as form NAS point of view, another RRC Resume procedure will be requested, NAS will for example provide a new resume cause. Hence, a new DCCH would have to carry at least resumeCause anyway and would be similar to RRC resume request message. The security issue for using CCCH message can be easily addressed by using a new KRRCint for generating shortMAC-I. This would have much less specifications impacts than specifying a whole new message just for serving the same purpose as current RRCResumeRequest message.

Proposal 13: In case SA3 concludes repeating the same security material in the second RRC Resume Request message is a security issue, non-SDT indication can be performed by reusing RRCResumeRequest message where the shortMAC-I is calculated using a UE’s new KRRCint key (i.e. the one calculated when triggering SDT).

# 2.7 SRB handling

In the RAN#91e meeting, the following description was added to the updated SDT WID: “Specify configuring of SRB1 and SRB2 for small data transmission in RRC_INACTIVE state by reusing the framework for DRBs”.

In the current specifications, NAS messages are usually carried over SRB2 using ULInformationTransfer message. SRB1 can only be used for ULInformationTransfer message in case SRB2 is not yet established and also for sending “initial NAS message” within MSG5, i.e. RRCResumeComplete and RRCSetupComplete messages, e.g. for RRCResumeComplete this is captured as follows in TS 38.331:

TS 38.331 1> set the content of the of RRCResumeComplete message as follows: 2> if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers; 2> if upper layers provides a PLMN and UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast: 3> set the selectedPLMN-Identity from the npn-IdentityInfoList; [… …]

For downlink, NAS messages can be carried either in RRCReconfiguration message or via DLInformationTransfer message (for DLInformationTransfer, also SRB2 is used whenever it is established). It should be noted that neither of the messages normally carried over SRB1 and which can include NAS containers will be used during SDT procedure, so the only option to carry NAS messages during SDT is using ULInformationTransfer/DLInformationTransfer message.

Observation 8: During SDT procedure, it is only possible to carry NAS messages using ULInformationTransfer/DLInformationTransfer message.

At the same time, it is not possible to use SRB1 for UL/DL Information Transfer in case SRB2 is established and SRB2 is always established when the UE is in RRC Inactive state.

Proposal 14: During SDT procedure, NAS messages can only be carried over DL/UL Information Transfer messages over SRB2.

The UE in the RRC_INACTIVE state resumes only the DRBs that are configured for SDT when initiating the SDT procedure. We think that for NAS message transfer on SRB2, we should follow the same behaviour as for the SDT DRBs, that is, whether SRB2 is resumed for SDT is up to network configuration. If SRB2 is configured for SDT, the UE resumes the SRB2 upon initiating the SDT procedure and NAS messages can be carried using SDT procedure. Otherwise, the UE triggers normal Resume procedure in case it has NAS messages to transfer.

Proposal 15: Whether SRB2 is resumed for SDT is up to network configuration (similar as for DRBs). If SRB2 is configured for SDT, the same behaviour is applied to SRB2 as agreed for DRBs configured for SDT.

# Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Observation 1: The reason to specify both CG-SDT and RA-SDT is that they are targeting different kinds of services.

Observation 2: I-RNTI and resumeMAC-I have to be included in the initial RRC message sent by the UE for SDT for the network to identify and authenticate the UE.

Observation 3: Not all resume causes are applicable to SDT, but the potential gains of limiting the number of causes in SDT UL CCCH message are very limited (2-3 bits).

Observation 4: There is no good motivation to introduce a new resume cause for SDT.

Observation 5: Any subsequent SDT data that needs to be exchanged between the UE and the network during a multi-shot SDT session must be exchanged between the first UL RRC message/ RRCResumeRequest and RRCRelease message that terminates the SDT procedure.

Observation 6: According to current NAS specifications, NAS will trigger another Resume procedure according regardless of which option is selected to indicate the non-SDT data arrival.

Observation 7: The main difference between using DCCH and using CCCH message for non-SDT data arrival is that for DCCH a new KRRCint is used while, according to current specifications, for shortMAC-I in CCCH, the UE would use its old KRRCint.

Observation 8: During SDT procedure, it is only possible to carry NAS messages using ULInformationTransfer/DLInformationTransfer message.

Proposal 1: If uplink data arrives at the UE for DRBs not configured for SDT, the UE shall initiate the legacy RRC resume procedure (i.e. the UE does not trigger SDT transmission even if SDT data is also available).

Proposal 2: It should be possible to configure different SDT data volume threshold for CG-SDT and for RA-SDT.

Proposal 3: There is no need for uplink carrier selection for CG-SDT, i.e. the UE can use CG-SDT configurations from both SUL and UL simultaneously, if configured and valid (according to CG-SDT validity criteria).

Proposal 4: For RA-SDT, an existing uplink carrier selection threshold/procedure can be reused.

Proposal 5: Fallback to non-SDT RRC Connection Resume procedure can be explicitly indicated by network.

- For 2-step RA based SDT, the fallback is indicated by fallbackRAR. UE falls back to non-SDT 4-step RA

- For 4-step RA based SDT, the fallback can be realized by gNB providing a smaller grant in RAR and optionally indicating that the UE should only include RRC message (no user data) in msg3

- For CG-based SDT, fallback indication is included in DCI.

Proposal 6: RRCResumeRequest message is reused as the first UL RRC message sent by the UE for RRC-based SDT.

Proposal 7: The downlink RRC message in response to uplink SDT RRC message can be RRCResume, RRCSetup, RRCRelease with suspendConfig, RRCRelease without suspendConfig or RRCReject message (i.e. the same as in the legacy procedures).

Proposal 8: A new downlink RRC message that does not terminate the SDT procedure, should be defined. This DL RRC message can be used for sending configuration parameters for CG resources for subsequent data transmissions or for providing PUCCH/SR configuration or RLM/BM configuration to the UE during the SDT procedure

Proposal 9: Lossless data transmission should be addressed when cell reselection occurs during small data transmission.

Proposal 10: To avoid data loss when cell reselection is performed by the UE during SDT procedure, the UE stays in RRC_INACTIVE after a cell reselection, suspends SDT DRBs and PDCP entities and performs SDT/RRC Resume procedure in the reselected cell. If needed, RAN2 may check with SA3 whether there is a security concern with reusing the same NCC in the new cell in this case.

Proposal 11: The UE can send a MAC CE with assistance information to the network indicating the expected subsequent UL and DL transmissions, similar to the one used for RAI in LTE.

Proposal 12: SA3 should be consulted to verify whether repeating the same security material in the second RRC Resume Request message is a security issue.

Proposal 13: In case SA3 concludes repeating the same security material in the second RRC Resume Request message is a security issue, non-SDT indication can be performed by reusing RRCResumeRequest message where the shortMAC-I is calculated using a UE’s new KRRCint key (i.e. the one calculated when triggering SDT).

Proposal 14: During SDT procedure, NAS messages can only be carried over DL/UL Information Transfer messages over SRB2.

Proposal 15: Whether SRB2 is resumed for SDT is up to network configuration (similar as for DRBs). If SRB2 is configured for SDT, the same behaviour is applied to SRB2 as agreed for DRBs configured for SDT.

# Reference

- 3GPP RAN WG2 Meeting #113bis-e, [Post113-e][503][SDT] T319 cell reselection and re-establishment (InterDigital)

- R2-2101340 for Correction on the configuration of Type 1 configured grant, Huawei, HiSilicon