3GPP TSG-RAN WG2#113bis-e Meeting	Tdoc R2-2103930

# Online, 12th- 20th April 2021

Agenda Item:		8.13.3.2

Souce:	Samsung

Title:	R17 Logged MDT issues (on overwrite, IRAT/ MR-DC, logging non camping freqs, IDC and OSI)

Document for:		Discussion and decision

# Introduction

RAN2 made some agreements regarding logged MDT enhancements for R17 during R2#113 and continued discussion by post 113 e-mail [854]. This contribution discusses the remaining issues, covering:

- UE based mechanism to avoid overwrite of sig-LogMDT

- IRAT continuation/ handling in EN-DC

- Capturing agreements on logging for EMR/ non Cell-Reselection frequencies

- IDC

- On demand SI

In brief, the paper includes following proposals:

- Baseline UE assistance mechanism to avoid overwrite of sig-LogMDT comprising

  - Single flag, indicated upon transition to connected, transferred within network and configured by dedicated signalling

- Neither enhancements for continuation upon IRAT mobility nor SN configuration of SCG frequencies not used for camping

- Mechanism for logging of SCG frequencies not used for camping

  - Discuss if there is need to avoid that a legacy node receives/ processes results for non camping frequencies

    - If so, introduce a flag in broadcast, controlling setting of availability and which results to include in UE information response (and no further changes)

  - Discuss whether to also introduce field to specify IRAT target frequencies

- IDC

  - Upon detecting IDC, continue event based logging. If triggered, add a field if UE experienced IDC

  - Upon detecting IDC, suspend periodic logging and add entry indicating UE experienced IDC (as in LTE)

- On-Demand SI

  - Extend RA report to cover failure of RA for OSI request and clarify use for posSIB

  - Extend report to include: SIBs acquired by UE, time elapsed between OSI request and success/ failure, flag indicating use of NUL or SUL

# Discussion

## A: UE based mechanism to avoid overwrite of sig-LogMDT

Continueing from [854]

As part of [Post113-e][854][NR17 SON/MDT] Logged MDT (CMCC) the proposal is to adopt a mechanism in which UE provides assistance by which network can avoid overwriting of an MDT configuration. Unfortunately there was no conclusion whether there is a need to avoid logged MDT configuration in the following cases:

1)	Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Logged MDT configuration is released, but UE still has un-retreived results that would be discarded upon accepting a new configuration

During the e-mail discussion companies did not really address the two cases separately. It seems that most companies focused on 2nd case. For this case, there was some support but some companies questioned the need. Some companies suggested that network can retrieve results. There however seem to be quite common cases in which this this is not possible e.g. cases in which UE, following T330 expiry, did not connect to the RAT that configured sig-LogMDT. We thus think it’s good to aim for avoiding that sig-LogMDT results are discarded, and hence propose following starting point:

Proposal A.1	Try to avoid discarding of sig-LogMDT not yet retrieved by network (e.g. if UE following T330 expiry, did not connect to the RAT that configured sig-LogMDT)

Further details UE assistance procedure (capturing previous)

In this section we propose the further details of a UE assistance mechanism, aiming to keep things as simple as possible.

We first of all think it is sufficient to introduce a single flag and we merely need to clarify which cases it concerns. We think it can be limited to sig-LogMDT, intra-PLM and that there is no need to indicate whether pending sig-LogMDT concerns current or other RAT. Hence we propose:

Proposal A.2	To support UE assistance introduce a flag (single bit) that UE sets if it has pending LogMDT i.e. ongoing and/ or (unretrieved) logging results are available

  - Limited to sig-LogMDT and intra-PLMN cases

  - No distinction whether pending sig-LogMDT concerns current or other RAT (i.e. used upon return from idle, possibly involving IRAT reselection

We furthermore think UE mainly transfers the information upon transition to connected. There seems no strong need to provide updated assistance during connected i.e. there should be sufficient alternative UEs to configure logMDT for

Proposal A.3	UE provides the logMDT pending indication in the following cases:

  - Upon transition to connected: I.e. same cases as for reporting availability of logMDT i.e. in setupComp, resumeComp, reconfigComp (includes HO to NR), reestablishComp. By extension of IE UE-MeasurementsAvailable (NR case)

  - (do not report status change e.g. upon T330 expiry, retrieval)

We think the information should also be transferred upon mobility in connected and hence propose:

Proposal A.4	Transfer UE assistance between network nodes (change of MN, including IRAT mobility)

Finally, there seems need by which network can control whether UE provides the assistance. For this we have two options i.e:

- Introduce field in dedicated signaling i.e. field in OtherConfig and a corresponding UE capability

- Introduce an indication in SIB

We have no strong view but appreciate that SIB option has the merit that we can support cell specific configuration and avoid UE capability. Hence, we propose:

Proposal A.5	Consider making the feature conditionally mandatory (i.e. for R17 UE supporting logMDT)

    - If agreed, introduce a field in SI/ broadcast indicating network support of the feature

    - Otherwise, introduce a field in OtherConfig by which network configures whether UE shall provide the assistance and a corresponding UE capability

## B: IRAT continuation/ handling in EN-DC

Continueing from [854]

As part of [Post113-e][854][NR17 SON/MDT] Logged MDT (CMCC) four options were discussed but no conclusion was reached. The options are listed in the table below. The discussion seems to be dragging on for some time, partly because there seems to be some confusion. We tried to clarify what the options comprise, see table below, also regarding which options really concern EN-DC and which options rather concern IRAT mobility (see main remarks).

| No | Original description | Clarified description | Main remarks |
| --- | --- | --- | --- |
| 1 | LTE and NR logged MDT configurations are independent, and UE performs logging based on the logged MDT configuration of the same RAT it camps | a) Per RAT configuration i.e. at T1 UE receives a configuration in RAT1 and at T2, after IRAT reselection, UE can receive logMDT config in RAT2 in which case UE does not overwrite config received in RAT1 b) When camping in RATN, UE behaves according to RATN config (can include target freqs for RAT2) i.e. performs logging, reports availability for RATN and, if network initiates retrieval, provides results for RATN | We understand this solution is for IRAT mobility i.e. does not improve situation for SCG frequencies for which camping is not supported |
| 2 | In EN-DC where UE cannot camp on NR cells, UE logs the NR measurements based on network configurations, i.e. for the NR logged MDT measurements in NSA, LTE SIB24 broadcasts the NR frequencies and indicates the UE does not need to reselect to NR cells | UE broadcasts NR cells in SIB24, but in a manner such that UE does not consider these for cell re-selection | NR SCG frequencies not used for cell re-selection are not measured by UE, unless included in EMR configuration in SIB5 We should stick to general principle was that logging is only done for available results, and never implies UE needs to perform additional measurements |
| 3 | Do not introduce SN configuration for logged MDT (neither for camping nor for non-camping/ EMR specific frequencies) |  | MN is capable to set LogMDT configuration for SCG frequencies (not too detailed/ specific and similar to what it provides for camping frequencies) |
| 4 | Enhanced areaConfiguration wherein a list of areaConfig could be provided to the UE wherein one list is associated to LTE and the other is associated is NR | Extend areaConfig to include IRAT cells, so logging need not stop upon IRAT re-selection | We understand this solution is for IRAT mobility i.e. does not improve situation for SCG frequencies for which camping is not supported |

Main remarks:

- Although all options were labelled as options for logged MDT in EN-DC, we think option 1 and 4 are only relevant for IRAT mobility i.e. they do not change situation for NR SCG frequencies not used for cell re-selection (i.e. not relevant for EN-DC/ MRDC)

- We think option 2 violates the general MDT principle that logging only concerns available measuremnts and shall not require UE to perform additional measurements. I.e. NR SCG frequencies not used for cell re-selection should be included in SIB5

We thus think there are two independent issues that should be discussed/ concluded seperately

Enhancing IRAT mobility

We see no real need to enhance logged MDT procedures for IRAT mobility i.e. we don’t see why we would introduce enhancements at this stage. If however there is majority to enhance, we somewhat prefer option 4) above option 1) as it is more consistent with current framework

EN-DC/ MR-DC

Logged MDT configuration for SCG frequencies not used for camping: We think MN is capable to set the LogMDT configuration for SCG frequencies. I.e. it does not concern very detailed/ specific parameters and is similar to what it provides for camping frequencies. We therefore see no need to introduce SN configuration/ to enhance compared to what we have in R16.

Proposal B.1	Do not introduce enhancements for IRAT mobility i.e. neither option 1 (per RAT configuration) nor option 4 (extending area config to cover IRAT reselection)

Proposal B.2	Do not introduce enhancements for logged MDT in EN-DC/ MRDC (i.e. MN configures logged MDT for SCG frequencies not used for camping)

## C: Further details logging for NR SCG frequencies not used for camping (EMR)

R2#113 agreements and remaining aspects to discuss

During the RAN2#113 meeting RAN2 reached some agreements relevant for logging for NR SCG frequencies not used for camping (EMR), see below.

R2-2102143	Report of [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I		Huawei

Agreement:

The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies. Whether a flag is needed should be FFS.

We think there are 3 main aspects that still need to be concluded:

- What configuration option to introduce (whether to introduce flag in logMeasConfig message)

- Whether to introduce changes for retrieval procedure (whether to introduce additional availability indication and/ or request field, which results to include in response message)

- How to signal the results concerning nonSIB frequencies e.g. whether to introduce additional field for separation purposes

The 3 issues are somewhat connected, and we will handle them from bottom up as this seems most relevant order (i.e. information from lower issue is relevant for deciding upper one)

C1: Signalling of results (seperation)

There was some discussion about the need to seperate the results of nonSIB frequencies. Below the relevant ASN.1 is shown as well as 2 options for transferring results of nonSIB frequencies by extensions fields.

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...,

[[

-- extensionOption2

logMeasInfoList-v17xy                LogMeasInfoList-v17xy        		OPTIONAL

]]

}

LogMeasInfoList-v17xy ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-v17xy

LogMeasInfo-v17xy ::=                  SEQUENCE {

measResultNeighCellsEMR-r17          SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

}

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

...,

[[

-- extensionOption1

measResultNeighCellsEMR-r17          SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

}

]]

}

The following table summarises the options

| Option | Description | Remarks |
| --- | --- | --- |
| 0 | No changes Results are included in a separate entry of the per frequency list (i.e. according to existing ASN.1) | + No changes - Legacy nodes will process results alike SIB frequency, if received |
| 1 | Introduce fields in LogMeasInfo-r16 for nonSIB freqs (one for inter freq and one for inter-RAT) ASN.1 changed by introducing parallel list of measurement entries, with each entry including separate lists for non-SIB4/ non-SIB5 frequencies (green highlights) | - Most complex i.e. assuming parallel list to avoid duplication of general information like time stamp, location information and so + Legacy nodes will discard results of non-SIB frequencies, if received |
| 2 | Introduce fields in LogMeasReport-r16 for nonSIB freqs (one for inter freq and one for inter-RAT) ASN.1 changed by introducing separate lists for non-SIB4 & non-SIB5 frequencies (yellow highlights) | - Medium complex + Legacy nodes will discard results of non-SIB frequencies, if received |

As ASN.1 is same, it may not be a problem if legacy nodes receive results for nonSIB frequencies. Furthermore, it should be possible to avoid that legacy node receives results of nonSIB frequencies. Hence there may be no real need to introduce specification changes

Observ 1	Only if legacy nodes process the logMDT results while there are no other ways to avoid legacy node receives results of nonSIB frequencies, there may be a need to introduce specification changes i.e. to carry results of nonSIB frequencies by extension field

C2: Retrieval related signaling (availability, request, contents of response)

In this section we will discuss whether to introduce changes for retrieval procedure. In the following we show options by 2 tables that respectively cover:

- Options for Availability indication i.e. whether to introduce additional availability indication or a flag on broadcast to control setting of legacy field

- Options for controlling and setting contents of UEInformationResponse: whether to have an additional request field and or a flag on broadcast controlling which results to include in the response message.

| Option | Description | Remarks |
| --- | --- | --- |
| 1 | No changes Re-use existing availability indication and always consider logging results for nonSIB frequencies when setting this field i.e. may also be set when UE only has results for nonSIB frequencies | + No changes - Legacy nodes will receive results for nonSIB frequencies, even if no results for SIB frequencies |
| 2 | Introduce field(s) in SI indicating support for nonSIB frequencies (can be common for interFreq and Inter-RAT) Only if field indicates network support, UE considers logging results for nonSIB frequencies when setting availability indication. I.e. field can avoid retrieval by network not supporting nonSIB frequencies | + Minor change + Legacy will not initiate retrieval when UE only has results for nonSIB frequencies - Legacy nodes will receive results for nonSIB frequencies |
| 3 | Introduce additional field to indicate availability of logging results for nonSIB frequencies (can be common for interFreq and Inter-RAT) | - Largest change, also because it’s useful only in combination with separate request + In such case, legacy nodes will not receive results for nonSIB frequencies |

Tab C1-1: Options for Availability indication

| Option | Description | Remarks |
| --- | --- | --- |
| 1 | No changes Re-use existing request field and always report available results for nonSIB frequencies | + No changes - Legacy nodes will receive results for nonSIB frequencies, even if no results for SIB frequencies |
| 2 | Introduce field(s) in SI indicating support for nonSIB frequencies (can be common for interFreq and Inter-RAT) Only if field indicates network support, UE report available results for nonSIB frequencies | + Minor change + Legacy nodes will not receive results for nonSIB frequencies |
| 3 | Introduce additional field to request reporting of availability results for nonSIB frequencies (can be common for interFreq and Inter-RAT) | - Largest change, also because it’s useful only in combination with availability indication + In such case, legacy nodes will not receive results for nonSIB frequencies |

Tab C1-2: Options for controlling and setting contents of UEInformationResponse message

We think we should consider combinations of availability and retrieval options, focusing on the following subset:

| Option | Description | Remarks |
| --- | --- | --- |
| 1 | No changes Re-use existing availability and request fields and always report available results for nonSIB frequencies | + No changes - Legacy nodes will receive results for nonSIB frequencies, even if no results for SIB frequencies |
| 2a | Introduce field(s) in SI indicating support for nonSIB frequencies (can be common for interFreq and Inter-RAT) Only if field indicates network support, UE considers logging results for nonSIB frequencies when setting availability indication UE always reports available results for nonSIB frequencies | Medium change + Legacy nodes will neither initiate retrieval when UE only has results for nonSIB frequencies nor receive results for nonSIB frequencies |
| 2b | Introduce field(s) in SI indicating support for nonSIB frequencies (can be common for interFreq and Inter-RAT) Only if field indicates network support, UE considers logging results for nonSIB frequencies when setting availability indication Only if field indicates network support, UE reports available results for nonSIB frequencies | Medium change + Legacy nodes will neither initiate retrieval when UE only has results for nonSIB frequencies nor receive results for nonSIB frequencies |
| 3 | Complete seperation Introduce additional fields for both availability indication and retrieval request | - Largest change + Legacy nodes will not receive results for nonSIB frequencies |

Tab C1-3: Combinations to consider

Observ 2	A combination with complete separation provides most flexibility/ control, but involves most specification changes. A combination merely having a flag on broadcast can avoid legacy nodes impact (avoid receiving results for nonSIB frequencies as well as avoid initiating retrieval when UE only has results for nonSIB frequencies) with much less specification changes

A: Network control

Regarding network control, the following table provides an overview of the different options we have identified.

| Option | Description | Remarks |
| --- | --- | --- |
| 1 | Configure target freqs (logMeas msg) Network sets aeaConfig including targetFreqList, that includes nonSIB freqs | + Enables per UE control and per individual frequency - Requires UE capability - Requires network to specify area config, serving cells/ TACs/ TAIs for which UE performs logging - Requires network to specify target frequencies, also for non SIB frequencies - Supported for interFreq but not for IRAT frequencies. I.e. will involve additional changes (also offering further control) |
| 2 | Flag in logMeas | + Enables per UE control, but not per individual frequency - Requires UE capability |
| 3 | Flag in SIB | + Enables per cell control i.e. can avoid network retrieval |

Some remarks:

Compared to other indications discussed in the previous, dedicated configuration can be used to stop a UE performing logging to store results for nonSIB frequencies. However, as UE merely logs available results, the benefit seems limited. I.e. it might just somewhat reduce the size of the variable

We see no real need for per UE control for the storing of available results i.e. network already has the option to use per UE control for performing EMR measurements and that seems sufficient

Observ 3	There main feature dedicated configuration could bring compared to previously discussed options is per UE control for the storing of available results for nonSIB frequencies. However, there seems no real motivation to avoid this (i.e. the storage of available results)

Synthesis

Remarks, summarizing my preferred solution:

- Existing ASN.1 supports reporting for nonSIB frequencies. Legacy RAN nodes are therefore capable of forwarding these results to TCE

  - Only if RAN processes the logMDT results, there may be a need to avoid reporting for nonSIB frequencies

- Should RAN2 agree there is a need to avoid that a legacy node receives/ processes results for nonSIB frequencies, the main options to consider are:

  - Introduce an extension field for the results of nonSIB frequencies

  - Introduce a flag on broadcast with UE reporting results only if network indicates support

- Introducing a flag on broadcast seems the preferred way forward, as it can also avoid that RAN initiates retrieval when UE only has results for nonSIB frequencies

- A completely independent question is whether to introduce filtering for IRAT frequencies i.e. it will not really help to avoid the remaining issue regarding logging of non-SIB frequencies

Altogether we hence propose:

Proposal C.1	Discuss and conclude if there is a need to to avoid that a legacy node receives/ processes results for nonSIB frequencies

Proposal C.2	If so, introduce a flag in SI/ broadcast indicating network support for nonSIB frequencies

    - One flag, common for interFreq (nonSIB4) and interRAT (nonSIB5), as extension of SIB1

    - Only if flag is set, UE reports results of nonSIB frequencies

    - Only if flag is set, UE will consider nonSIB frequencies for setting availability

Proposal C.3	Do not introduce further signaling changes, meaning that:

- No separate flag in logMDT config indicating whether UE shall log available results for nonSIB frequencies

- No separate field to indicate availability of results for non-SIB frequencies (but setting of existing availability indication is affected by field specifying target frequencies or flag)

- No separate field by which network requests results for non-SIB frequencies (but provides results of non-SIB frequencies based on field specifying target frequencies or flag)

- No separate field for results for non-SIB frequencies i.e. results are provided in separate entry of per frequency list of neighbouring results (according to current ASN.1)

Proposal C.4	Discuss and conclude whether to introduce a field indicating target IRAT frequencies

UE capabilities

We understand that network should be able to include a nonSIB frequency in the field defining the target frequencies that is in the LogMDT configuration. We think network should however only do so for a UE that supports the concerned functionality. Regarding the UE capability, we see the following options:

- Introduce a new capability, possibly separate for inter quency and inter-RAT

- Make the feature conditionally mandatory in R17 i.e. mandatory for an R17 UE supporting logMD and EMR

As the additional functionality seems limited, there may be no strong need for a UE capability. However, we have no strong view.

Proposal C.5	Consider making the feature conditionally mandatory in R17 (mandatory for an R17 UE supporting logMD and EMR)

## D: IDC and logged MDT (event triggered and periodic)

RAN2 reached some agreements regarding logged MDT and IDC:

NR MDT support IDC mechanism, including:

-           UE resumes the measurement logging when the IDC problem is resolved

-           upon detection of IDC, the UE suppress logging and tag MDT report with InDeviceCoexDetected flag

In NR we introduced event triggered measurement logging i.e. are two event based logging: 1) OOC and 2) eventL1 (like eventA2 in MR). As this option does not exist in LTE, we think there is a need to discuss how this is affected by IDC. Some remarks:

- IDC might merely affect one of the neighbouring frequencies. As a consequence, the UE may not be able to move to the affected frequency and as a result the configured event may happen. I.e. the event may actually be caused by the IDC problem

- If UE suspends event based measurement logging upon detecting IDC, the network will not be made aware that the problem reflected ty the concerned events occurs. It would however seem useful for network to know that the events actually occurred, and whether it may have been caused by IDC problems. We thus think the following seems preferrable:

  - Baseline: Upon detecting IDC problems, the UE continues measurement logging and if an event occurs, the UE adds an entry

  - Add-on, option 1: When creating an entry in VarLogMeasReport, the UE adds a field if it experiences IDC problems

  - Add-on, option 2: UE creates an entry when IDC problems are detected as well as when they are resolved (may not be reported if Variable only includes IDC entries)

- We think option 1 is more in line with the procedures as defined in LTE and hence propose:

Proposal D.1	Upon detecting IDC, the UE continues event based measurement logging. When adding an entry in VarLogMeasReport due to triggering an event, the UE includes a field indicating the UE experiended IDC, if applicable

It the above proposal is agreed, RAN2 could consider whether for periodical reporting, UE should also continue periodic measurment logging. We think that unless UE informs network about which frequencies are affected by IDC, this does not really make sense. We see no real need to enhace periodic measurement logging by adding information concerning affected frequencies and hence propose:

Proposal D.2	Upon detecting IDC, the UE suspends periodic measurement logging and if VarLogMeasReport has entries it adds an entry only comprising a field indicating that IDC was detected. When IDC problems are resolved, the UE resumes periodic measurement logging. I.e. do same as in LTE

  - E: SI Request Optimization

In RAN2#113e, RAN2 made the following agreements:

=>	 UE records the on demand SI related information for following scenarios:

1. Failed on-demand SI request

2. Successful on-demand SI request

1 	One specific raPurpose is introduced for MSG3 based on demand SI request.

From the agreements above, it is assumed to extend RA Report to log the SI-request related information. On the other hand, the current RA Report is used to log the RA-related information on successfully completed Random Access only. Hence, the RA Report is required to be improved so that it can consider even failed RA, but the failed RA should be related to on-demand SI request only. If considered in RA Report, a new indicator may be introduced to indicate if the RA due to SI request succeeded or failed.

Proposal E.1	RA report is extended to consider even failure of RA initiated due to on-demand SI request only.

In Rel-16 Positioning, PosSI messages and PosSIBs have been introduced. RAN2 is required to clarify if it’s in this scope.

Proposal E.2	RAN2 clarifies whether to consider Positioning SI/SIB in SI request optimization.

From on-demand SI related information, networks may expect to optimize to (re)group SIB(s) with each SI message. For instance, SIBs frequently requested should be included into a SI message periodically broadcast, while SIBs infrequently requested belong to SI message not periodically broadcast. Thus, it is useful for UEs to report the SIBs actually acquired by the UEs, targeting all SI messages regardless of whether to broadcast or not.

The time spent until the successful SI acquisition or the acquisition failure would impact the network performance. If it takes too much time to obtain the requested SI, the network should increase the PRACH resource for SI request. Thus, it is useful for UE to log time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure.

SI Request operation can be either via NUL or SUL.

Proposal E.3	For on-demand SI optimization, extend RA report to include the following information:

- SIBs actually acquired by UE.

- Time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure

- Indicating if SI request was performed over  NUL or SUL

# Conclusion & recommendation

The paper discusses several logged MDT related enhancements for R17. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal A.1	Try to avoid discarding of sig-LogMDT not yet retrieved by network (e.g. if UE following T330 expiry, did not connect to the RAT that configured sig-LogMDT)

Proposal A.2	To support UE assistance introduce a flag (single bit) that UE sets if it has pending LogMDT i.e. ongoing and/ or (unretrieved) logging results are available

  - Limited to sig-LogMDT and intra-PLMN cases

  - No distinction whether pending sig-LogMDT concerns current or other RAT (i.e. used upon return from idle, possibly involving IRAT reselection

Proposal A.3	UE provides the logMDT pending indication in the following cases:

  - Upon transition to connected: I.e. same cases as for reporting availability of logMDT i.e. in setupComp, resumeComp, reconfigComp (includes HO to NR), reestablishComp. By extension of IE UE-MeasurementsAvailable (NR case)

  - (do not report status change e.g. upon T330 expiry, retrieval)

Proposal A.4	Transfer UE assistance between network nodes (change of MN, including IRAT mobility)

Proposal A.5	Consider making the feature conditionally mandatory (i.e. for R17 UE supporting logMDT)

- If agreed, introduce a field in SI/ broadcast indicating network support of the feature

- Otherwise, introduce a field in OtherConfig by which network configures whether UE shall provide the assistance and a corresponding UE capability

Proposal B.1	Do not introduce enhancements for IRAT mobility i.e. neither option 1 (per RAT configuration) nor option 4 (extending area config to cover IRAT reselection)

Proposal B.2	Do not introduce enhancements for logged MDT in EN-DC/ MRDC (i.e. MN configures logged MDT for SCG frequencies not used for camping)

Proposal C.1	Discuss and conclude if there is a need to to avoid that a legacy node receives/ processes results for nonSIB frequencies

Proposal C.2	If so, introduce a flag in SI/ broadcast indicating network support for nonSIB frequencies

    - One flag, common for interFreq (nonSIB4) and interRAT (nonSIB5), as extension of SIB1

    - Only if flag is set, UE reports results of nonSIB frequencies

    - Only if flag is set, UE will consider nonSIB frequencies for setting availability

Proposal C.3	Do not introduce further signaling changes, meaning that:

- No separate flag in logMDT config indicating whether UE shall log available results for nonSIB frequencies

- No separate field to indicate availability of results for non-SIB frequencies (but setting of existing availability indication is affected by field specifying target frequencies or flag)

- No separate field by which network requests results for non-SIB frequencies (but provides results of non-SIB frequencies based on field specifying target frequencies or flag)

- No separate field for results for non-SIB frequencies i.e. results are provided in separate entry of per frequency list of neighbouring results (according to current ASN.1)

Proposal C.4	Discuss and conclude whether to introduce a field indicating target IRAT frequencies

Proposal C.5	Consider making the feature conditionally mandatory in R17 (mandatory for an R17 UE supporting logMD and EMR)

Proposal D.1	Upon detecting IDC, the UE continues event based measurement logging. When adding an entry in VarLogMeasReport due to triggering an event, the UE includes a field indicating the UE experiended IDC, if applicable

Proposal D.2	Upon detecting IDC, the UE suspends periodic measurement logging and if VarLogMeasReport has entries it adds an entry only comprising a field indicating that IDC was detected. When IDC problems are resolved, the UE resumes periodic measurement logging. I.e. do same as in LTE

Proposal E.1	RA report is extended to consider even failure of RA initiated due to on-demand SI request only.

Proposal E.2	RAN2 clarifies whether to consider Positioning SI/SIB in SI request optimization.

Proposal E.3	For on-demand SI optimization, extend RA report to include the following information:

- SIBs actually acquired by UE.

- Time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure

- Indicating if SI request was performed over  NUL or SUL

# References

[1] TS 38.331 Radio Resource Control