3GPP TSG-RAN WG2#113bis-e Meeting	Tdoc  R2-2103933

# Online, 12th- 20th April 2021

Agenda Item:		8.13.3.1

Souce:	Samsung

Title:	Introducing additional CHO related failure/ success info, including multiple events

Document for:		Discussion and decision

# Introduction

RAN2 made some agreements regarding additional CHO related information to be reported upon failure (RLF/ CHOF). So far the focus was on time related info, but in the recent e-mail [Post113-e][851][NR17 SON/MDT]  HO related SON changes addressed measurement, other information and touched upon the handling of multiple successive events. In this TDoc we are trying to summarise the current status, and review/analyse this means for the case of specific multiple events (i.e. dual failures, or success/ failure combinations). Along the lines we raise some technical considerations/ concerns regarding some of the proposed agreements.

RAN2 discussed whether in case of successive reporting events the UE provides a single or multiple reporting entries. We think that before concluding this, we should carefully review the information reported in different cases. Moreover, we think that it is important to aim for a sound general framework rather deciding the approach based on a few parameters agreed at this stage.

This contribution we subsequently touch the following aspects:

- Which CHO related scenarios to consider when designing the signalling

- Which measurement results are relevant, addressing for which time points and handling of CHO candidages

- Which other information is relevant/ have been agreed to be relevant (times, cellId’s, ..)

- How to signal the information, considering use of single and multiple reporting events

In brief, the paper includes following proposals (TBD):

- Reporting of measurements for CHO

- Single or multiple entries for successive reporting events

# Discussion

We think it’s beneficial if we can agree some abbreviations for the different failure/ sucdess events, particular when discussing scenario’s. In the remainder of this contribution we use the following conventions.

| HOF/ CHF | Failed HO/ CHO | ACHOF | Failed recovery Attempt on CHO candidate |
| --- | --- | --- | --- |
| HOS/ CHO | Succesful HO/ CHO | ACHOS | Succesful recovery Attempt on CHO candidate |

## Clarifications and comments regarding proposed outcome of eMails

Failure enhancements for CHO

We think there is a need to correct/ clarify some of the proposals:

- P1: In case of RLF after successful HO/ CHO, there are no CHO candidates remaining on which recovery can be done (attemptCondReconfig), so some clarification is required (mainly concerns scenario 3)

- P2: We assume b) means

- a single bit it introduced indicating whether CHO execution condition was fulfilled (i.e. no further details regardless actual conditions that were configured)

- done by including a flag within existing measResultNeighCells

- P4: Item a) and b) seem to address the same, except that b) merely also covers candidates not belonging to the best measured cells. I.e. we shoud chose between a) and b)

- P5: Item a seems already covered by P2 item b. Only additional thing may be indication of the candidate actually selected candidate

Company views/ concerns

- P2: Measurements: We think it is sufficient to introduce flag (single bit) within existing measResultNeighCells indicating whether cell concerns a CHO candidate which execution condition was fulfilled. See further remarks below

- The proposals that concern reporting of configuration can only conclude after receiving feedback from RAN3 regarding whether we can assume network has the CHO config

- Further analysis is required regarding the proposal regarding scenario’s involving multiple events, in particular regarding whether to have a single or multiple reports. This contribution attempts to progress this aspect

Success reporting for HO/ CHO

We think there is a need to correct and/or clarify some of the proposals:

- We understand P16 and P17 merely meant to exclude options i.e. to define which proposals to still consider (i.e. rather than to agree these)

- P16a/b: Is really the intention that for CHO UE only reports measurements of CHO candidates but not any other good neighbours detected (nCells)

- P16e: We assume this concerns measurement of source cell at HO/ CHO i.e. at same time as in 16a and 16b. If so, we prefer to align formulation (to avoid confusion)

- P17a/b: These seems more detailed versions of 16f i.e. we might have either but not both

For HOS/ CHOS, our company views/ concerns are provided in a separate contribution

## Failure report information, summary

The following table attempts to provide an overview of fields in R16 NR RLF-Report, as well as field (proposed to be) agreed for R17 (green) or proposed to be discussed/ concluded (blue, TBD). We used TBC for cases for which we think there is a mistake in current status, and for which we like our undrstanding to be confirmed

| Timers | Use cases | Remarks |
| --- | --- | --- |
| timeUntilReconnection (from failure until entering connected) | RLF, HOF, CHOF | 113: Agreed to be re-used for CHO |
| timeConnFailure (from last HO to failure) |  |  |
| timeSinceFailure (from failure to report) | RLF, HOF, CHOF | 113: Agreed to be re-used for CHO |
| P3/ P17c: Select either: - Time from CHO config until CHO exec (C, TBC) - Time from CHO exec until CHOF (D, Alt) | CHOF, CHOS (C?) | Same option for CHOF and CHOS is preferable TBD for CHOS |
| Cells | Use cases | Remarks |
| PreviousPCellId | HOF, CHOF, HOS? |  |
| FailedPCellId | RLF |  |
| ReconnectCellId | RLF, HOF, CHOF |  |
| ReestablishmentCellId |  |  |
| P5: TBD on CellId of - CHO candidate selected for exec e.g. if cond fulfilled for multiple candidates (FFS) - CHO candidate UE attempted to recover on after 1st failure - CHO candidate UE attempted to recover on after 2nd failure | CHOF | TBD for CHOS |
| Measurement related | Use cases | Remarks |
| measResultLastServCell | RLF, HOF |  |
| measResultNeighCells | HOF, HOS | TBC: Assumed relevant also at CHOF and CHOS |
| P2b: Flag indicating if CHO conditions fulfilled |  |  |
| P2c/ P16b: Latest results for CHO candidates (e.g. poor ones not in legacy nCell field) | CHOF, CHOS | TBD for CHOS |
| P16d: Latest results of source at latest CHO config | CHOS | TBD for CHOS |
| P16e: Latest results of source at HO | HOS | TBD for HOS |
| P16f: Flag indicating RLM issue in source during DAPS | HOS | TBD for CHOS |
| P17a: Elapsed time for T310 for HO/ CHO | HOS, CHOS | TBD for HOS/ CHOS Seems more detailed version of P16f |
| Config (R3 dependent) |  |  |
| P2a/P16c: Execution cond (e.g. event, TTT) of CHO candidates | (C)HOF, (C)HOS | TBD for HOS/ CHOS |
| P4a/ P4b: Indication of CHO candidates either by - Flag for candidates in measResultNeighCells - Full list of CHO candidates |  |  |
| Other |  |  |
| CRNTI | HOF, HOS? |  |
| connectionFailureType |  |  |
| rlf-Cause | HOF |  |
| locationInfo | HOF |  |
| noSuitableCellFound (T311 exp) |  |  |
| ra-InformationCommon |  |  |
| 112: Some indication that HOF concerned CHO (explicit/ implicit) | CHOF |  |

Tab 1: Overview of failure/ success information to be reported

We think it is important to establish a proper overview i.e. to ensure properly use of existing fields and to ensure our agreements result in a consistent approach. This is particularly relevant when discussing scenari’s involving multiple events, see next section.

## Scenario’s involving multiple events (failure, success)

[Post113-e][851] proposes to conclude the scenario’s to focus on. These include cases involving multiple events, covering following types:

- (F,) F,F: i.e. dual or triple failure

- (F,) F,S: i.e success after single or dual failure

- (F,) F,F/S: fail/ success after single or dual failure

- HOS/ CHOS, RLF, Restablishment on non CHO candidate

Failure can concern:

- RLF in source, HOF/ CHOF (T304), RLF in target shortly after HOS/ CHOS (CHO config cleared during HOS/ CHOS no, so no subsequent recovery on CHO candidate)

Succesful case can concern:

- HOS, CHOS, ACHOS (Restablishment/ recovery Attempt on CHO candidate), Reestablishment on other candidate?

We understand intention is to cover all these successful cases, depending on final conclusion regarding actual information to be reported.

Now let’s evaluate at the relevance of parameters in case of scenario’s involving multiple events. Some initial remarks:

- In many cases, there is some time between 1st and 2nd event. I.e. in many cases 2nd event concerns recovery and it may take some time to find a cell suitable for this. Moreover, in several cases the 2nd event concerns a failure which takes some time to determine/ declare.

- In case a subsequent event concerns a failure, this typically concerns a radio problem meaning the measurement situation is different from when starting the recovery resulting in subsequent failure. Measurement results upon detecting the subsequent event can help network in determining whether configuration needs adjustment, in particular for failure cases

- This particularly seem interesting for recovery on a CHO candidate (ACHOF/ACHOS), an event that’s part of in many of the primary scenario’s

- Measurement provide indication whether other good NCells were available that network missed to configure as CHO candidate (or other way around)

Now let’s have a closer look at some of the scenario’s the eMail proposes to focus on. Note that 1, 2 in column 1b means that the concerned parameter seems relevant for both 1st and 2nd event of scenario 1b.

| Parameter | 1b | 3e | Remarks |
| --- | --- | --- | --- |
| timeUntilReconnection | 1, 2 |  |  |
| timeConnFailure |  |  |  |
| timeSinceFailure | 1, 2 |  |  |
| timeCHO-ExeToCfg |  |  |  |
| previousPCellId | 2? |  |  |
| failedPCellId | 1 |  | Same for both |
| reconnectCellId |  |  |  |
| reestablishmentCellId |  |  |  |
| cho-SelectedCellID |  |  |  |
| attemptXCHO-CellID |  |  |  |
| measResultLastServCell | 1, 2 |  |  |
| measResultNeighCells | 1, 2 |  |  |
| c-RNTI | 1, 2 |  | In case of successive failures, value is same |
| connectionFailureType | 1, 2 |  |  |
| rlf-Cause | 1, 2 |  |  |
| locationInfo | 1, 2 |  |  |
| noSuitableCellFound | 1, 2 |  |  |
| ra-InformationCommon | 1, 2 |  |  |
| hof-WasActuallyCHO |  |  |  |

Some observations from considering these cases:

- For most of the parameters, it seems useful to have a value for each of the event involved in the scenario and typically the value is different. Some parameters are of course not applicable for a given event, but these are simply omitted largely in accordance with current specification

- If UE does not provide a report per event, additional fields would need to be defined for each event. E.g. for each failure event we may need to introduce separate time fields e.g. timeSinceFailure or timeUntilReconnection and separate failureType and possibly rlf-Cause for each failure event..

- Different cellID fields may apply for different (failure) events. The field in a single report may accidentally  be sufficient to cover 2 failure events, but it may not be obvious which field applies for 1st or 2nd failure event. I.e. this may require additional rules/  complexity

- Assuming (serving and neighbouring cell) measurements are useful for several successive events, as indicated in the previous, a single report approach would require additional fields for these measurements also

- An approach with a single report and several additional fields requires furher rules to clarify which fields belong together i.e. which fields concern the 2nd event, a 3rd event, ..

- An approach with a single report seems difficult to extend in future i.e. will soon become overly complex. Moreover, when quite some paramers are required per event(as shown above), a multiple report approach seems most sensible/ natural

We think RAN2 has so far not really performed this exercise, but given the previous analysis and also taking into account the general design considerations in the following section, we propose:

Proposal 1	In case UE experiences multiple report triggers/ events, the UE stores multiple reports that network can retreive

## General design considerations a.o. on measurements

General

We think there somewhat seems to be a tendency to introduce quite specific fields in on a case by case basis, and decision is taken not so much based on technical consideration but based on popularity. We however think that in general radio protocols should preferably aim to design and adopt general purpose mechanisms. E.g. we have an RRM mechanism that network can use for a variety of purposes, and this is mostly left up to network implementation. It seems that for measurement related information, we have a similar choice:

- Should UE provide specific indications for CHO candidates, or should UE provide results of best neighbours leaving it up to network to infer that apparently some of the CHO candidates were quite poor

- If there is a sequence of events, as in the primary scenario’s to focus on, should UE provide measurement results at occurrence of the first event, of each event or should we consider other generic means e.g. reporting results a certain conditions or periodically

  - E.g. if UE would provide results periodically, network can infer a lot of information including whether TTT may have been met

- If we introduce reporting of additional information, we should use existing generic information as baseline. I.e. we should avoid introducing new fields specifically to provide results for CHO candidates. I.e. if there is need to convey that CHO contition is met, we should just extend the existing field listing results of neibhour cells by a field indicating this cell concerns a CHO candidate which execution condition was fulfilled

  - E.g. if we introduce reporting for succesfull HO, we can take information reported in case of failure as baseline

  - In accordance with general priciples, we should try to avoid creating new ASN.1 information structures if there is large overlap in content (i.e. rather use optionality)

Although initially introducing specific solutions/ signalling may look simpler, we think they will increase UE complexity especially when standards evolve. This is because the number of specific indications tends to grow significantly over the years while we may also run into some difficult combinations. Hence, we suggest:

Proposal 2	RAN2 is requested to adopt generic solution approaches as much as possible e.g. exend existing fields rather than creating new specific fields with similar contents

Measurements to assist SON for setting CHO configuration

We think the UE should provide information enabling network to tune CHO configurations a.o. covering:

- Which of the configured candidates were not needed/ useful and merely resulted in waste of reserved network resources and increase of the measurement burden on the UE

- Which neighbouing cells would have been good candidates, that would have improved CHO robustness and/ or avoided failure of CHO (CHOF)

- Which conditions result in optimal balance between mobility robustness while limiting resource wastage due to early preparation at configuration of a CHO candidate

In the previous section we indicated that in scenarios involving multiple event, it seems useful to provide results applicable at each such event so network has proper understanding of the measurement conditions at the time of each event, in particular for failure events. It actually seems beneficial for network to have measurements not just at the particular (failure) event times. We note that for normal HO, network has the option to configure event triggered periodic measurement reporting i.e. network can make UE trigger MR upon entering HO region and periodically after that. Network can use this feature not just to for tuning actual HO also for IMM MDT purposes. It may be beneficial to have somewhat similar functionality to self-optimise the CHO configurations.

Several companies indicated they want UE to provide measurement results of cells that are really bad, just because they are CHO candidate. We think it is sufficient if UE reports the best cells and are open to discuss increasing the number of cells UE reports. However, think there is no value in introducing separate fields for really bad CHO candidates. We actually think that reporting of measurement results at more time instances is definitely more useful to the network’s SON purposes.

Proposal 3	RAN2 is requested to discuss and conclude for which time instances UE should provide available measurement results to facilitate SON for the CHO config and support measurements at least for each failure event

# Conclusion & recommendation

The paper discusses additional CHO related information proposed to be introduced for different failure/ success reporting cases. It furthermore addresses the signaling in case of scenario’s involving multiple events. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1	In case UE experiences multiple report triggers/ events, the UE stores multiple reports that network can retreive

Proposal 2	RAN2 is requested to adopt generic solution approaches as much as possible e.g. exend existing fields rather than creating new specific fields with similar contents

Proposal 3	RAN2 is requested to discuss and conclude for which time instances UE should provide available measurement results to facilitate SON for the CHO config and support measurements at least for each failure event

# References

[1] TS 38.331 Radio Resource Control

# Background (annex)

## Proposed agreements from [Post113-e][851][NR17 SON/MDT]  HO related SON changes

Previous agreements (summary)

In case of CHO failure (#112)

a) Time between last corresponding CHO config and CHO exec (C)

b) Indication that HO concerned CHO (implicit/ explicit)

c) Measurements of source and target CHO (cell and beam)

RLF report contents (#113)

d) Time elapsed since CHO execution until connection failure (D, either explicitly or implicitly)

e) Reuse timers also for CHO: timeUntilReconnection, timeSinceFailure

f) Include latest measurement results

Proposed results from latest e-mail

Scenario’s

P1: Focus on following scenarios (FFS whether to merge some e.g. 1b/1c, 2a/2b)

1 Too late HO:

1a: RLF in source; succes RRE on CHO cand

1b: RLF in source; failed RRE on CHO cand, succes RRE (non CHO cand)

1c: RLF in source; failed RRE on CHO cand, failed RRE (non CHO cand)/ no cell found

1d: RLF in source; success/ failed RRE (non CHO cand)/ no cell found <Dif to legacy: CHO configured)

2 (too early HO):

2a1: failed CHO, success/ failed RRE (source/ non CHO cand)

2a2: Success CHO, RLF in target, success/ failed RRE (source/ non CHO cand)

2b: failed (C)HO, failed RRE on CHO cand(2), succes RRE (non CHO cand)

3 (HO to wrong cell):

3a: failed CHO cand1, success RRE on CHO cand2

3b: failed CHO cand1, success/ failed RRE on non CHO cand

3c: failed HO, success RRE on CHO cand

3e: failed CHO cand1, failed RRE on CHO cand2, success/ failed RRE (non CHO cand)/ no cell found

3f: failed HO, failed RRE on CHO cand2, success/ failed RRE (non CHO cand)/ no cell found

Tabular representation, using acronyms

| Case | Event 1 | Event 2 | Event 3 |
| --- | --- | --- | --- |
| 1a | RLF in source | ACHOS |  |
| 1b |  | ACHOF | RRES |
| 1c |  | ACHOF | RREF/NCF |
| 1d |  | RREF/NCF |  |
| 2a1 | CHOF | RRE |  |
| 2a2 | CHOS | RLF | RRE |
| 2b | HOF/ CHOF | ACHOF | RRES |
| 3a | CHOF | ACHOS |  |
| 3b | HOF | RRE |  |
| 3c | HOF | ACHOS |  |
| 3e | CHOF | ACHOF | RRE/NCF |
| 3f | HOF | ACHOF | RRE/NCF |

Timers

P3: Conclude whether to include

D. Time elapsed between CHO exec until the first HOF/RLF (RAN3)

C. Time elapsed between last CHO config and CHO exec (previously RAN2 agreed)

Measurements

P2: Introduce in RLF-Report following info of CHO candidates:

a.	Execution condition config (events, TTT)

b.	Indication whether CHO execution condition(s) were fulfilled (i.e. single bit)

c.	Latest radio measurement results of CHO candidates

Depend on R3

P4: Introduce in RLF-Report following CHO related info

a.	In existing measResultNeighCells include flag indicating if cell concerns CHO cand

b.	List of candidate cells IDs

P5: Conclude whether to include

a.	List of CHO candidates for which execution condition(s) were fulfilled and candidate selected for CHO exec (E)

b.	CellID of CHO candidate in which UE attempted recovery after 1st failure (G)

c.	CellID of CHO candidate in which UE attempted recovery after 2nd failure (x)

P6: In case of multiple events (failure/ success), use single report with additional fields rather than using report entry per event

## Proposed agreements from [Post113-e][85x][NR17 SON/MDT]  HO related SON changes (successful HO)

Previous agreements (RAN2#113) on contents of the HO success report:

1. source cell and target cell related identifiers and

2. measurements (superseded by e-mail, see below).

P16 Measurements related info to consider

a.	HO: Latest measurement results of nCell @ HO

b.	CHO: Latest measurement results of CHO cand’s @ execution

c.	CHO execution condition config (events, TTT) of CHO cand’s

d.	Measurement results of source cell @ receipt of latest CHO config?

e.	Latest measurement results of source cell @ HO/ HO command receipt

f.	Flag indicating if UE detected RLM issues in source cell during DAPS HO

P17 Timer related info to consider

a.	Elapsed time for T310 timer for normal HO/ CHO (relates to 16f)

c.	Time elapsed between the CHO execution and receipt of the latest corresponding CHO config (i.e. concerning same CHO candidate; same as option C for P3)

## Previous agreements (full list)

During the RAN2#111 meeting RAN2 reached some agreements relevant for the discussion in this contribution, see below.

Agreements:

=>	The following scenarios are considered:

1) Successful CHO and HO (i.e. no failure happens). FFS consideration in RAN2/3

2) Unsuccessful CHO due to late CHO execution.

3) Unsuccessful CHO after CHO execution.

4) Successful or Unsuccessful CHO after unsuccessful CHO or handover failure.

Note: other scenarios are not ruled out…

=>	RAN2 should study what CHO failure information can be stored in RLF report.

=>	RAN 2 to discuss the method for distinguishing between different handover types in RLF report. FFS the details, e.g., explicitly way or not.

During the RAN2#112 meeting RAN2 reached some agreements relevant for the discussion in this contribution, see below.

From RAN2#112 agreements:

The following time information is as part of the UE RLF report:

Time between the first CHO execution and the corresponding CHO command received at UE at least in the CHO failure case.

Focused scenarios:

In case of successive CHO related failures, the UE stores and reports both RLF related information in the RLF report. The successive failure referred above, includes at least the following scenarios.

a.	A UE that has CHO configuration declares RLF in the source cell. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell.

b.	A UE that has CHO configuration executes the CHO towards the target cell upon fulfilling the configured condition and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell.

c.	A UE that has CHO configuration executes the normal HO towards the target cell and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell using CHO procedure.

Note: other scenarios still can be discussed.

FFS: Further clarification on the successful reestablishment.’

The following cells’ related cell and beam measurements are included in the RLF report associated to CHO failure:

a.	Source cell of the CHO. FFS the detail on cell ID. Try our best to reuse the existing information.

b.	The target cell towards which the CHO was executed, if CHO related condition was satisfied. FFS the detail on cell ID. Try our best to reuse the existing information.

c.	The cell in which the re-establishment is performed after the CHO failure or source RLF. Try our best to reuse the existing information. FFS on the related measurements.

FFS:	Candidate target cells as configured in the CHO configuration.

RLF-report shall contain information to differentiate an ordinary HO failure from the CHO failure and CHO recovery failure. FFS: implicit indication vs explicit indication.

RAN2 subsequently had an email during which the following options were considered:

| A | Timeline relationship between two consecutive RLF reports for cases of successful or unsuccessful CHO after unsuccessful CHO or handover failure |
| --- | --- |
| B | Time between the UE receiving the CHO command and RLF |
| C | UE reports the time elapsed since CHO execution until connection failure |
| D | In case of multiple failures case, UE includes the time elapsed since CHO execution until connection failure (TimeConnFailure) and time elapsed since the last radio link or handover failure (TimeSinceFailure) in each RLF-Report |
| E | The time between CHO execution and successful reestablishment to a third cell after CHO failure towards the candidate target cell selected at CHO execution |
| F | The time elapsed since CHO configuration until the immediate HO reception or execution |
| G | The related cell and beam measurements of candidate target cells as configured in the CHO configuration |

Upon which RAN2 reached the following agreement:

=>	Regarding the CHO-related timers, Option D, E, F will not be included in the RLF report and other options will continue discussion through email mail after this meeting.

During the RAN2#113 meeting RAN2 reached some further agreements relevant for the discussion in this contribution, see below.

R2-2101451	[Post112-e][853][NR R17 SON/MDT] R17 Information needed in UE report for CHO cases (Ericsson)	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

Agreements:

1	Include in the RLF report the “Time elapsed since CHO execution until connection failure”. How to convey this information is FFS. (email discussion 886, Qualcomm)

2	Reuse the following legacy timers in the RLF report also for CHO: timeUntilReconnection, timeSinceFailure.

3	In the RLF report for CHO, the UE includes of the latest radio measurement results. FFS: to indicate whether or not it is candidate target cell. (email discussion 887, Ericsson)

FFS: Separate IEs/fields within the existing RLF-report are used to represent the second HOF. Also consider the second HO is successful case together. What measurements also need to be considered.

R2-2102146	The report of [Offline-e][886][NRR17 SONMDT]  How to address time information	Qualcomm

-	Based on the agreements that “Include in the RLF report the “Time elapsed since CHO execution until connection failure”.

-	Figure out how to convey this information.

Agreements:

UE reports "Time elapsed since CHO execution until connection failure" implicitly or explicitly, i.e. UE either explicitly provides the aforementioned timing information or provides sufficient information for the network to compute it.

R2-2102145	[AT113-e][887][NR/R17 SON/MDT]  Indication of candidate target cell	Ericsson

-	Based on the agreements that “In the RLF report for CHO, the UE includes of the latest radio measurement results. FFS: to indicate whether or not it is candidate target cell.”.

-	Figure out the necessity of introducing the indication.

=>	Continue the discussion ”UE shall include the latest radio measurement results of the candidate target cells in the RLF-report.” through email. (Ericsson)

=>	Before agreeing on including an indication indicating whether a neighbor cell, included as part of neighbor cell measurement result, is associated to a CHO candidate target cell or not, RAN2 waits RAN3 to confirm whether the source cell can keep the UE context, at least up to the point the RLF-report is received by the source cell. Draft LS to RAN3 for this.(#899, Ericsson)