3GPP TSG-RAN WG2 #113-bis-e	Tdoc R2-2103940

Online, April 12th – 20th 2021

Agenda Item:	8.4.2

Source:	Ericsson

Title:	On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#113e, the following topics were endorsed for further discussion and possible enhancements to topology-wide fairness, multi-hop latency, and congestion in IAB network.

Chair: On the agreed issues below, the agreement doesn’t mean that we have agreed that there need to be a solution for it in R17. Furthermore, liberal interpretation of the text is ok. ISSUES: eIAB work on topology-wide fairness will focus on the following issues - IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes) - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement. - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load) ISSUES: In the first instance, eIAB work on multi-hop latency will focus on the following issues: - IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16 - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel - IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion. Both IC-1 and CI-7 are related to RAN3. RAN3 seems to also work on this, so to what extent R2 shall work on this is currently not clear.

This paper analyses the above issues and presents our point of views as well as possible solutions.

# 2	Discussion

This section provides our perspective on each topic agreed for further discussion in RAN2, and that are documented in [1].

## 2.1	Topics Related to Topology-wide fairness

- IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes).

Addressing IF-1, i.e., how to provide route link quality information to schedulers of IAB nodes, in our view not only requires considerable specification work but also will inflict significant signalling overhead during network operation. Besides, it is not clear how this additional information will improve topology-wide fairness, and most importantly, why the scheduler of an IAB node should take into account the quality of other links. The schedulers of the various IAB nodes should rather act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in underutilization or unbalanced utilization of the radio link capacity. Additionally, by the time this radio link quality indication is received by an IAB node of a previous hop, such radio link quality may have already changed which will make the usage of such information even more troublesome.

Considering all these points, in our view RAN2 should deprioritize this topic (IF-1) for further discussion and rather focus on other important topics related to topology-wide-fairness.

It is not clear what benefits the sharing of the link qualities over multiple hops brings, especially considering the significant specification work, and the large overhead.

The schedulers of the various IAB nodes should act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in an unbalanced utilization of the radio link capacity.

RAN2 to downprioritize issue IF-1.

- IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement.

- IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load)

The issues highlighted in IF-2 and IF-4 are interrelated, hence, here we discussed them together. The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation, for instance, by letting the CU to properly balance the traffic across the available/configured BH RLC channels.

The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation.

Concerning the issue of how to enable an IAB node to know about the number of UE bearers mapped to BH RLC channels (i.e., for N:1 mapping scheme) so that the IAB node scheduler can provide adequate radio resources accordingly, there are several ways to handle this issue. Some companies proposed to use F1 signaling for communicating this information to IAB nodes, while other companies argued to add additional fields in the BAP header for this purpose. Both these approaches require specification work and cause signalling overhead.

Enabling the IAB node (via F1 signaling or additional BAP header fields) to know about the number of DRBs/UEs served by a BH RLC channel for QoS purposes might inflict significant signalling overhead.

In particular, it is unclear to see the advantage of the solution in which F1 signalling is used. Besides the overhead to provide an update whenever there is a change on the number of DRBs/UE, it is not clear why the CU could not simply reconfigure via F1 the BH RLC channels, whenever that is needed, e.g. when there is the need to provide a better balance in the traffic load or when there is a congestion. To this end, for example, RAN3 has agreed to introduce a new F1 signal that allows the IAB node to indicate the congestion status of the various BH RLC channels that it is handling. This information can be used by the network to properly redistribute the load of the affected BH RLC channels.

It is not clear the advantage of using F1 signalling to provide updates on the number of UEs/DRBs conveyed by a BH RLC Channel. The CU could simply reconfigure the BH RLC channels when there is a high imbalance in the traffic they are handling.

In general, a simple implementation solution for this issue would be to assign a uniform number of DRBs per BH RLC channel and later reconfigures (via F1 signaling) the BH RLC channels, whenever there is a significant imbalance in the traffic load (i.e., number of UE DRBs) carried by such BH RLC channels. In our understanding, it remains unclear what additional benefits the specification solutions (e.g., using F1 signaling and BAP header) would provide over the implementation solution for this issue.

A simple approach for ensuring fair radio resources to BH RLC channels could be to assign a uniform number of DRBs per BH RLC channel and later reconfigures the BH RLC channels when there is a high imbalance in the traffic they are handling.

Furthermore, when considering whether providing the information to IAB nodes about the number (actual or average) of DRBs mapped to each BH RLC channel is an accurate approach for improving topology-wide fairness. Because, the traffic patterns of the UEs/DRBs mapped to a certain BH RLC channel might be very much different from each other, depending not only on the QoS of the traffic, but also on the traffic model generated by the end-user, the radio conditions, the congestion situations at the various intermediate IAB nodes, the number of hops apart, etc. Thus, assuming that a BH RLC channel is always at “full load”, i.e. all packets of this BH RLC channel always convey data for all the UEs/DRBs associated with this BH RLC channel, is not a correct assumption in a real-world scenario.

Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending on a number of factors.

Hence, in our view, topology-wide fairness issues can be addressed via implementation, at least to some extent, e.g., by simply letting the CU properly balance the traffic across the available BH RLC channels. If more granularity is required, RAN2 could consider more efficiently utilize the extended LCID space (i.e., 16 bits) for IAB networks. For example, establishing several BH RLC channels rather than one BH RLC channel for carrying all UE bearers traffic served by an IAB node. In addition, the IAB-donor CU can map the same number of UE bearers to each BH RLC channel and can then maintain/update this mapping whenever the network status changed due to either new UEs connecting to or some existing UEs depart from the network.

RAN2 to assume that IF-4 and IF-2 can be addressed by network implementation.

In any case, as observed in Observation 5 and Observation 7, if RAN2 eventually decides to pursue some enhancements to the current framework, such enhancements should be adopted in the BAP header rather than F1 signalling, because F1 signaling might be inefficient for frequently updating the mapping of UE bearers to BH RLC channels, and would not give significant gains compared with existing techniques, i.e. the CU reconfigures the BH RLC channels when needed. Including this information in the BAP header would also give the advantage to the IAB node scheduler to always know the exact number of UEs/DRBs contained in a BAP packet, which may vary over time depending on a number of factors.

If RAN2 decides to design solutions for IF-2/IF-4, RAN2 should prioritize solutions in which the number of UEs/DRBs is conveyed in a BAP header, i.e. solution F2-3 in [1].

## 2.2	Topics Related to Multi-hop latency

- IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency

To provide some background, the network gives a configuration (to IAB nodes) to meet the required QoS on a bearer basis, and Rel-16 IAB-donor CU knows the network topology under its domain, i.e., each backhaul channel and each route that is set up may have different levels of QoS, hence can lead to different latency. When routing packets, the network may choose an appropriate route to meet the QoS requirements including latency requirements. In addition, the pre-BSR might be able to assist the network to minimize UL latency.

Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters and the pre-emptive BSR can provide minimum UL latency.

Additionally, for ensuring PDB management, according to Rel-16 specifications, it is already possible for the CU to configure via F1 the IAB node with a PDB requirement per BH RLC channel. Since the Rel-16 IAB-donor CU knows the network topology under its domain, it can configure the PDB HbH at each node along the path towards an access IAB node. Based on this information, a network node can discard a packet if its scheduler cannot satisfy the PDB for the associated BH RLC channel, without the need to specify any specific discarding rule.

In Rel-16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

Some companies argued to enhance the current ways to handle HbH latency. One such enhancement is to extend the granularity of PDB configuration, making it possible for the IAB-donor CU to indicate to the IAB node the PDB per BAP destination for traffic mapped to a backhaul RLC channel, rather than just per backhaul RLC channel.

It can also be discussed, if needed, whether information related to the remaining PDB for a given packet should be transferred from one node to another. However, RAN2 should aim at minimum specification changes, and avoid specifying unnecessary procedures that can be left to the IAB node implementation, e.g., whether to discard a packet or not on the basis of PDB fulfillment. Considering that the PDB is already available at the IAB node, it should be then up to the network implementation to determine when/how to discard a certain packet, as it already happen in non-IAB networks

If RAN2 identifies the need for solutions to solve IL-1, RAN2 should focus on the following solutions:

  - Introducing a PDB per BH RLC channel per destination, i.e. solution L1-4 in [1].

  - Introducing an indication in each packet of the remaining PDB at previous hop, i.e. solution L2-2 in [1].

RAN2 to avoid specifying latency-related solutions that can be handled by the IAB node implementation, e.g., packet discarding.

- IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs

We acknowledge the issue highlighted in IL-2. In fact, this topic of “enhancement of LCGs per BH link” was briefly discussed during Rel-16 but was postponed to Rel-17. In our view, this topic can be discussed in the context of ensuring QoS requirements for UL traffic in a multi-hop IAB network.

RAN2 to consider increasing the number of LCGs to address the issue IL-2, i.e. solution L4-3 in [1].

- IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16

This topic, i.e., “different ways to calculate buffer size for pre-emptive BSR” have been thoroughly discussed in Rel-16 and it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. The fact that different vendors may use different ways to calculate the pre-emptive BSR has not to be seen as a problem. IAB nodes are network nodes, and 3GPP standardization work does not aim at aligning network implementations of different vendors. Additionally, it will be ambiguous and probably require a lot of standardization work to converge on one single mechanism. Given that the objective of the WID is “enhancements to improve topology-wide fairness, multi-hop latency, and congestion mitigation”, it is not clear how to prove that the selected mechanism will really improve the IAB performances.

During IAB Rel-16, it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. Hence, it is not clear what benefit would bring standardizing the Preemptive BSR calculation, especially considering the potential large standardization work.

- IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free

- IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel

In our view, these issues are more RAN3 specific and are related to E2E latency and flow control, which are already under discussion in RAN3. In fact, RAN3 has agreed upon CP-based congestion indication report (reuses the F1AP GNB-DU Status Indication procedure) from IAB node DUs to CU. Hence, RAN2 does not need to further discuss these topics unless RAN3 asks for RAN2-related specification work needed for their agreed solution.

The issues highlighted in IL-5 and IL-6 are more RAN3 specific and are related to E2E latency and flow control, which are already under discussion in RAN3.

RAN2 should not discuss potential solutions for issues highlighted in IL-5 and IL-6 since they are RAN3 specific.

## 2.3	Topics Related to Congestion mitigation

- IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets

- IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion.

IC-1 endorses the concept that HbH flow control mechanism can only solve the temporary congestion on a local BH link, not long-term congestion. However, for long-term congestion, E2E flow control (RAN3 domain) mechanism can be used and improved (if needed) rather than further enhancing the HbH flow control. In fact, RAN3 has been discussing how to enhance both UP-based and CP-based approaches for DL congestion mitigation and baseline agreements have been made for both approaches.

For long term congestion, E2E flow control (RAN3 domain) mechanism can be used and improved (if needed) rather than further enhancing the HbH flow control.

RAN3 has been discussing how to enhance both UP-based and CP-based approaches for DL congestion mitigation and baseline agreements have been made for both approaches.

When it comes to IC-7, we acknowledge this issue, however, in our view RAN2 standardization work is not required, since RAN3 is already discussing ways to convey via F1 signalling more granular congestion indications (e.g. per BH RLC channels) to the CU.

It is up to RAN3 to address, if needed, the issues IC-1 and IC-7.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is not clear what benefits the sharing of the link qualities over multiple hops brings, especially considering the significant specification work, and the large overhead.

Observation 2	The schedulers of the various IAB nodes should act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in an unbalanced utilization of the radio link capacity.

Observation 3	The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation.

Observation 4	Enabling the IAB node (via F1 signaling or additional BAP header fields) to know about the number of DRBs/UEs served by a BH RLC channel for QoS purposes might inflict significant signalling overhead.

Observation 5	It is not clear the advantage of using F1 signalling to provide updates on the number of UEs/DRBs conveyed by a BH RLC Channel. The CU could simply reconfigure the BH RLC channels when there is a high imbalance in the traffic they are handling.

Observation 6	A simple approach for ensuring fair radio resources to BH RLC channels could be to assign a uniform number of DRBs per BH RLC channel and later reconfigures the BH RLC channels when there is a high imbalance in the traffic they are handling.

Observation 7	Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending on a number of factors.

Observation 8	If RAN2 decides to design solutions for IF-2/IF-4, RAN2 should prioritize solutions in which the number of UEs/DRBs is conveyed in a BAP header, i.e. solution F2-3 in [1].

Observation 9	Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters and the pre-emptive BSR can provide minimum UL latency.

Observation 10	In Rel-16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

Observation 11	During IAB Rel-16, it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. Hence, it is not clear what benefit would bring standardizing the Preemptive BSR calculation, especially considering the potential large standardization work.

Observation 12	The issues highlighted in IL-5 and IL-6 are more RAN3 specific and are related to E2E latency and flow control, which are already under discussion in RAN3.

Observation 13	For long term congestion, E2E flow control (RAN3 domain) mechanism can be used and improved (if needed) rather than further enhancing the HbH flow control.

Observation 14	RAN3 has been discussing how to enhance both UP-based and CP-based approaches for DL congestion mitigation and baseline agreements have been made for both approaches.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to downprioritize issue IF-1.

Proposal 2	RAN2 to assume that IF-4 and IF-2 can be addressed by network implementation.

Proposal 3	If RAN2 identifies the need for solutions to solve IL-1, RAN2 should focus on the following solutions:

a.	Introducing a PDB per BH RLC channel per destination, i.e. solution L1-4 in [1].

b.	Introducing an indication in each packet of the remaining PDB at previous hop, i.e. solution L2-2 in [1].

Proposal 4	RAN2 to avoid specifying latency-related solutions that can be handled by the IAB node implementation, e.g., packet discarding.

Proposal 5	RAN2 to consider increasing the number of LCGs to address the issue IL-2, i.e. solution L4-3 in [1].

Proposal 6	RAN2 should not discuss potential solutions for issues highlighted in IL-5 and IL-6 since they are RAN3 specific.

Proposal 7	It is up to RAN3 to address, if needed, the issues IC-1 and IC-7.

# 4	References

- R2-2101168, Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung), Samsung (Rapporteur), 3GPP TSG-RAN WG2 #113-e