3GPP TSG-RAN WG2 Meeting #113bis electronic		R2-2103946

Online Meeting, April 12 – April 20, 2021

Agenda item:	8.1.3 Idle and Inactive mode UEs

Title:	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs

Source:	Convida Wireless

Document for:	Discussion

# Introduction

At RAN #86, a work item on supporting of multicast and broadcast services was approved [1] which was further updated at RAN #88-e as [2]. The WI aims to provide the support in RAN for enabling general MBS services over 5GS and the use cases identified that could benefit from this feature include (but are not limited to) public safety and mission critical, V2X applications, transparent IPv4/IPv6 multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications. The objectives of the WI include:

- Specify RAN basic functions for multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

Restrictions and assumptions:

Physical layer: limit the scope of this WI to current Rel-15 numerologies, physical channels (PDCCH/PDSCH) and signals.

FR2: we assume that there are no issues to provide Multicast / Broadcast transmissions in FR2. If any enhancements is needed it should be treated with lower priority compared to the minimum set of objectives above.

In this contribution, we provide our views on NR multicast and broadcast for RRC_IDLE UEs and RRC_INACTIVE UEs.

# Discussion

In RAN2 #112e, two delivery modes were defined:

- Delivery mode 1, which supports the transmission of MBS data with high QoS requirement, e.g. high reliability and/or low latency, where the UE can receive MBS data in RRC_CONNECTED, and mechanism to guarantee required high QoS requirement, e.g. transmission feedback and/or PTP assistance, is needed. It was agreed that this mode would be applicable only to multicast sessions for Release 17.

- Delivery mode 2, which supports the transmission of MBS data with low QoS requirement, where the UE can receive MBS data in RRC_IDLE/RRC_INACTIVE and neither transmission feedback nor PTP assistance for reliability guarantee is needed. It was agreed that this mode would be applicable to at least broadcast sessions.

Based on discussions post RAN2 #112e [3] [4], it seems that most companies view delivery mode 2 as an NR version of LTE SC-PTM, which is supported in all states. However, LTE SC-PTM only applies to broadcast sessions, and as a result, the applicability of delivery mode 2 to multicast sessions was FFS. The agreement from RAN2 #113e includes

Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other).

In our view, not all multicast sessions have high QoS requirements, and using delivery mode 1 for these services will result in issues with power consumption, as the UE may be  required to be in RRC_CONNECTED to receive the MBS service. For these multicast sessions with “low QoS”, delivery mode 2 would be more power efficient, and result in lower network signaling overhead.

Proposal 1: RAN2 agree to support Delivery Mode 2 for multicast sessions with “low QoS” requirements.

Like the RRC_CONNECTED state, the BWP used for MBS in RRC_INACTIVE state and in RRC_IDLE state needs to be defined. In unicast scenario, the initial BWP is used in RRC IDLE/INACTIVE to support paging delivery and system information delivery, etc. However, if we further consider supporting MBS, the initial BWP may not be large enough. Therefore, enhancements on BWP operation are needed to support MBS for UEs in RRC_IDLE/ RRC_INACTIVE states.

Proposal 2: Enhancements on BWP operation are needed to support MBS for UEs in RRC_IDLE/ RRC_INACTIVE states.

In addition, one of the issues, to support MBS for UEs in RRC_IDLE/RRC_INACTIVE states, is that the network may not know the UE location, e.g., due to the UE’s mobility.  A similar problem occurs for delivering paging message, and beam sweeping was introduced to solve the issue. From our view, we think that the beam sweeping mechanism can also be considered for supporting MBS for UEs in RRC_IDLE/ RRC_INACTIVE states to help the gNB to deliver the MBS transmission to all the UEs within the cell. The details can be for further study.

Proposal 3: Beam sweeping mechanism can be considered for supporting MBS for UEs in RRC_IDLE/ RRC_INACTIVE states. The details can be for further study.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: RAN2 agree to support Delivery Mode 2 for multicast sessions with “low QoS” requirements.

Proposal 2: Enhancements on BWP operation are needed to support MBS for UEs in RRC_IDLE/ RRC_INACTIVE states.

Proposal 3: Beam sweeping mechanism can be considered for supporting MBS for UEs in RRC_IDLE/ RRC_INACTIVE states. The details can be for further study.

# References

- RP-193248, “New Work Item on NR support of Multicast and Broadcast Services”, Huawei, RAN #86, Spain, December 9th – 12th, 2019

- RP-201038, “WID revision: NR Multicast and Broadcast Services”, Huawei, RAN #88-e, E-meeting, June 29 - July 3, 2020

- R2-2100177, [Post112-e][069][MBS] Delivery mode 2, MediaTek, RAN2 #113e, January 25th – February 5th, 2021

- Draft_R2-113-e_Meeting_Report_v1, RAN2 #113e