3GPP TSG-RAN WG2 #113bis-e	R2-2103950

E-Meeting, 2021-04-12 - 2021-04-20	Revision of R2-2101493

Agenda Item:	8.10.2.2

Source:	Ericsson

Title:	On scheduling, HARQ, and DRX for NTNs

Document for:	Discussion, Decision

# Introduction

At the RAN#86 meeting, the Study Item on “Study on solutions for NR to support non-terrestrial network” was completed [1] and the corresponding work item was approved and later updated at RAN#91 [2]. The new work item aims to specify the enhancements identified for NR NTN and has the following RAN2 objectives on user plane aspects:

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

- RLC

  - Status reporting: Extension of the value range of t-Reassembly

  - Sequence Numbers: extension of the SN space only for GEO scenarios

- PDCP

  - SDU discard: Extension of the value range of discardTimer.

  - Sequence Numbers: extension of the SN space for GEO scenarios.

At RAN2#112e the following agreements were made:

- For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset)

Agreement from Friday CB session:

- From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded

Agreements via email - offline 103:

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

- At least the following are FFS in Rel-17 NTN:

- Report UE-calculated TA in e.g. msg3/msg5/msgA

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

- LCP impact caused by disabling HARQ UL retransmission

At RAN2#113e the following agreements were made

Agreements:

- Both Type 1 and Type 2 configured grant are feasible in NTN.

- From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN.

- No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN.

- UE in NTN can have both 2-step RACH and configured grant configurations at the same time.

Agreements via email - from offline [103]:

- For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started.

- FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled.

Agreements online:

- From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8)

- For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

In this paper, we discuss HARQ, scheduling, DRX, configured grant scheduling and SPS issues in adapting NR to NTN.

# HARQ in NTN

In this section we go through the necessary changes to HARQ handling and scheduling for support of NTNs.

## Enabling and disabling HARQ feedback and HARQ retransmissions

TR 38.821 [1] states that the enabling / disabling of HARQ feedback should be configurable on a per UE and per HARQ process.

For UL the, TR states that enabling / disabling of HARQ uplink retransmissions could be configurable on a per UE, per HARQ process and per LCH basis.

The justification part of the WID, [2], is more limited than the TR, the WID says:

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

RAN1 addressed the number of HARQ processes in this agreement:

RAN1#102-e:

Agreement:

The extension of maximal HARQ process number can be considered with following assumptions:

- The maximal supported HARQ process number is up to 32.

- FFS: Support on the maximal HARQ process number is up to UE capability

- Minimizing the impacts on specification and scheduling

The benefit of disabling HARQ processes for NTN is to enable the gNB and UE to reuse a HARQ process ID before a full HARQ RTT has elapsed. This enables peak throughput even when the number of available HARQ processes IDs (HP IDs) are less than the number of slots during the NTN HARQ RTT. The drawback is that there can be no HARQ retransmissions on the reused HARQ process ID. Since the HARQ feedback for downlink transmissions becomes useless for HARQ processes IDs, that are intended to be reused before the feedback arrives at the transmitter, HARQ feedback shall be disabled for those DL HP IDs.

There is no HARQ feedback for uplink transmissions in Rel-15 NR.

There is no HARQ feedback for uplink transmissions, besides for NR-U that is unlikely to be usable for satellite operation. In the case of NR-U where there is HARQ feedback for UL transmissions, the gNB can always decide to not send any HARQ feedback at all or send ACK = allow new transmission on that HP ID (if the HP ID is configured for CG) or send NACK = UE will send a retransmission on that HP ID (if the HP ID is configured for autonomous CG retransmissions).

The UL HARQ feedback for NR-U is controlled by gNB and gNB may select not to send it, if sent it can be used to get retransmissions or new transmissions on a HP ID configured for configured grants.

This means in Rel-16 the gNB is in control of what the UE shall do per HP ID and there is no need to introduce an artificial restriction for some UL HP IDs (no need to disable UL HARQ retransmissions for some HP IDs).

There is no meaning in the sentence from the TR “enabling / disabling of HARQ uplink retransmissions could be configurable on a per UE, per HARQ process and per LCH basis” besides for LCP and monitoring of the PDCCH which we discuss further in the sections below on LCP and DRX. The disabling of uplink HARQ retransmissions was intentionally left out of the WID.

The RAN2 WID objective “Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]” has been met.

The UE shall always follow the grants/assignments that the gNB send to it, be it with or without the NDI toggled. If the UE receives a retransmission grant for a HARQ process that have “disable uplink HARQ retransmission” it is ambiguous what the UE shall do.

The UE shall always follow the received grants and assignments as in legacy.

Disabling uplink HARQ retransmissions cannot mean that the UE can ignore a received grant

There is no support in the WID for “disable uplink HARQ retransmissions”. RAN2 will not further study solutions for enabling/disabling uplink HARQ retransmissions.

### Continuously scheduling the uplink

RAN1 specifies restrictions on when a grant for a specific HARQ process can be sent to the UE, see 38.214 clause 6.1:

The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

The interpretation of last sentence for UL scheduling is up for discussion in RAN1, but the conservative interpretation (in Rel 15) is that the UE cannot receive a new grant for same HP ID before the end of the PUSCH transmission. Thus, about one slot (depending on CORSET, timing advance, Tproc2 and SCS, see 38.214 section 6.4) must elapse before gNB can reuse (retransmission or new transmission) the same HP ID in an UL grant. This is acceptable, and with a few HARQ processes it is possible to achieve peak throughput and avoid HARQ stalling using only NR Rel-15.

In NTNs functionality from NR Rel-15 support scheduling the UL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after one slot.

Today in MAC spec one HP ID is not expected to be reused while a drx-HARQ-RTT-TimerUL is running:

-    drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

There is no reason for restricting sending retransmission grants while drx-HARQ-RTT-TimerUL is running when considering the RAN1 limitations of reusing HP IDs for any grant (new or retransmissions). If the UE is listening to the PDCCH, because of some reason, and RAN1 spec allows a grant, then MAC spec shall not limit the UE to receive a retransmission grant.

While drx-HARQ-RTT-TimerUL is running for an HARQ process, the UE can expect grants for new transmissions or retransmissions for that HARQ process.

In Figure 1, we have illustrated a TN when the propagation RTT is small in relation to the slot length and the grant (with k2=1) is sent in one of the first symbols in the DL slot and the long Tproc2 is used. The gNB can send grants (G1/G2) continuously using only two HARQ processes, and each HARQ process ID is reused before one HARQ RTT has elapsed.

Figure 1 Continuous uplink grants and transmissions with small propagation RTT.

In Figure 2, we have illustrated an NTN when the propagation RTT is 8 slots, TA is tuned to be 8 slots, and Koffset+k2 is 9 slots and otherwise the same UE processing time. Here the gNB can, again, continuous transmit grants using only two HARQ processes and for the UE there is no difference other than the numbering of the slots differs compared to TN example in Figure 1.

Figure 2 Continuous uplink grants and transmissions with long propagation RTT.

In Figure 3 we have illustrated an NTN with two UEs at different propagation RTT using the same Koffset+k2. The UE2 is at the cell edge experiencing the maximum propagation RTT supported by this Koffset+k2, while the UE1 has a shorter propagation RTT. In this case the gNB can continously send grants to UE2 using only two HARQ processes, while for UE1 six HARQ processes are needed.

Data transmission for both UEs in Figure 3 have the same delay, as they have the same Koffset+k2.

Figure 3 NTN with two UEs at different delay using the same Koffset+k2

In Figure 4 we have illustrated the same two UEs as in Figure 3 but we have adapted the individual Koffset+k2 to match TA+1 of each UE. Here we can see that we only need two HARQ processes for each UE to continously schedule them, each UE have a different delay, and the grants for transmissions that are received in the same slot are sent at different times from the gNB.

Figure 4 NTN with two UEs at different delay using the individual Koffset+k2

### Continuously scheduling the downlink

RAN1 specifies restrictions on not reusing a downlink HARQ process ID (for retransmission or new transmission) in 38.214 clause 5.1:

The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6]

…

Thus, for DL, the gNB cannot send a new DL assignment for a given HARQ process ID until the end of the HARQ-ACK transmission, this corresponds to a UE processing time for receiving an assignment and preparing and transmitting a HARQ feedback transmission, we may call this time period W. In TNs, W is approximately half the RTT (assuming UE and gNB processing times are similar), and the UE processing time will not change in NTNs where the propagation time is significantly longer. Thus, for NTNs, if W is the same it will be possible to schedule the UE continuously in the DL by using a number of HPs to cover W. In TNs that is (about) half of the HP IDs.

This means the NW can send a new assignment (retransmission or new transmission) on a HP ID while the previous assignment on for the same HP ID is still propagating through the air, but the same HP IDs will always be at least W apart in time at any location (e.g., when received at the UE).

In NTNs functionality from NR Rel-15 support scheduling the DL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after a time period corresponding to the TN time between receiving a DL PDSCH until after transmitting the HARQ feedback.

At RAN1#104e the following agreement was made

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

When no HARQ feedback is to be transmitted, it is thus possible to reuse a HARQ process X after the end of the PDSCH.

When DL HARQ feedback is disabled, the gNB can reuse a HARQ process X after the end of the PDSCH transmission.

Today in MAC spec one HP ID is not expected to be reused while a drx-HARQ-RTT-TimerDL is running:

-    drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

There is no reason for restricting sending retransmission assignments while drx-HARQ-RTT-TimerDL is running when considering the RAN1 limitations of reusing HP IDs for any assignment (new or retransmissions). If the UE is listening to the PDCCH, because of some reason, and RAN1 spec allows an assignment, the MAC spec shall not limit the UE to receive a retransmission.

While drx-HARQ-RTT-TimerDL is running for an HARQ process, the UE can expect assignments for new transmissions or retransmissions for that HARQ process.

We discuss the DRX timers further in section 4.3 below.

### Blind retransmissions

At RAN2#113e the following agreements were made

Agreements via email - from offline [103]:

- For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started.

- FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled.

At RAN1#104e the following agreement was made

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

This RAN1 agreement with Proposal 6 (or setting drx-HARQ-RTT-TimerDL to zero) resolves the FFS of the RAN2 agreement as the gNB can send a new assignment (for new or retransmission) X after the end of the PDSCH transmission.

Further methods for blind retransmission for HARQ processes with HARQ feedback disabled are not needed.

## Logical Channel Prioritization

By toggling or not toggling the NDI, the gNB will control if an uplink grant will be for a retransmission or a new transmission. This is independent of if RAN2 chooses to use signalling (for example RRC configuration) for disabling uplink HARQ retransmissions or not.

Some important data (for example a BSR MAC CE where the MAC entity will cancel the BSR when it is transmitted) may be lost with fewer (or zero) HARQ retransmissions, but HARQ failure happens in legacy too (but less common).

In legacy, UL HARQ failure can happen.

For important LCHs, usually the setting for polling (pollPDU, pollByte) will result in frequent polling and in case a PDU is sent with the poll bit set and no status report is received, then the UE will autonomously retransmit the latest SDU when t-PollRetransmit expires if no status report for that SN has been received.

For BSRs we can tune the retxBSR-Timer and the periodicBSR-Timer to counter a higher HARQ failure rate.

Is there any UL data that really require HARQ retransmissions? We do not think so.

There is no uplink data that always require HARQ retransmissions.

### LCP with “disable uplink HARQ retransmissions”

Changing logical channel prioritization (LCP) to have some LCHs only allowed on one HP ID type means sometimes delaying data due to the wrong type of HP ID in a grant. Possibly we must then create a new Scheduling Request for this “SR for data requiring HARQ retransmissions” and/or “SR for data not requiring HARQ retransmissions”.

If some LCH is not sent because a grant has the “wrong” type of HP ID, the LCH will be delayed, probably it will require an SR for every time this happens to make the gNB aware. This will occur every time the gNB schedules with the “wrong” HP ID type, which on average will add longer delay than not changing LCP at all because of the added delay of SR/BSR procedure.

Changing the LCP procedure to restrict LCHs using HP IDs with or without retransmissions will incur delay and possibly require new type of SRs for LCHs blocked in LCP.

### Legacy LCP in NTN

As the gNB often knows what data the UE has from the BSRs and SRs (each LCH may have its own SR configuration) and from the received data in previous decoded TBs, the gNB can estimate the UE buffer content and schedule accordingly. If some data would require retransmissions, gNB can schedule and not reuse the HP ID until gNB has decoded the transmission correctly after zero or more retransmissions.

Using an uplink HARQ process of wrong HP ID type will be a rare event as gNB can estimate what data the UE has in its buffer from SRs, BSRs and decoded received data.

If some LCH is sent using “wrong” type of HP ID, if the gNB decodes the TB correctly, all is fine. If gNB fails to decode a TB, the gNB can immediately send an RLC status report (and possibly an UL grant) to trigger an early RLC retransmission (it does not matter if the lost TB have data that is more or less important, gNB knows it is lost).

If UL decoding fails, gNB implementation can proactively send an RLC status report to trigger early RLC retransmission.

Link adaptation in NTNs will most likely aim at a lower block error rate (BLER) because the pathloss differences are smaller over a cell and the long RTT has severe drawbacks as retransmissions are delayed for long. A commonly used BLER in TNs is 10%, in NTNs the target may be as low as 1% or even lower.

Most of the time gNB knows what data the UE has and even when retransmissions is not used >95% of the transmissions will be successful (assuming link adaptation adjust for <5% BLER, likely BLER target will be even lower) so block errors will be rare.

Block errors when scheduling data without retransmissions will be rare, thus not using retransmission for important data will likely not lead to failed transmissions.

The gNB is aware of the decoded data and failed decoding and can adapt the scheduling and/or link adaptation and/or the gNB estimation algorithm for UE buffer status.

When scheduling data without retransmission, gNB can detect and adapt scheduling and/or link adaptation and/or the estimation of the UE buffer status.

The IIoT WI introduced means to specify in LCP if a LCH may use a specific dynamic grant or configured grant and other parameters that affect the QoS for a LCH, see 38.321 section 5.4.3.1.1:

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission.

Legacy parameters allow reserving a certain type of grant for some LCHs and to control the QoS of each LCH.

The logical channel prioritization is not updated for NTNs.

See section 2.5 for more methods that the gNB can use to improve the communication without any change to the LCP.

## Using drx-HARQ-RTT-TimerUL for motivating signalling to “disable uplink HARQ retransmissions”

It is a rare case that drx-HARQ-RTT-TimerUL makes a difference (drx-HARQ-RTT-TimerUL can only take effect at the end of an UL data burst as any new grant/assignment will restart the drx-InactivityTimer) and, as BLER target will be lower in NTNs than in TNs, retransmissions will be rare in NTNs.

We can use the value zero for both drx-HARQ-RTT-TimerUL and drx-RetransmissionTimer and if a retransmission is needed the gNB can rely on the drx-InactivityTimer or the drx-OnDurationTimer to reach the UE. This may sometimes give a small increase in delay – but relative to the RTT this will be a minor increase, and it will happen seldom as the BLER target will be lower in NTNs.

Usually drx-InactivityTimer will be in the order of 100s of ms to cover TCP ACKs, so no problem in LEO. In GEO the relative increase compared to the RTT, of 541 ms, is minor.

Using the value zero for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL will not increase delay in LEO scenarios and will on rare occasions give a small relative delay increase in GEO scenarios.

Regarding having the same behaviour for drx-HARQ-RTT-TimerUL and drx-HARQ-RTT-TimerDL, today they already have the following differences:

- Described on different lines in the spec

- Started at different points in time on separate lines in the spec

- Different actions at expiry that are described at different places in the spec

- Configured separately in RRC

They are only similar in the name, thus there is no reason to try having the same agreements for them in NTNs.

The drx-HARQ-RTT-TimerUL is not differentiated based on if retransmissions are used for a HARQ process.

DRX is discussed further in section 4 below.

## Scheduling flexibility

If gNB need to schedule important data with retransmissions, but the HARQ processes with retransmissions are all in use – the important data may be delayed.

Splitting the HARQ processes into groups based on if retransmissions are enabled or disabled will require new management algorithms for this in the gNB (and signalling when redistribution of the HARQ processes is needed), but more serious is that if there are available resources in the gNB at the end of a data burst, the gNB may not be able to schedule a retransmission because the HARQ process was configured with ”disable uplink HARQ retransmissions” (leading to added delay at the burst end).

Splitting uplink HARQ processes in two groups limits gNB scheduling flexibility and may incur increased delay at the end of a data burst. It also leads to signalling overhead for configuring and manage the two groups.

## Handling HARQ failures without HARQ retransmissions

If there are no HARQ retransmissions we must rely on RLC AM for robustness. Here we consider how to improve the communication by increasing the frequency of RLC status reports and by enabling the UE and gNB to faster detect that there are outstanding RLC AM SDUs that may need retransmission as well as increasing the gNB knowledge of the UE uplink buffer content.

The gNB may tune the RLC parameters t-PollRetransmit, t-Reassembly, pollPDU, pollByte and t-StatusProhibit of each bearer to increase the frequency of status reports and handle the long RTT.

For the DL, the gNB may proactively poll for RLC status reports more often to increase the number of status reports so that RLC retransmissions can be faster if a HARQ transmission fails.

For the UL, the gNB may proactively send RLC status reports, for example if a HARQ transmission fails, however it is difficult for the gNB to know which LCH the data belongs to.

### Adding in-flight BSRs

To help the gNB in estimating what data is lost in transmissions to know which LCH to send RLC status reports for, we propose to add an “in-flight BSR”. The in-flight BSR shall report the RLC AM buffer status of RLC AM data that is waiting for RLC ACK/NACK (the data in RLC that will be retransmitted if an RLC NACK is received).

When the gNB receives the BSR with in-flight buffer indication it can compare the LCH data received with the in-flight buffer indication and deduce that data has been lost since the RLC entity has in-flight buffer but no data was received.

Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

For example, in uplink if RLC data lost by HARQ failure did not contain an RLC poll (not the last data in the RLC buffer), RLC receiver in gNB need to detect an outstanding RLC SN (if next transmission do not fail HARQ) and start t-Reassembly, then RLC t-Reassembly need to expire and gNB can send a status report. With the new BSR, the gNB can send an RLC status report immediately when a new BSR is received, which may be at the same time as the t-Reassembly would be started in this example.

For example, in uplink if RLC data lost by HARQ failure did contain an RLC poll (and it was the last data in the RLC buffer), the RLC transmitter in the UE would after t-PollRetransmit expires send a new poll (if UE has some new RLC data to send for the bearer, or blindly retransmit some RLC data waiting for RLC ACK/NACK if the RLC new/retransmission buffer is still empty) and the gNB would then reply with a RLC status report. With the new BSR, the gNB can send an RLC status report immediately if a new BSR is received, which may be long before t-PollRetransmit expires.

The in-flight BSR may be triggered periodically or triggered from the RLC entity (for example after a poll bit is sent then the following UL transmission can have an in-flight BSR in case the poll PDU failed in HARQ). It may be triggered after a certain number of HARQ transmissions.

### Polling for BSR

To improve the gNB knowledge of the data the UE has in the buffer, we can have the gNB poll for a BSR.

Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

This will be beneficial if BSRs happens to fail in HARQ or gNB estimate of UE buffer content indicates the UE has data but the gNB did not receive any BSR lately.

# Efficient UL scheduling and latency reduction

One major hurdle for enabling faster uplink transmissions is for the UE to quickly request and receive uplink resources in order to transmit its UL message. There are currently a set of options to reduce the latency until the UE can make use of the uplink resources. One option is for the UE to make use of SR request on PUCCH resources.

## SR-BSR procedure

If the UE buffer status changes and a BSR is triggered to be sent to the network, the UE first needs to acquire UL resources to send the BSR. If the UE does not have any PUSCH resources to transmit the BSR and has no ongoing DL signaling with accompanying PUCCH resources, the UE must send a scheduling request, SR, if configured for the UE, otherwise the UE shall use the random access procedure to request UL resources.

The SR-BSR framework is commonly used but the problem related to this is that it takes several steps before the network is made aware of the buffer status and even longer before the network can properly schedule the UE. This is seen in Figure 1 where it takes 4 steps before the UE can be scheduled properly according to the buffer size.

Figure 5. Propagation and processing delays.

While this procedure has advantages such as resource efficiency since the network will not over-provision UE resources when not needed, there are several cases where the number of steps could result in undesired delay. Since each step is associated with a propagation delay, the total delay for NTN with long RTT may have a profound impact on the overall delay.

The SR-BSR procedure may introduce a large scheduling delay due to the long RTT in NTNs.

## The sr-ProhibitTimer

For the case that we rely on the SR-BSR procedure, we need to adjust the sr-ProhibitTimer to handle the large RTT of NTNs.

For prioritized services we may allow the UEs to send multiple SRs during an RTT, to decrease the delay in case the gNB do no detect the first SR. Therefore, this timer shall be extended with higher values in the value range instead of adding an RTT offset to the value.

The sr-ProhibitTimer in NTN is sr-ProhibitTimer-r15 + factor*RTT where factor can take values lower than one and RTT is the round-trip time as used for other timers in MAC, factor and RTT are new RRC parameters.

## 2-step Random access for BSR

The SR-BSR procedure takes at least two HARQ RTTs before the gNB send the UE a grant based on the BSR. We may provide the UE with a configured grant, where the UE can send its buffer status at an early stage. But configured grant requires a lot of resources with short periodicity in order to reduce the delays, which may be wasteful.

Using configured grant to handle BSRs may not be resource-efficient.

An alternative could be to use the 2-step random access procedure i.e. to utilize the preconfigured contentious 2-step RA resources to allow for transmission of BSR when the UE does not have any PUSCH resources. Compared to configured grant, this can be more resource-efficient due to the fact that several UEs can be multiplexed on the same 2-step RA resources, allowing for more frequent MsgA PUSCH allocations which in-turn allows for faster network scheduling due to that the buffer status report reaching the gNB in a shorter time compared to the SR-BSR procedure.

This can be achieved in different ways, for example by letting a triggered BSR trigger a 2-step RA, or by letting an SR due to BSR trigger a 2-step RA. By not configuring PUCCH SR resources and having an TB size for MsgA that can accommodate the C-RNTI MAC CE and the BSR MAC CE, the UE will send BSR in MsgA. We observe

SR due to BSR can trigger a 2-step RA, if we do not configure PUCCH SR resources, and BSR can be included in MsgA if grant is big enough.

# DRX enhancements for NTN

The DRX is about when the UE shall monitor PDCCH, and the UE may save energy if it does not need to monitor PDCCH.

## Discussion on uplink and downlink relative timing

In 38.211 we have the uplink-downlink timing relations at the UE:

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start  before the start of the corresponding downlink frame at the UE where  is given by [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used.

Figure 4.3.1-1: Uplink-downlink timing relation.

Each frame is 10 milliseconds. In TNs the UL and DL are reasonable aligned, for example for cell sizes of up to 150 km, the TA is less than a millisecond.

In NTNs the RTT and the timing advance will be much longer than one frame. Thus, at the UE, the uplink frame i do not overlap in time with the downlink frame i.

In NTNs at the UE, the uplink system frame i will never overlap in time with downlink system frame i. This is true when timing reference point is at the gNB and at the satellite.

When UL and DL are aligned at the gNB, basing the start of the ra-ContentionResolutionTimer or DRX timers on the uplink timing is not logical as ra-ContentionResolutionTimer is for deciding what the UE shall do on the downlink (monitor PDCCH). Therefore, we suggest implementing the starting of the ra-ContentionResolutionTimer and the DRX timers from the corresponding symbol of the downlink frame instead of the uplink frame. We observe:

Basing the start of UE timers, for monitoring PDCCH, on the UL timing will require a timing offset and risk starting the UE timer late or early when Timing Advance is varying or is not correct.

When UL and DL are not aligned at the gNB, basing the start of UE timers, for monitoring PDCCH, on the DL timing will require a timing offset and risk starting the UE timer late or early.

When UL and DL are aligned at the gNB, basing the start of UE timers, for monitoring PDCCH, on the DL timing will start the UE timer at the correct time no matter what the timing difference between DL and UL is at the UE and no timer offset is required.

## Discussion on UE Active time

The UE is required to monitor the PDCCH when the UE is in Active time. When in Active time, the UE is also required to monitor the PDCCH continuously and is thus reachable from the network. Part of the times when the UE is active, are controlled by timers configured by the network such as C-DRX related timers and RA related timers. A UE with a C-DRX configuration can leave active time and discontinuously monitor the PDCCH to save energy.

During the Rel-16 SI on NTN, it was acknowledged that an NTN UE will need an offset for the drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL, to allow the UE not to enter Active time too soon and to avoid monitoring the PDCCH in vain. The UE is also allowed to enter Active time triggered by certain events. See below, pasted from TS 38.321:

------------------------------------------------------------------------------------------------------------

When a DRX cycle is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

------------------------------------------------------------------------------------------------------------

When a UE has a sent an SR, and it is pending, it will enter Active time and start monitoring the PDCCH as described in the yellow highlighted part above. Given the large RTT of an NTN, the UE can’t expect a response to the SR until RTT ms has elapsed.

With current DRX procedure, the UE will monitor the PDCCH in vain for RTT ms after sending an SR.

Figure 6. Example of the active time period after sending SR.

Following the proposed way of handling DRX and RA timers, it is proposed to introduce an offset to when a UE should enter Active time and start monitoring for a response to an SR. Meanwhile, the UE should make use of DRX and thus be reachable by the network while waiting for the response to the SR.

For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

When uplink and downlink are aligned in the gNB, we can start the monitoring of PDCCH based on the DL timing instead of using an offset after sending the SR (using for example the used Timing Advance as offset) as this gives the exact timing for the DL.

When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

If monitoring the PDCCH after Scheduling Request transmission is not as described in Proposal 14 then the UE need a timing offset that risk monitoring late or early.

When the UE performs CFRA it will not start the contention resolution timer that controls the PDCCH monitoring during CBRA. Thus, the green highlighted part above, from TS 38.321, requires the UE to enter Active time and start the monitoring of the PDCCH. As such, and similar as for sending SR, the UE will enter Active time and start monitoring the PDCCH already after receiving Msg2 (RAR). For CFRA, the UL message sent on PUSCH, according to the grant received in the RAR, is by the network seen as an acknowledgment that the UE received the RAR message and the network will typically not schedule the UE before the granted transmission in the RAR message has been received. Given the RTT of an NTN, the UE will therefore not receive a response until at least RTT ms has elapsed.

For CFRA, the network typically waits to receive msg3 granted in the RAR, as an acknowledgment that the UE has received the RAR, before it schedules the UE.

With current DRX procedure, after receiving the RAR message for CFRA, the UE will monitor the PDCCH in vain for at least RTT ms if the network does not schedule the UE in the meantime.

We propose to introduce a configurable offset to when a UE should enter Active time when doing CFRA. When the UE has sent the PUSCH message as granted in the received RAR, it would not expect a response until RTT ms have elapsed if the network does not schedule the UE in the meantime. Thus, the UE can offset the start of the active time by RTT ms.

In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

Thus, if gNB intends to wait for the msg3 as an ACK that the UE have received the RAR before it schedules the UE, then gNB can configure the UE to save energy after sending msg3 by not monitoring PDCCH until the corresponding DL slot.

If monitoring the PDCCH after msg3 transmission in CFRA is not as described in Proposal 16 then the UE need a timing offset that risk monitoring late or early or it will waste energy by always monitoring.

## Discussion on HARQ and DRX for NTN

Existing DRX and HARQ procedures at the PHY/MAC layer have been designed for terrestrial networks where the round-trip propagation delay is typically restricted to be within 1ms. With existing HARQ protocol, the transmitting side needs to wait for the feedback from the receiving side before the HARQ process ID can be reused to retransmit or to send new data. This stop-and-wait (SAW) procedure introduces inherent latency to the communication protocol, which may reduce the link throughput. To alleviate this issue, the existing HARQ procedure allows activating multiple HARQ processes with different IDs at the transmitter (N-SAW). That is, the transmitter may initiate multiple transmissions in parallel without having to wait for each individual HARQ process’s acknowledgment. For example, in NR with a maximum of 16 HARQ process IDs, the gNB may send 16 DL transport blocks on separate HARQ processes without waiting for an acknowledgment. If the RTT is longer than 16 slots (assuming each slot is scheduled with on HARQ process), the NW must stall future DL TBs until feedback is received. When this HARQ stalling occurs, also the UE needs to wait about RTT milliseconds before any new or retransmitted TBs can be received and will thus monitor the PDCCH in vain, draining the UE battery.

With disabled HARQ feedback for DL transmissions, this HARQ stalling and wasted monitoring of PDCCH is avoided. Similarly, a few uplink HARQ processes can be used for continuously transmissions in uplink and the HARQ stalling is avoided. The PDCCH monitoring due to drx-HARQ-RTT-TimerDL(UL) expiring and starting drx-RetransmissionTimerDL(UL) is further discussed in the following sections.

### On DRX with HARQ enabled

When reliability is prioritized over bitrate and for delay insensitive traffic scenarios, the HARQ protocol is still seen as a needed framework to ensure correctly received and acknowledged packets. In these cases, HARQ stalling will be seen whenever the RTT (in slots) are larger than the available number of HARQ processes. The last DCI received on PDCCH with the NDI toggled to indicate new data will restart the drx-InactivityTimer, controlling the length of PDCCH monitoring before entering DRX. If the most recently received DCI makes use of the last available HARQ process ID, the UE will be stalled, and no new data will be received until RTT ms has elapsed from when the oldest, not yet acknowledged, HARQ process ID was received (see Figure 7). Since the drx-InactivityTimer is running, the UE will be forced to monitor the PDCCH in vain, wasting energy (see Figure 8). This monitoring can be minimized by always sending a MAC CE in the DL TB using the last available HARQ process ID, forcing the UE to enter DRX. This would however cause some overhead to be sent every x configured HARQ process IDs, which might be undesirable when the HARQ process IDs are limited.

Figure 7. Stalling due to limited number of HARQ processes.

To allow the UE not to monitor the PDCCH in vain when HARQ staling occurs, the UE should be allowed to enter DRX upon detecting the stalling conditions, i.e. when all configured HARQ process IDs have been received. This would enable energy savings by not forcing the UE to monitor the PDCCH while the drx-InactivityTimer is running while the stalling condition is true.

Allow the UE to enter DRX when HARQ stalling occurs.

For NTN with long RTTs and commonly used values configured for the drx-InactivityTimer, the UE will enter DRX due to HARQ stalling when the drx-InactivityTimer expires. As the end of a RTT, as seen from the oldest not yet acknowledged HARQ process ID, will not in general coincide with a multiple of the UE configured DRX cycles (see Figure 8), the network will need to delay the next to be transmitted packet to the first available onDuration period after that RTT milliseconds have elapsed, causing an extra unnecessary delay introduced by DRX.

Figure 8. Unnecessary energy consumption and extra delay caused by DRX in NTN due to HARQ stalling.

If a longer DRX cycle is used to minimize the monitoring and the energy consumption, the delay due to DRX could be in the order of the RTT. Since the RTT of an NTN is significant, RAN2 should discuss how to minimize this delay.

It has been agreed that an offset is added to the HARQ RTT timers. For the UL, the retransmission timer is always started when the HARQ-RTT-UL expires and will make the UE monitor the PDCCH during the time of the retransmission timer and would probably solve the problem of the extra delay added by the use of DRX. But in the DL, the retransmission timer is only started if a decoding error has occurred and thus, if no error occurred, the UE will not start the retransmission timer and will not wake up to monitor the PDCCH even if an offset is used for the HARQ RTT.

So, in DL, a short DRX cycle will lead to that the UE monitors the PDCCH in vain during its onDuration periods but minimizes the time between the end of the RTT and the first available onDuration. A long DRX cycle avoids unnecessary monitoring during the RTT but will with high probability introduce a delay in the same order as the RTT which should be avoided.

One way to resolve this would be to allow the UE to wake up RTT milliseconds after the first not yet acknowledged DL TB was received regardless if this coincides with the UE onDuration period or not (see Figure 8). This would avoid the extra delay caused by DRX and enable the network to send new DCIs on PDCCH as soon as RTT milliseconds have elapsed.

Allow the UE to wake up after RTT ms regardless of DRX state to minimize the delay introduced by DRX and HARQ stalling

### On DRX with HARQ disabled

#### DRX and HARQ feedback enabled per HARQ process ID for downlink TB

If HARQ is disabled but enabled for a specific set of HARQ process IDs, the DRX framework should not force the UE to wake up to monitor the PDCCH for retransmissions for all HARQ processes, just the HARQ process IDs that have HARQ feedback enabled.

With the interpretations of TS 38.321 as in the TR 38.821, the drx-HARQ-RTT-TimerDL plus an RTT offset would then only be started for the HARQ process IDs that have HARQ feedback enabled and thus the drx-RetransmissionTimerDL will only be started if a negative acknowledgement is sent.

The current DRX framework plus an RTT offset allows the network to configure the UE with suitable DL DRX timers that will not force the UE to monitor the PDCCH for retransmissions that will never occur but will enable monitoring when needed.

At RAN2#113 we agreed:

- For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

When the drx-HARQ-RTT-TimerDL is accurately copnesated for the RTT, then drx-RetransmissionTimerDL will be started at the correct time and there is no need to extend it.

There is no need to extend the drx-RetransmissionTimerDL.

#### DRX and HARQ retransmissions for uplink TB

The drx-HARQ-RTT-TimerUL is started a bit differently when compared to the downlink. The drx-HARQ-RTT-TimerUL is always started after a PUSCH transmission. In TS 38.321, sub-clause 5.7, it says:

…

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

…

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

…

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

Thus, for UL transmissions, the retransmission timer is started, and the UE will face the risk of monitoring the PDCCH in vain when HARQ retransmission is not used. Like DL, the drx-RetransmissionTimerUL is not started if the drx-HARQ-RTT-TimerUL is not started.

If the drx-InactivityTimer is running, the UE already monitors the PDCCH and the start of the retransmission timer has no effect. If the drx-InactivityTimer has expired, the UE will wake up to monitor for retransmissions, but no retransmissions grants will be sent when HARQ retransmission is not used. This can be avoided by setting the drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL to zero. Then the network must configure the drx-InactivityTimer, drx-OnDurationTimer, and other parameters of the DRX cycle, in a way that the network can reach the UE without too much delay.

Using zero value for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL will only have an effect at the end of a transmission burst when the UE have no more data to send (when drx-InactivityTimer is not restarted by new UL transmissions and drx-RetransmissionTimerUL could have started if its value was not zero) and the UE is waiting for further grants. Any extra delay added by not utilizing drx-RetransmissionTimerUL will be relatively small compared to the long RTT of NTNs.

For uplink HARQ processes in NTN, the existing DRX framework is well adapted to avoid unnecessary PDCCH monitoring for UL grants.

No NTN adaptation needed for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL.

Since the NW is in control of the HARQ process IDs and it has knowledge of the UE’s DRX cycle, it can always reach the UE in any of the configured onDuration periods by sending a new DCI that will be received when the UE monitors the PDCCH. The NW also control if HARQ retransmission is needed for some HP ID, the NW then do not reuse the HP ID for new transmissions, though it may reuse it for blind retransmissions, until a RTT has elapsed and based on the reception of the HARQ process decide if a retransmission is needed.

The NW is in control of all uplink HARQ transmissions, be it new or retransmissions.

# Configured grant

For a configured grant configuration, the gNB may configure configuredGrantTimer or not.

If configuredGrantTimer is configured for a CG-config, it means HARQ processes will not be used for a new CG transmission while it is running, and this allows the gNB to schedule retransmissions of that HP ID, if needed.

If configuredGrantTimer is not configured for a CG-config, it means HARQ processes can be used for new CG transmission at any point in time, retransmissions of a HP ID may not be possible except if periodicity is longer than the HARQ RTT.

Configured grants can be configured with or without HARQ retransmissions in Rel-15. No spec change needed to support CG in NTNs.

For the DRX HARQ RTT timers we can, as proposed above, use the value zero and instead rely on inactivityTimer and or onDuration.

# Semi-persistent scheduling

When semi-persistent scheduling (SPS) is configured in NTNs, we can use the already agreed methods to enable/disable HARQ feedback per HP ID. Thus, we can configure the HP IDs of configured downlink assignment were some HP IDs are with HARQ feedback and some without.

DL SPS can use the already agreed method of enabling/disabling HARQ feedback in NTNs.

In 38.321 we have:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

While in 38.213 we have:

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release.

When SPS is configured and HARQ feedback happens to be disabled for one or more HP IDs, there is a question on how HARQ feedback shall be interpreted in the case of SPS deactivation.

In the DCI for SPS activation/deactivation with multiple SPS configurations, the HP ID field is used for indicating the sps-ConfigIndex.

We think the correct interpretation of the SPS deactivation is that the DCI is not related to any HP ID, and thus the HARQ feedback shall always be sent from the UE (for the gNB to know the UE have received an SPS deactivation).

HARQ feedback shall always be sent for SPS deactivation.

# Conclusion

In the above sections we made the following observations and proposals:

Observation 1	There is no HARQ feedback for uplink transmissions in Rel-15 NR.

Observation 2	The UL HARQ feedback for NR-U is controlled by gNB and gNB may select not to send it, if sent it can be used to get retransmissions or new transmissions on a HP ID configured for configured grants.

Proposal 1	The RAN2 WID objective “Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]” has been met.

Observation 3	The UE shall always follow the received grants and assignments as in legacy.

Observation 4	Disabling uplink HARQ retransmissions cannot mean that the UE can ignore a received grant

Proposal 2	There is no support in the WID for “disable uplink HARQ retransmissions”. RAN2 will not further study solutions for enabling/disabling uplink HARQ retransmissions.

Proposal 3	In NTNs functionality from NR Rel-15 support scheduling the UL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after one slot.

Proposal 4	While drx-HARQ-RTT-TimerUL is running for an HARQ process, the UE can expect grants for new transmissions or retransmissions for that HARQ process.

Proposal 5	In NTNs functionality from NR Rel-15 support scheduling the DL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after a time period corresponding to the TN time between receiving a DL PDSCH until after transmitting the HARQ feedback.

Observation 5	When DL HARQ feedback is disabled, the gNB can reuse a HARQ process X after the end of the PDSCH transmission.

Proposal 6	While drx-HARQ-RTT-TimerDL is running for an HARQ process, the UE can expect assignments for new transmissions or retransmissions for that HARQ process.

Proposal 7	Further methods for blind retransmission for HARQ processes with HARQ feedback disabled are not needed.

Observation 6	In legacy, UL HARQ failure can happen.

Observation 7	There is no uplink data that always require HARQ retransmissions.

Observation 8	Changing the LCP procedure to restrict LCHs using HP IDs with or without retransmissions will incur delay and possibly require new type of SRs for LCHs blocked in LCP.

Observation 9	Using an uplink HARQ process of wrong HP ID type will be a rare event as gNB can estimate what data the UE has in its buffer from SRs, BSRs and decoded received data.

Observation 10	If UL decoding fails, gNB implementation can proactively send an RLC status report to trigger early RLC retransmission.

Observation 11	Block errors when scheduling data without retransmissions will be rare, thus not using retransmission for important data will likely not lead to failed transmissions.

Observation 12	When scheduling data without retransmission, gNB can detect and adapt scheduling and/or link adaptation and/or the estimation of the UE buffer status.

Observation 13	Legacy parameters allow reserving a certain type of grant for some LCHs and to control the QoS of each LCH.

Proposal 8	The logical channel prioritization is not updated for NTNs.

Observation 14	Using the value zero for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL will not increase delay in LEO scenarios and will on rare occasions give a small relative delay increase in GEO scenarios.

Proposal 9	The drx-HARQ-RTT-TimerUL is not differentiated based on if retransmissions are used for a HARQ process.

Observation 15	Splitting uplink HARQ processes in two groups limits gNB scheduling flexibility and may incur increased delay at the end of a data burst. It also leads to signalling overhead for configuring and manage the two groups.

Proposal 10	Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

Proposal 11	Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

Observation 16	The SR-BSR procedure may introduce a large scheduling delay due to the long RTT in NTNs.

Proposal 12	The sr-ProhibitTimer in NTN is sr-ProhibitTimer-r15 + factor*RTT where factor can take values lower than one and RTT is the round-trip time as used for other timers in MAC, factor and RTT are new RRC parameters.

Observation 17	Using configured grant to handle BSRs may not be resource-efficient.

Observation 18	SR due to BSR can trigger a 2-step RA, if we do not configure PUCCH SR resources, and BSR can be included in MsgA if grant is big enough.

Observation 19	In NTNs at the UE, the uplink system frame i will never overlap in time with downlink system frame i. This is true when timing reference point is at the gNB and at the satellite.

Observation 20	Basing the start of UE timers, for monitoring PDCCH, on the UL timing will require a timing offset and risk starting the UE timer late or early when Timing Advance is varying or is not correct.

Observation 21	When UL and DL are not aligned at the gNB, basing the start of UE timers, for monitoring PDCCH, on the DL timing will require a timing offset and risk starting the UE timer late or early.

Observation 22	When UL and DL are aligned at the gNB, basing the start of UE timers, for monitoring PDCCH, on the DL timing will start the UE timer at the correct time no matter what the timing difference between DL and UL is at the UE and no timer offset is required.

Observation 23	With current DRX procedure, the UE will monitor the PDCCH in vain for RTT ms after sending an SR.

Proposal 13	For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

Proposal 14	When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

Observation 24	If monitoring the PDCCH after Scheduling Request transmission is not as described in Proposal 14 then the UE need a timing offset that risk monitoring late or early.

Observation 25	For CFRA, the network typically waits to receive msg3 granted in the RAR, as an acknowledgment that the UE has received the RAR, before it schedules the UE.

Observation 26	With current DRX procedure, after receiving the RAR message for CFRA, the UE will monitor the PDCCH in vain for at least RTT ms if the network does not schedule the UE in the meantime.

Proposal 15	In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

Proposal 16	When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

Observation 27	If monitoring the PDCCH after msg3 transmission in CFRA is not as described in Proposal 16 then the UE need a timing offset that risk monitoring late or early or it will waste energy by always monitoring.

Proposal 17	Allow the UE to enter DRX when HARQ stalling occurs.

Proposal 18	Allow the UE to wake up after RTT ms regardless of DRX state to minimize the delay introduced by DRX and HARQ stalling

Proposal 19	There is no need to extend the drx-RetransmissionTimerDL.

Observation 28	For uplink HARQ processes in NTN, the existing DRX framework is well adapted to avoid unnecessary PDCCH monitoring for UL grants.

Proposal 20	No NTN adaptation needed for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL.

Observation 29	The NW is in control of all uplink HARQ transmissions, be it new or retransmissions.

Proposal 21	Configured grants can be configured with or without HARQ retransmissions in Rel-15. No spec change needed to support CG in NTNs.

Observation 30	DL SPS can use the already agreed method of enabling/disabling HARQ feedback in NTNs.

Proposal 22	HARQ feedback shall always be sent for SPS deactivation.

Here is a list of the proposals made:

Proposal 1	The RAN2 WID objective “Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]” has been met.

Proposal 2	There is no support in the WID for “disable uplink HARQ retransmissions”. RAN2 will not further study solutions for enabling/disabling uplink HARQ retransmissions.

Proposal 3	In NTNs functionality from NR Rel-15 support scheduling the UL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after one slot.

Proposal 4	While drx-HARQ-RTT-TimerUL is running for an HARQ process, the UE can expect grants for new transmissions or retransmissions for that HARQ process.

Proposal 5	In NTNs functionality from NR Rel-15 support scheduling the DL continuously without using all HARQ processes available in TNs by reusing HARQ process IDs after a time period corresponding to the TN time between receiving a DL PDSCH until after transmitting the HARQ feedback.

Proposal 6	While drx-HARQ-RTT-TimerDL is running for an HARQ process, the UE can expect assignments for new transmissions or retransmissions for that HARQ process.

Proposal 7	Further methods for blind retransmission for HARQ processes with HARQ feedback disabled are not needed.

Proposal 8	The logical channel prioritization is not updated for NTNs.

Proposal 9	The drx-HARQ-RTT-TimerUL is not differentiated based on if retransmissions are used for a HARQ process.

Proposal 10	Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

Proposal 11	Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

Proposal 12	The sr-ProhibitTimer in NTN is sr-ProhibitTimer-r15 + factor*RTT where factor can take values lower than one and RTT is the round-trip time as used for other timers in MAC, factor and RTT are new RRC parameters.

Proposal 13	For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

Proposal 14	When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

Proposal 15	In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

Proposal 16	When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

Proposal 17	Allow the UE to enter DRX when HARQ stalling occurs.

Proposal 18	Allow the UE to wake up after RTT ms regardless of DRX state to minimize the delay introduced by DRX and HARQ stalling

Proposal 19	There is no need to extend the drx-RetransmissionTimerDL.

Proposal 20	No NTN adaptation needed for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL.

Proposal 21	Configured grants can be configured with or without HARQ retransmissions in Rel-15. No spec change needed to support CG in NTNs.

Proposal 22	HARQ feedback shall always be sent for SPS deactivation.

# References

TR 38.821 “Study on solutions for NR to support non-terrestrial networks”

RP-210908, Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#91, e-Meeting, March 2021