3GPP RAN WG2 Meeting #113bis-e	R2-2103972

eMeeting April 12th – April 20th, 2021

# Title:	[Draft] LS to SA WG3 on re-use of same NCC and I-RNTI value for RRC Resume procedure in different cells during small data transmission procedure.

# Release:	Release 17

Source:	InterDigital [to be RAN2]

To:	SA3

Contact Person:

Name:	Dylan Watts

E-mail Address:	Dylan.watts@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 has been discussing RRC-based small data transmission (SDT), where UE in INACTIVE state initiates a SDT session by transmitting an RRC message (e.g. RRCResumeRequest) along with small data in first UL transmission.  Upon initiating the resume procedure during SDT initiation, the UE re-establishes PDCP entities and applies the security keys for sending the data in SDT-DRBs and/or SRBs securely.  After initial UL transmission, multiple UL/DL packets can be transmitted/received on dedicated grants during the same SDT session while in RRC inaction (i.e. without UE transitioning to RRC_CONNECTED).   RAN2 design assumes SDT session is terminated upon reception of RRCRelease.

According to Rel-16, if UE transmits RRCResumeRequest and performs cell re-selection before receiving RRC response message (e.g. RRCRelease or RRCResume), UE transitions to IDLE. However, this may result in data loss if RRCResume procedure was used to initiate a SDT session since UE may transmit/receive multiple packets before an RRC response message is expected.

An alternative solution is for UE to remain in INACTIVE and attempt a new SDT session in new cell. However, per TS 33.501 UE is provided with updated I-RNTI and NCC in subsequent RRCRelease with suspendConfig messages. If UE attempts the new SDT session before completion of the first, it will not have received updated I-RNTI and NCC. One potential solution discussed in RAN2 is to temporarily allow re-use of the NCC and I-RNTI from the former cell to initiate SDT session in the new cell.

2. Actions:

To SA WG3

ACTION:	RAN2 kindly asks SA WG3 if NCC and I-RNTI from a former cell can be re-used to initiate an SDT session in a new cell, and to list possible security impacts (if identified).

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#114- e	 	May 19th – 27th, 2020		Online meeting