3GPP TSG-RAN WG2 Meeting #113b-e	R2-2103979

Online, 12 Apr – 20 Apr 2021

Agenda item:	8.2.2.3

Source: 	Huawei, HiSilicon

Title: 	SCG activation

Document for:	Discussion and Decision

# 1. Introduction

In the last meeting, RAN2 has the following agreements.

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 1b 	SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 3 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

- Agreements

- 1	NW-triggered SCG activation is indicated to the UE via the MCG.

- 9	NW-triggered SCG deactivation can be indicated to the UE via the MCG. FFS via SCG.

- 8	Further discuss the comparison between

- -  define a mechanism for SCG activation upon UL data arrival on SCG bearers

- -	use split bearer with primary path on MCG (network sees UL data and can initiate activation)

- 2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

- option 1)	similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

- option 2)	in certain cases:

- -	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

- -	the SCG can schedule data transmission on the PDCCH

- The UE decides not to perform random access (one option to be selected):

- option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

- option 2b) based on the contents of the SCG activation indication

- FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

- -	will perform random access upon reception of the next SCG activation indication from the MCG

- -	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

- 7	Further discuss the format and content of the SCG activation indication from the MCG to the UE after there is more progress on solution 2.

- 5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

- 6	Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

In this contribution, we will further discuss the SCG activation procedure.

# 2. Discussion

## 2.1 signalling flow

In the last meeting, RAN2 has agreed that SCG activation can be requested by MN/SN/UE. We discuss the signalling flows in these three cases respectively.

In our understanding, in most cases, the MN will request to (re)activate the SCG based on the data transmission requirements. We discuss this for the bearer types using the SCG, i.e. MN terminated split bearer, MN terminated SCG bearer, SN terminated split bearer and SN terminated SCG bearer (the MN terminated MCG bearer and SN terminated MCG bearer are not relevant to this discussion).

- For the MN terminated split bearer

For DL data arrival, the MN can see the DL traffic, so it can decide when to request the SCG (re)activation.

For UL data arrival,

If the UL PDCP duplication is activated (although we think the network will not deactivate the SCG when the duplication is used for the services), the MN can still receive the UL data. Therefore, the MN can decide when to request the SCG reactivation, e.g. based on the BSR or the data volume received.

If the UL PDCP duplication is not configured or activated and primary path is the MCG, the UE will transmit the UL data via the MCG. Therefore, the MN can know that UL data are arriving and decide when to request the SCG reactivation, e.g. based on the BSR or the data volume received.

If the UL PDCP duplication is not configured or activated and primary path is the SCG, the UE cannot transmit the UL data to MN because the SCG is deactivated. In our understanding, the activation should be decided by the network. Therefore, the UE can change the primary path to the MCG or the UE can inform the network of the UL data arriving via the MCG. In our understanding, it is better that the UE changes the primary path in order to reduce the delay of activation. Therefore, the MN can decide when to request the SCG reactivation, e.g. based on the data volume received.

- For the SN terminated split bearer

For DL data arrival, the SN can immediately forward the DL data to the MCG leg and then the MN knows that DL data are arriving and how much.

For UL data arrival, as discussed above for the MN terminated split bearer, the MN can receive the UL data from the UE and be aware of the amount of UL traffic.

Even though the SN is the termination point, the MN has sufficient QoS information to judge the need for SCG activation. Of course, this does not preclude optimizations to provide the MN with further information on how to make that judgment.

- For the MN/SN terminated split bearers, the MN can determine whether to activate the SCG based on QoS information and observed data volume on the MCG RLC bearers. The UE does not need request SCG activation because of UL data arrival because the UE can send the UL data via the MCG. In addition, DL/UL data will not be delayed until the SCG is activated.

-		For the MN terminated SCG bearers

For DL data arrival, the MN can see the DL traffic, so it can decide when to request the SCG activation.

For UL data arrival, if the SCG is deactivated, the UE cannot send the data without the SCG being activated first. The following alternative options could be supported (only one option is sufficient):

1)	the UE initiates random access towards the SCG

2)	the UE indicates to the MN (details FFS) that it has uplink data to transmit on an SCG bearer

With option 1, the UE would use contention-based random access unless the SCG reserves contention-free random access resources for the UE even though the SCG is deactivated. In addition, the SN would inform the MN that the SCG was activated. In this option, the SCG is activated without any RRC message and the configuration remains the same.

With option 2, upon receiving the UE indication, the MN would trigger the SCG activation involving the SN. If the MN and the SN are not co-located, this is likely to take some more time than option 1 but this is more flexible as it allows the MN and the SN to modify the UE configuration and/or allocate dedicated random access resources.

If delay for initial sending of UL data on a particular SCG bearer is a strong concern, the network may prefer to allocate an MCG RLC bearer (making it a split bearer) before deactivating the SCG. Therefore, option 2 seems sufficient.

-		For the SN terminated SCG bearer

In this case, only the SN can see the traffic. For DL data arrival, the SN could send an activity notification to the MN so that the MN initiates SCG activation. Alternatively, if SN-initiated modification can be used to trigger SCG activation, this procedure could be used.

If the SN and the MN are not co-located, SN-initiated modification could be a little faster because it requires 2 inter-node messages instead of 3, but if this triggers an MN-initiated SN modification there is no such benefit.

Besides, as already mentioned for MN-terminated SCG bearers, if delay for initial sending of UL data on a particular SCG bearer is a strong concern, the network may prefer to allocate an MCG RLC bearer (making it a split bearer) before deactivating the SCG.

For UL data arrival, the situation is the same like for MN-terminated SCG bearers.

- For the MN/SN terminated SCG bearers, the UE needs to notify the MN or access the SCG in case of UL data arrival. For MN-terminated bearers, the MN can initiate SCG activation upon DL data arrival. For SN-terminated SCG bearers, the MN can determine whether to activate the SCG based on the activity notification from the SN or the SN could initiate an SN modification to activate the SCG.

- For MN/SN terminated SCG bearers, SCG activation could be faster using direct access to the SCG (in case of UL data arrival) or SN-initiated SN modification (in case of DL data arrival) but only if this does not trigger any MN-initiated modification. However, allocating an MCG leg before SCG deactivation would avoid any delay.

Based on the above observations, we propose:

If the UE has uplink data to send on a split bearer while the SCG is deactivated, the UE sends the uplink data on the MCG leg, regardless of the primary path.

If the UE has uplink data to send on an SCG bearer while the SCG is deactivated, the UE informs the network via the MCG (details FFS).

Activity notification can be used to request SCG activation (there is no need to use SN-initiated modification for SCG activation).

Also in the last meeting, some companies think the UE can request the SCG activation for the MCG fast recovery. In our understanding, it can avoid the RRC reestablishment. Therefore, there are some benefits to support it. In MCG fast recovery, the UE send the MCG failure information to the MN via the SCG. In the last meeting, RAN2 has agreed that there is no PUSCH transmission on deactivated SCG. Therefore, the UE should resume the SCG transmission for the MCG fast recovery.

In case of MCG RLF while fast MCG link recovery is configured and the SCG is deactivated, the UE can perform contention-based access toward the SCG in order to resume transmission on split SRB1 or SRB3 and send the MCGFailureInformation message.

One question is whether, when the MN or the SN initiates SCG activation, the other node can reject the request.

For the MN initiated SCG activation, the SN is responsible for the resources of SCG. Therefore, the SN may reject the SCG activation request from MN. Also in the last meeting, RAN3 has agreed that the SN can send the SCG activation result after receiving the SCG activation request from the MN.

MN initiated SN modification procedure Add a new IE, e.g., “SCG activation requested” with two codepoints in the SN modification request message in order to indicate the SCG is requested to activate or de-activate. Add a new IE, e.g., “SCG activation result” with two codepoints in the SN modification response message in order to indicate the SCG is activated or de-activated.

If the SN requests activation (via activity notification), the MN may prefer to change all bearers to MN-terminated or to change SCG so the MN may not activate the SCG.

The SN can reject the SCG activation request from the MN. It has been agreed by RAN3.

The MN may not activate the SCG upon SN request.

## 2.2 UE behaviour when the SCG activation is indicated to the UE

In the last meeting, there are some options for the UE behaviours when the SCG activation is indicated to the UE. In our understanding, it depends on the UE behaviours when the SCG is deactivated. Therefore we think we can discuss it after there are some agreements for the UE behaviours in the deactivated SCG.

## 2.3 How to send the SCG activation command

In previous RAN2 meeting, it was agreed that the SCG activation state (activated/deactivated) can be configured at PSCell addition/change, RRC resume or HO.

In this case, an RRC reconfiguration is anyway needed, so we discuss whether this SCG activation state is indicated in the MN RRC message or the SN RRC message. As discussed in [1] for SCG deactivation and above for SCG activation, both the MN and the SN need to be aware of the SCG activation state and it is the MN that will send the RRC message after inter-node exchange with the SN and the SN cannot change the SCG activation state without informing the MN . Therefore, the SCG activation state can be generated by MN and provided to the UE together with the SCG configuration in the MN RRC reconfiguration message at PSCell addition/change, RRC resume or HO.

At PSCell addition/change, RRC resume or HO, the SCG activation state is indicated to te UE in the MN RRC reconfiguration message.

Apart from the above mobility related cases (i.e. PSCell addition/change, RRC resume or HO), for general reconfiguration case, on how to signal the SCG activation, the following options can be considered:

- RRC signalling

- MAC CE

- DCI

In R16, the network uses the MN RRC reconfiguration including the SCG addition command to inform the UE to add the SCG. RAN4 specifies the requirements of adding the SCG.

### 8.9.2 PSCell Addition Delay Requirement The requirements in this clause shall apply for the UE configured with only PCell in FR1. Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in subframe n + Tconfig_PSCell: where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms. Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the PSCell configuration command: - the UE has sent a valid measurement report for the PSCell being configured and - One of the SSBs measured from the PSCell being configured remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being configured also remains detectable during the PSCell configuration delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

According to the agreement in the last meetings, UE will continue the PSCell RRM measurement in the deactivation SCG. Therefore we think the Tsearch = 0 ms. Also we think Tprocessing can be reduced because the UE will store the SCG configuration when the SCG is deactivated. Also we think T∆ can be reduced if UE can perform the fine time tacking when the SCG is deactivated. But it is up to RAN4 to decide. In our understanding, the TPSCell_ DU is the same for all the options. Therefore the only difference between the option of using the MAC CE and the option of using the RRC is the processing time. If we assume in most cases (e.g. the UE is not moving fast) the SCG configuration does not need to be changed during SCG activation and deactivation, then the RRC reconfiguration is not needed. Thus using MAC CE to activate SCG could reduce the delay of the SCG activation using the RRC.

In our understanding, the main use cases of deactivating/activating SCG is for the burst traffic. If RAN2 uses the RRC to deactivate/activate the SCG, the network will send the RRC reconfiguration message frequently during the traffic. It will bring significant signalling overhead.

- The delay of the SCG activation using the MAC CE could be significantly smaller than using RRC signalling.

- Using MAC CE to activate the SCG can reduce the signalling overhead than using RRC.

Therefore we think using the MAC CE to indicate the SCG deactivation/action is better than using the RRC message.

Also based on the following reasons, we think the DCI method should be avoided:

- According to the WID, the WG of the efficient SCG activation/deactivation mechanism does not involve L1

- Support efficient activation/de-activation mechanism for one SCG and SCells - Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4]

- If using the DCI, the RAN does not know whether the UE has received the DCI.

- It does not align with the design of SCell deactivation/activation.

Support MAC CE to indicate SCG activation to the UE in cases where no reconfiguration is needed.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

- If the UE has uplink data to send on a split bearer while the SCG is deactivated, the UE sends the uplink data on the MCG leg, regardless of the primary path.

If the UE has uplink data to send on an SCG bearer while the SCG is deactivated, the UE informs the network via the MCG (details FFS).

Inform RAN3 that it is possible to use SN-initiated modification for SCG activation but it is also possible to rely on the activity notification instead.

In case of MCG RLF while fast MCG link recovery is configured and the SCG is deactivated, the UE can perform contention-based access toward the SCG in order to resume transmission on split SRB1 or SRB3 and send the MCGFailureInformation message.

The SN can reject the SCG activation request from the MN. It has been agreed by RAN3.

The MN may not activate the SCG upon SN request.

At PSCell addition/change, RRC resume or HO, the SCG activation state is indicated to te UE in the MN RRC reconfiguration message.

Support MAC CE to indicate SCG activation to the UE in cases where no reconfiguration is needed.

# 4. Reference

[1] R2-2103977, SCG deactivation, Huawei, Hisilicon