3GPP TSG-RAN WG2 Meeting #113e	R2- 2103993

# Online Meeting, 2021 April 12 – April 20

Agenda item:	8.7.2

Source:	LG Electronics Inc.

Title: 	Relay UE selection criterion using SL-unicast and discovery message

Document for:	Discussion and Decision

1.	Introduction

In the LTE Relaying operation (3GPP TS 38.331), Remote UE selects Relay UE using only the value of SD-RSRP (Discovery message-RSRP) to (re-)select SL Relay. However, in the previous Relay SI TR(3GPP TR 38.836), we’ve discussed that Remote UE may measure sidelink quality using SL-RSRP(sidelink unicast-RSRP) and/or SD-RSRP.

[3GPP TR 38.836] Further details on the PC5 radio measurements criteria, e.g., in case of no transmission on the sidelink unicast link can be discussed in WI phase. How to perform RSRP measurement based on RSRP of discovery message and/or SL-RSRP if Remote UE has PC5-RRC connection with Relay UE can be decided in WI phase.

In this contribution, we will study some specific methods of using SD-RSRP and/or SL-RSRP when Remote UE (re-)select a Relay UE.

2. Discussion

2.1 Initial Relay UE selection

If Remote UE does not have a Relay UE, the Remote UE can select a Relay UE by measuring SD-RSRP. Remote UE cannot use SL-RSRP until there is a connection with a Relay UE. So, when the Remote UE selects a Relay UE initially, the Remote UE can only measure SD-RSRP.

Observation 1: When Remote UE initially selects a Relay UE, the Remote UE can measure only SD-RSRP.

Proposal 1: When Remote UE initially selects a Relay UE, the Remote UE uses only SD-RSRP.

2.2. SL measurement after connection with Relay UE

After Remote UE makes a connection with a Relay UE, the Remote UE can use SD-RSRP and SL-RSRP to measure SL quality between Remote UE and Relay UE. However, to use SD-RSRP, we can consider the following cases depending on the discovery model.

- In the case of discovery model A, Relay UE broadcast discovery messages periodically. And the Remote UE can measure SL quality by using SD-RSRP between connected Remote UE and Relay UE.

- In the case of discovery model B, when the Remote UE sends out a solicitation message, the Relay UE received the solicitation message transmits discovery messages to the Remote UE as the response. However, the Relay UE may not broadcast discovery message after SL connection (PC5-S, SL-RRC) with the Remote UE. In this case, it’ hard to measure the SL quality using SD-RSRP continuously between connected Remote UE and Relay UE.

So, when the Remote UE uses SD-RSRP for measuring SL quality between connected Relay UE and itself, it can be some restriction depends on the discovery model. For example, in the case of discovery mode B, the Relay UE may broadcast discovery messages continuously if the Relay UE has at least one SL connected Remote UE.

Observation 2: In discovery model B, Relay UE may not broadcast discovery messages continuously.

In another case, we can consider that Remote UE uses SD-RSRP for Relay (re-)selection. The Remote UE measures connected SL quality by only using SL-RSRP and other information (e.g., the number of consecutive NACK/DTX, CBR, etc.) until SL-RLF occurs. If SL-RLF occurs between connected Relay UE and Remote UE, the Remote UE can trigger transmitting solicitation message to measure SD-RSRP. Or if the SL-RSRP and other information (e.g., the number of consecutive NACK/DTX, CBR, etc.) exceed/below (pre-)defined configuration, the Remote UE may trigger solicitation message transmission to measure SD-RSRP (in the case of Discovery model B). When Relay UE broadcasts the discovery message, Remote UE can compare SD RSRP between connected Relay UE and other candidate Relay UE(s).

Observation 3: it’s possible to measure SL quality between connected Relay UE and Remote UE by using SL-RSRP and other information (e.g., the number of consecutive NACK/DTX).

Proposal 2: Remote UE uses SL-RSRP to measure the link quality with the currently connected Relay UE. And the Remote uses SD-RSRP to compare the link quality with other candidate Relay UE(s).

Proposal 3: SL-RSRP and other information (e.g., the number of consecutive NACK/DTX) can trigger transmitting solicitation messages in Discovery model B.

On the other hand, if it is assumed that Relay UE broadcast discovery message continuously after establishing SL connection with Remote UE, we can also consider that Remote UE measures SL quality by using SL-RSRP and SD-RSRP together. When Remote UE uses SL-RSRP and SD-RSRP together, the Remote UE can get some advantages at the point of having more granularity for measurement. However, SL-RSRP is a controlled-power value. So, it’s not proper to use SD-RSRP and SL-RSRP directly together. The controlled -power SL-RSRP should be modified for measuring SL quality with SD-RSRP. To do solve this problem, one of the solutions may be that the unicast message from Relay UE includes TX power value. The Remote UE can modify power-controlled SL-RSRP by using this value. And then the modified SL-RSRP can compare or add to the SD-RSRP.

Observation 4: SL-RSRP is not a fixed-power value. It is a controlled-power value.

Proposal 4: If Remote UE wants to use SL-RSRP with SD-RSRP together for increasing measurement granularity, Remote UE has to know TX power of SL unicast link.

3.  Conclusion

Observation 1: When Remote UE initially selects a Relay UE, the Remote UE can measure only SD-RSRP.

Observation 2: In discovery model B, Relay UE may not broadcast discovery messages continuously.

Observation 3: it’s possible to measure SL quality between connected Relay UE and Remote UE by using SL-RSRP and other information (e.g., the number of consecutive NACK/DTX).

Observation 4: SL-RSRP is not a fixed-power value. It is a controlled-power value.

Proposal 1: When Remote UE initially selects a Relay UE, the Remote UE uses only SD-RSRP.

Proposal 2: Remote UE uses SL-RSRP to measure the link quality with the currently connected Relay UE. And Remote UE used SD-RSRP to measure the link quality with other candidate Relay UE(s).

Proposal 3: SL-RSRP and other information (e.g., the number of consecutive NACK/DTX) can trigger transmitting solicitation messages in Discovery model B.

Proposal 4: If Remote UE wants to use SL-RSRP with SD-RSRP together for increasing measurement granularity, Remote UE has to know TX power of SL unicast link.

4.  References

- RP-210904, “New WID on NR Sidelink Relay”

- 3GPP TR 38.836