3GPP TSG-RAN WG2 Meeting #113e	R2- 2103994

# Online Meeting, 2021 April 12 – April 20

Agenda item:	8.7.3

Source:	LG Electronics Inc.

Title: 	Relay (re-)selection and path switching

Document for:	Discussion and Decision

1.	Introduction

Objective Work Item objectives on aspects common to both L2 and L3: - Specify mechanisms for U2N relay discovery and (re)selection for L3 and L2 relaying [RAN2, RAN4] - Re-use LTE relay discovery and (re)selection as baseline - Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3] - Re-use LTE as baseline Work Item objectives specific to Layer-2 (L2) relaying: - Specify mechanisms for E2E, i.e. PC5 and Uu, QoS management [RAN2]: - Specify mechanisms for service continuity - Limited to intra-gNB cases [RAN2] - Specify mechanisms for U2N Adaptation layer design [RAN2] - For bearer mapping and Remote UE identification, incl. RAN related security aspects if any - Specify Control Plane procedures for U2N, including RRC connection management, system information delivery, paging mechanism and access control for Remote UE [RAN2, RAN3] NOTE 1: RAN requests RAN2 to strive for completion of the common parts (objective 1) by RAN#92 (June). RAN understands that RAN2 will also initially work on other aspects that have cross-group dependencies. NOTE 2: For L2 UE-to-Network Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release. NOTE 3: Only NR Uu interface, i.e. gNB, and 5GC is considered, and it is limited to NR SA scenario in this release. NOTE 4: Work specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”, and the group mobility is not supported in this release.

In this contribution, we investigate when Relay re-selection is triggered and who triggers the Relay re-selection. And according to the scope of the new WID on NR sidelink relay, we investigate whether the LTE relay re-selection method in TS 36.331 will use as the same in the aspect of service continuity.

2. Discussion

2.1 Relay (re-)selection triggering

The condition of relay (re-)selection is captured in current Relay TR (3GPP TR 38.836) as follows:

[3GPP TR 38.836] Relay reselection should be triggered if the NR Sidelink signal strength of current Sidelink relay is below a (pre)configured threshold. Also, relay reselection may be triggered if RLF of PC5 link with current Relay UE is detected by Remote UE.

According to the above Relay TR, it seems that only Remote UE triggers relay reselection. However, we think Relay UE can trigger relay reselection, and gNB can also trigger relay (re-)selection when we consider service continuity. Besides, the relay reselection conditions are seemed to be limited to the above two cases in the Relay TR. However, the following cases could be relay reselection triggering condition.

- When Uu link RLF occurs:

When Uu RLF occurs between Relay UE and gNB, the Remote UE should trigger relay reselection. Relay UE informs Uu RLF to the Remote UE. Or Relay UE directly should trigger relay reselection of Remote UE. That's why the Remote UE cannot notice Uu RLF of the Relay UE at all.

- When Relay UE should do HO (HandOver):

Relay UE can select a new cell after cell measurement and decide to move to the new cell. In this case, the Remote UE connected to the Relay UE can be left in a current cell because this release does not support group mobility. So, when Relay UE performs HO, the Relay UE informs it to the Remote UE. Or Relay UE directly should trigger relay reselection of the Remote UE.

- When Relay UE is barred:

Relay UE can get barring configuration from gNB. When Relay UE is barred, Relay UE cannot do a relaying operation. So, the Relay UE informs it to the connected Remote UE. Or Relay UE directly should trigger relay reselection of Remote UE.

Observation 1: relay reselection triggering condition in current Relay TR(3GPP 38.836) is as follows:

- NR Sidelink signal strength of current Sidelink relay is below a (pre)configured threshold

- RLF of PC5 link with current Relay UE is detected by Remote UE.

Proposal 1: relay re-selection triggers by Relay UE, Remote UE, and gNB(considering service continuity)

Proposal 2: the Relay re-selection triggering condition from Relay UE can be as follows:

- When Uu link RLF occurs

- When Relay UE should do HO

- When Relay UE is barred

2.2. Relay (re-)selection and path switching

In the case of path switching that we’ve discussed in SI period, service continuity is satisfied through normal HO procedure. The normal HO means the series of processes that Remote UE reports cell measurement to the gNB, gNB decides HO, and then gNB configures RRCReconfiguration to the Remote UE. However, when relay reselection is triggered without the normal HO procedure, due to PC5-RLF, Uu RLF, Relay UE HO, or Relay UE barred, we need to further study these cases.

The new WID on NR sidelink relay (RP-210904-WID) has some limitations to support service continuity. The path switching between indirect and indirect path is not supported and intra-cell path switching is supported in the point of service continuity. So, even though Remote UE doesn’t take a normal HO procedure, it could be a benefit that Remote UE connects directly with the same serving cell for maintaining service continuity. Because the same cell may still have the context of the Remote UE. Therefore, when relay reselection is triggered by abnormal conditions, Remote UE has a higher priority to select a direct path in the same cell than to select a direct path or an indirect path in different cells.

Besides, when relay reselection is triggered without a normal HO procedure and Remote UE selects a direct path with the same serving cell, it’s advantageous for the Remote UE to transmit information for service continuity to the serving gNB during RACH operation. For example, the Remote UE can provide C-RNTI of Remote UE, some information about previous connected Relay UE (e.g., SRC / DST / LOCAL ID), or the reason for having a direct connection to the serving cell without a normal HO procedure. The serving gNB may not provide service continuity as perfect as normal HO cases. But the transmitted information from Remote UE may be helpful that the gNB maintains the best-effort service continuity.

Observation 2: when relay reselection is triggered without normal HO procedure (e.g., PC5-RLF, Uu RLF, Relay UE HO, or Relay UE barred), it’s advantageous in service continuity that the Remote UE selects a direct path with the serving gNB.

Proposal 3: when relay reselection is triggered without normal HO procedure (e.g., PC5-RLF, Uu RLF, Relay UE HO or Relay UE barred), the Remote UE directly connects if the Uu-RSRP of its serving cell is above the (pre-)defined configuration.

In this case, the Remote UE delivers the following information to gNB during RACH procedure.

- C-RNTI of Remote UE itself

- Some information about sidelink connected Relay UE (e.g., SRC / DST / LOCAL ID)

- The reason for having to direct connection to the serving cell without a normal HO procedure.

In LTE Relay operation, Remote UE selects a new Relay UE by using SD-RSRP and Remote UE selects a direct path or indirect path by using Uu-RSRP. The specific selection methods are following as (3GPP TS 36.331):

5.10.11.4 Selection and reselection of sidelink relay UE A UE capable of sidelink remote UE operation that is configured by upper layers to search for a sidelink relay UE shall: 1> if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4], clause 11.4; or 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below threshHigh within remoteUE-Config : 2> search for candidate sidelink relay UEs, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected sidelink relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the filterCoefficient in SystemInformationBlockType19 (in coverage) or the preconfigured filterCoefficient as defined in 9.3(out of coverage), before using the SD-RSRP measurement results; NOTE 1: The details of the interaction with upper layers are up to UE implementation. 2> if the UE does not have a selected sidelink relay UE: 3> select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst; 2> else if SD-RSRP of the currently selected sidelink relay UE is below q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage); or if upper layers indicate not to use the currently selected sidelink relay: (i.e. sidelink relay UE reselection): 3> select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst; 2> else if the UE did not detect any candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst: 3> consider no sidelink relay UE to be selected; NOTE 2: The UE may perform sidelink relay UE reselection in a manner resulting in selection of the sidelink relay UE, amongst all candidate sidelink relay UEs meeting higher layer criteria, that has the best radio link quality. Further details, including interaction with upper layers, are up to UE implementation. 5.10.11.5 Sidelink remote UE threshold conditions A UE capable of sidelink remote UE operation shall: 1> if the threshold conditions specified in this clause were not met: 2> if threshHigh is not included in remoteUE-Config within SystemInformationBlockType19; or 2> if threshHigh is included in remoteUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHigh by hystMax (also included within remoteUE-Config): 3> consider the threshold conditions to be met (entry); 1> else: 2> if threshHigh is included in remoteUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHigh (also included within remoteUE-Config): 3> consider the threshold conditions not to be met (leave);

In the LTE spec, a method on which the Remote UE selects a new direct or indirect path is based on L3-relay. So, we need to consider whether the same method is proper in L2-relay. In specially, considering service continuity in L2-relay, the specific operation can be different. For example, in the LET Relay operation, the Remote UE connected indirectly selects direct communication when any Uu-RSRP is above threshHigh. But, in the L2-Relay, it may be beneficial in service continuity that the Remote UE selects a direct path when the Uu-RSRP of the current serving cell is above threshHigh.

Observation 3: relay selection and remote UE threshold condition (TS 36.331: 5.10.11.4 and 5.10.11.5) in LTE spec is based on L3-relay.

Proposal 4: we need to study in the aspect of service continuity whether to apply relay selection and remote UE threshold condition (TS 36.331: 5.10.11.4 and 5.10.11.5) in the same way in LTE spec.

3.  Conclusion

Observation 1: relay reselection triggering condition in current Relay TR(3GPP 38.836) is as follows:

- NR Sidelink signal strength of current Sidelink relay is below a (pre)configured threshold

- RLF of PC5 link with current Relay UE is detected by Remote UE.

Observation 2: when relay reselection is triggered without normal HO procedure (e.g., PC5-RLF, Uu RLF, Relay UE HO, or Relay UE barred), it’s advantageous in service continuity that the Remote UE selects a direct path with the serving gNB.

Observation 3: relay selection and remote UE threshold condition (TS 36.331: 5.10.11.4 and 5.10.11.5) in LTE spec is based on L3-relay.

Proposal 1: relay re-selection triggers by Relay UE, Remote UE, and gNB(considering service continuity)

Proposal 2: the Relay re-selection triggering condition from Relay UE can be as follows:

- When Uu link RLF occurs

- When Relay UE should do HO

- When Relay UE is barred

Proposal 3: when relay reselection is triggered without normal HO procedure (e.g., PC5-RLF, Uu RLF, Relay UE HO or Relay UE barred), the Remote UE directly connects if the Uu-RSRP of its serving cell is above the (pre-)defined configuration.

In this case, the Remote UE delivers the following information to gNB during RACH procedure.

- C-RNTI of Remote UE itself

- Some information about sidelink connected Relay UE (e.g., SRC / DST / LOCAL ID)

- The reason for having to direct connection to the serving cell without a normal HO procedure.

Proposal 4: we need to study in the aspect of service continuity whether to apply relay selection and remote UE threshold condition (TS 36.331: 5.10.11.4 and 5.10.11.5) in the same way in LTE spec.

4.  References

- RP-210904, “New WID on NR Sidelink Relay”

- 3GPP TR 38.836