3GPP TSG-RAN WG2 Meeting #113bis electronic          	          R2-2104025

Online, April 12 – April 20, 2021

Agenda Item:	5.4.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on BCS of a fallback band combination

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the issues about the BCS (Bandwidth Combination Set) of a fallback band combination.

Discussion

In the current specification TS 38.306, the definition of fallback is as follows:

Fallback band combination: A Uu band combination that would result from another Uu band combination by releasing at least one SCell or uplink configuration of SCell, or SCG. A PC5 band combination that would result from another PC5 band combination by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination.

Fallback per band feature set: A feature set per band that has same or lower capabilities than the reported capabilities from the reported feature set per band for a given band.

Fallback per CC feature set: A feature set per CC that has lower capabilities of UE supported MIMO layers and BW while keeping the numerology and other parameters the same from the reported feature set per CC for a given carrier per band.

As can be seen from the definitions above, fallback band combination comes from releasing of SCell or uplink configuration releasing of SCell or SCG. Generally, fallback band combination has the same or lower capability compared with the original one. However, the fallback bandwidth capability regarding BCS of the fallback band combination has never been discussed in RAN2.

For a given band combination, the supported channel bandwidth of each band is specified by the BCS ID according to TS 38.101 [1]. However, in TS 38.101, the same BCS ID in different band combination represents different channel bandwidth for a specific band. For example:

BC n3A-n78C, BCS 0 represents the channel bandwidth for n78 is {50,60,80,100} for 30kHz/60kHz SCS;

BC n3A-n78A, BCS 0 represents the channel bandwidth for n78 is {10,15,20,40,50,60,80,90,100}MHz for 30kHz/60kHz SCS.

Observation 1: The supported channel bandwidths corresponding to the same BCS ID may be different for a specific band between a fallback band combination and the super band combination.

Based on the example above, BC n3A-n78A is a fallback band combination of BC n3A-n78C, the supported channel bandwidths for band n78 of n3A-n78A are more or different compared with it for band n78 of n3A-n78C. So when a fallback band combination supports the same BCS ID(s) of the super band combination, there are two possible understandings for the supported channel bandwidths of a fallback BC regarding the BCS:

- The supported channel bandwidths for a specific band of a fallback BC regarding the BCS are determined by the supported BCS ID(s) of the fallback BC as specified in the TS 38.101, in which the BCS ID(s) is supported for the super BC

- The supported channel bandwidths for a specific band of a fallback BC regarding the BCS are determined by the channel bandwidths of the super BC as specified in the TS 38.101

For the example above, the supported channel bandwidths for band n78 of n3A-n78A are:

- Based on understanding 1): {10,15,20,40,50,60,80,90,100}MHz for 30kHz/60kHz SCS

- Based on understanding 2): {50,60,80,100} MHz for 15kHz/30kHz/60kHz SCS

In our view, the first understanding is more reasonable, that when a fallback band combination supports the BCS ID(s) of the super band combination, the BCS of a fallback band combination is determined by the BCS ID, and the supported channel bandwidth for a band of a fallback band combination is subject to the channel bandwidths corresponding to the BCS ID specified in TS 38.101. Of course, the supported channel bandwidth of a fallback band combination also considers the per band capability reported in channelBWs-DL and channelBWs-UL in BandNR and per feature set capability. Moreover, if the BCS ID(s) are defined in RAN4 for super band combination but not defined for fallback band combination, it can be considered that such BCS ID(s) are not supported by the fallback band combination. In this case, the UE shall report the fallback band combination explicitly so that the network can identify the channel bandwidths of the fallback band combination according to the reported BCS ID(s).

While for the second understanding, the UE has to report a different fallback band combination with the same BCS ID(s) if only UE supports more channel bandwidths for the fallback band combination, or the network cannot configure some channel bandwidths for particular cases. Considering that a higher capability for a fallback band combination is a more common case, this understanding will lead to unnecessary signalling overhead.

Proposal 1: The supported channel bandwidths for a specific band of a fallback BC regarding the BCS are determined by the supported BCS ID(s) (defined in RAN4 for the fallback BC) of the fallback BC, in which the BCS ID(s) is explicitly signalled for the super BC.

However, if the supported channel bandwidths for a band of the fallback band combination corresponding to the same BCS ID are less than that in the super band combination after releasing a SCell in this band, then the network needs to reconfigure UE according to the channel bandwidths of the fallback band combination, which seems contradictory with the initial intention of fallback mechanism. To avoid this situation, UE should report a different fallback band combination with more BCS ID(s) according to the channel bandwidths of the super BC.

Proposal 2: If the supported channel bandwidths for a band of the fallback BC corresponding to the same BCS ID are less than that in the super BC, UE reports a different fallback BC with more BCS ID(s) according to the channel bandwidths of the super BC.

Based on the above Proposal 1 and Proposal 2, when it comes to the BCS of a fallback band combination, there may be some aspects to be considered for the CA configuration table in TS 38.101. For the baseline, the channel bandwidths for a band of a fallback band combination defined for all possible BCS IDs should be no less than the channel bandwidths of the super band combination, so that the UE does not need to additionally report more BCS ID(s). Generally speaking, for a specific BCS ID corresponding to both the fallback band combination and the super band combination, the channel bandwidths for a band of the fallback band combination should be designed carefully to make sure that they can cover the channel bandwidths of the super band combination.

Proposal 3: Send a LS to RAN4 that for a specific BCS ID supported by both the fallback BC and the super BC, the channel bandwidths for a band of the fallback BC should be no less than that in the super BC.

Conclusion

In this contribution, we discuss the issue on the BCS of a fallback band combination. Observation and proposals are summarized as below.

Observation 1: The supported channel bandwidths corresponding to the same BCS ID may be different for a specific band between a fallback band combination and the super band combination.

Proposal 1: The supported channel bandwidths for a specific band of a fallback BC regarding the BCS are determined by the supported BCS ID (defined in RAN4 for the fallback BC) of the fallback BC, in which the BCS ID is explicitly signalled for the super BC.

Proposal 2: If the supported channel bandwidths for a band of the fallback BC corresponding to the same BCS ID are less than that in the super BC, UE reports a different fallback BC with more BCS ID(s) according to the channel bandwidths of the super BC.

Proposal 3: Send a LS to RAN4 that for a specific BCS ID supported by both the fallback BC and the super BC, the channel bandwidths for a band of the fallback BC should be no less than that in the super BC.

References

- 3GPP TS 38.101, User Equipment (UE) radio transmission and reception.