3GPP TSG-RAN WG2 Meeting#113bis-e		       					  R2-2104061

Electronic, 12th - 20th April 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on network sharing with multiple SSBs in a carrier

Agenda item:		8.17

Document for: 	Discussion and Decision

# Introduction

SA5 is working on the Rel-17 WI “Management Aspects of 5G Network Sharing (MANS)”. During the discussion, the companies have different opinions on whether the feature multiple SSBs in a carrier can be used to support NG-RAN sharing. The following questions have been asked from SA2 to RAN2/3 for clarification [1]:

1)Whether the feature multi-SSBs in a carrier is mature and stable in RAN specification? Why the Annex B.2 Multiple SSBs in a carrier in TS 38.300 is informative?

2)Is the feature multiple SSBs in a carrier specific for NG-RAN sharing?

3)Whether the feature multiple SSBs in a carrier can be used to support NG-RAN sharing (i.e. the cell associated to different SSBs can be used by different operators)?

In this paper, we share some understanding on the multi-SSBs in a carrier and the relation between SSBs, cells and operators identified by PLMN as well as answer questions from SA5s.

# Discussion

## Understanding of the multi-SSBs in a carrier in RAN specification

To ensure that UEs in both wide-band CC operation and intra-band contiguous CA operation can access the network, multiple SSBs within a wide-band carrier has been introduced since RAN1#89.

Follow-up discussion in RAN2 has been completed to support multi-SSBs in a carrier since Rel-15. By introducing definition of Cell-Defining SSB (CD-SSB, an SSB with an RMSI associated), the relationship between cell and SSB for the multi-SSB in a carrier case has been clarified: different CD-SSBs would be modeled by UE as individual cells during cell search, cell (re)selection, initial access and connection establishment. For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells during measurements and mobility handling with details captured in RAN2 and RAN4 specifications.

With the above analysis on the status of the multi-SSBs in a wide-band carrier, we understand the feature multi-SSBs in a carrier is mature and stable in RAN specification. Regarding the Annex B in TS38.300, we understand it is mainly used to capture examples of deployment scenarios. To avoid unnecessary limitation on implementation, the example of multi-SSB in a carrier has been captured as informative text in Annex B.

Observation 1: The feature multi-SSBs in a carrier is mature and stable in RAN specification.

## Cell/SSB/PLMN relationship

Figure 1: Example of multiple SSBs in a carrier

As defined in TS38.300[2], the Cell-Defining SSB refers to an SSB with an RMSI associated.

For the example given in Figure 1, multiple SSB can be transmitted within the bandwidth of a wide-band carrier. Each SSB can associate with a RMSI. From UE perspective, a cell is associated with a single SS Block (i.e. CD-SSB). Also considering that the initial BWP refers to the MIB->pdcch-ConfigSIB1for DL and SIB1->servingCellConfigCommon->uplinkConfigCommon for UL, different UEs may search and camp on one of the two different SSBs (i.e. SSB1 or SSB3) and take the camped SSB as the CD-SSB.

For instance, UE1 and UE2 may camp on SSB1. Then UE1and UE2 will take SSB1 as the CD-SSB and monitor system information and paging in their corresponding initial DL BWP and performs random access in their corresponding initial DL/UL BWP. While UE3 will camp on SSB3, takes SSB3 as CD-SSB and use the corresponding initial BWP in system information/paging reception and random access.

In other words, the two SSBs associated with the corresponding RMSI as illustrated in Figure 1 are modeled as two individual cells by UEs in the Idle/Inactive mode. Different system information can be broadcast from these cells and the PLMNs broadcast in SIB1->cellAccessRelatedInfo->plmn-IdentityList can also be different to support different operators.

Observation 2: For multi-SSB in a carrier, the PLMNs broadcast in RMSI associated with each CD-SSB can be different to support different operators.

With the above analysis, we understand that although the multi-SSB in a carrier is not introduced for RAN sharing, it can be used to support NG-RAN sharing (i.e. the cell associated to different SSBs can be used by different operators).

Proposal 1: RAN2 answers to Question 1 in S5-212403 in the following way:

The feature multi-SSBs in a carrier is mature and stable in RAN specifications. To avoid unnecessary limitation on implementation, the example of multi-SSB in a carrier has been captured as informative text in Annex B as deployment scenario examples.

Proposal 2: RAN2 answers to Question 2 and 3 in S5-212403 in the following way:

For multi-SSB in a carrier, the PLMNs broadcast in RMSI associated with each CD-SSB can be different to support different operators.

Proposal 3: Agree the reply LS [3].

# Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Observation 1: The feature multi-SSBs in a carrier is mature and stable in RAN specification.

Observation 2: For multi-SSB in a carrier, the PLMNs broadcast in RMSI associated with each CD-SSB can be different to support different operators.

Proposal 1: RAN2 answers to Question 1 in S5-212403 in the following way:

RAN2 understand the feature multi-SSBs in a carrier is mature and stable in RAN specifications. To avoid unnecessary limitation on implementation, the example of multi-SSB in a carrier has been captured as informative text in Annex B as deployment scenario examples.

Proposal 2: RAN2 answers to Question 2 and 3 in S5-212403 in the following way:

For multi-SSB in a carrier, the PLMNs broadcast in RMSI associated with each CD-SSB can be different to support different operators.

Proposal 3: Agree the reply LS [3].

# References

- R2-2102676-S5-212403 LS on network sharing with multiple SSBs in a carrier (Contact: ZTE)

- TS 38.300 V16.5.0

- R2-2104062_Draft_reply LS on network sharing with multiple SSBs in a carrier