3GPP TSG-RAN WG2 #113bis-e	R2-2104144

E-meeting, Apr 12 – Apr 20, 2021

Agenda Item	: 8.10.2.2 (NR_NTN_solutions-Core)

Source	: LG Electronics Inc.

Title	: Discussion on UL scheulding and UL retranmission

Document for	: Discussion and Decision

Introduction

In this contribution, we show our view on the enhancement of the UL scheduling and UL retransmission.

Discussion

UL scheduling enhancement

In RAN2#113e meeting, how to enhance the UL scheduling was further discussed. However, there is meaningful agreement was not made for the enhancement of UL scheduling. In this section, we show our view on the enhancement of UL scheduling using configured grant and BSR over 2-step RACH.

A new triggering condition for 2-step RACH

In POST 112#152 email discussion, the scheduling enhancement was discussed for BSR transmission via 2-step RACH and the following options were proposed.

- Option 1. BSR can be sent over 2-step RACH which is triggered by existing events in RRC_CONNECTED

- Option 2. BSR can trigger 2-step RACH.

In Rel-16, when triggering BSR, the UE triggers an SR to get the UL grant to transmit the BSR if the UL-SCH resource is not available. Then, if the SR resource is not available, the UE triggers the RA procedure. When performing the RA procedure, the UE selects the RA type, e.g., 2-step RACH or 4-step RACH, based on the RSRP threshold. In this case, if the 2-step RACH is triggered, the BSR is transmitted on the MsgB. Therefore, we understand that Option 1 is already supported.

Observation 1. Option 1 is already supported in a legacy RA procedure.

In case there is no SR resource configured to a UE, we think option 2 is what the UE is doing already today. Although it has not been presented how option 2 works in detail, it may be that the UE directly triggers 2-step RACH for BSR transmission, i.e., without triggering SR even with SR resource and without comparing the RSRP threshold.

In legacy, the reason of comparing RSRP for selection of either 2-step or 4-step RACH is to ensure successful transmission of MsgB as PUSCH resource. As, in option 2, the UE would select 2-step RACH regardless of RSRP, there is a risk that the MsgB is transmitted under bad radio condition and failed. The latency caused by failed transmission may not be negligible and we would prefer to keep a conservative approach, i.e., using RSRP threshold for selection of 2-step RACH.

In the meanwhile, it could be argued that saving SR resource is another benefit. However, the network may instead provide more reliable/larger PUSCH resource for MsgB transmission, which shows there is certainly a trade-off.

With the above reasons, we do not consider option2 is beneficial in terms of latency and resource saving.

Proposal 1. Do not introduce a new triggering condition of 2-step RACH for BSR transmission.

BSR transmission on configured grant and RACH

In RAN2#113e meeting, for scheduling enhancement, it was agreed that UE in NTN can have both 2-step RACH and configured grant configurations at the same time. However, it was not discussed which resources between configured grant and RACH is selected for BSR transmission.

For example, the BSR is triggered at T1. In this case, the UE should decide whether the BSR is transmitted via RACH or configured grant.

Ideally, the BSR transmission should be transmitted as soon as possible. Thus, if the BSR is triggered at T1, it seems a good to transmit the BSR via RA procedure. This is because the next RA occasion is earlier than the configured grant occasion. However, considering that the RACH is used not only for BSR transmission but also for other purposes and the RACH preamble is limited, it would lead to more collision of RA procedure. Consequently, the RACH for BSR transmission should be used to a limited level.

Thus, in order not to decrease the RACH performance, the configured grant should be prioritized over the RACH for BSR transmission. For example, if the BSR is triggered at T1, the UE selects one resource among the configured grant and RACH. In this case, if the next configured grant occasion is within a specific time frame, the UE selects the next configured grant occasion for BSR transmission even if the next RA occasion is earlier than the configured grant occasion.

Proposal 2. The configured grant should be prioritized over the RACH for BSR transmission if configured grant occurs within a certain time from a BSR trigger.

LCH impact on UL retransmission

In RAN2#112e and the RAN2#113e meeting, the following agreements were made.

RAN2#112e - From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded RAN2#113e - From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8)

During discussion, some companies show the concern on the LCP procedure for disabling UL retransmission. With this reason, the agreement on “Companies can contribute next meeting to describe the LCP impacts.” was made for disabling UL retransmission.

In our understanding, the main concern is that the UE does not expect whether the UL HARQ retransmission for a UL grant can be scheduled or not if the UL grant is a new transmission. More specifically, the data from RBs with different QoS, i.e., data not requiring UL HARQ retransmission and data requiring UL HARQ retransmission, can be multiplexed in a MAC PDU. In this case, it is hard for the network to decide whether to schedule the UL HARQ retransmission for the MAC PDU or schedule another new transmission before receiving the MAC PDU. This is because that the network would not know whether the MAC PDU contains the data from RBs with same QoS or data from RBs with different QoS without receiving a MAC PDU. Thus, the network anyway need to wait until it receives the MAC PDU and it may cause the HARQ stalling problem in NTN with longer propagation delay.

To be able to schedule new transmission even before reception of a MAC PDU, it would be desirable that different QoS are not multiplexed in one MAC PDU. Then, the network can predict whether the MAC PDU needs retransmission or not priori and plan retransmission scheduling easily.

Proposal 3. Avoid QoS multiplexing in one MAC PDU

One simple way to avoid QoS multiplexing in one MAC PDU is that the network indicates which logical channels are supposed to be included in the MAC PDU via UL grant. For this purpose, the network can use existing parameters such as logical channel priority or logical channel group. If they are not sufficient a new QCI parameter can be considered, e.g., indication indicating whether the RB requires UL HARQ retransmission or not. Then, when receiving the UL grant, the UE generates a MAC PDU containing the data from RBs that is allowed as indicated by the UL grant. As the network can predict what RBs are to be included in the MAC PDU, it can have a plan whether to schedule a retransmission or not easily.

Given that we already have some kind of LCP restriction based on RRC parameters, MAC change seems not so big but there could be RAN1 impact.

Proposal 4. For an UL grant, RAN2 discuss how the UE applies LCP procedure for RBs having same QoS level.

Conclusion

In this contribution, we show our view on the enhancement of the UL scheduling and UL retransmission and the following proposals are made.

Observation 1. Option 1 is already supported in a legacy RA procedure.

Proposal 1. Do not introduce a new triggering condition of 2-step RACH for BSR transmission.

Proposal 2. The configured grant should be prioritized over the RACH for BSR transmission if configured grant occurs within a certain time from a BSR trigger.

Proposal 3. Avoid QoS multiplexing in one MAC PDU

Proposal 4. For an UL grant, RAN2 discuss how the UE applies LCP procedure for RBs having same QoS level.