3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2104150

Online, April 12th – 20th, 2021

Agenda item:		8.1.2.1

Source:		Futurewei, Qualcomm Inc., Intel

Title:		Comparison of L2 Reliability Solutions for MRB with Dynamic PTM/PTP Switch

Document for:		Discussion and Decision

# Introduction

During RAN2#113e, email discussion [AT113-e][038][MBS] UP architecture decisions [01] discussed  following aspects.

The main two points that seems to need resolution/consolidation are the following

A.	L2 ARQ for PTM for normal data transfer

B.	Which layer anchors the PTM PTP switch, i.e. at PTM PTP switch which layer remains the same, (and might be responsible for service continuity).

Both point A and B are included here because several companies indicate that they are inter-dependent,

For A. there seems to be the following options on the table:

A1. No L2 ARQ for PTM

A2. L2 ARQ by PDCP for PTM

A3. L2 ARQ by RLC-AM for PTM

For B. There seems to be the following options on the table:

B1. PDCP anchored PTM/PTP switch

B2. RLC anchored PTM/PTP Switch

Summary proposals:

Proposal 1: (A1+B1), For the case that both PTM and PTP are RLC-UM, No L2 ARQ with PDCP  anchored PTM – PTP switching shall be supported,

Proposal 2:  Discuss whether to support any of:

- A1+B1 for PTM RLC-UM + PTP RLC-AM, possibly with some kind of data recovery in the  switching procedure.

- A2+B1 for PTM RLC-UM + PTP RLC-AM

- A3+B2(+B1) For PTM RLC-AM + PTP RLC-AM

RAN2#103e Agreement and Chairman Notes:

For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast).

Chair understanding is that actually all the proposals on the table could support the high reliability requirement. Indeed RLC AM proposal would be expected to be the most efficient by using PTM to greater extent (at least greater than A1), and by retransmitting segments. TO be continued at a later meeting.

This contribution analyzes and compares the specification impact and expected performance of applying L2 retransmission at either PDCP sublayer or RLC sublayer to enhance reliability of a PTM transmission.

# Discussion

Though PDCP duplication has been specified in Rel-15 and Rel-16, its operation is not based on the status report of a receiver. Duplication is done blindly for all PDCP PDUs to support URLLC services, at the expense of radio resource utilization. Given the potentially large amount of MBS traffic, it is not wise to apply PDCP duplication for reliable PTM transmission.

Observation 1: PDCP duplication blindly transmits multiple copies of every PDCP PDU, and it is not suitable to achieve radio efficient Multicast transmission.

PDCP status report can be generated when [2]: “

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

-	upper layer requests a uplink data switching;

-	upper layer reconfigures the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 38.331 [3].”

That is, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU. On the other hand, RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU [3].

Observation 2: While RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU.

When loss of RLC PDU is detected and RLC status report is triggered by the receiving side of an AM RLC entity, the lower edge of RLC receiving window RX_Next is not moved, and the retransmitted RLC PDU will still fall within the RLC receiving window [3] – “

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.”

When loss of a PDCP PDU is detected, however, the lower edge of PDCP reception buffer RX_DELIV is moved forward, and any retransmission of missing PDCP PDU would fall out of reception buffer [2] – “

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.”

Observation 3: While RLC sublayer already supports the retransmission of a lost RLC PDU, the lower edge of PDCP reception buffer RX_DELIV is moved forward when loss of a PDCP PDU is detected, and any retransmission of missing PDCP PDU would fall out of reception buffer.

In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified such that its lower edge is not moved until the corresponding PDCP PDU is received. In other words, the procedure of handling t-Reordering timer expiration at PDCP sublayer should be modified in a way similar to the handling of t-Reassembly timer expiration at RLC sublayer. This basically leads to the replication of existing RLC functionality at PDCP sublayer.

Observation 4: In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified, practically replicating existing RLC functionality at PDCP sublayer.

The triggers for PDCP based retransmission for PTM is different from existing triggers for PDCP status report. To support PDCP based retransmission for PTM, similar mechanisms as RLC status report needs to be introduced. Contributions submitted to RAN2#113-e meeting propose various triggers for PDCP status reporting: expiry of t-Reordering, polling, number of failed PDCP PDUs, periodic reporting etc. It is obvious that standardization efforts are required to support PDCP based ARQ.

Observation 4bis: Retransmission of lost PDCP PDU without RRC involvement requires significant changes on both gNB and UE sides to introduce similar mechanisms as RLC status report (e.g. expiry of t-Reordering, polling, number of failed PDCP PDUs, periodic reporting).

A PDCP entity may be associated with multiple RLC entities, which in turn are associated with different logical channels. For MRB with dynamic PTP/PTM switch, these RLC entities/logical channels may operate at different modes, like one in UM and another one in AM. This makes it difficult to configure proper t-Reordering timer, as different legs would take various time to finish transmission of a PDCP PDU.

Observation 5: Retransmission of lost PDCP PDU without RRC involvement suffers performance degradation in MRB with dynamic PTP/PTM switch, as a single t-Reordering timer at PDCP sublayer wouldn’t fit to transmission time budgets of both UM and AM RLC legs.

Furthermore, retransmission of lost PDCP PDU without RRC involvement can lead to two loops of L2 retransmission on PTP leg at RLC sublayer and PDCP sublayer, respectively. It is not clear how timers and state variables at PDCP and RLC sublayers can be configured and operated to avoid the intertwining of PDCP retransmission and RLC retransmission – (n+1)th retransmission of a PDCP PDU starts when RLC still performs retransmission on the n-th transmission of a PDCP PDU.

Observation 6: Retransmission of lost PDCP PDU without RRC involvement generates conflict with RLC AM operation, and it doesn’t work well on RLC AM PTP leg of the MRB with dynamic PTM/PTP switch.

There had been extensive discussions during the study of NR in Rel-14 on where L2 retransmission should be done [4] – “ R2-166828	Second level retransmissions in NR	Ericsson	discussion

-	CATT understand that ARQ should not deal with front haul errors but ARQ above the front haul can bring benefits in multiconnectivity scenarios to help cope with link failures. We should not preclude that ARQ is in the CU

-	LG would like to discuss this again after reordering is decided

=>	Comeback to discuss P3

-	LG think that ARQ could be in PDCP based on PDCP SN.

-	MediaTek support ARQ in RLC especially for DC cases.

-	CATT think it should be possible to configure ARQ in PDCP, to deal with the fact that the link can break quickly. It should not be ruled out at the moment. Ericsson think if it is in PDCP then we have backhaul issues. This doesn't preclude duplication in PDCP to send packets on different links and it is also a lower delay solution. Huawei also think that an eNB could resend packets on the other link if it detects low throughput on the other link.

-	ZTE agree with the proposal.

Agreement

-	The ARQ will be supported in RLC.

”

Performing L2 retransmission at PDCP layer may subject a system to additional delay over front-haul between DU and CU, and degrade system performance.

Observation 7: Extensive study in Rel-14 led to the agreement of having ARQ supported in RLC sublayer to avoid performance degradation from additional delay over front-haul between DU and CU.

Unlike for RLC AM DRB, RLC transmitter is always at network for MRB. Hence there’d be no extra specification work for the transmit operation of an MRB RLC AM entity. There is additional requirement on network implementation for PTM scheduling, which stems from the fact that one transmission needs to target at multiple UEs. This functional requirement doesn’t change whether scheduling is implemented at RLC sublayer or PDCP sublayer. That is, if retransmission is done at PDCP sublayer, then PDCP sublayer also needs to use a transmission window, and to move the transmission window based on the status report of all UEs receiving from the PTM transmission. As scheduling is at a DU (where MAC and RLC sublayers reside), handling transmission window at PDCP sublayer, which resides at a CU, further incurs front haul delay between CU and DU in transmission window management and scheduling operation.

Observation 8: As the transmit operation of an MRB RLC AM entity is always at gNB, transmission window management is part of network implementation, and it doesn’t impose additional impact on RLC specification.

Observation 8bis: Scheduling requirement on network implementation to accommodate status reports from multiple UEs exists no matter whether L2 retransmission is done at RLC sublayer or PDCP sublayer. Retransmission of lost PDCP PDU without RRC involvement incurs extra front haul delay between CU and DU in transmission window management and scheduling of retransmission.

RLC AM receive operation is per UE, and it doesn’t depend on if other UEs receive the same packet. And RLC entity receive operation is not tied to any particular RNTI; whether a RLC PDU is received through G-RNTI or C-RNTI can be kept transparent to the receive operation of an RLC AM entity. Hence, there’d be no impact on the specification of receive operation of RLC AM entity, if it is used to receive transmission from a PTM leg.

Observation 9: As the receive operation of RLC AM entity is not dependent on if other UEs receive the same packet or on whether a RLC PDU is received through G-RNTI or C-RNTI, there’d be no impact on the specification of receive operation of RLC AM entity, if it is used to receive transmission from a PTM leg.

With a MRB consisting of PTM and PTP legs, a UE’s reception feedback can be carried on the UL of its PTP leg, either as PDCP status report or as RLC status report, without much specification impact. Hence, for transmission over PTM, the status reports from MBS UEs can be transmitted over their respective dedicated logical channel over PTP leg.

Observation 10: Without much specification impact, either PDCP status report or RLC status report can be carried on the UL of a UE’s PTP leg in a MRB with dynamic PTM/PTP switch.

One may argue that option A1+B1 for PTM RLC-UM + PTP RLC-AM can also support the service of high reliability and moderate latency with the PTP leg using RLC-AM. However, it should be emphasized that from air interface perspective, the main benefit of using MBS is the radio efficiency. With that in mind, for service with high reliability and moderate latency, RLC AM for PTM is a good way to achieve radio efficiency and high reliability simultaneously. If PTP is always used for such services and PTM is switched to PTP based on radio condition, we lose the benefits of using MBS. In addition, purely relying on switching from PTM to PTP might not support services with high reliability well since there is potential physical layer error (NACK to ACK), which cannot be known by gNB if L2 feedback is not provided from UE to gNB. There is also gNB implementation complexity increase for option A1 since gNB implementation needs to switch UE operation between PTM and PTP legs based on suitable criteria so that reliability requirement can be satisfied.

Observation 11: PDCP based L2 re-transmission without L2 feedback method (i.e. solution A1) is not reliable for PTP/PTM switching and is radio inefficient due to unicast L2 re-transmission of whole PDCP PDUs.

Table 1 summarises the comparison among different options.

Table 1: Comparison of MRB L2 reliability solutions

|  | A1. No L2 ARQ for PTM | A2. L2 ARQ by PDCP for PTM | A3. L2 ARQ by RLC-AM for PTM |
| --- | --- | --- | --- |
| How it works | Based on HARQ feedback and gNB cross-layer implementation, gNB decides at PDCP level whether to re-tx via PTP RLC leg. | Based on UE’s PDCP status reporting, gNB decides at PDCP level whether to re-tx via PTP RLC leg. | Based on UE’s RLC status reporting, gNB decides at RLC level whether to re-tx via multicast or unicast manner. |
| PTM efficiency | Lower due to unicast L2 re-transmission of a whole PDCP PDU. | Lower due to unicast L2 re-transmission of a whole PDCP PDU. | High due to multicast or unicast L2 re-transmission of RLC PDU (i.e., PDCP PDU segments). |
| PTM reliability and performance | Low due to switching decision based on L1 HARQ feedback, which has limited reliability. | High | High |
| Spec Complexity | Lower (but may have issues with dropping of packets when PDCP PDUs received outside PDCP Rx window) | Very High, PDCP has to implement RLC AM kind of functionality, i.e., receive window management, status reporting enhancements, new t_reassembly timer, polling etc. | Low |

Based on the above analysis, when compared to PDCP based solution, RLC based ARQ solution offers high radio efficiency, high reliability for PTM leg, and limited specification changes for MRB with dynamic PTM/PTP switch.

Proposal 1: RAN2 agrees to introduce RLC based ARQ solution A3 for reliable PTM transmission in MRB with dynamic PTM/PTP switch.

# Conclusions

This paper analyzed the specification impact and expected performance of applying L2 retransmission at either PDCP sublayer or RLC sublayer to enhance reliability of a PTM transmission, which leads to the following observations and proposals:

Observation 1: PDCP duplication blindly transmits multiple copies of every PDCP PDU, and it is not suitable to achieve radio efficient Multicast transmission.

Observation 2: While RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU.

Observation 3: While RLC sublayer already supports the retransmission of a lost RLC PDU, the lower edge of PDCP reception buffer RX_DELIV is moved forward when loss of a PDCP PDU is detected, and any retransmission of missing PDCP PDU would fall out of reception buffer.

Observation 4: In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified, practically replicating existing RLC functionality at PDCP sublayer.

Observation 4bis: Retransmission of lost PDCP PDU without RRC involvement requires significant changes on both gNB and UE sides to introduce similar mechanisms as RLC status report (e.g. expiry of t-Reordering, polling, number of failed PDCP PDUs, periodic reporting).

Observation 5: Retransmission of lost PDCP PDU without RRC involvement suffers performance degradation in MRB with dynamic PTP/PTM switch, as a single t-Reordering timer at PDCP sublayer wouldn’t fit to transmission time budgets of both UM and AM RLC legs.

Observation 6: Retransmission of lost PDCP PDU without RRC involvement generates conflict with RLC AM operation, and it doesn’t work well on RLC AM PTP leg of the MRB with dynamic PTM/PTP switch.

Observation 7: Extensive study in Rel-14 led to the agreement of having ARQ supported in RLC sublayer to avoid performance degradation from additional delay over front-haul between DU and CU.

Observation 8: As the transmit operation of an MRB RLC AM entity is always at gNB, transmission window management is part of network implementation, and it doesn’t impose additional impact on RLC specification.

Observation 8bis: Scheduling requirement on network implementation to accommodate status reports from multiple UEs exists no matter whether L2 retransmission is done at RLC sublayer or PDCP sublayer. Retransmission of lost PDCP PDU without RRC involvement incurs extra front haul delay between CU and DU in transmission window management and scheduling of retransmission.

Observation 9: As the receive operation of RLC AM entity is not dependent on if other UEs receive the same packet or on whether a RLC PDU is received through G-RNTI or C-RNTI, there’d be no impact on the specification of receive operation of RLC AM entity, if it is used to receive transmission from a PTM leg.

Observation 10: Without much specification impact, either PDCP status report or RLC status report can be carried on the UL of a UE’s PTP leg in a MRB with dynamic PTM/PTP switch.

Observation 11: PDCP based L2 re-transmission without L2 feedback method (i.e. solution A1) is not reliable for PTP/PTM switching and is radio inefficient due to unicast L2 re-transmission of whole PDCP PDUs.

Proposal 1: RAN2 agrees to introduce RLC based ARQ solution A3 for reliable PTM transmission in MRB with dynamic PTM/PTP switch.

# References

R2-2102313:[AT113-e][038][MBS] UP architecture decisions

3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”.

3GPP TS 38.322, “NR; Radio Link Control (RLC) protocol specification”.

R2-167461, “Report of 3GPP TSG RAN WG2 meeting #95bis”.