3GPP TSG-RAN WG2 Meeting #113bis-e                          R2-2104176

Online, April 12 – April 20 2021

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on slice based cell reselection

Document for:	Discussion & Decision

Introduction

One of the objective [1] to enhance RAN support of network slicing is as follows:

- Support slice based cell reselection, specify mechanisms and signalling including [RAN2]

- To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message.

- To assist cell reselection, include slice info (with similar information as in SI message) in RRCRelease message.

Note: The use of Rel-17 RAN slicing enhancements in given cells shall not prevent from accessibility for Rel-15 and Rel-16 UEs.

Note: RAN3 objectives to be added after RAN3 study phase is finished.

Note: This work item should take SA2 output on slicing enhancement into consideration if RAN impacts are identified, e.g. the relation between Tracking Area and S-NSSAI is expected to impact the solution for slice based cell reselection.

This contribution discusses slice based cell reselection.

Discussion

SA2 concluded to support homogeneous slice support in all cells of a registration area as Rel-15 and Rel-16 [1]. Specifically, the following conclusions are agreed:

- The following is assumed e.g. to support legacy UEs:

  - all cells of a tracking area support the same S-NSSAI(s); and

  - the S-NSSAI(s) of the Allowed NSSAI are supported by all tracking areas in a registration area.

- To enable the possibility where certain frequencies, supported by the network i.e. PLMN or SNPN, cannot be used to access a network slice, a solution without UE impacts is required as to ensure any UE can be directed towards a frequency/cell from which the UE can access the network slice.

  - The principles of solution #17/46 are used as basis for network based solution with no UE impacts. The 5GC may provide to the RAN, in addition to the Allowed NSSAI as in Rel-16, additional assistance information to steer the UE to another cell. The assistance information can be at least one of the rejected S-NSSAIs, the Configured NSSAI, or Target NSSAI and RFSP. This assistance information allows the RAN to redirect the UE to a cell supporting network slices not available in a current cell and RA.

Observation 1: SA2 concluded to support homogeneous slice support in all cells of a registration area as Rel-15 and Rel-16.

Observation 2: In case the UE is not allowed to access a certain network slice in a current cell and RA, UE can be directed towards a frequency/cell from which the UE can access the network slice.

As studied in [2], its main intention is to enable UE fast access to the cell supporting the intended slice(s) UE wants to use. However, according to the SA2 conclusion, RAN can ensure to allocate registration area supporting the intended slice(s) UE wants to use after initial registration procedure. It means that in most cases UE will end up reselecting a cell supporting UE's intended slice(s) without cell reselection related slice information in broadcasted system information. We think that the benefits of broadcasting the supported slice information per cell and cell reselection priority per slice seems quite marginal considering its signalling overhead as well as potential security issues.

Proposal 1: Do not broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message.

As RAN receives the Allowed NSSAI, the rejected S-NSSAIs, the Configured NSSAI, or Target NSSAI and RFSP from the 5GS, it may be beneficial to include slice information in RRCRelease message to assist UE to reselect a cell supporting the intended slice(s) UE wants to use.

Proposal 2: RAN2 to discuss whether to include slice information in the RRCRelease message.

Conclusion

In section 2, we made the following observations:

Observation 1: SA2 concluded to support homogeneous slice support in all cells of a registration area as Rel-15 and Rel-16.

Observation 2: In case the UE is not allowed to access a certain network slice in a current cell and RA, UE can be directed towards a frequency/cell from which the UE can access the network slice.

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: Do not broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message.

Proposal 2: RAN2 to discuss whether to include slice information in the RRCRelease message.

References

[1] RP-210912, New WID on enhancement of RAN Slicing for NR

[2] 3GPP TR 38.832 V1.0.0