3GPP TSG-RAN WG2 Meeting #113bis-e	   R2-2104185(rev R2-2101562)

Online, April 12th - April 20th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Clarification on the Intra-band and Inter-band EN-DC Capabilities

Agenda item:		5.4.3

Document for: 	Discussion and Decision

- Introduction

In the last meeting, some CRs were agreed on intra-band combination capability issues, e.g. singleUL-Transmission [1][2], SupportedBandwidthCombinationSetEN-DC [3][4][5]. Meanwhile, in the last plenary meeting, there was also some discussion on intraBandENDC-Support [6]. In this paper, we try to further clarify some other intra-band and inter-band (NG)EN-DC related capabilities, including:

dualPA-Architecture/ul-TimingAlignmentEUTRA-NR/pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/ simultaneousRxTxInterBandENDC.

- Discussion

2.1 (NG)EN-DC/NE-DC band combination types definition

Before the discussion, first we need give a clear clarification on the current (NG)EN-DC/NE-DC band combination types definition. Though companies have achieved some consensus on the terminologies of the different kinds of  (NG)EN-DC/NE-DC band combination types in the [Post113-e][009][NR15] EN-DC BCS (Nokia)[5], it’s not clearly described in the chairman notes or in the spec, which would cause confusion for the readers who didn’t attend this post meeting discussion. To solve this confusion, it’s better to give a clarification on the (NG)EN-DC/NE-DC band combination types definition in 38306 or in the chairman note at least.

According to the discussion in the [5], the (NG)EN-DC/NE-DC band combinations that have the same band component at NR and Eutra side were all defined as intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR and LTE CA component), otherwise, it would be defined as inter-band (NG)EN-DC/NE-DC combination. In detail, for the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/ LTE CA component), besides the case that Pcell and PScell works on the same band, it also includes the case that SPcell works on the same band with a scell of another CG and the case that there are scells from the different CGs working on the same band.

Proposal 1: Ran2 confirm that the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component) in 38306 means the (NG)EN-DC/NE-DC band combinations that have the same band component at NR and Eutra side (irrespective of SPcell or Scell), for other cases, it would be defined as inter-band (NG)EN-DC/NE-DC combination.

According to the terminologies in the SupportedBandwidthCombinationSet/ supportedBandwidthCombinationSetIntraENDC, the intra-band (NG)EN-DC/NE-DC combination further includes 3 types:

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination supporting only UL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

Meanwhile, another BC type that also has the intra-band EN-DC requirement is “Inter-band EN-DC configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. [7][8][9]”

Observation 1: The (NG)EN-DC/NE-DC band combination can be divided in to 5 types:

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination supporting only UL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. [7][8][9].

Based on this observation, we will further clarify the following parameters one by one in the below chapters.

dualPA-Architecture/ul-TimingAlignmentEUTRA-NR/pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/ simultaneousRxTxInterBandENDC.

Note: For the intraBandENDC-Support, RAN4 further input was needed, thus in this paper, we didn’t touch this element.

In the below sub-chapters, we first analyze which BC types shall these capabilities be used for, then discuss whether some clarification/modifications are needed.

2.2 BC types for each UE capability

    - 2.2.1 ul-TimingAlignmentEUTRA-NR/dualPA-Architecture/pa-PhaseDiscontinuityImpact

The definition of the ul-TimingAlignmentEUTRA-NR/dualPA-Architecture/pa-PhaseDiscontinuityImpact is as below:

| ul-TimingAlignmentEUTRA-NR Indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC. If this field is absent, UE shall be capable of handling a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous (NG)EN-DC network, as specified in TS 38.133 [5]. If this capability is included in an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part, this capability is used to indicate the restriction to the intra-band (NG)EN-DC BC part. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| pa-PhaseDiscontinuityImpacts Indicates incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band (NG)EN-DC/NE-DC, intra-band CA and FDM based ULSUP. | FS | No | N/A | N/A |

These 3 capabilities are for the intra-band UL, obviously, it would not be adopted for the type 3/4.

Proposal 2: The ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts is for the Type 1/2 BC, and not for the Type 3/4 BC.

For the type 5, according to the RAN4 LS [3], RAN4 has agreed that intra-band EN-DC requirements shall apply for inter-band EN-DC configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. Thus we think this parameter shall also be indicated for the Type 5 BC. Anyway, the confirmation from RAN4 is also needed before any conclusion.

Proposal 3: Confirm with Ran 4 whether the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts shall be adopted for the Type 5 BC.

    - 2.2.2 asyncIntraBandENDC

The definition of the asyncIntraBandENDC is as below:

| asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. | BC | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

This capability is for asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD, thus it shall be only adopted for 1/2.

Proposal 4: The asyncIntraBandENDC is for Type 1/2 BC.

    - 2.2.3 Inter-band EN-DC parameters

The definition of simultaneousRxTxInterBandENDC is as below:

| simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Note: There is a reply LS [10] from RAN4 and an LS out [11] on this capability, but it focus on the NR-DC, to avoid confusion, we only focus on (NG)EN-DC/NE-DC in this paper.

This parameter was introduced for the case that the TDD band receiver can’t suppress the interface from the transmitter. Obviously, this parameter is not for the Type 1 BC, and it shall be adopted by the Type 4 BC. For the type 2 and type 3 BC, the transmission on one band may also introduce unacceptable interference to the receiving band, thus this parameter shall also be adopted for the type 2/3 BC.

For the type 5 BC e.g. DC_B42_n77 and DC_B42_n78, considering that the same TDD DL_UL_pattern would be adopted for the two bands, thus this capability is not needed for the type 5 BC. Anyway an LS can be sent to RAN4 for the further confirmation.

Proposal 5: The simultaneousRxTxInterBandENDC is for Type 2/3/4 and send a LS to RAN4 to confirm whether it is needed also for the type 5 BC.

2.5 BC types summary

Based on the above discussion, the BC types for each UE capabilities can be summarized as below:

| UE capability | corresponding BC types | Current Terminology in the field description |
| --- | --- | --- |
| ul-TimingAlignmentEUTRA-NR | BC type 1/2 | intra-band contiguous (NG)EN-DC |
| ul-TimingAlignmentEUTRA-NR | BC type 2 | an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part |
| dualPA-Architecture | BC type 1/2 | intra-band band combination |
| pa-PhaseDiscontinuityImpacts | BC type 1/2 | intra-band (NG)EN-DC/NE-DC |
| asyncIntraBandENDC | BC type 1/2 | intra-band (NG)EN-DC |
| simultaneousRxTxInterBandENDC | BC type 2/3/4 | inter-band (NG)EN-DC/NE-DC |

From this table, we can see that for the “dualPA-Architecture/ ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts/ asyncIntraBandENDC” the intra-band (NG)EN-DC (/NE-DC) doesn’t include the Type 3 BC, e.g. Intra-band (NG)EN-DC/NE-DC combination supporting only UL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component. Meanwhile, for the type 2 BC, in the filed description of “SupportedBandwidthCombinationSet/ supportedBandwidthCombinationSetIntraENDC”, it was called “Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component”, but in the “ul-TimingAlignmentEUTRA-NR”, it was called “an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part”. Similarly, for the “simultaneousRxTxInterBandENDC”, besides the “inter-band (NG)EN-DC/NE-DC”, it can also be reported for the Type 2/3 BC.

Observation 2: The current terminologies of the (NG)EN-DC/NE-DC BC type in each UE capability are inconsistent with that in the SupportedBandwidthCombinationSet/ supportedBandwidthCombinationSetIntraENDC

To solve these problems and make the terminological consistent among these capabilities, some modification are needed as in draft CR [12][13].

Proposal 6: Agree the CR in [12][13].

For the type 5 BC, which kinds of Intra-band and Inter-band (NG)EN-DC capabilities shall be adopted, a LS can be sent to RAN4 first.

Proposal 7: Agree the LS to RAN1/4 in [14].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Ran2 confirm that the intra-band (NG)EN-DC/NE-DC combination (with or without additional inter-band NR/LTE CA component) in 38306 means the (NG)EN-DC/NE-DC band combinations that have the same band component at NR and Eutra side (irrespective of SPcell or Scell), for other cases, it would be defined as inter-band (NG)EN-DC/NE-DC combination.

Observation 1: The (NG)EN-DC/NE-DC band combination can be divided in to 5 types:

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination supporting only UL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. [7][8][9].

Proposal 2: The ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts is for the Type 1/2 BC, and not for the Type 3/4 BC.

Proposal 3: Confirm with Ran 4 whether the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts shall be adopted for the Type 5 BC.

Proposal 4: The asyncIntraBandENDC is for Type 1/2 BC.

Proposal 5: The simultaneousRxTxInterBandENDC is for Type 2/3/4 and send a LS to RAN4 to confirm whether it is needed also for the type 5 BC.

Observation 2: The current terminologies of the (NG)EN-DC/NE-DC BC type in each UE capability are inconsistent with that in the SupportedBandwidthCombinationSet/ supportedBandwidthCombinationSetIntraENDC

Proposal 6: Agree the CR in [12][13].

Proposal 7: Agree the LS to RAN1/4 in [14].

- Reference

- R2-2102391	Clarfication on single uplink operation capability report (LS Contact)	Huawei, HiSilicon	CR	Rel-15	38.306	15.12.0	0524	1	F	NR_newRAT-Core

- R2-2102392	Clarfication on single uplink operation capability report (LS Contact)	Huawei, HiSilicon	CR	Rel-16	38.306	16.3.0	0525	1	A	NR_newRAT-Core

- R2-2102216	Clarification on the supportedBandwidthCombinationSetIntraENDC capability	Nokia, Nokia Shanghai Bell

- R2-2102217	Clarification on the supportedBandwidthCombinationSetIntraENDC capability	Nokia, Nokia Shanghai Bell

- R2-2102215	Summary of [Post113-e][009][NR15] EN-DC BCS (Nokia)	Nokia, Nokia Shanghai Bell

- RP-210640  Clarifications on intra-band contiguous and non-contiguous EN-DC combinations  Apple

- R2-2000034 LS on UE capability of intraBandENDC-Support RAN4 to RAN2

- R2-2002349	UE capability of intra-band requirements for inter-band EN-DC/NE-DC	NTT DOCOMO, INC.	CR	Rel-15	38.331	15.8.0	1501	1	F	NR_newRAT-Core	Late

- R2-2002350	UE capability of intra-band requirements for inter-band EN-DC/NE-DC	NTT DOCOMO, INC.	CR	Rel-15	38.306	15.8.0	0259	1	F	NR_newRAT-Core	Late

- R4-2016988  Draft LS on simultaneous Rx/Tx capability  3GPP RAN4

- R2-2102495  Reply LS on simultaneous Rx/Tx capability	Huawei, HiSilicon	LS out

- R2-2104186	CR on the Intra-band and Inter-band EN-DC Capabilities-R15	ZTE Corporation, Sanechips

- R2-2104187	CR on the Intra-band and Inter-band EN-DC Capabilities-R16	ZTE Corporation, Sanechips

- R2-2104188	Draft LS on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips

About the (NG)EN-DC/NE-DC band combination, in the current 38306 spec, the following terminologies were used in the field descriptions:

| Terminology | Related UE capability |
| --- | --- |
| inter-band (NG)EN-DC/NE-DC without intra-band (NG)EN-DC/NE-DC component | supportedBandwidthCombinationSet |
| intra-band (NG)EN-DC/NE-DC without additional inter-band NR and LTE CA component |  |
| intra-band (NG)EN-DC/NE-DC band combination |  |
| intra-band (NG)EN-DC/NE-DC with additional inter-band NR CA component |  |
| intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component |  |
| intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part. |  |
| intra-band (NG)EN-DC |  |
| inter-band (NG)EN-DC/NE-DC combination |  |
| inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part |  |