3GPP TSG-RAN WG2 Meeting #113bis-e	   R2-2104212

Online, April 12th - April 20th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Further Clarification on the supportedBandwidthCombinationSet

Agenda item:		5.4.3

Document for: 	Discussion and Decision

- Introduction

In [1][2][3], most of the intra-band BC (NG)EN-DC/NE-DC BC cases have been discussed, however for the case that an intra-band (NG)EN-DC/NE-DC BC with only single NR carrier was not touched. Meanwhile, in [4], some BCS related issues were also mentioned for the super BC and its fallback BCs, e.g. example 5.1~5.6 of [4]. Furthermore, in the [5][6], the modification to the channelBWs-UL/DL and supportedBandwidthDL/UL didn’t cover the case of 90M bandwidth, but based on the [1][2][3], for some intra-band BC (NG)EN-DC/NE-DC BC that with 90M bandwidth, it may also report SupportedBandwidthCombinationSetEN-DC, e.g. BC_(n)41AA-n78A.

In this paper, we will give our further analysis/classifications on these issues one by one.

- Discussion

2.1 Intra-band (NG)EN-DC/NE-DC BC with only single NR carrier

According to the current spec, the BCS reporting for the Intra-band (NG)EN-DC/NE-DC BC can be summarized as below: (Note: The BCS for the Eutra part was not include in this table)

| BC Type UE capability BCS for each part | BCS for the Intra-band Part | BCS for the NR-part |
| --- | --- | --- |
| Intra-band (NG)EN-DC/NE-DC BC without inter-band component | supportedBandwidthCombinationSet | N/A |
| Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier e.g. DC_1A-(n)41AA | SupportedBandwidthCombinationSet or supportedBandwidthCombinationSetIntraENDC?? | N/A |
| Intra-band (NG)EN-DC/NE-DC BC with LTE single carrier and NR inter-band CA | supportedBandwidthCombinationSetIntraENDC | supportedBandwidthCombinationSet |
| Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR inter-band CA | supportedBandwidthCombinationSetIntraENDC | supportedBandwidthCombinationSet |

From this table, we can see that it’s not clear whether the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported by SupportedBandwidthCombinationSet or by supportedBandwidthCombinationSetIntraENDC. Take the DC_1A-(n)41AA as an example, besides the BCS for the Eutra side CA1A_41A, the UE shall also report the BCS for the intra_band part (n)41AA. But it’s not clear which element (SupportedBandwidthCombinationSet or supportedBandwidthCombinationSetIntraENDC) shall be reported. Based on the current definition, in the field description of supportedBandwidthCombinationSetIntraENDC, it is clearly mentioned the case of intra-band (NG)EN-DC/NE-DC with additional inter-band CA component(s) of LTE and/or NR. Thus, the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSetIntraENDC.

Observation 1: Based on the current field description, the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSetIntraENDC.

However, as discussed in RAN2, the supportedBandwidthCombinationSetIntraENDC was introduced for the cases that can’t be covered by supportedBandwidthCombinationSet. From this point of view, the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSet.

Observation 2: The supportedBandwidthCombinationSetIntraENDC was introduced for the case that can’t be covered by supportedBandwidthCombinationSet. From this point of view, the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSet.

Based on the observation 1 and the observation 2, RAN2 shall confirm which capability element (SupportedBandwidthCombinationSet or supportedBandwidthCombinationSetIntraENDC) shall be adopted for the NR single carrier case.

Proposal 1: Ran2 to confirm which capability element (SupportedBandwidthCombinationSet or supportedBandwidthCombinationSetIntraENDC) shall be adopted to report the intra-band Part of “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier”

Based on the proposal 1, if go to the supportedBandwidthCombinationSet, obviously, the similar modification as in [2][3] shall also be added for the supportedBandwidthCombinationSet, e.g. add the below clarification to the  intra-band (NG)EN-DC/NE-DC with additional LTE CA component and NR single carrier:

- It is mandatory if it supports both UL and DL intra-band (NG)EN-DC/NE-DC parts (e.g. DC_1A- (n)41AA);

- It is optional if it doesn’t support UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part.(e.g. DC_1A-(n)41AA)

The related modification was also provided in the first change of the draft CR [7][8].

Proposal 1.1: If RAN2 confirm that the SupportedBandwidthCombinationSet shall be reported, agree the related change in the draft CR [7][8]

If go to the supportedBandwidthCombinationSetIntraENDC, the modification would be simple, we only need to give some clarification to the last sentence of the field description, e.g.

supportedBandwidthCombinationSet Defines the supported bandwidth combination set for a band combination as defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. For NR SA CA, NR-DC, inter-band (NG)EN-DC without intra-band (NG)EN-DC component, inter-band NE-DC without intra-band NE-DC component and intra-band (NG)EN-DC/NE-DC with additional inter-band NR CA component, the field defines the bandwidth combinations for the NR part of the band combination. For intra-band (NG)EN-DC/NE-DC without additional inter-band NR and LTE CA component, the field indicates the supported bandwidth combination set applicable to intra-band (NG)EN-DC/NE-DC band combination. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. It is mandatory if the band combination has more than one NR carrier (at least one SCell in an NR cell group) or is an intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component.

For this modification, it has been included in another papers/CRs, which intends to unify the terminologies for the BC types in the field description of different UE capabilities [9][10][11].

2.2 Different bandwidth limitations with the same BCS for the super BC and fallback BCs

As mentioned in [4], even for the same BCS (e.g. BCS0), the supper BC and its fallback BCs may have different bandwidth limitation. As shown in the table Example 5.2 of [4], for the CA_n25A-n41A-n66A, the BCS0 supports 30M for the band 41, but for the CA_n25A-n41A, the BCS0 doesn’t support 30M on the band 41, which would cause confusion at the network side. For example, when the network configure a CA with 3 DLs CA_n25A-n41A-n66A and 30M on band 41, when it fall backs to the CA_n25A-n41A, how to configure the bandwidth on the band 41.

Example 5.2 of [4]

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | ... | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A | CA_n25A-n41A | n25 | 15 | ... |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 | ... |  |  |  |  |  |  |  |  |
|  |  |  | 60 | ... |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 | ... |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 | ... |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 | ... |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | ... | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | - | n25 | 15 | ... |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 | ... |  |  |  |  |  |  |  |  |
|  |  |  | 60 | ... |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 | ... | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 | ... | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 | ... | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n66 | 15 | ... |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 | ... |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 | ... |  | Yes |  |  |  |  |  |  |

Some other similar mismatch examples are also mentioned in the [4], to solve these issues, there are 2 main options for the fall back BC.

Option 1: The network configure the UE always based on the BCS table of super BC.

Option 2: The network configure the UE based on the BCS table of the fallback BC.

Though these issues were mentioned for the BCS4 Topic, they still need to be clarified in the R15/16. To keep alignment with the RAN4 spec, the option 2 is slightly preferred.

Proposal 2: RAN2 confirm that for the fall back BC, the network configure the UE based on the BCS table of the fallback BC.

Based on proposal 2 a sentence can be added to the current field description of supportedBandwidthCombinationSet as below, which was also included in the first change draft CR [7] [8] .

supportedBandwidthCombinationSet Defines the supported bandwidth combination set for a band combination as defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. For NR SA CA, NR-DC, inter-band (NG)EN-DC without intra-band (NG)EN-DC component, inter-band NE-DC without intra-band NE-DC component and intra-band (NG)EN-DC/NE-DC with additional inter-band NR CA component, the field defines the bandwidth combinations for the NR part of the band combination. For intra-band (NG)EN-DC/NE-DC without additional inter-band NR and LTE CA component, the field indicates the supported bandwidth combination set applicable to intra-band (NG)EN-DC/NE-DC band combination. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. It is mandatory if the band combination has more than one NR carrier (at least one SCell in an NR cell group) or is an intra-band (NG)EN-DC/NE-DC combination or both. Note: For the case that the supper BC and its fallback BCs have different bandwidth limitations for the same BCS, for the fallback BC, the network configure the UE based on the BCS table of the fallback BC.

2.3 90M limitation

In the [5][6], the modification to the channelBWs-UL/DL and supportedBandwidthDL/UL didn’t cover the case of 90M bandwidth, but based on the [1][2][3], for some intra-band BC (NG)EN-DC/NE-DC BC that with 90M bandwidth, it may also report SupportedBandwidthCombinationSetEN-DC, especially for the band 41 which can support both 90M and intra-band EN-DC. Take the BC (n)41AA-n78A as an example:

The UE shall report SupportedBandwidthCombinationSetEN-DC for the (n)41AA part, meanwhile report SupportedBandwidthCombinationSet for the n41A-n78A part.

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)38101-1

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 |  | 10 |  | 80 | 90 | 100 | 1 |
|  |  | n78 |  | 10 | 70 | 80 | 90 | 100 |  |

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands) 38101-3

| EN-DC configuration | Uplink EN-DC configuration(NOTE 1) |
| --- | --- |
| DC_(n)41AA-n78A | DC_41A_n78A |

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC 38101-3

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
| ... |  |  |  |  |  | 1 |

Thus, based on above observation, to determine whether the UE supports a channel bandwidth of 90 MHz, the network shall also validate SupportedBandwidthCombinationSetEN-DC, though currently it only happens for the BC with the band 41.

Proposal 3: To determine whether the UE supports a channel bandwidth of 90 MHz, the network shall also validate SupportedBandwidthCombinationSetEN-DC.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: Based on the current field description the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSetIntraENDC.

Observation 2: The supportedBandwidthCombinationSetIntraENDC was introduced for the case that can’t be covered by supportedBandwidthCombinationSet. From this point of view, the BCS for the Intra-band Part of a “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier” shall be reported in the supportedBandwidthCombinationSet.

Proposal 1: Ran2 to confirm which capability element (SupportedBandwidthCombinationSet or supportedBandwidthCombinationSetIntraENDC) shall be adopted to report the intra-band Part of “Intra-band (NG)EN-DC/NE-DC BC with LTE inter-band CA and NR single carrier”

Proposal 1.1: If RAN2 confirm that the SupportedBandwidthCombinationSet shall be reported, agree the related change in the draft CR [7][8]

Proposal 2: RAN2 confirm that for the fall back BC, the network configure the UE based on the BCS table of the fallback BC.

Proposal 3: To determine whether the UE supports a channel bandwidth of 90 MHz, the network shall also validate SupportedBandwidthCombinationSetEN-DC.

- Reference

- R2-2102215	Summary of [Post113-e][009][NR15] EN-DC BCS (Nokia)	Nokia, Nokia Shanghai Bell

- R2-2102216	Clarification on the supportedBandwidthCombinationSetIntraENDC capability	Nokia, Nokia Shanghai Bell

- R2-2102217	Clarification on the supportedBandwidthCombinationSetIntraENDC capability	Nokia, Nokia Shanghai Bell

- R2-2102401	CR on the SupportedBandwidth and channelBWs(R15)	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.12.0	0515	1	F	NR_newRAT-Core

- R2-2102402	CR on the SupportedBandwidth and channelBWs(R16)	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.3.0	0516	1	A	NR_newRAT-Core

- R4-2016453 An alternative to creating new BCSs T-Mobile USA, Deutsche Telekom, AT&T, TELUS, Bell Mobility, Rogers Communications, Telstra, Telecom Italia, KDDI, Vodafone, BT plc, Ericsson

- R2-2104213	CR on the supportedBandwidthCombinationSet-R15	ZTE Corporation, Sanechips

- R2-2104214	CR on the supportedBandwidthCombinationSet-R16	ZTE Corporation, Sanechips

- R2-2104185	Clarification on the Intra-band and Inter-band EN-DC Capabilities	ZTE Corporation, Sanechips

- R2-2104186	CR on the Intra-band and Inter-band EN-DC Capabilities-R15	ZTE Corporation, Sanechips

- R2-2104187	CR on the Intra-band and Inter-band EN-DC Capabilities-R16	ZTE Corporation, Sanechips