3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2104219
Online, April 12 – April 20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2560 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the TRP definition for eMIMO and positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi Communications, Samsung, OPPO, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core, NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-4-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the current RRC specificaitons, the term of TRP (or mult-TRP) is used for several fields (e.g. dl-PRS-ID, dl-PRS-ResourceId, multiDCI-MultiTRP-r16, multiDCI-multiTRP-Parameters, harqACK-separateMultiDCI-MultiTRP-r16 and harqACK-jointMultiDCI-MultiTRP-r16). However two different definitions of TRP are given in 38.300 and 38.355 respectively. The 37.355 definition of TRP is only for the co-located antenna for the positioning purpose. The 38.300 definition of TRP is not restricted to a specific antenna location (i.e. which could be either co-located or non-collocated). The field description of dl-PRS-ID already states that the TRP for dl-PRS-ID refers to the 38.355 TRP (i.e. only the co-located scenario). It is not clear which TRP definition/scenario other fields refer to. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.1.2, it is clarified that the term TRP is defined in TS 38.300, unless explicility stated otherwise. In the field description of dl-PRS-ResourceId, it is clarified that this field refers to the 38.355 TRP definition (i.e. only the co-located scenario). Impact analysis Impacted functionality: TRP Interoperability analysis: If the UE implements the change, but the gNB does not implements the change. The location information of the UE may not be accurate. If the UE does not implement the change, but the gNB implements the change. No interoperability problem is foreseen. 5G architectures: All |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE location information may not be accurate if the TRPs are using non-collocated antennas. The TRP deployment scenarios for eMIMO might be restricted to the co-located antenna. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.1.2	General requirements

The UE shall:

1>	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE:	Network may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	within a clause execute the steps according to the order specified in the procedural description;

1>	consider the term 'radio bearer' (RB) to cover SRBs and DRBs unless explicitly stated otherwise;

1>	consider the term TRP as defined in TS 38.300 [2] unless explicitly stated otherwise;

1>	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from the network that triggered the response message;

1>	upon receiving a choice value set to setup:

2>	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1>	upon receiving a choice value set to release:

2>	clear the corresponding configuration and stop using the associated resources;

1>	in case the size of a list is extended, upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether the network signals more entries in total); apply the following generic behaviour unless explicitly stated otherwise:

2>	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2>	for the combined list, created according to the previous, apply the same behaviour as defined for the original field.

Next change

6.3.2	Radio resource control information elements

…

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

End of changes