3GPP TSG-RAN2 Meeting #113bis-e 	R2-2104258

# E-Meeting, 12 – 20 April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0570 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS video capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Currently, there is no way for the UE to indicate whether the UE supports IMS video over E-UTRA/5GC, NR or SCG bearer of NE-DC. It is poroposed to use the existing capability indications of IMS voice over E-UTRA/5GC, NR and SCG bearer of NE-DC to indicate capabilities of IMS video over E-UTRA/5GC, NR and SCG bearer of NE-DC, respectively. - The UE should not be reqired to support IMS video over split bearer for NR-DC and NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - If the UE supports IMS video, this field also indicates whether the UE supports IMS video over E-UTRA via 5GC. - If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR. - If the UE supports IMS video, this field also indicates whether the UE supports IMS video over SCG bearer of NE-DC. - IMS video over split bearer is not supported for NR-DC and NE-DC. Impact analysis Impacted 5G architecture options: NR SA, E-UTRA/5GC, NR-DC, NE-DC Impacted functionality: IMS video over NR SA IMS video over E-UTRA/5GC IMS video over split bearer for NR-DC and NE-DC Inter-operability: If the network does not implement the CR and configures an IMS video call over E-UTRA/5GC, NR or SCG bearer of NE-DC the UE not supportting IMS video over E-UTRA/5GC, NR or SCG bearer of NE-DC, respectively, the UE fails to establish the IMS video call. The UE triggers the RRC connection re-establishment procedure due to the invalid configuration. Otherwise, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE may fail to establish an IMS video call over E-UTRA/5GC, NR or SCG bearer of NE-DC because the UE does not support IMS video over E-UTRA/5GC, NR or SCG bearer of NE-DC, respectively. - The UE is required to support IMS video call over the split bearer, which imposes an implementation burden on the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.13	IMS Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| voiceOverEUTRA-5GC Indicates whether the UE supports IMS voice over E-UTRA via 5GC. It is mandated to the UE if the UE is capable of IMS voice over E-UTRA via 5GC. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over E-UTRA via 5GC. | UE | No | No | No |
| voiceOverNR Indicates whether the UE supports IMS voice over NR. It is mandated to the UE if the UE is capable of IMS voice over NR. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR. | UE | No | No | Yes |
| voiceOverSCG-BearerEUTRA-5GC Indicates whether the UE supports IMS voice over SCG bearer of NE-DC. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over SCG bearer of NE-DC. | UE | No | No | N/A |

NOTE:	In this release of specification, neither IMS voice nor IMS video over split bearer is supported for NR-DC and NE-DC.