3GPP TSG-RAN WG2 Meeting #113-bis-e                         R2-2104274

Electronic meeting, 12 April – 20 April 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on latency reduction

Agenda item:		8.11.2

Document for: 	Discussion and Decision

- Introduction

The WI on NR positioning enhancement has been approved in RAN#91e(RP-210903)[1], and the following objectives have been agreed in WID:

-----------------objectives in RP-210903-----------------

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

-----------------objectives in RP-210903-----------------

The intention of this contribution is to share some views on the latency reduction of NR positioning.

- Periodic SRS transmission for optimization cases

- At the end of RAN2#113e, RAN2 finally agreed to add the basic procedure of RAT-dependent positioning in Rel-16 due to lack of consensus. The procedure and latency performance analysis for UL-TDOA/UL-AoA and multi-RTT can be found in the last TR38890 and annex. Currently, the steps for SP-SRS and aperiodic SRS are implemented in the positioning procedure(step 6,7,8 in  UL-TDOA/UL-AoA , multi-RTT). Based on our understanding, either SP SRS or aperiodic SRS is not the best choice for SRS measurement in the optimization scenarios. Though in some scenarios that the SP SRS or aperiodic SRS may have better performance on the latency, from only the ideal SRS measurement perspective, the periodic SRS should have the same performance on the measurement compared with the other SRS. If the system decides to use periodic SRS, the latency of step 6,7, and 8 can be skipped without any further modification of Rel-16 mechanism.

- Proposal 1: RAN2 shall consider to only use periodic SRS for the optimization latency performance in Rel-16.

- CG-based transmission

- After RAN2#113e, companies have made consensus that latency reduction related to the reporting of the measurements can be achieved by CG-based transmission. Based on our understanding, the current defined CG-based transmission mechanism can be implemented in positioning field only if the NRPPa supports to transmit PRS period data between LMF and gNB. Considering RAN3 is responsible for the NRPPa modification, we prefer to send an LS to RAN3 and trigger the discussion on CG-based transmission.

- Proposal 2: RAN2 shall send an LS to RAN3 and trigger the discussion on NRPPa about PRS data transportation for CG-based positioning data transmission.

- Conclusion and proposals

- Proposal 1: RAN2 shall consider to only use periodic SRS for the optimization latency performance in Rel-16.

- Proposal 2: RAN2 shall send an LS to RAN3 and trigger the discussion on NRPPa about PRS data transportation for CG-based positioning data transmission.

- References

- 3GPP RP-202094,“Work Item Description: Study on NR Positioning Enhancements”, RAN,Sep,18, 2020.

- 3GPP RP-210903,“Work Item Description: NR Positioning Enhancements”, RAN,Mar,26, 2021.

- 3GPP TR 38.857, “Study on NR Positioning Enhancements”, Release 17,v1.1.0, Feb, 2020.

- 3GPP Chairman notes in RAN2#113e, Feb, 2021.

- Annex

    - 8.1.3.2	Latency analysis for UL-TDOA/UL-AoA

Referred to [27], Figure 8.1.3.2-1 shows the messaging between the LMF, the AMF, the gNBs and the UE to perform UL-TDOA and UL-AoA procedure.

Figure 8.1.3.2-1: UL-TDOA/UL-AoA positioning procedure

The latency performance analysis for UE assisted UL-TDOA and UL-AoA are provided in table 8.1.3.2-1.

Table 8.1.3.2-1: Latency performance analysis for UE assisted UL-TDOA and UL-AoA

| Step | Delay Value [ms] | Description of Latency Component |
| --- | --- | --- |
| Step 1 LPP Request capabilities | 18-34.5 | Processing delays: 14 ms - UE: TUEProc-RRCDLInfo - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF= 1-10 ms Note 1: the LPP capability processing delay is counted together in response message. Note 2: Should not be counted if the LMF does not need the capability, e.g. only use Rel-15 SRS for UL positioning. |
| Step 2 LPP Provide Capabilities | 25-54.5 | Processing delays: 21-34 ms - UE: - TUEProc-RRCULInfo - TUEProc-LPPCapab - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5 ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 3 NRPPa POSITIONING INFORMATION REQUEST | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 4 RRC SRS configuration | 13-13.5 | Processing delays: 13 ms - UE: TUEProc-RRCReconf - gNB: TgNBProc-RRC Signalling delay:0-0.5ms - UE-gNB: TUE-gNB Note 3: Should not be counted if the SRS configuration has been configured before the procedure. |
| Step 5 NRPPa POSITIONING INFORMATION RESPONSE | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 6 NRPPa Request UE SRS activation | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 5: Should not be counted if the periodic SRS is used. |
| Step 7 MAC Activate UE SRS transmission | 1-3.5 | Processing delays: 13ms - UE: TUEProc-MAC-SRSAct Signalling delay:0-0.5ms - UE-gNB: TUE-gNB Note 6: Should not be counted if the periodic or aperiodic SRS is used. |
| Step 8 NRPPa Request UE SRS activate Response | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 7: Should not be counted if the periodic SRS is used. |
| Step 9 NRPPa MEASUREMENT REQUEST | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 10 UL SRS measurement | 12 | TUL-Meas |
| Step 11 NRPPa MEASUREMENT RESPONSE | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 12 LMF calculation | 2-30 | TLMF-Calc |
| Total values | 149-322 |  |

    - 8.1.3.3	Latency analysis for Multi-RTT

Referred to [27], Figure 8.1.3.3-1 shows the messaging between the LMF, the AMF, the gNBs and the UE to perform Multi-RTT procedure.

Figure 8.1.3.3-1: Multi-RTT positioning procedure

The latency performance analysis for UE assisted Multi-RTT are provided in table 8.1.3.3-1.

Table 8.1.3.3-1: Latency performance analysis for UE assisted Multi-RTT

| Step | Delay Value [ms] | Description of Latency Component |
| --- | --- | --- |
| Step 1 LPP Request capabilities | 18-34.5 | Processing delays: 14 ms - UE: TUEProc-RRCDLInfo - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 1: the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | Processing delays: 21-34 ms - UE: - TUEProc-RRCULInfo - TUEProc-LPPCapab - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5 ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 3 NRPPa POSITIONING INFORMATION REQUEST | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 4 RRC SRS configuration | 13-13.5 | Processing delays: 13 ms - UE: TUEProc-RRCReconf - gNB: TgNBProc-RRC Signalling delay:0-0.5ms - UE-gNB: TUE-gNB Note 3: Should not be counted if the SRS configuration has been configured before the procedure. |
| Step 5 NRPPa POSITIONING INFORMATION RESPONSE | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 6 NRPPa Request UE SRS activation | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 5: Should not be counted if the periodic SRS is used. |
| Step 7 MAC Activate UE SRS transmission | 1-3.5 | Processing delays: 13ms - UE: TUEProc-MAC-SRSAct Signalling delay:0-0.5ms - UE-gNB: TUE-gNB Note 6: Should not be counted if the periodic or aperiodic SRS is used. |
| Step 8 NRPPa Request UE SRS activate Response | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 7: Should not be counted if the periodic SRS is used. |
| Step 9 NRPPa MEASUREMENT REQUEST | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 8: Step 9 (NRPPa Measurement Request) can be performed in parallel with Steps 10/11 (LPP signalling). Hence, only the bigger number of the two procedures are considered (i.e., the latency for NRPPa Measurement Request is not counted in the summation). |
| Step 10 LPP Provide Assistance Data | 28-44.5 | Processing delays: 21-34 ms - UE: - TUEProc-RRCDLInfo - TUEProc-LPPAssi - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5 ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 11 LPP Request Location Information | 23-39.5 | Processing delays: 19 ms - UE: - TUEProc-RRCDLInfo - TUEProc-LPPLocationRe - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 12 RRC Location Measurement Indication | 5-8.5 | Processing delays: 5-8 ms - UE: TUEProc-RRCLocationMeas - gNB: TgNBProc-RRC Signalling delay:0-0.5ms - UE-gNB: TUE-gNB |
| Step 13 RRC Measurement Gap configuration | 13-13.5 | Processing delays: 13 ms - UE: TUEProc-RRCReconf - gNB: TgNBProc-RRC Signalling delay:0-0.5ms - UE-gNB: TUE-gNB |
| Step 14 a DL PRS measurement | 88.5 | TDL-Meas |
| Step 14 b UL SRS measurement | 12 | TUL-Meas Note : Step 14b (UL SRS measurement) can be performed in parallel with Step 14 a (DL PRS measurement). Hence, only the bigger number of the two procedures are considered (i.e., the latency for UL SRS measurement is not counted in the summation). |
| Step 15 LPP Provide Location Information | 20-39.5 | Processing delays: 16-19 ms - UE: - TUEProc-RRCULInfo - TUEProc-LPPLocationRe - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5 ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| Step 16 NRPPa MEASUREMENT RESPONSE | 13-29 | Processing delays: 9 ms - gNB: TgNBProc-NRPPa - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20 ms - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF Note 9: Step 16 (NRPPa Measurement Response) can be performed in parallel with Step 15 (LPP Provide Location Information). The UL- and DL- measurements are made concurrently, hence the results are send at about the same time. Only the bigger number of the two procedures need to be considered (i.e., the latency for NRPPa Measurement Response is not counted in the summation). |
| Step 17 LMF calculation | 2-30 | TLMF-Calc |
| Total values | 288.5-486 |  |