3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104320

e-Meeting, 12th-20th Apr, 2021

Source:	vivo

Title:	Summary of AI 8.3.4: Paging with service indication

Agenda item:	8.3.4

Document for:	Discussion and Decision

# 1	Introduction

This document aims to summarize the contributions submitted to agenda item 8.3.4 of RAN2 #113bis-e. Based on the proposals of these papers, two main issues discussed by multiple contributions are identified as following and captured in section 2:

- Solutions to discriminate type of MT service, i.e. voice, non-voice and any service (Network does not support paging cause feature)

- Solutions to indicate pagingCause

Besides, several trivial issues raised by one or two contributions are captured in the section 3.

# 2	Main Issues

## 2.1	Solutions to discriminate type of MT service

When receiving the paging message in Network B, a MUSIM UE communicating with Network A needs to decide whether to switch to Network B. UE would prefer to do so if the incoming traffic is a high priority service, e.g. voice. A new paging cause for voice service, which allows the UE to identify the reason for paging without requiring RRC connection establishment, was concluded to be supported in SA2 as follows [1]:

The following conclusions are agreed for the baseline functionality: - For both EPS and 5GS, only the voice is supported to be indicated as the paging cause to the UE. The indicated Paging Cause as voice can be determined based on the PPI related to voice. - The network applies the Paging Cause only for UEs that have provided indication they operate in MUSIM mode. NOTE 1: The CS call indicator from MSC is handled differently (see TS 23.272) to this Release 16 Paging Cause. - The Paging Cause Solution shall ensure that a UE can detect when the feature is not supported in the cell where it is camping (either because the RAN or CN does not support the feature). NOTE 2: "voice" refers to MMTel voice (5GS and EPS) and CS domain voice (EPS only). Editor's note: Final conclusion depends on further feedback from RAN WG2, RAN WG3 and SA WG3.

Accordingly, it is proposed in RAN2 [2] that a paging cause is preferred to be added to the NR/LTE Paging message to allow the MUSIM UE to decide if interrupting the ongoing service on the network where the UE is currently connected and to respond to the paging in the other network.

Given that SA2 has concluded the network applies the Paging Cause only for UEs that have provided indication they operate in MUSIM mode, it is further proposed that a new optional IE containing the Paging cause would be needed to indicate if the service is VoLTE/VoNR. It will be included in the Paging Records carried in the Paging message, only for the intended MUSIM UEs [2].

Based on the above, the rapporteur proposes RAN2 to confirm:

- An IE containing the Paging cause is included in the paging message, only for intended MUSIM UEs.

Besides, SA2#143-e has made further progress on this and sent a new LS to RAN2 [3] indicating that: SA2 has concluded that the UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service) by a RAN node not supporting the Paging Cause, either because the RAN node does not support the Paging Cause feature, or because in case of RAN sharing it is configured to operate without the Paging Cause feature for some of the connected CNs.

According to the above conclusion and LS, we have the following observation：

- From SA2 requirement, a UE in MUSIM mode should have enough information to discriminate the type of MT service as follows: voice, non-voice, any service (i.e. UE’s serving CN or eNB/gNB does not support the Paging Cause feature).

To meet the requirement in Observation 1, the following solutions are proposed according to contributions submitted to this meeting:

- Solution 1: Two values defined for pagingCause [2], [4~7]

In this solution, an optional pagingCause IE is included for each paged MUSIM UE in paging message. The pagingCause IE has values for {voice, non-voice}. By not receiving the IE, one MUSIM UE will know that the network does not support Paging Cause Feature and the UE is paged for any service. Otherwise, the UE will obtain the type of incoming service via the received pagingCause IE.

- Solution 2: A single value for pagingCause IE + indication of network support of Paging Cause feature

In this solution, an optional pagingCause IE is included for each paged MUSIM UE in paging message. The pagingCause IE has a single value for {voice}. Network will inform the MUSIM UEs whether it supports the Paging Cause feature or not. If the network supports the Paging Cause feature, one MUSIM UE could further deduce the paging cause as following: if the pagingCause IE is present, the UE is paged for voice; otherwise, the UE is paged for non-voice.

3 sub-solutions are proposed on how the network informs the MUSIM UEs whether it supports the Paging Cause feature or not, as follows.

- Solution 2.1 [8]: The support of the paging cause is delivered in SIB1 or Paging DCI.

Given CN nodes of different operators may have different capabilities, the rapporteur assumes, in case of RAN sharing scenario, the support of the paging cause feature needs to be indicated for each PLMN, respectively.

- Solution 2.2 [9]: According to some paging message extension solutions (details can be found in solution 3 of section 2.2), the ASN.1 encoding for paging message itself can be used as an indication of whether the network supports the Paging Cause feature. A separate list of paging records is to be introduced for MUSIM UEs only is included to indicate whether they are paged for voice service or non-voice service. And MUSIM UE can know if the network support Paging Cause feature as follows:

- If a MUSIM UE paging ID is included in legacy pagingRecordList, then the network does not support paging cause feature, and

- If a MUSIM UE paging ID is included in the new pagingRecordList for MUSIM UE, then the network supports Paging Cause feature.

- Solution2.3 [10]: MUSIM UE is informed whether its serving CN supports Paging Cause feature via NAS. In case the CN node supports Paging Cause feature, a MUSIM UE further deduces whether its serving RAN node supports Paging Cause feature via ASN.1 coding principle. An example of such ASN.1 coding can be found in solution 4 of section 2.2: a parallel list is introduced in paging message to carry the pagingCause IE for MUSIM UEs, and a paged UE could deduce that this is a legacy cell if the whole parallel list is not present.

- Solution 3: Different paging IDs indicates different paging causes [11]

In this solution, the UE and network negotiate the use of alternative identifier for sending paging message meant for service type of VoLTE. Based on the received identifier (i.e. legacy or alternative identifier) in paging message, one MUSIM UE can identify the service type. One MUSIM UE will consider it is paged for any service (i.e. the network does not support paging cause feature) before the negotiation procedure is performed.

Therefore, the rapporteur proposes RAN2 to discuss and select one of the above described solutions.

- RAN2 to discuss which solution should be used for MT service type discrimination

        - Solution 1: Two values defined for pagingCause [

        - Solution 2: A single value for pagingCause IE + indication of network support of Paging Cause feature

        - Solution 3: Different paging IDs indicates different paging causes

As SA2 expects RAN2’s response and some solutions (e.g. solution 2.3) may require SA2 and RAN3 involving. Hence, the rapporteur proposes:

- A reply LS to SA2 and RAN3 is needed to inform the RAN2 decision or preference.

## 2.2 Solutions to indicate pagingCause

Different solutions are proposed to extend paging message to include the pagingCause, details are as follows:

- Solution 1: Including the number of UEs paged for voice in paging message [12]

If only one value for paging cause is defined and the network always puts the paging records with paging cause at the very first/last part of the pagingRecordList, the network can only transfer the number of the paging records with paging cause to UE. For example, at most 32 records can be included in pagingRecordList for NR, 5 bits is enough to indicate UE that how many pagingRecords in the very first/last of the pagingRecordList have the paging cause. The number of the pagingRecord with paging cause is included in paging message or DCI scheduling paging message. One example can be found in Figure 1:

Figure 1 Example of indicating the number of PagingRecords including paging cause

The solution is very simple and introduces very low overhead. Only 5 extra bits are introduced if the number of the pagingRecord with paging cause is delivered in paging message. If the number of pagingRecords with paging cause is delivered in paging DCI, reserved bits in paging DCI is used.

- Solution 2: Including a separate list of pagingRecords for MUSIM UEs paged for voice in paging message [13]

In this solution, new PagingRecordList per PagingCause is signaled in the Paging message. One example can be found in Figure 2:

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                   OPTIONAL,

nonCriticalExtension                SEQUENCE{Paging-v17xy-IEs}     OPTIONAL

}

Paging-v17xy-IEs ::=                SEQUENCE {

voicePagingRecordList-r17          VoicePagingRecordList-r17     OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE{}                OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

VoicePagingRecordList-r17  ::=      SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

Figure 2 Example of indicating the paging cause via a PagingRecordList dedicated for voice

In this example, if a UE that operates in MUSIM mode is paged via the VoicePagingRecordList-r17, this indicates that the UE is being paged for voice. If a UE that operates in MUSIM mode is paged via the legacy PagingRecordList this indicates that the UE is being paged for a cause other than voice.

- Solution 3: Including a separate list of pagingRecords for MUSIM UEs in paging message [9]

It is proposed to define a new separate list (i.e. PagingRecordList2-r17 in Figure 3) including the complete pagingRecords for only MUSIM UEs to indicate whether they are paged for voice service, details for NR paging message can be seen in the ASN.1 example below.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                   OPTIONAL,

nonCriticalExtension                SEQUENCE{Paging-v17xy-IEs}     OPTIONAL

}

Paging-v17xy-IEs ::=                SEQUENCE {

pagingRecordList2-r17             PagingRecordList2-r17     OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE{}                OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList2-r17  ::=         SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-r17 ::=                SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

paginingCause-r17                   ENUMERATED  {voice}     OPTIONAL -- Need S

}

Figure 3 Example of including a separate list of paging records for MUSIM UEs

In this example, if a MUSIM UE finds its paging ID in legacy pagingRecordList, it will know the network does not support paging causes. If a MUSIM UE finds its paging ID in the new pagingRecordList for MUSIM UE (i.e. pagingRecordList2-r17), the UE decides the paging cause according to the pagingCause IE.

- Solution 4: Including a parallel list of pagingCause in paging message [2], [6], [7], [10], [14]

It is proposed to define a new parallel list (i.e. PagingRecordList-v17xy in Figure4) including the pagingCause for MUSIM UEs to indicate whether they are paged for voice service, details for NR paging message can be found in the ASN.1 example in case two values for paging cause is assumed below.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                       OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                           OPTIONAL,

nonCriticalExtension                Paging-v17xy-IEs                       OPTIONAL

}

Paging-v17xy-IEs ::=			     SEQUENCE {

pagingRecordList-v17xy			     PagingRecordList-v17xy				OPTIONAL,	-- Need N

nonCriticalExtension			     SEQUENCE {}						OPTIONAL

}

PagingRecordList ::=              	 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v17xy ::=			 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v17xy

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v17xy ::=				  SEQUENCE {

pagingCause-r17						  ENUMERATED {voice, other}			OPTIONAL		-- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

Figure 4 Example of including a separate list of pagingCause for UEs

If gNB includes PagingRecordList-v17xy, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix). Since the pagingCause IE is only present for MUSIM UEs, the additional bits of the pagingCause are expected to incur neglectable impacts to the RRC Paging message, both regarding the message size and the possible coverage shrinking.

- Solution 5: Different paging causes are indicated implicitly with different paging IDs [11]

In this solution, the UE and AMF negotiate the use of alternative identifier for sending paging message meant for service type of VoLTE/SMS. In this case the RRC paging message needs not to include additional cause IE. MUSIM UE in RRC-IDLE mode will monitor the paging message for its primary GUTI value and alternative identifier assigned for the service type. AMF may need to ensure that these two identifiers have same UE-ID value so that UE can monitor these two identifiers over the same paging occasion. Based on the received identifier in paging message, one MUSIM UE can identify the service type. A similar mechanism can also be applied to RAN paging. The solution avoids changes to RRC paging message and RAN3 interface.

Therefore, the rapporteur proposes RAN2 to discuss and select one of the solutions described above.

- RAN2 to discuss which solution should be used for paging message extension:

- Solution 1: Including the number of UEs paged for voice in paging message

- Solution 2: Including a separate list of pagingRecords for MUSIM UEs paged for voice in paging message

- Solution 3: Including a separate list of pagingRecords with pagingCause for MUSIM UEs in paging message

- Solution 4: Including a parallel list of pagingCause in paging message

- Solution 5: Different paging causes are indicated implicitly with different paging IDs

# 3	Other Issues

## 3.1 Security and Privacy issues

Some security and privacy issues have been raised in [14] and [15]. Given SA3 has not concluded their study on security related issues on Multi-SIM, for the sake of progress in RAN2, a way forward is proposed as follows:

- Postpone the security issue for paging cause.

- Based on SA3 progress on paging cause, RAN2 may re-evaluate the overhead and feasibility on paging cause

Hence, the rapporteur proposes:

- The discussion related to security/privacy issue for paging cause is postponed in RAN2 and can be triggered according to SA3 progress.

## 3.2 UE behaviour upon reception of the paging cause

If RAN2 agrees to add the paging cause information in the paging message, the UE behaviour should be also considered when receiving the paging cause. If RAN2 agrees that the UE AS needs to forward the paging cause information to the UE NAS, the behaviour may need to be specified in TS 36.331. One company suggests RAN2 to discuss whether the UE behaviour upon reception of the paging cause can be specified in LTE, given some companies don’t want to specify MUSIM functions in TS 36.331 [16].

However, specifying the paging cause for EPS has been agreed in SA2. And according to the MUSIM WID objective on paging cause is described as follows:

3）Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR[ RAN2]. - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

From the rapporteur understanding, the UE behaviour upon reception of the paging cause should be specified in both NR and LTE specifications.

Therefore,

- UE behavior upon reception of the paging cause is specified in both NR and LTE specifications.

## 3.3 Define spare paging cause value

Given that SA2 agreed to support only single paging cause, it seems unnecessary to define some spare paging cause values as it may have a negative impact on the size of paging message. Hence, the following is proposed: do not define spare paging cause values in paging message [9].

The rapporteur tends to agree with this analysis and proposes:

- Do not define spare paging cause values in paging message.

# 4	Conclusion

Based on all the papers submitted to AI 8.3.4 in RAN2 #113bis-e, we propose the following:

- An IE containing the Paging cause is included in the paging message, only for intended MUSIM UEs.

- RAN2 to discuss which solution should be used for MT service type discrimination.

        - Solution 1: Two values defined for pagingCause [

        - Solution 2: A single value for pagingCause IE + indication of network support of Paging Cause feature

        - Solution 3: Different paging IDs indicates different paging causes

- A reply LS to SA2 and RAN3 is needed to inform the RAN2 decision or preference.

- RAN2 to discuss which solution should be used for paging message extension.

- Solution 1: Including the number of UEs paged for voice in paging message

- Solution 2: Including a separate list of pagingRecords for MUSIM UEs paged for voice in paging message

- Solution 3: Including a separate list of pagingRecords with pagingCause for MUSIM UEs in paging message

- Solution 4: Including a parallel list of pagingCause in paging message

- Solution 5: Different paging causes are indicated implicitly with different paging IDs

- The discussion related to security/privacy issue for paging cause is postponed in RAN2 and can be triggered according to SA3 progress.

- UE behavior upon reception of the paging cause is specified in both NR and LTE specifications.

- Do not define spare paging cause values in paging message.

# References

[1] 3GPP TR 23.761, "Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM).";

[2] R2-2103758, Introduction of a Paging cause indication	Ericsson	discussion;

[3] S2-2102039, LS on System support for Multi-USIM devices, SA2;

[4] R2-2104158, Paging with service indication	MediaTek Inc.	discussion;

[5] R2-2103226, Paging Cause and Busy Indication 	Qualcomm Incorporated	discussion;

[6] R2-2103195, Support for Multi-SIM paging cause from SA2 LS	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core;

[7] R2-2103304, Introduction of paging cause	China Telecommunications	discussion;

[8] R2-2103246, Supporting of Paging Cause Solution detection	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM-Core;

[9] R2-2102913, Discussion on support of paging cause for multi-USIM devices	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core;

[10] R2-2103348, Introduction of Paging Cause	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core;

[11] R2-2103186, Further analysis on Service type indication in paging and signalling mechanism for BUSY indication	Nokia, Nokia Shanghai Bells	discussion	Rel-17;

[12] R2-2103248, Discussion on the transmission of paging cause	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM-Core

[13] R2-2103958, Discussion on paging service indication for MUSIM	Futurewei Technologies	discussion;

[14] R2-2102794, Paging with Service Indication	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core;

[15] R2-2103483, Discussion on the paging with service indication	Huawei, HiSilicon	discussion	Rel-17;

[16] R2-2103574, Support of Paging Cause	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core;

[17] R2-2104171, Discussion of the paging cause support for MUSIM	Xiaomi Communications	discussion