3GPP TSG RAN WG2 Meeting #113bis-e                                                    R2-2104367

# Electronic meeting, 12th-20th April 2021

Agenda item:		8.12.3.1

Source:			Intel Corporation

Title:		Summary of offline 101 - [REDCAP] eDRX cycles - second round

Document for:	 	Discussion and decision

# Introduction

The intention is to continue the discussion the following topics as part of the offline [AT113-e][101][RedCap] eDRX cycles (Intel) taken into consideration the related proposals on submitted TDocs [1]-[14] and phase-1 discussion [17]:

- FFSs on eDRX configuration, e.g. based on p4.x from R2-2104360 [17]

- p5, p6, p7 from R2-2102852 [3]

It is also important to keep in mind RAN2 agreements taken on this topic during this meeting:

Agreements:

- RAN decides and configures eDRX via RRC for RRC_INACTIVE (FFS on the need and details of coordination with the CN)

- At least for eDRX cycle, the configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different (FFS for PTW, e.g. length and starting point, when eDRX cycles are longer than 10.24s)

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

The deadlines for this offline are:

- Final deadline (for companies' feedback): Friday 2021-04-16 02:00 UTC

- Final deadline (for rapporteur's summary in R2-2104367): Friday 2021-04-16 06:00 UTC

- Proposals marked "for agreement" in R2-2104367 that are not challenged until Monday 2021-04-19 10:00 UTC will be declared as agreed via email by the session chair. For the rest the discussion will continue online.

For reference Annex A includes sections 7.1 and 7.3 of TS 36.304 addressing LTE eDRX operation and Annex B includes relevant discussion points from the 1st round of this offline.

# Phase 2: Discussion

## Configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

During online session RAN2 agreed that at least eDRX cycle could have different value configured for RRC_IDLE and RRC_INACTIVE. However, it was left FFS for PTW (e.g. length and starting point) as well as the expected operation when UE is configured with different eDRX cycles. This section aims to address those FFS considering some of the options discussed during the online session, as well as, the inputs provided in related discussion points during the 1st round of this offline (related details kept in Annex B for reference).

- To address the FFS captured on PTW configuration for RRC_IDLE / RRC_INACTIVE, please indicate your preferred option(s) in relation to PTW operation for eDRX cycle >10.24sec in RRC_IDLE and RRC_INACTIVE, considering the following options:

- It is left up to network implementation any coordination on how to decide the PTW configurations for RRC_IDLE and for RRC_INACTIVE (i.e. PTW may be configured same or different).

- A common PTW configuration is provided for RRC_IDLE and for RRC_INACTIVE.

- A common PTW_start only is provided for RRC_IDLE and for RRC_INACTIVE (i.e. PTW_end can be different).

- PTW for RRC_IDLE overlaps with PTW for RRC_INACTIVE, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller).

- Other options.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Intel | 1) | We understand that network should guarantee an operation as explained by options 4) or even 2) and 3) but we are not sure whether this needs to be specified. |
| OPPO | Option 2 | From UE power saving point of view, we prefer to use a common PTW length and a common PTW starting point for CN paging and RAN paging |
| Xiaomi | Option2 | Currently, CN configures the PTW considering the factors such as SFN misalignment among gNBs, latency of N2 interface, gNB scheduling and so on. For RAN paging, some of these same factors may potentially affect RAN paging PTW, for example synchronization inaccuracy and gNB processing time. Since the CN is expected to have the overall knowledge of the synchronization performances in both the UE and the gNB, it seems feasible that the CN configures the PTW for both CN and RAN paging for simplicity. As such, the RAN do not need to decide (PH, PTW). Also, we think the eDRX cycle contributes more than the PTW to the power consumption gain. As such, a common PTW configuration is feasible. |
| CATT | 1), 2) | The more overlap between the PTW for idle and inactive, the more power saving benefit the UE can obtain, so we think coordination is needed between CN and RAN to configure a common PTW configuration for RRC idle and inactive. |
| Huawei, Hisilicon | Option 4) with updates | To save UE powering, PTW for RRC_IDLE should overlap with PTW for RRC_INACTIVE. There are some potential methods to ensure this: 1) by network implementation 2) by putting constraints on the configuration so the PTWs always overlap We propose to delete “, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller)” in option4, to make the proposal more general. We support the updated Option 4 as baseline. The details can be discussed later. In our view, option 4 covers options 2,3,4 at high level, i.e. allow UE to monitor only one “overlapping” PTW for power saving by. constraining the NW to configure overlapping PTWs. We understand “common” in option 2 as same length and same start point. So, there is only one PTW and only one eDRX cycle and one Length is used to calculate this PTW. |
| LGE | Option 2 | A common PTW configured by the CN is preferred for simplicity. |
| MediaTek | Option 2 | Prefer Option 2, because common PTW configuration could help to reduce the UE implementation complexity and this could be beneficial for RedCap UEs. |
| Ericsson | Option 1/ 4 / 5 ? | Option 1 is not fully clear – what does “left up to network implementation any coordination” exactly mean? We assume there is some assistance information e.g. from CN to RAN (as in LTE baseline, and indeed specified in NGAP already) which could be used by the RAN to configure the PTW according to the other options. In option 2, what is the understanding of “common PTW”? Per agreement, RAN should be able to configure a different eDRX cycle from CN, so does this only refer to PTW length and starting point, where the frequency (i.e. PH) of the PTW depends on the RAN configuration in case the cycle is shorter than the CN cycle? If CN configures a long PTW and RAN configures a shorter RAN paging eDRX cycle, it does not make sense to use such a long PTW for RAN paging, thus we think this option might not be always technically feasible. In option 3, if we assume the eDRX cycle lengths are powers of 2 then depending on the design, it can be this option is fulfilled automatically. We think like Huawei above that Option 4 is something that could be considered as target for the design, however we don’t think the RAN configuration should be restricted in the specifications. In any case, as agreed RAN should be able to configure a different eDRX cycle and this should not be limited by any PTW options. |
| Qualcomm | 1 or 3 | Option 4 is fine in theory but harder to capture in spec than other options. Option 2 is |
| Apple | Opt 2 is the first preference, but 1 or modified 4 is ok. | The main aim of separate RAN config for eDRX is for paging with reduced latency by RAN to move the UE back to CONNECTED (compared to CN DRX), but with PTW/PH in operation, we think the UE is already delay tolerant, so RAN can just re-use the CN DRX config which leads to option-2. However, with the agreement to allow RAN to configure separate eDRX config for INACTIVE, the main aim is to cover the case where the UE is reachable in INACTIVE if the NW thinks the UE is in IDLE. From that perspective, if NW can ensure this, then opt-1 is ok. Opt-4 does say that PTWs are to be overlapped. We are trying to understand the reasoning behind having the IDLE PTW length longer than INACTIVE PTW in opt-4. If the aim is to allow CN to reach the UE with CN page (while the UE thinks it is in INACTIVE), it would be better to have the RRC_IDLE PTW completely within the INACTIVE PTW, so that the UE does not miss the CN page? We have assumed that the RAN T is shorter than the CN T or the same so that atleast some RAN eDRX cycle can coincide with the CN DRX during the overlapped PTWs. |
| Futurewei | 1? / 4 | Although it is up to network implementation to decide the PTW configurations for RRC_IDLE and for RRC_INACTIVE, a sensible network implementation (for the benefit of UEs) is to try to maximize the overlap between the PTWs for RRC_IDLE and RRC_INACTIVE. Option 2 is too restrictive. Option 3 restricts the starting point. We can consider relaxing option 4 as suggested by Huawei. |
| DENSO | 1 | Agree with Intel views, as a result of setting eDRX for IDLE and for INACTIVE, those settings may match option 2, option 3, but we are not sure whether they need to be specified. |
| Sharp | Option2 |  |
| vivo | Option4 | As the eDRX cycle for RRC_IDLE may be equal or longer than the eDRX cycle for RRC_INACTIVE, the PTW for RRC_IDLE should not be smaller than the PTW for RRC_INACTIVE to ensure the network has enough opportunities to deliver the CN paging message. Besides, the duration of one PTW for RRC_IDLE should cover the whole duration of one PTW for RRC_IDLE, to minimize the UE’s DRX ON duration for paging monitoring and power consumption. |
| CMCC | Option2/4 | It’s better to have more overlap between PTW for Idle and Inactive considering UE power saving, so Option 2 could be considered. And for Option 4, we have similar view with Huawei. |
| ZTE | Option 2 | For UE power saving, it is beneficial to configure a PTW for RRC IDLE always overlaps the PTW for RRC INACTIVE (assuming eDRX cycle for RRC IDLE is no shorter than RRC IDLE). Thus, within the overlapped PTW, UE monitor CN and RAN paging on the same PO. In the PTW for RRC INACTIVE only, UE monitor RAN paging. For option 1, we understand proponent think it can be left to NW implementation to ensure common PTW. However, if the PTW calculation mechanism in TS38.304 is reused, it is hard for network to make sure the PTW_start for idle and Inactive are the same. For example, network can configure eDRX cycle for RRC IDLE as a integer multiple of RRC INACTIVE. According to PH formula in LTE-M, the PH for RRC IDLE can always overlap the PH for RRC INACTIVE. The PH is the H-SFN satisfying the following equation: - SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H), However, according to PTW_start formula, the PTW_start calculated based on two different eDRX cycle may probably be different: PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 256* ieDRX, where - ieDRX = floor(UE_ID_H /TeDRX,H) mod 4 As a result, the UE have monitor two PTWs within the same PH. This is not UE power efficient and unnecessary. For PTW length, the intention is to provide multiple chances for paging message for robustness. The benefit of having different PTW length for IDLE and INACTIVE is unclear to us. Because we think CN paging and RAN paging have same requirement of paging robustness. |
| Sequans | 1 or 4 Not 2 | NW implementation should be fine. If we wish to specify, 4 is least restrictive. If it proves to complex, 3 is fine as well. 2 seems too restrictive |
| Samsung | 1) | We have assumed any coordination is required to maximize UE power saving gain while guaranteeing the flexibility. |

- Do you support defining different and/or specific behaviour depending on whether eDRX cycle for RRC_INACTIVE is configured differently than for RRC_IDLE? If yes, please justify your response and explained your scenario and preferred approach.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | No |  |
| OPPO | No |  |
| Xiaomi | No | Even if from the network point of view, gNB configures eDRX cycle for RRC_INACTIVE other than for RRC_IDLE, from UE’s point of view, a common framework is desirable for simplicity. |
| CATT | No but | … we still need to specify UE behavior based on the eDRX configuration combinations for idle and inactive, per below (and check if all are allowed). Not sure if it can be guaranteed the UE behavior is same or different across all cases. A least from our perspective, we should aim at minimizing UE behavior differences across all cases: 1) eDRX only configured for idle, cases eDRX ≤ 10.24sec and eDRX >10.24sec 2) eDRX only configured for inactive, cases eDRX ≤ 10.24sec and eDRX >10.24sec 3) eDRX configured both for inactive and idle 3-a) both eDRX ≤ 10.24sec 3-b) one of the eDRX≤ 10.24sec and the other eDRX>10.24s 3-c) both of the eDRX>10.24s (Whether allow eDRX cycle for inactive longer than eDRX cycle for idle can be further discussion. We just list all the possible cases.) |
| Huawei, Hisilicon | Postpone | Although the behaviours should be very similar, there may be some specific details. This can be discussed in later stage. |
| LGE | No |  |
| MediaTek | FFS | Depends on the meaning of “different/specific behavior”. This should be further studied based on companies’ input. |
| Ericsson | - | Not sure what the discussion point is really asking, like MTK, what is “different / specific behavior” referred to in the question We think the UE monitors paging during PTW (or PTWs) similarly as the current eDRX is specified for LTE. In case there is no PTW e.g. for RAN paging then RAN paging is monitored without PTW (like for LTE-M). Details can be discussed further once the overall design agreed. |
| Qualcomm | No | We do not see justifications for that |
| Apple | FFS | Similar view as MediaTek. |
| Futurewei | FFS |  |
| DENSO | No |  |
| Sharp |  | Similar view as Ericsson |
| vivo | No |  |
| CMCC | FFS | Similar view with Ericsson. |
| ZTE | FFS | Same view as MediaTek and Huawei |
| Sequans | Probably not | We don’t see any difference in principle, but some minor details may need to be clarified later (e.g where one is with and one without PTW) |
| Samsung | No |  |

- From RAN2 point of view, do you support that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC) [3][6][8]? If not, please justify your response.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes | In TS 23.501 section 5.31.7.2.1:“If the UE supports eDRX in RRC inactive, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC-INACTIVE up to the value for the UE's idle mode eDRX cycle as provided by the AMF in "RRC Inactive Assistance Information" or up to 10.24 seconds (whichever is lower).” That is a potential solution on the table. |
| CATT | Yes | Assistance to RAN to decide suitable eDRX for Inactive |
| Huawei, Hisilicon | Yes | At least reuse eDRX config IE, defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC. RAN can take it as reference to determine eDRX parameters for RRC_INACTIVE. |
| LGE | Yes |  |
| MediaTek | Yes | Existing mechanisms for E-UTRA/5GC can be re-used. |
| Ericsson | Yes |  |
| Qualcomm | Yes | Useful to have it |
| Apple | Yes |  |
| Futurewei | Yes |  |
| DENSO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes | To maximize UE’s power saving gain, RAN should set the eDRX configuration for RRC_Inactive in a manner to maximize the PO overlapping duration for CN and RAN paging. Hence, AMF should provide RAN with the eDRX configuration for RRC_Idle. |
| CMCC | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes |  |
| Samsung | Yes |  |

- From RAN2 point of view, do you support that RAN provides necessary eDRX assistance information for RRC_INACTIVE to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration [6]? If yes, please justify your response.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Not sure | We are not sure for this proposal. E.g. we do not support the proposal if the indication is sent in every transition into RRC_INACTIVE. On other hand, even if the intention is to send a single indication, this might need to send every time that RAN modifies one of the eDRX parameters. |
| OPPO | No | We don’t know what the benefit is by doing so. |
| Xiaomi | No | Not convinced. We agree the intension that CN should be aware whether the UE is not reachable or not. If RAN configures the UE with an e-DRX cycle in RRC-INACTIVE up to the value used for the UE's idle mode e-DRX cycle as provided by the AMF, or up to the maximum value allowed based on the NAS retransmission timers (whichever is lower). Does the problem solved? |
| CATT | Not sure | It might help CN “adapting” its eDRX configuration that could fit different gNB requirements covered by CN. Not sure though what this would exactly be. So we can keep it FFS for now. |
| Huawei, Hisilicon | Yes | CN needs to configure suitable NAS retransmission timers according to eDRX configuration of RRC_INACTIVE. |
| LGE | FFS | We think the RAN can configure eDRX for RRC_INACTIVE based on the information provided by the CN, but not sure whether the opposite direction is also needed in this stage. We are wondering if RAN3 and SA2 needs be involved in this issue. |
| MediaTek | Wait for SA2 | Too early to say. This will also depend on SA2/CT1 response to question in the LS on eDRX. Whether the RAN sends assistance information to CN or RAN follows the limitations provided by CN may need to be discussed after the response is received. |
| Ericsson | Yes | The benefit is that the CN knows when the UE is reachable for transmission of MT data. How would it know otherwise? According to our agreement, RAN configures eDRX cycle for RRC_INACTIVE, therefore CN cannot know when the UE is reachable for paging or deliver of DL data unless this information is provided to the CN from RAN in some way. Note that in RRC_INACTIVE the UE is in CM_CONNECTED mode in core network, thus CN assumes UE is reachable all the time, which would not be true (and for this reason we are sending the LS to SA2/CT1, to check for feasibility and solutions). The exact details should be coordinated with RAN3/SA2, but for RAN2 point of view we cannot think of any reason why such information should not be provided. |
| Qualcomm | Depends | If SA2 confirms eDRX cycle for RRC Inactive can be longer than 10.24s, it may be useful to have RAN send that information to CN. Otherwise, we do not think it is necessary. |
| Apple | wait for SA2 | We think the discussion would be clearer with SA2 LS reply. |
| Futurewei | Yes, if | If SA2 confirms eDRX cycle for RRC Inactive can be longer than 10.24s. |
| DENSO | No | If it’s worth doing it, we may consider. |
| Sharp | Wait for SA2 | It depends on whether SA2 needs some information to enhance the NAS retransmission timer etc. |
| vivo | No | With the assumption that CN provides eDRX configuration for RRC_IDLE to RAN, we see no extra benefit for the RAN to provide eDRX configuration for RRC_IDLE to CN. |
| CMCC | Wait for SA2 | We agree that there may be some benefit, but it could be discussed depend on SA2/CT1 response to the LS on eDRX. |
| ZTE | Wait for SA2 | It depends on how CN handle the NAS retransmission timer issue, e.g. buffer DL data in CN or enlarge NAS retransmission timer. We suggest to wait. |
| Sequans | Wait for SA2 |  |
| Samsung | Wait for SA2 |  |

- Please indicate if there are other scenarios that need to be discussed in relation to the eDRX configuration, and if so, explain your preferred approach.

| Company’s name | Comments, if any |
| --- | --- |
| CATT | See our comment to discussion point 2). We think we should also discuss if all configuration combinations are allowed. |

## Paging eDRX mechanism for RRC_IDLE / RRC_INACTIVE

This section aims to define paging eDRX mechanism for UEs in RRC_IDLE / RRC_INACTIVE. For reference, legacy LTE eDRX operation from TS 36.304 is included in Annex A (sections 7.1 and 7.3).

### Paging mechanism for eDRX up to 10.24sec

- Do you confirm that for UE side, a common paging mechanism is enabled for eDRX ≤ 10.24sec in RRC_IDLE and RRC_INACTIVE (with details to be discussed in following discussion points)? If not, please justify your response and explained your preferred approach.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes |  |
| OPPO | Yes | For a UE in RRC IDLE, if CN eDRX cycle do not exceed 10.24s, the UE monitors PO with eDRX cycle T, where T is determined by CN eDRX cycle. For a UE in RRC INACTIVE, if both CN eDRX cycle and RAN eDRX cycle do not exceed 10.24s, the UE monitors PO with eDRX cycle T, where T is determined by the shortest of CN eDRX cycle and RAN eDRX cycle. |
| Xiaomi | Yes | For eDRX cycles no longer than 10.24s for both RRC_IDLE and RRC_INACTIVE, the PTW and PH are not used. The details can be further considered. |
| CATT | Yes | We understand the question as eDRX is configured ≤ 10.24sec for both Idle and Inactive. The question does not cover the mixed cases, if allowed, mentioned in our response to discussion point 2). |

| Huawei, Hisilicon | Yes | As long as SA2/CT1 has no concern on our WID assumption for eDRX = 10.24s, it is fine. |
| --- | --- | --- |
| LGE | Yes | How to determine T. We think the UE should monitor the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX in RRC_IDLE, and the shortest of the UE specific DRX value, if allocated by upper layers, the RAN paging cycle in RRC_INACTIVE. |
| MediaTek | FFS | In principle, we agree with the intention to have as much commonality as possible. However, it is unclear what is being asked here. The details of the IDLE/INACTIVE mechanisms are still under discussion, and it is possible that a common mechanism can be applied, but it is FFS for now. |
| Ericsson | Maybe | Not exactly clear what is being asked – what is “common paging mechanism”? - but we assume we use the LTE baseline for this case, and as the WID states no PTW or PH are used. If this was the intention of the question then our answer is “yes”. |
| Qualcomm | Yes | From UE’s perspective, we prefer common operations between the two RRC states. |
| Apple | FFS | Same view as MediaTek |
| Futurewei | Yes |  |
| DENSO | Yes | We think that a common mechanism is suitable from the viewpoint of UE power consumption for cycles of 10.24 seconds or less. |
| Sharp | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes with comments | We understand common paging operation means for eDRX ≤ 10.24sec, no PTW is applied, and set T=eDRX cycle in the PF/PO formula (LTE-M/NB-IoT mechanism of eDRX cycle =5.12 sec). If the understanding is correct, we confirm common paging mechanism for both RRC IDLE and RRC INACTIVE. Further, we think when both eDRX cycle ≤ 10.24sec, it is beneficial to have overlapped POs for both CN and RAN paging. |
| Sequans | Yes |  |
| Samsung | Yes |  |

For eDRX ≤ 10.24sec in RRC_IDLE and RRC_INACTIVE, the assumption is that PTW/PH model is not used as captured in the agreed WID [15].

- Please indicate the applicable options to enable paging eDRX mechanism for eDRX ≤ 10.24sec in RRC_IDLE and RRC_INACTIVE:

- When UE is configured with eDRX ≤ 10.24sec, UE only monitors UE-specific paging eDRX applicable to the corresponding RRC state (instead of the smallest of the configured ones). i.e. a UE in RRC_INACTIVE monitors eDRX cycle configured for RRC_INACTIVE, and a UE in RRC_IDLE monitors eDRX cycle configured for RRC_ IDLE. [3]

- When UE is configured with eDRX ≤ 10.24sec, the UE in RRC ACTIVE monitors PO with eDRX cycle T, where T is determined by the shortest of CN eDRX cycle and RAN eDRX cycle. [2]

- Other options.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Intel | 1) |  |
| OPPO | Option 2 | We need to clarify the option 2 applies to RRC INACTIVE. For a UE in RRC IDLE, if the UE is configured with eDRX ≤ 10.24sec, the UE monitors PO with eDRX cycle T, where T is determined by CN eDRX cycle. |
| Xiaomi | - | Option2 seems a simple way. But more details can be discussed. |
| CATT | 3) | When eDRX is configured ≤ 10.24sec for both Idle and Inactive, - In Idle, UE monitors paging with eDRX cycle T, where T is determined by CN eDRX cycle. - In Inactive, UE may receive both CN and RAN pages, so it monitors paging according to both eDRX cycles T1 and T2, where T1 and T2 are determined by CN and RAN eDRX cycles, respectively. Complete freedom in configuring T1 and T2 should be avoided and we should preferably have T1 and T2 commensurable (T1 = nT2). |
| Huawei, Hisilicon | 2), but | As long as SA2/CT1 has no concern on our WID assumption, it is fine. Option 2) should be extended to the scenario where only eDRX for one RRC state is configured. |
| LGE | Option 2 |  |
| MediaTek | Option 1 | We think this aligns with the LTE (eMTC) mechanism when eDRX cycle = 512 RFs. RAN should ensure that CN paging can be delivered in the POs defined for RAN in RRC_INACTIVE. |
| Ericsson | Option 2 | Agree with OPPO on RRC_IDLE operation. Agree also with Xiaomi that this starts to be close to stage-3 details but we can capture this as a design goal. If RAN paging is configured < CN paging, RAN paging should be monitored even if the CN paging cycle is ≤ 10.24 s. |
| Qualcomm | 1 | We prefer to reuse the LTE procedure |
| Apple | 1 | Same view as MediaTek and Qualcomm |
| Futurewei | 2 |  |
| DENSO | 1 | UE should be able to choose each monitoring state. |
| Sharp | 1 |  |
| vivo | Option 2 | Same view as OPPO. |
| CMCC | Option 1 |  |
| ZTE | Option 3 | First, PO is calculated based on T, and different T values may result in different PO positions (i.e. subframe). For Option 1, seems it cannot fulfil the requirement that UE in RRC INACTIVE monitor both CN paging and RAN paging. For Option 2, this option may cause paging failure when RRC state mismatches. For example, in the case eDRX cycle for RRC IDLE is larger than RRC INACTIVE, if UE thinks it is in RRC INACTIVE while network thinks UE is in RRC IDLE, UE will monitor PF/PO based on eDRX cycle for RRC INACTIVE. While the network transmit paging message based on eDRX cycle for RRC IDLE. According to current NR PF/PO formula, the PF according to the two eDRX cycle will overlap, but the PO are different. In this case, UE will miss CN paging. Our understanding is aligned with CATT. On top of it, for RRC_INACTIVE, if we want to ensure overlapped POs for CN paging and RAN paging, we can consider to always use the eDRX cycle configured for IDLE for PO calculation for RAN paging. In summary, we think these details can be further discussion, there is no need to make decision in this meeting. |
| Sequans | 1 | Agree with MediaTek, QC |
| Samsung | Option 2 |  |

### Paging mechanism for eDRX above 10.24sec

- Do you confirm that for UE side, a common paging mechanism is enabled for eDRX >10.24sec in RRC_IDLE and RRC_INACTIVE (with details to be discussed in following discussion points)? If not, please justify your response and explained your preferred approach.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes |  |
| OPPO | Yes | For a UE in RRC IDLE, if CN eDRX cycle is above 10.24s, the UE monitors PO with T during PTW, where T is determined by the shortest of CN configured DRX value and default DRX cycle. For a UE in RRC INACTIVE, if both CN eDRX cycle and RAN eDRX cycle are above 10.24s, the UE monitors PO with T during PTW, where T is determined as follows: - For CN PTW, T is determined by the shortest of CN configured DRX value, and default DRX cycle and RAN paging cycle. - For RAN PTW, T is RAN paging cycle. - For common PTW, T is determined by the shortest of CN configured DRX value, default DRX cycle and RAN paging cycle. |
| Xiaomi | Yes | For eDRX cycles are longer than 10.24s for both RRC_IDLE and RRC_INACTIVE, the PTW and PH are used. The details can be further considered. |
| CATT | Yes | We understand the question as eDRX is configured > 10.24sec for both Idle and Inactive. The question does not cover the mixed cases, if allowed, mentioned in our response to discussion point 2). |
| Huawei, HiSilicon | See comments | What is the relationship with Discussion point 2)? seem to be the same. |
| LGE | Yes |  |
| MediaTek | FFS | In principle, we agree with the intention to have as much commonality as possible. However, it is unclear what is being asked here. The details of the IDLE/INACTIVE mechanisms are still under discussion, and it is possible that a common mechanism can be applied, but it is FFS for now. |
| Ericsson | Maybe | Again not very clear what is being asked. The exact solution is also pending feasibility of the longer cycles from SA2/CT1, but in general our view is that we use the LTE baseline for RRC_IDLE eDRX and for RRC_INACTIVE we try to achieve similar behaviour. That is, the paging is monitored during a PTW according to the DRX configuration of the UE/the cell. The details can be discussed further. |
| Qualcomm | Depends | If PTW is configured, RRC Idle and RRC Inactive may use different T within PTW. Otherwise, the same operation is used in the two RRC states. |
| Apple | FFS | We agree to have a common framework where possible, and in this question it is about both IDLE and INACTIVE with eDRX >10ses, but we also need to discuss the handling of different combnations of IDLE/INACTIVE where both of them are NOT >10.24 sec and that point is yet to be discussed. |
| Futurewei | Yes |  |
| DENSO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes | PH/PTW mechanism is applied for both eDRX cycles. However, the detailed mechanism for paging monitoring in CN/RAN/common PTW differs, as commented by OPPO. |
| Sequans | Maybe | Agree with Erisson |
| Samsung | Yes |  |

- Please indicate the applicable options to enable paging eDRX mechanism for eDRX > 10.24sec in RRC_IDLE and RRC_INACTIVE:

- eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC. [2][3][5][6]

- NR PTW definition is different than LTE PTW (which is defined as fixed to multiples of 256 SFNs), e.g. considering configurable by the network. [12]

- Other options.

Note: whether different handling is required when eDRX cycle is configured with different is discussed separately in previous Discussion point 2).

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Intel | 1) |  |
| OPPO | Option1/2 | We are ok to use LTE eDRX mechanism as baseline. And some optimization could be considered, e.g. in order to further balance the paging load in time domain, the determination of PTW_start could be different from LTE. |
| XIaomi | Option1 | Agree that operation defined for E-UTRA/5GC is used as baseline. |
| CATT | 1) 2) | We should start with the LTE mechanism as baseline, and adapt it considering NR specific feature(s), e.g. multiple beams. |
| Huawei, Hisilicon | Option 1 | Changing PTW definition has additional CN impact. |
| LGE | Option 1 |  |
| MediaTek | Option 2 | A more flexible PTW_start configuration is useful to evenly spread the paging load. |
| Ericsson | Option 1 | Use LTE as baseline, we can discuss any modifications later if there is a good reason to change. |
| Qualcomm | 1, 2 | We have the same comment as OPPO |
| Apple | Op 1 | This can be the starting baseline. Op2 can be FFS. |
| Futurewei | 1 | We prefer the LTE mechanism as a baseline. |
| DENSO | 1&2 | We agree to use E-UTRA/5GC as baseline, but we should consider each time what may have an effect on the system like option 2. |
| Sharp | Option1 |  |
| Vivo | Option1 and Option2 | We are fine to reuse eDRX mechanism in LTE as the baseline. But in NR with multiple beam scenario, PTW definition may be updated to adapt beam sweeping, like the PO definition in NR. |
| CMCC | Option1 |  |
| ZTE | Option 1 | We think option 1 can be the baseline. |
| Sequans | 1 as baseline, 2 FFS | Start with LTE mechanism as baseline and change it only if needed |
| Samsung | Option 1 |  |

### Other details and cases related to paging eDRX mechanism

- Do you confirm that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN [1]? If not, please justify your response and explained your preferred approach.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes | We do not see the reason to configure different paging frame offset for CN paging and RAN paging |
| CATT | Yes |  |
| LGE | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes | In principle this sounds fine but this is also a stage-3 detail which does not need to be decided yet before we have concluded stage-2 discussions. Let’s use LTE as baseline, we can discuss any modifications later if there is a good reason to change. |
| Qualcomm | Yes | Those parameters are cell specific and depend on how gNB configures paging channel resources. It helps ensure there are always some overlapping POs between RAN paging and CN paging. |
| Apple | Yes | Anyway it’s RAN that has to carry the CN paging on the air interface. |
| Futurewei | Yes |  |
| DENSO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes |  |
| Samsung | YEs |  |

- Do you confirm that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX) [12]? If not, please justify your response and explain your preferred approach.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Intel | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes |  |
| Huawei, Hisilicon | Yes |  |
| LGE | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes | We can look into further details if needed later. |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| Futurewei | Yes |  |
| DENSO | Yes |  |
| Sharp | Yes |  |
| vivo | Yes |  |
| CMCC | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes |  |
| Samsung | Yes |  |

- Please indicate if there are other topics related to paging eDRX mechanism that needs to be discussed.

| Company’s name | Comments, if any |
| --- | --- |
| MediaTek | Whether to indicate the eDRX support in a cell by a flag (e.g. eDRX-Allowed) in SI (e.g. SIB1) should be discussed [12]. |
| Ericsson | As brought up in our contribution R2-2102965 [6], we are wondering what are the companies’ views on how the buffering should be done in case there is pending DL data and both IDLE and INACTIVE cycles are relatively long. Our preference would be to consider buffering in CN in this case e.g. for better support of UE mobility during sleep and as there is already buffering mechanism for the RRC_IDLE case (at least in EPC). This is not an issue to be resolved by RAN2 alone but for the sake of progressing the work we provided this view in the draft LS at the end of our contribution. |

## Minimum value of eDRX

- Please indicate your preferred minimum value allowed for the eDRX cycle in RRC_IDLE and RRC_INACTIVE, considering the following options:

- 2.56 sec. [3][9][13]

- 5.12 sec. [8][10][14]

- Other options.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Intel | 2) | We are ok allowing 2.56sec as the min. eDRX cycle that can be set in RRC_IDLE / RRC_INACTIVE if TS impact is minimal (i.e. follows the same operation as defined for any other eDRX value ≤ 10.24sec). |
| OPPO | Option 3 | We prefer to reuse the same eDRX lower bound as LTE for RedCap. For RedCap UEs that expect to receive emergency broadcast services within the required delay budget of 4s, the existing DRX mechanism could be used. |
| Xiaomi | Option3 | We prefer the lower boundary of LTE to be reused. Option2 can be further discussed. |
| CATT | 3) | RedCap UEs that need to receive emergency broadcast services are not expected to request to be configured with eDRX. |
| Huawei, Hisilicon | Option 3) |  |
| LGE | Option 3 |  |
| MediaTek | Option 2 | Option 2 will enable wearables to monitor paging with the periodicity of 2.56s. This is difficult to configure in legacy deployments as the common RAN paging cycle is typically lower than 2.56s. |
| Ericsson | Option 3 | We prefer to follow the LTE baseline and additionally we don’t consider shorter cycles to be “eDRX”. |
| Qualcomm | 2 | We do not see strong need for eDRX cycle of 1.28s in typical deployments. But 2.56s is useful, as discussed during the SI phase. |
| Apple | 2 | MediaTek reflects our view accurately. |
| Futurewei | 3 | We prefer to reuse the lower boundary in LTE. |
| DENSO | 2 | We agree with Intel, MediaTek. |
| Sharp | 2 |  |
| vivo | 3)/2) | We prefer to reuse the eDRX lower bound of LTE. eDRX should not be configured if the UE needs to receive ETWS/CMAS. We are also fine to accept 2) if many companies want. |
| CMCC | Option 3 |  |
| ZTE | Option 3 |  |
| Sequans | Probably 2 | But it depends on the exact solution adopted |
| Samsung | Option 3 |  |

# Report: summary and proposals

This report summarizes the views of 19 companies (Intel, OPPO, Xiaomi, CATT, Huawei, Hisilicon, LGE, MediaTek, Ericsson, Qualcomm, Apple, Futurewei, DENSO, Sharp, vivo, CMCC, ZTE, Sequans, Samsung).

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

## Configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

#### 1st Discussion Point

Discussion point 1 addresses the FFS captured on PTW configuration for RRC_IDLE / RRC_INACTIVE:

- Option 1) is supported by 7 companies (Intel, Ericsson, Qualcomm, Futurewei, DENSO, Sequans, Samsung)

- Option 2) is supported by 9 companies (OPPO, Xiaomi, CATT, LGE, MediaTek, Apple, Sharp, CMCC, ZTE).

- Option 3) is supported by 2 companies (CATT, Qualcomm).

- Option 4) is supported by 7 companies (Huawei, Hisilicon, Ericsson, Futurewei, vivo, CMCC, Sequans)

  - (Huawei, HiSilicon, Ericsson) Updated option 4 “PTW for RRC_IDLE overlaps with PTW for RRC_INACTIVE, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller).

  - ”

- Option 5) is supported by 1 companies (Ericsson)

Rapporteur: only options 1) 2) and 3) are listed for discussion

- [To discuss] For eDRX >10.24sec, to discuss whether PTW configuration for RRC_IDLE / RRC_INACTIVE follows: option 1) [6/19] It is left up to network implementation any coordination on how to decide the PTW configurations for RRC_IDLE and for RRC_INACTIVE (i.e. PTW may be configured same or different); option 2) [9/19] A common PTW configuration is provided for RRC_IDLE and for RRC_INACTIVE; option 4) [7/19] PTW for RRC_IDLE overlaps with PTW for RRC_INACTIVE, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller)

#### 2nd Discussion Point

Discussion point 2 - Do you support defining different and/or specific behaviour depending on whether eDRX cycle for RRC_INACTIVE is configured differently than for RRC_IDLE?

- 10 companies do not support this (Intel, OPPO, Xiaomi, CATT, LGE, Qualcomm, , DENSO, vivo, Sequans, Samsung)

- 9 companies left this FFS or asked to postpone this discussion (Huawei, Hisilicon, MediaTek, Ericsson, Apple, Futurewei, Sharp, CMCC, ZTE).

- (CATT) we need to specify UE behavior based on the eDRX configuration combinations for idle and inactive: 1) eDRX only configured for idle, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 2) eDRX only configured for inactive, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 3) eDRX configured both for inactive and idle, 3-a) both eDRX ≤ 10.24sec, 3-b) one of the eDRX≤ 10.24sec and  the other eDRX>10.24s, 3-c) both of the eDRX>10.24s

- [FFS] [9/19] whether to specify UE behavior based on the eDRX configuration combinations for idle and inactive: 1) eDRX only configured for idle, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 2) eDRX only configured for inactive, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 3) eDRX configured both for inactive and idle, 3-a) both eDRX ≤ 10.24sec, 3-b) one of the eDRX≤ 10.24sec and  the other eDRX>10.24s, 3-c) both of the eDRX>10.24s.

#### 3rd Discussion Point

Discussion point 3 - From RAN2 point of view, do you support that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC)?

- All companies support this.

- [To agree] [19/19] RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC).

#### 4th Discussion Point

Discussion point 4 - From RAN2 point of view, do you support that RAN provides necessary eDRX assistance information for RRC_INACTIVE to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration?

- 3 (or 5) companies support this (Huawei, Hisilicon, Ericsson).

  - Companies supported with buts (Qualcomm, Futurewei,

- 4 Companies do not support this (OPPO, Xiaomi, DENSO, vivo).

- 10 Companies are not sure, ask to postpone its discussion (e.g. waiting for SA2) or prefer marking it as FFS (Intel, CATT, LGE, MediaTek, Apple, Sharp, CMCC, ZTE, Sequans, Samsung)

- [To agree] [10/19] To wait for SA2 input on whether RAN provides necessary eDRX assistance information for RRC_INACTIVE to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration.

#### 5th Discussion Point

Discussion point 5 - please indicate if there are other scenarios that need to be discussed in relation to the eDRX configuration, and if so, explain your preferred approach

Rapporteur: Inputs provided here is already addressed in discussion point 2 above

## Paging eDRX mechanism for RRC_IDLE / RRC_INACTIVE

#### 6th Discussion Point)

Discussion point 6 - Do you confirm that for UE side, a common paging mechanism is enabled for eDRX ≤ 10.24sec in RRC_IDLE and RRC_INACTIVE (with details to be discussed in following discussion points)?

- 16 companies do support this (Intel, OPPO, Xiaomi, CATT, Huawei, Hisilicon, LGE, Qualcomm, Futurewei, DENSO, Sharp, vivo, CMCC, ZTE, Sequans, Samsung).

- 4 companies leave this as FFS or were not sure (MediaTek, Ericsson, Apple).

- [To agree] [16/19] From UE point of view, a common paging mechanism/behaviour is enabled for eDRX ≤ 10.24 sec in RRC_IDLE and/or RRC_INACTIVE.

#### 7th Discussion Point)

Discussion point 7 - applicable options to enable paging eDRX mechanism for eDRX ≤ 10.24sec in RRC_IDLE and RRC_INACTIVE.

- Option 1) is supported by 8 companies (Intel, MediaTek, Qualcomm, Apple, DENSO, Sharp, CMCC, Sequans)

- Option 2) is supported by 8 companies (OPPO, Huawei, Hisilicon, LGE, Ericsson, Futurewei, vivo, Samsung)

  - (Huawei, Hisilicon) scenario should be extended when only one of them is configured

- Option 3) is supported by 2 companies (CATT, ZTE)

  - (CATT) When eDRX is configured ≤ 10.24sec for both Idle and Inactive,

    - In Idle, UE monitors paging with eDRX cycle T, where T is determined by CN eDRX cycle.

    - In Inactive, UE may receive both CN and RAN pages, so it monitors paging according to both eDRX cycles T1 and T2, where T1 and T2 are determined by CN and RAN eDRX cycles, respectively.

    - Complete freedom in configuring T1 and T2 should be avoided and we should preferably have T1 and T2 commensurable (T1 = nT2).

  - (ZTE)  Our understanding is aligned with CATT. On top of it, for RRC_INACTIVE, if we want to ensure overlapped POs for CN paging and RAN paging, we can consider to always use the eDRX cycle configured for IDLE for PO calculation for RAN paging.

- Companies that prefer further discussion (Xiaomi)

- [To discuss] applicable option to enable paging eDRX mechanism for eDRX ≤ 10.24sec in RRC_IDLE and/or RRC_INACTIVE, considering at least option 1) [8/19] UE only monitors UE-specific paging eDRX applicable to the corresponding RRC state (instead of the smallest of the configured ones). i.e. a UE in RRC_INACTIVE monitors eDRX cycle configured for RRC_INACTIVE, and a UE in RRC_IDLE monitors eDRX cycle configured for RRC_ IDLE, and option 2) [8/19] the UE in RRC ACTIVE monitors PO with eDRX cycle T, where T is determined by the shortest of CN eDRX cycle and RAN eDRX cycle

### Paging mechanism for eDRX above 10.24sec

#### 8th Discussion Point

Discussion point 8 - Do you confirm that for UE side, a common paging mechanism is enabled for eDRX >10.24sec in RRC_IDLE and RRC_INACTIVE (with details to be discussed in following discussion points)?

- 14 companies do support this (Intel, OPPO, Xiaomi, CATT, Huawei, Hisilicon, LGE, Futurewei, DENSO, Sharp, vivo, CMCC, ZTE, Samsung).

- 4 companies leave this as FFS or were not sure (MediaTek, Ericsson, Apple, Qualcomm, Sequans).

- [To agree] [14/19] As baseline from UE point of view, a common paging mechanism/behaviour is enabled for eDRX >10.24sec in RRC_IDLE and/or RRC_INACTIVE

#### 9th Discussion Point

Discussion point 9 - applicable options to enable paging eDRX mechanism for eDRX > 10.24sec in RRC_IDLE and RRC_INACTIVE:

- Option 1) is supported by 18 companies (Intel, OPPO, Xiaomi, CATT, Huawei, Hisilicon, LGE, Ericsson, Qualcomm, Apple, Futurewei, DENSO, Sharp, vivo, CMCC, ZTE, Sequans, Samsung)

- Option 2) is supported or want to be further discussed by 7 companies (OPPO, CATT, MediaTek, Qualcomm, DENSO, vivo, Sequans)

- [To agree] [18/19] eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC

- [FFS] whether NR PTW definition is different than LTE PTW (which is defined as fixed to multiples of 256 SFNs), e.g. considering configurable by the network.

### Other details and cases related to paging eDRX mechanism

#### 10th Discussion Point

Discussion point 10 - Do you confirm that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN?

- All companies agree to this.

- [To agree] [19/19] RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

#### 11th Discussion Point

Discussion point 11 - Do you confirm that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)?

- All companies agree to this.

- [To agree] [19/19] RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

#### 12th Discussion Point

Discussion point 12 - indicate if there are other topics related to paging eDRX mechanism that needs to be discussed

- (MediaTek) Whether to indicate the eDRX support in a cell by a flag (e.g. eDRX-Allowed) in SI (e.g. SIB1) should be discussed

- (Ericsson) how the buffering should be done in case there is pending DL data and both IDLE and INACTIVE cycles are relatively long

- [FFS] [1] Whether to indicate the eDRX support in a cell by a flag (e.g. eDRX-Allowed) in SI (e.g. SIB1).

- [FFS] [1] whether to discuss and if so, how the buffering should be done in case there is pending DL data and both IDLE and INACTIVE cycles are relatively long.

## Minimum value of eDRX

#### Discussion Point )

Discussion point 13 - Please indicate your preferred minimum value allowed for the eDRX cycle in RRC_IDLE and RRC_INACTIVE:

- 2.56sec is preferred by 8 companies (Intel, MediaTek, Qualcomm, Apple, DENSO, Sharp, vivo, Sequans)

- 5.12sec is preferred by 12 companies (OPPO, Xiaomi, CATT, Huawei, Hisilicon, LGE, Ericsson, Futurewei, vivo, CMCC, ZTE, Samsung)

- [To discuss] preferred minimum value allowed for the eDRX cycle in RRC_IDLE and RRC_INACTIVE is option 2) [8/19] 2.56 sec or option 3) [12/19] 5.12 sec.

# Conclusion

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

The proposals captured are the following:

Proposal 1.[To discuss] For eDRX >10.24sec, to discuss whether PTW configuration for RRC_IDLE / RRC_INACTIVE follows: option 1) [6/19] It is left up to network implementation any coordination on how to decide the PTW configurations for RRC_IDLE and for RRC_INACTIVE (i.e. PTW may be configured same or different); option 2) [9/19] A common PTW configuration is provided for RRC_IDLE and for RRC_INACTIVE; option 4) [7/19] PTW for RRC_IDLE overlaps with PTW for RRC_INACTIVE, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller)

Proposal 2.	[FFS] [9/19] whether to specify UE behavior based on the eDRX configuration combinations for idle and inactive: 1) eDRX only configured for idle, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 2) eDRX only configured for inactive, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 3) eDRX configured both for inactive and idle, 3-a) both eDRX ≤ 10.24sec, 3-b) one of the eDRX≤ 10.24sec and  the other eDRX>10.24s, 3-c) both of the eDRX>10.24s.

Proposal 3.	[To agree] [19/19] RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC).

Proposal 4.	[To agree] [10/19] To wait for SA2 input on whether RAN provides necessary eDRX assistance information for RRC_INACTIVE to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration.

Proposal 5.	[To agree] [16/19] From UE point of view, a common paging mechanism/behaviour is enabled for eDRX ≤ 10.24 sec in RRC_IDLE and/or RRC_INACTIVE.

Proposal 6.	[To discuss] applicable option to enable paging eDRX mechanism for eDRX ≤ 10.24sec in RRC_IDLE and/or RRC_INACTIVE, considering at least option 1) [8/19] UE only monitors UE-specific paging eDRX applicable to the corresponding RRC state (instead of the smallest of the configured ones). i.e. a UE in RRC_INACTIVE monitors eDRX cycle configured for RRC_INACTIVE, and a UE in RRC_IDLE monitors eDRX cycle configured for RRC_ IDLE, and option 2) [8/19] the UE in RRC ACTIVE monitors PO with eDRX cycle T, where T is determined by the shortest of CN eDRX cycle and RAN eDRX cycle

Proposal 7.	[To agree] [14/19] As baseline from UE point of view, a common paging mechanism/behaviour is enabled for eDRX >10.24sec in RRC_IDLE and/or RRC_INACTIVE

Proposal 8.	[To agree] [18/19] eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC

Proposal 9.	[FFS] whether NR PTW definition is different than LTE PTW (which is defined as fixed to multiples of 256 SFNs), e.g. considering configurable by the network.

Proposal 10.	[To agree] [19/19] RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

Proposal 11.	[To agree] [19/19] RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

Proposal 12.	[FFS] [1] Whether to indicate the eDRX support in a cell by a flag (e.g. eDRX-Allowed) in SI (e.g. SIB1).

Proposal 13.	[FFS] [1] whether to discuss and if so, how the buffering should be done in case there is pending DL data and both IDLE and INACTIVE cycles are relatively long.

Proposal 14.	[To discuss] preferred minimum value allowed for the eDRX cycle in RRC_IDLE and RRC_INACTIVE is option 2) [8/19] 2.56 sec or option 3) [12/19] 5.12 sec.

The following order is suggested for the online discussion:

Proposals for potential agreement

Proposal 3.	[To agree] [19/19] RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC).

Proposal 4.	[To agree] [10/19] To wait for SA2 input on whether RAN provides necessary eDRX assistance information for RRC_INACTIVE to CN so that CN can estimate when the UE is unreachable, e.g. by providing the RAN paging configuration.

Proposal 5.	[To agree] [16/19] From UE point of view, a common paging mechanism/behaviour is enabled for eDRX ≤ 10.24 sec in RRC_IDLE and/or RRC_INACTIVE.

Proposal 7.	[To agree] [14/19] As baseline from UE point of view, a common paging mechanism/behaviour is enabled for eDRX >10.24sec in RRC_IDLE and/or RRC_INACTIVE

Proposal 8.	[To agree] [18/19] eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC

Proposal 10.	[To agree] [19/19] RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

Proposal 11.	[To agree] [19/19] RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

Proposals for potential discussion online

Proposal 1.[To discuss] For eDRX >10.24sec, to discuss whether PTW configuration for RRC_IDLE / RRC_INACTIVE follows: option 1) [6/19] It is left up to network implementation any coordination on how to decide the PTW configurations for RRC_IDLE and for RRC_INACTIVE (i.e. PTW may be configured same or different); option 2) [9/19] A common PTW configuration is provided for RRC_IDLE and for RRC_INACTIVE; option 4) [7/19] PTW for RRC_IDLE overlaps with PTW for RRC_INACTIVE, and PTW length for RRC_IDLE can be same or larger than PTW length for RRC_INACTIVE (but not smaller)

Proposal 6.	[To discuss] applicable option to enable paging eDRX mechanism for eDRX ≤ 10.24sec in RRC_IDLE and/or RRC_INACTIVE, considering at least option 1) [8/19] UE only monitors UE-specific paging eDRX applicable to the corresponding RRC state (instead of the smallest of the configured ones). i.e. a UE in RRC_INACTIVE monitors eDRX cycle configured for RRC_INACTIVE, and a UE in RRC_IDLE monitors eDRX cycle configured for RRC_ IDLE, and option 2) [8/19] the UE in RRC ACTIVE monitors PO with eDRX cycle T, where T is determined by the shortest of CN eDRX cycle and RAN eDRX cycle

Proposal 14.	[To discuss] preferred minimum value allowed for the eDRX cycle in RRC_IDLE and RRC_INACTIVE is option 2) [8/19] 2.56 sec or option 3) [12/19] 5.12 sec.

Proposals for potential discussion in future meetings

Proposal 2.	[FFS] [9/19] whether to specify UE behavior based on the eDRX configuration combinations for idle and inactive: 1) eDRX only configured for idle, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 2) eDRX only configured for inactive, cases eDRX ≤ 10.24sec and eDRX >10.24sec, 3) eDRX configured both for inactive and idle, 3-a) both eDRX ≤ 10.24sec, 3-b) one of the eDRX≤ 10.24sec and  the other eDRX>10.24s, 3-c) both of the eDRX>10.24s.

Proposal 9.	[FFS] whether NR PTW definition is different than LTE PTW (which is defined as fixed to multiples of 256 SFNs), e.g. considering configurable by the network.

Proposal 12.	[FFS] [1] Whether to indicate the eDRX support in a cell by a flag (e.g. eDRX-Allowed) in SI (e.g. SIB1).

Proposal 13.	[FFS] [1] whether to discuss and if so, how the buffering should be done in case there is pending DL data and both IDLE and INACTIVE cycles are relatively long.

# Annex A: Legacy LTE eDRX operation

References to legacy LTE eDRX operation ≥ 10.24sec from TS 36.304.

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. In P-RNTI transmitted on MPDCCH case, PO refers to the starting subframe of MPDCCH repetitions. In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

One Paging Narrowband (PNB) is one narrowband, on which the UE performs the paging message reception.

PF, PO, and PNB are determined by following formulae using the DRX parameters provided in System Information:

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation:

i_s = floor(UE_ID/N) mod Ns

If P-RNTI is monitored on MPDCCH, the PNB is determined by the following equation:

PNB = floor(UE_ID/(N*Ns)) mod Nn

If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation:

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

System Information DRX parameters stored in the UE shall be updated locally in the UE whenever the DRX parameter values are changed in SI. If the UE has no IMSI, for instance when making an emergency call without USIM, the UE shall use as default identity UE_ID = 0 in the PF, i_s, and PNB formulas above. If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

The following Parameters are used for the calculation of the PF, i_s, PNB, and the NB-IoT paging carrier:

-	T: DRX cycle of the UE.

Except for NB-IoT: If a UE specific extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T =512. Otherwise, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied. In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, and the default paging cycle, if allocated by upper layers. Otherwise, in RRC_INACTIVE state when extended DRX is configured by upper layers, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

-	nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024.

-	N: min(T,nB)

-	Ns: max(1,nB/T)

-	Nn: number of paging narrowbands (for P-RNTI monitored on MPDCCH) or paging carriers (for P-RNTI monitored on NPDCCH) determined as follows:

If UE monitors GWUS according to clause 7.5.1:

this is the number of paging narrowbands (paging carriers) that are configured with GWUS.

else:

this is the number of paging narrowbands (paging carriers) provided in system information.

-	UE_ID:

If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell:

5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH.

5G-S-TMSI mod 16384, if P-RNTI is monitored on NPDCCH or MPDCCH.

else

IMSI mod 1024, if P-RNTI is monitored on PDCCH.

IMSI mod 4096, if P-RNTI is monitored on NPDCCH.

IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information.

-	W(i): Weight for NB-IoT paging carrier i.

-	W: Total weight of all NB-IoT paging carriers, i.e. W = W(0) + W(1) + … + W(Nn-1). If UE monitors GWUS according to clause 7.5.1, Total weight of all NB-IoT paging carriers configured with GWUS.

IMSI is given as sequence of digits of type Integer (0..9), IMSI shall in the formulae above be interpreted as a decimal integer number, where the first digit given in the sequence represents the highest order digit.

For example:

IMSI = 12 (digit1=1, digit2=2)

In the calculations, this shall be interpreted as the decimal integer "12", not "1x16+2 = 18".

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [39]. 5G-S-TMSI shall in the PF and i_s formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

## 7.3	Paging in extended DRX

The UE may be configured by upper layers with an extended DRX (eDRX) cycle TeDRX. Except for NB-IoT, the UE may operate in extended DRX only if the UE is configured by upper layers and the cell indicates support for eDRX in System Information. For NB-IoT, the UE may operate in extended DRX only if the UE is configured by upper layers. If the UE is configured with a TeDRX cycle of 512 radio frames, it monitors POs as defined in 7.1 with parameter T = 512. Otherwise, a UE configured with eDRX monitors POs as defined in 7.1 (i.e, based on the upper layer configured DRX value and a default DRX value determined in 7.1 or if the UE is in RRC-INACTIVE based on the upper layer configured DRX value,default DRX cycle and RAN paging cycle determined in 7.1), during a periodic Paging Time Window (PTW) configured for the UE or until a paging message including the UE's NAS identity is received for the UE during the PTW, whichever is earlier. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formulae:

The PH is the H-SFN satisfying the following equation:

H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H), where

-	UE_ID_H:

- 10 most significant bits of the Hashed ID, if P-RNTI is monitored on PDCCH or MPDCCH

- 12 most significant bits of the Hashed ID, if P-RNTI is monitored on NPDCCH

-	T eDRX,H : eDRX cycle of the UE in Hyper-frames, (TeDRX,H =1, 2, …, 256 Hyper-frames) (for NB-IoT, TeDRX,H =2, …, 1024 Hyper-frames) and configured by upper layers.

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 256* ieDRX, where

-	ieDRX = floor(UE_ID_H /TeDRX,H) mod 4

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window length (in seconds) configured by upper layers

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of S-TMSI or 5G-S-TMSI. 5G-S-TMSI is used for Hashed-ID if the UE supports connection to 5GC and NAS indicated to use 5GC for the selected cell.

S-TMSI = <b39, b38, …, b0> as defined in TS 23.003 [35]

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [35].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any S-TMSI or 5G-S-TMSI value. An example of hashed ID calculation is in Annex B.

# Annex B: Related topics from phase-1

## Phase-1: Configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

Discussion point 2) For configuration of the eDRX for RRC_IDLE / RRC_INACTIVE, do you prefer (a) using the same configuration [4][7][8][11], or (b) allowing the network to choose different configurations [1][2][3][6][9]? Note that the following Discussion point 3)  and Discussion point 4)  addressed detailed approaches proposed by the different supportive companies.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | b) | DL Data buffering in the NW is different in IDLE and INACTIVE and it would be beneficial to have flexibility for the NW to configure different configuration including eDRX cycle for IDLE and INACTIVE. |
| OPPO | Option b | We think RAN should be allowed to configure a different eDRX cycle for RAN paging, which could provide more flexible configuration compared with a common eDRX cycle configuration. Otherwise, RAN could only page the UE based on eDRX cycle configured by CN, but has no ways to page the UE with a shorter latency (if it wants). |
| Ericsson | b) | This depends, at least partly, on the outcome of the discussion on which node configures the RRC_INACTIVE parameters. If RAN does not have choice to configure other than what CN configures, then it is functionally CN which configures. Agree with Nokia and OPPO comments, but on top of OPPO view we would also let RAN configure PTW. |
| Apple | We prefer a | (b) would be avoided by CN config for INACTIVE. |
| Qualcomm | b) | We do not think always forcing common eDRX configurations between RRC Idle and Inactive are fully justified, as argued in our paper [1]. There are use cases in which it is beneficial to have different eDRX configurations (at least different eDRX cycles) between the two RRC states. If in some cases network does want to have common eDRX configurations, it can use the existing signaling to do so without the need for new enhancements. |
| Intel | b) |  |
| Samsung | b) | It seems beneficial to have a flexibility. |
| Xiaomi | b) | As the RAN buffers DL packets for the UE in RRC_INACTIVE, it is natural that RAN decides the e-DRX parameters for RAN paging. To guarantee that a UE in RRC_INACTIVE can monitor both RAN paging and CN paging, it is better that the RAN paging e-DRX configuration can be decided using some assistance information from the CN, e.g. a shorter e-DRX cycle. |
| Sharp | b) | Different eDRX cycle can be configured for RRC_INACTIVE from for RRC_IDLE. |
| NEC | b) | RAN should decide the paging for INACTIVE UE. CN takes INACTIVE UE CMM_CONNECTED but takes IDLE UE CMM IDLE, so CN may send data for INACTIVE UE, in this case, RAN should trigger the paging for INACTIVE UE. So INACTIVE/IDLE UE should have different eDRX. |
| Futurewei | b) |  |
| MediaTek | b) | Because the scenarios and the requirements for Idle and Inactive modes can be different, it may be beneficial to use different configurations. |
| CATT | (a) | Simplest as it does not require any complex “merge” procedure of different configurations when they have to be supported simultaneously to cope with states mismatch. |
| vivo | b) | For DRX, UE specific DRX cycle for inactive and idle can be different. For eDRX, there is no motivation to have common eDRX configuration for idle and inactive. We prefer to have similar flexibility for eDRX configuration. |
| DENSO | a&b | Basically, eDRX is used in the same configuration for IDLE and INACTIVE, but if different configurations are set for IDLE and INACTIVE, the eDRX settings corresponding to the different configurations should be reflected in the system. |
| ZTE | b) | We prefer to support different configurations for flexibility, e.g. a shorter eDRX cycle for RRC INACTIVE. In our paper[7], we propose to configure overlapped PTW for RRC idle and RRC inactive as much as possible, i.e. eDRX cycle for RRC IDLE and INACTIVE can be different, but the PTWs are partial overlapped. Following figure show an example of our preference. For IDLE and INACTIVE eDRX cycle equal to or shorter than 10.24s, we are fine to support different eDRX configuration for RRC IDLE and INACTIVE. |
| Huawei, HiSilicon | b) | We should not limit the flexibility of the RAN. RAN can configure same or different eDRX configuration for RRC_INACTIVE based on its implementation. |
| CMCC | a) & b) | In our opinion, common eDRX configuration could be a start point, and we also see the benefit of different eDRX cycle for IDLE and INACTIVE, as analysed in our contribution. And considering UE power consumption, common PTW could be considered. Besides, we have the similar concern with OPPO, some approaches for Option a) seem like approaches for Option b). |
| Sequans | b | This flexibility seems beneficial as mentioned above |
| LGE | b) |  |
| Convida | Option b | We prefer that the eDRX cycle for the RRC_IDLE and RRC_INACTIVE can have different configurations for flexibility. It is worth noting that in the case of WB-E-UTRA and LTE-M connected to 5GC, NR-configure the eDRX cycle for RRC_INACTIVE that may be different from eDRX cycle for RRC_IDLE as specified in TS 23.501 section 5.31.7.2.1. “If the UE supports eDRX in RRC inactive, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC-INACTIVE up to the value for the UE's idle mode eDRX cycle as provided by the AMF in "RRC Inactive Assistance Information" or up to 10.24 seconds (whichever is lower).” |

Discussion point 3) If option a) same configuration of the eDRX for RRC_IDLE / RRC_INACTIVE were supported, please indicate any of your preferred approach(es):

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- Common eDRX cycle defined as min{ eDRX cycle given by AMF, eDRX cycle given by RAN }and common PTW (i.e. PTW start/ PTW length) [4].

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- For eDRX 10.24s, set the common eDRX cycle as T in PF/PO determination [7]

- Common eDRX configuration, i.e. eDRX cycle and when applicable, PTW [8] [11].

- Other approaches to consider.

| Company’s name | Approach(es) [y/n] |  |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a.1 | a.2 | a.3 | a.4 | a.5 | a.6 |  |
| Apple |  |  |  |  |  | Y | Much simpler to let CN configure the DRX for INACTIVE (in other words have same config for IDLE/INACTIVE, assuming that CN does not distinguish between IDLE/INACTIVE). |
| CATT | n | n | n | n | y | n | Other approaches but a.5 are more in the option (b) in our view. |
| DENSO | y |  | y |  |  |  | From a UE perspective, we think that if UE is used differently in IDLE and INACTIVE, it will require different eDRX settings. Since different eDRX are set for some reason, the setting should be reflected properly as a system. |
| CMCC | Y |  | Y |  | Y |  | Common eDRX configuration could be a start point. Considering the difference between IDLE and INACTIVE, a different eDRX cycle could be considered, and this will provide more flexibility. Besides, from the UE power consumption point, a common PTW is preferred. |

Discussion point 3) If option b) different configuration of the eDRX for RRC_IDLE / RRC_INACTIVE were supported, please indicate any of your preferred approach(es):

- Any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation [1] [3] [6].

- RAN guarantees that guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE [3]. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other [6].

- Different eDRX configuration, i.e. eDRX cycle and when applicable, PTW [1][2][3][6][9].

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- Other approaches to consider.

| Company’s name | Approach(es) [y/n] |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- |
|  | b.1 | b.2 | b.3 | b.4 | b.5 |  |
| Nokia, Nokia Shanghai |  |  | X |  |  |  |
| OPPO | N | N | N | Y |  | We prefer (b.4), i.e. different eDRX cycle and common PTW length for RRC IDLE and RRC INACTIVE. We think RAN should be allowed to configure a different eDRX cycle for RAN paging, which could provide more flexible configuration compared with a common eDRX cycle configuration. Otherwise, RAN could only page the UE based on eDRX cycle configured by CN, but has no ways to page the UE with a shorter latency (if it wants). Regarding PTW, from UE power saving point of view, the PTWs for CN paging and RAN paging should be overlapped as much as possible, so a common PTW length can be used for both CN paging and RAN paging. |
| Ericsson | N | N | Y | N |  | If RAN configures the parameters for inactive eDRX, we don’t think there should be restrictions on the configuration. However, we do agree that it would be beneficial to have a common, or overlapping, PTW. We think this should be left up to NW / RAN implementation to configure. We also prefer to specify coordination so that operation is possible e.g. in multi-vendor situations, however details of such signaling are up to other WGs. |
| Qualcomm | Y | N | N | N | Y | If majority of companies prefer to have some constraints on the two eDRX configurations, it is fine with us if that constraint is for ensuring there are some overlapping POs between two PTWs (e.g. have a common PTW starting point). But that is necessary only when RRC Inactive has an eDRX cycle > 10.24 sec (whose feasibility is pending approval by SA2/CT1). |
| Intel | y | y | y | n |  | In our understanding, the behaviour described by points (b.2) and (b.3) would be handled by network implementation (as explained in point (b.1)). |
| Samsung | Y | Y | Y | N |  | We have assumed any coordination is required to maximize UE power saving gain while guaranteeing the flexibility. |
| Xiaomi |  | Y |  |  |  | Is b.2 included in b.4? We support that eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE, e.g., a shorter e-DRX cycle. We also prefer a common PTW. |
| Sharp | N | Y | N | Y |  | If the eDRX cycle values defined in RAN spec is sub-set of the eDRX cycle values defined in CN spec, RAN doesn’t need to do extra effort to guarantee the configuration and coordination between RAN and CN is not needed. Although CN and RAN configure different eDRX cycles for UE, the UE can monitor the minimum one to reduce complexity. Considering eDRX cycle for INACTIVE is sub-set of eDRX cycle for IDLE, UE will not miss any paging. |
| NEC | Y | Y | Y | N | N | We prefer option b.1 and we think b.2/b.3 can be covered by network implementation. |
| Futurewei |  |  | Y |  |  |  |
| MediaTek | Y | Y |  | Y | Y | The network (CN and RAN) should coordinate and make sure that the eDRX configurations satisfy the requirements for Idle and Inactive. For Inactive, RAN should provide the eDRX to the UE, while making sure that the CN requirements are not violated. To simplify the eDRX configurations for Idle and Inactive and to achieve some commonality between the two, Approach b.4 can be useful (i.e. with different eDRX cycles, and common PTW configuration). As it is already agreed that PTW will not be used for eDRX cycle<=10.24s, b.4 and b.5 look similar to us. |
| vivo | Y | Y | N | N | Y | Firstly, we think it is up to network implementation whether to have coordination for eDRX configuration between idle and inactive. As UEs in inactive mode will monitor both RAN paging and CN paging, it is better to guarantee that the configuration for eDRX in inactive is the sub-set of eDRX configuration in idle. Such coordination is required to maximize UE power saving gain. |
| DENSO | y |  | y |  |  | If there are use cases where it is more effective for the NW to adjust the eDRX settings, it is desirable to adjust the eDRX settings. However, we think that NW should avoid blindly adjusting. |
| ZTE | Y |  |  |  | Y | First of all, network (CN or RAN) can decide whether to configure same different eDRX configurations for IDLE and INACTIVE. Further, when both eDRX cycles are longer than 10.24s, we prefer b.5: IDLE PTW and INACTIVE PTW can overlap as much possible, i.e. an IDLE PTW always overlaps an INACTIVE PTW (share same PH and PTW starting point). We think this is beneficial for UE power saving. For PTW length, we think the same PTW length is enough and simpler for paging monitoring. Considering the intention of introducing PTW is to improve paging robustness with multiple paging chances during long eDRX cycle, there is no difference for IDLE PTW and INACTIVE PTW on this aspect. Following figure shows an example of our preference |
| Huawei, HiSilicon | N | Y | Y | N |  | In our view, (b.3) should be the baseline, i.e. no need to put restrictions on the RAN. We agree that, from the UE point of view, it would be beneficial to have common/overlapping PTW and we think it can be achieved by NW implementation. However, if there is strong support to have some restrictions on the configuration, we are open to discuss further on the realisation of (b.2). |
| CMCC |  |  |  | Y |  |  |
| Sequans | Y | Y | Y |  |  | What is the difference between b.1 and b.3? they both give full NW control. If we decide to limit NW control, then it makes sense to us to maximize the UE power saving, as per option b.2 We are also fine with the other options. |
| LGE |  | Y |  |  | Y | No strong view on b.1). We are thinking the network side behavior may need to be specified, but we are also fine to leave it to implementation. |
| Convida | N | N | Y | N |  | The latency/power savings requirements in RRC_IDLE and RRC_INACTIVE may vary, suggesting a flexible configuration. Although we think it will be beneficial to have a common or overlapping PTW, we do not see a compelling reason to allow for different eDRX cycles but mandate a common PTW. A smart NW/RAN implementation can always configure a common PTW where/when appropriate. |

## Phase-1: report summary on configuration of the eDRX for RRC_IDLE / RRC_INACTIVE

The discussion point 2) asked whether configuration of the eDRX for RRC_IDLE / RRC_INACTIVE should be (a) same or (b) different:

- Option (a) is supported by 4 companies (Apple, CATT, DENSO, CMCC)

- Option (b) is supported by 21 companies (Nokia, Nokia Shanghai Bell, OPPO, Ericsson, Qualcomm, Intel, Samsung, Xiaomi, Sharp, NEC, Futurewei, MediaTek, vivo, DENSO, ZTE, Huawei, HiSilicon, CMCC, Sequans, LGE, Convida).

The following comments were raised by supporting companies of option (a):

- CATT indicates that this would be simpler option.

- OPPO and CMCC clarifies that some approaches for Option a) seem applicable approaches for Option b).

The following comments were raised by supporting companies of option (b):

- Nokia clarifies that DL Data buffering in the NW is different in IDLE and INACTIVE and it would be beneficial to have flexibility for the NW to configure different configuration including eDRX cycle for IDLE and INACTIVE.

- OPPO and Ericsson clarifies that this allows RAN to configure UEs in INACTIVE with shorter eDRX cycles which would reduce latency when needed.

- ZTE explains that PTW for RRC idle and RRC inactive should overlap as much as possible, i.e. eDRX cycle for RRC IDLE and INACTIVE can be different, but the PTWs are partial overlapped

Rapporteur: propose for agreement on option (a) of the Discussion point 2) due to the large support.

Phase-1 Proposal 3. [To agree] [21/23] The configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different.

The Discussion point 3)  asked further details on the preferred approach if option a) of the Discussion point 2) were supported.

- Approach a.1) is supported by 2 companies (DENSO, CMCC).

- Approach a.3) is supported by 2 companies (DENSO, CMCC).

- Approach a.5) is supported by 2 companies (CATT, CMCC).

- Approach a.6) I supported by 1 company (Apple).

Rapporteur: No proposal for discussion is added considering that option (a) of the Discussion point 2)  is not supported by majority view as captured in previous  Error! Reference source not found..

The Discussion point 4)  aims to clarify further details if option b) of the Discussion point 2) were supported.

- Any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation [1].

- This approach (b.1) is supported by 9 companies (Qualcomm, Intel, Samsung, NEC, MediaTek, vivo, DENSO, ZTE, Sequans).

- RAN guarantees that guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE [3]. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other [6].

- This approach (b.2) is supported by 11 companies (Intel, Samsung, Xiaomi, Sharp, NEC, MediaTek, vivo, Huawei, HiSilicon, Sequans, LGE).

- Different eDRX configuration, i.e. eDRX cycle and when applicable, PTW [1][2][3][6][9].

- This approach (b.3) is supported by 12 companies (Nokia, Nokia Shanghai Bell, Ericsson, Intel, Samsung, NEC, Futurewei, DENSO, Huawei, HiSilicon, Sequans, Convida).

- Different eDRX cycle, but a common PTW length and the PTW starting point [2].

- This approach (b.4) is supported by 4 companies (OPPO, Sharp, MediaTek, CMCC).

- For eDRX > 10.24sec, common PTW but with different eDRX cycle [7].

- This approach (b.5) is supported by 5 companies (Qualcomm, MediaTek, vivo, ZTE, LGE).

- Other approaches to consider.

- None

Rapporteur: 21 companies provided inputs to this discussion point. The proposal is to only continue discussion on the proposals with larger support i.e. approach (b.1), (b.2), (b.3). Moreover, supporting companies also explain that points (b.2) and (b.3) would be handled by network implementation (as explained in point (b.1))

Phase-1 Proposal 4. [To discuss] To discuss the following details in relation to Phase-1 Proposal 3Error! Reference source not found.:

Phase-1 Proposal 4.1. [To discuss] [9/21] To consider that any coordination between eDRX configurations for RRC_IDLE / RRC_INACTIVE is left up to network implementation

Phase-1 Proposal 4.2. [To discuss] [11/21] To consider that RAN guarantees that the configuration provided for eDRX in RRC_INACTIVE is a sub-set of one provided for eDRX in RRC_IDLE. E.g. eDRX cycle in RRC_INACTIVE may be shorter than the eDRX cycle in RRC_IDLE, and the PTW should be either common or overlapping in the case either window is shorter than the other.

Phase-1 Proposal 4.3. [To discuss][12/21] To consider that different eDRX configuration refers to different eDRX cycle and when applicable, different PTW.

# Annex C: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Marta Martinez Tarradell | marta.m.tarradell@intel.com |
| Nokia, Nokia Shanghai Bell | Jussi-Pekka Koskinen | Jussi-pekka.koskinen@nokia.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Apple | Naveen Palle | naveen.palle@apple.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| Sharp | Lei Liu | lei.liu@cn.sharp-world.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| MediaTek | Pradeep Jose | pradeep[dot]jose[at]mediatek[dot]com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Sequans | Noam Cayron | noam.cayron@sequans.com |
| Convida | Jerome Vogedes | Vogedes.jerome@convidawireless.com |
| DENSO | Haruhiko Sogabe | haruhiko.sogabe.j4r@jp.denso.com |

# Reference

- R2-2102681	Discussions on eDRX configuration	Qualcomm Incorporated.

- R2-2102736	Discussion on eDRX  for RedCap UEs	OPPO.

- R2-2102852	Extend paging DRX for RedCap devices	Intel Corporation.

- R2-2102862	Discussion on e-DRX for Redcap Devices	Xiaomi Communications.

- R2-2102962	Work on eDRX for RedCap UEs	DENSO CORPORATION.

- R2-2102965	Discussion of eDRX for RedCap	Ericsson	discussion	NR_redcap-Core

- R2-2103039	Discussion on eDRX for RedCap UE	ZTE Corporation, Sanechips.

- R2-2103112	Discussion On eDRX for NR RRC Inactive and Idle	CATT.

- R2-2103530	eDRX for REDCAP	Nokia, Nokia Shanghai Bell.

- R2-2103622	eDRX for RedCap UEs in RRC_IDLE/RRC_INACTIVE	LG Electronics UK.

- R2-2103707	Discussion on eDRX for RedCap	CMCC.

- R2-2103783	Further considerations for eDRX	MediaTek Inc..

- R2-2103887	RedCap UE power-saving with 2.56 DRX cycle	Apple.

- R2-2104059	eDRX for RedCap UE	Huawei, HiSilicon.

- RP-210918	Revised WID on support of reduced capability NR devices, Rel-17 WID, March 2021.

- TR 38.875	Study on support of reduced capability NR devices, Rel-17.

- R2-2104360 	Summary of offline 101 - [REDCAP] eDRX cycles - first round	Intel Corporation.