3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2104520

# Online, Apr 12  -  20 , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0294 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on the field description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The description of the value in nr-RelativeTimeDifference-r16 is misleading. The mapping of reported value and the measured negative value should be specified. - A DL-PRS Positioning Frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer, i.e., dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, dl-PRS-PointA, dl-PRS-CombSizeN, dl-PRS-ResourceBandwidth and dl-PRS-StartPRB in TS38.214. However in the current TS37.355, a restriction in the field description of parameters configured by NR-DL-PRS-PositioningFrequencyLayer that all DL-PRS resource sets belonging to the same positioning frequency layer have the same value of the parameters(i.e. dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, dl-PRS-PointA) configured by NR-DL-PRS-PositioningFrequencyLayer is missing. - Different names exist for the field decription nr-PositionCalculationAssistance. This needs to be corrected. - Currently, the description of additionalPaths in NR-DL-TDOA-RequestLocationInformation and NR-Multi-RTT-RequestLocationInformation is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a description for the mapping of reported value and the measured negative value in the field description of nr-RelativeTimeDifference-r16. - Add a restriction that “all DL-PRS resource sets belonging to the same Positioning Frequency layer have the same value of the parameters configured by NR-DL-PRS-PositioningFrequencyLayer” in the field description of the following parameters configured by NR-DL-PRS-PositioningFrequencyLayer: - dl-PRS-SubcarrierSpacing, - dl-PRS-CyclicPrefix, - dl-PRS-StartPRB - The field name has been changed from nr-PositionCalculationAssistanceData to nr-PositionCalculationAssistance - Add the field description of additionalPaths in NR-DL-TDOA-RequestLocationInformation and NR-Multi-RTT-RequestLocationInformation. Impact analysis Impacted 5G architecture options: NR SA, MR-DC Impacted functionality: Positioning Assistance data request, NR-DL-TDOA-RequestLocationInformation, NR-Multi-RTT-RequestLocationInformation Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue forseen. - If the UE is implemented according to the CR and the network is not, improper DL-PRS may be configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The errors and unclarities described above still remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3, 6.5.10.1, 6.5.10.2, 6.5.10.5, 6.5.11.1, 6.5.11.2, 6.5.12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR also includes changes provided in R2-2104269 and R2-2103924 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2102920 |  |  |  |  |  |  |  |  |

START OF CHANGE

6.4.3	Common NR Positioning Information Elements

#### –	NR-AdditionalPathList

The IE NR-AdditionalPathList is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-AdditionalPath-r16 ::= SEQUENCE {

nr-RelativeTimeDifference-r16	CHOICE {

k0-r16					INTEGER(0..16351),

k1-r16					INTEGER(0..8176),

k2-r16					INTEGER(0..4088),

k3-r16					INTEGER(0..2044),

k4-r16					INTEGER(0..1022),

k5-r16					INTEGER(0..511),

...

},

nr-PathQuality-r16				NR-TimingQuality-r16					OPTIONAL,

...

}

-- ASN1STOP

NR-AdditionalPathListfield descriptions

nr-RelativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing of the reference resource. The mapping of reported values and measured quantity value is defined in TS 38.133 [46] clause 10.1.23.3.3 and 10.1.25.3.3. A positive value indicates that the particular path is later in time than the detected path of the reference; a negative value indicates that the particular path is earlier in time than the detected path of the reference.

nr-PathQuality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

NEXT CHANGE

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. the serving TRP.

NOTE 2:	The nr-DL-PRS-ReferenceInfo defines the "assistance data reference" TRP whose DL-PRS configuration is included in nr-DL-PRS-AssistanceDataList. The nr-DL-PRS-SFN0-Offset's and nr-DL-PRS-expectedRSTD's in nr-DL-PRS-AssistanceDataList are provided relative to the "assistance data reference" TRP.

NOTE 3:	The network signals a value of zero for the nr-DL-PRS-SFN0-Offset, nr-DL-PRS-expectedRSTD, and nr-DL-PRS-expectedRSTD-uncertainty of the "assistance data reference" TRP in nr-DL-PRS-AssistanceDataList.

NOTE 4:	For NR DL-TDOA positioning (see clause 6.5.10) the nr-DL-PRS-ReferenceInfo defines also the requested "RSTD reference".

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 		DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...

}

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16	INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...}

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.

nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.

nr-SSB-Config This field specifies the SSB configuration of the TRPs.

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.

nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

nr-PhysCellID This field specifies the physical cell identity of the TRP.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes.

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

nr-DL-PRS-Info This field specifies the PRS configuration of the TRP.

dl-PRS-SubcarrierSpacing This field specifies the subcarrier spacing of the DL-PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2. All DL-PRS Resources and DL-PRS Resource Sets in the same Positioning Frequency layer have the same value of dl-PRS-SubcarrierSpacing.

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-StartPRB.

dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.

dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N.

dl-PRS-CyclicPrefix This field specifies the Cyclic Prefix length of the DL-PRS Resource. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-CyclicPrefix.

NEXT CHANGE

#### 6.5.10.1	NR DL-TDOA Assistance Data

#### –	NR-DL-TDOA-ProvideAssistanceData

The IE NR-DL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-TDOA. It may also be used to provide NR DL-TDOA positioning specific error reason.

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present for UE based NR DL-TDOA; otherwise it is not present. |

NR-DL-TDOA-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-TDOA-Error This field provides DL-TDOA error reasons.

#### 6.5.10.2	NR DL-TDOA Assistance Data Request

#### –	NR-DL-TDOA-RequestAssistanceData

The IE NR-DL-TDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-TDOA-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16							OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs 	(0),

posCalc (1) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-DL-TDOA-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

NEXT CHANGE

#### 6.5.10.5	NR DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0) } (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

maxDL-PRS-RSTD-MeasurementsPerTRPPair This field specifies the maximum number of. DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation.

NEXT CHANGE

### 6.5.11	NR DL-AoD Positioning

This clause defines the information elements for NR downlink AoD positioning (TS 38.305 [40]).

#### 6.5.11.1	NR DL-AoD Assistance Data

#### –	NR-DL-AoD-ProvideAssistanceData

The IE NR-DL-AoD-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-AoD. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

NR-DL-AoD-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, -- Cond UEB

nr-DL-AoD-Error-r16					NR-DL-AoD-Error-r16					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present for UE based NR DL-AoD; otherwise it is not present. |

NR-DL-AoD-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-AoD-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-AoD-Error This field provides DL-AoD error reasons.

#### 6.5.11.2	NR DL-AoD Assistance Data Request

#### –	NR-DL-AoD-RequestAssistanceData

The IE NR-DL-AoD-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-AoD-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-AdType-r16					BIT STRING { 	dl-prs 	(0),

posCalc (1) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-DL-AoD-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

NEXT CHANGE

6.5.12.5	NR Multi-RTT Location Information Request

–	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq(0)} (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL	-- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiffMeasurementInfoRequest This field, if present, indicates that the target device is requested to report the DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) associated with the DL-PRS Resources(s) or the DL-PRS Resource Set(s) which are used in determining the UE Rx-Tx time difference measurements.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (0..5) corresponds to (k0..k5) used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-Multi-RTT-RequestLocationInformation.

END OF CHANGE