3GPP TSG-RAN WG2 Meeting #113bis electronic                                         R2-2104473

Online, April 12 – April 20, 2021

Title: 	[AT113bis-e][707][V2X/SL] Uu DRX impact to support SL

Source: 	CATT

Agenda item:	8.15.2

Document for:	Discussion and Decision

# Introduction

This paper handles the following email discussion:

[AT113bis-e][707][V2X/SL] Uu DRX impact to support SL (CATT)

Scope: Summarize and discuss Uu DRX impact to support SL based on the companies’ contributions (2.2 in R2-2102690, 2.4 in R2-2102771 and similar ones in others) including any need of separate timer to define additional Uu active time for SL mode 1 operation, etc.

Intended outcome: Discussion summary in R2-2104473.

Deadline: 4/19, 10:00am (UTC), R2-2104473 should be available before next Monday session on SL enhancement

This email discussion is divided in two phases:

- Phase I with the deadline on Thursday April 15 22:00 UTC for companies to provide their views on the questions.

- Phase II with the deadline on Monday April 19 10:00 UTC for rapporteur to finalize summary report based on input of Phase I.

# Discussion

In Rel-17, it is possible that SL DRX and SL resource allocation mode 1 are simultaneously configured for one UE. And in RAN2#113 meeting, it was agreed that:

Agreements on high-level principles for SL DRX

9: In mode 1, when in RRC_CONNECTED, if DRX is configured, the MAC entity monitors the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX Active Time. MAC entity does not need to monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX in-active Time.

The above agreement means if UE is configured with SL resource allocation mode 1 and there is SL data transmission requirement, the Uu should be kept in active time. Hence, it is obvious that SL DRX will impact the Uu DRX. In this email discussion, we will discuss the detailed SL DRX impacts on Uu DRX.

## Impact on the drx-onDurationTimer and drx-InactivityTimer in Uu

In [2], it mentioned that “when both DRX and NR SL mode 1 are configured, NG-RAN will properly configure drx-onDurationTimer and drx-InactivityTimer e.g. considering SL traffic patterns to be reported and DL/UL traffic patterns”. According to the rapporteur’ understanding, we should first make clear whether it is necessary to introduce SL-specific drx-onDurationTimer and drx-InactivityTimer in Uu?

Question 1-1: Whether to introduce SL-specific drx-onDurationTimer in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | No | In mode 1, UE would monitor PDCCH to obtain scheduled SL resource. The required UE behaviour is the same as in the legacy onduration timer. We prefer to reuse the existing onduration timer. |
| Ericsson | No | Agree with Xiaomi, it is not well motivated to have SL specific on-duration timer. if there is SL specific Uu DRX on-duration timer, which needs to be aligned with UL/DL DRX on-duration timer, in this case, there is no point to have specific timer in Uu DRX for SL Mode 1. gNB can configure a proper timer value for the common on-duration timer (i.e., common for both Uu UL/DL and SL mode 1), to accommodate PDCCH monitoring for both Uu UL/DL and SL mode 1. |
| LG | No |  |
| OPPO | No | The Uu drx-onDurationTimer can be directly used for SL traffic, nothing is different between SL and DL/UL grant reception regarding this timer. |
| Lenovo | No | There is no need to have some SL specific drx-On-DurationTimer in addition to to the drx-OndurationTimer. The configuration of the drx-onDurationTimer should also consider SL transmission activity. |
| CATT | No | Considering it had better align the Uu and SL DRX, there is no need to introduce SL-specific drx-onDurationTimer in Uu from the perspective of power saving. |
| InterDigital | No | The guaranteed wakeup time should be the same for SL scheduling and Uu scheduling. There is no need to separate these. |
| Spreadtrum | No |  |
| Huawei, HiSilicon | No | There is no need to introduce SL-specific onduration timer in Uu. |
| Sharp | No |  |
| Nokia | No |  |
| vivo | No | The legacrx-onDurationTimer configuration by the NW should properly take into account the SL transmission activity instead of introducing a new timer. |
| ZTE | No |  |
| Fraunhofer | No |  |
| Intel | No | Agree with Ericsson that we can rely on gNB configuration in this case to take SL operation into account and no new timer is needed |
| Apple | No |  |
| Convida Wireless | No |  |
| Qualcomm | No | No need for Mode 1 operation. |

Rapporteur comment:

Within 18 companies:

- Agree: All

Proposal 1[18/18]:  SL-specific drx-onDurationTimer is not introduced in Uu.

Question 1-2: Whether to introduce SL-specific drx-InactivityTimer in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | No | Currently, the inactivity timer would only be started by PDCCH indicating new UL or DL transmission. We could extend the triggering condition to include PDCCH indicating new SL transmission. |
| Ericsson | No | Agree with Xiaomi |
| LG | No |  |
| OPPO | No | The same reason as the reply to Q1-1. |
| Lenovo | No | There is in our understanding no need to have a SL-specific drx-InactivityTimer. However we need to introduce new SL related trigger conditions for drx-Inactivity (re)starting. |
| CATT | No | Drx-InactivityTimer is used for extending the Uu DRX active time, there is no need to identify Uu and SL. |
| InterDigital | Yes | Uu active time in DRX is controlled by an inactivity timer. For a SL UE, mode 1 scheduling is triggered by the UE sending SR/BSR when data arrives in the SL transmission buffers. Because the network schedules the UE in mode 1 based on the priority of the SL data, the amount of additional time the UE should remain active on PDCCH following each SL scheduling grant will depend on this priority. The timeline of such data arrival is therefore different than normal Uu traffic, and a different inactivity timer should be defined for the SL UE. |
| Spreadtrum | No |  |
| Huawei, HiSilicon | No | There is no need to introduce SL-specific inactivity timer in Uu. As in LTE DRX, the UE starts or restarts the drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission. |
| Sharp | No |  |
| Nokia | No |  |
| vivo | No | As reply in Question 1-1, the legacy Uu drx-InactivityTimer configuration should also take into account the SL transmission activity instead of introducing a new timer. |
| ZTE | No | The triggering condition of inactivity timer shall be added to consider the SL resource allocation. but new inactivity timer is unnecessary. |
| Fraunhofer | No |  |
| Intel | No |  |
| Apple | No |  |
| Convida Wireless | No | But we do need to restart the drx-InactivityTimer at the UE based on its monitoring the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI. |
| Qualcomm | No, comment | Generally OK with one Inactivity timer for extending active state for monitoring all DCI formats like On timer, if different time lines for Uu traffic and SL traffic are NOT taken into consideration for the Inactivity timer value setting. |

Rapporteur comment:

Within 18 companies:

- Yes: 1

- No: 17

Only one company thinks the SL-specific drx-InactivityTimer should be introduced because the timeline of SL traffic is different from Uu traffic. And all the other companies (17 companies) agree not to introduce SL-specific drx-InactivityTimer in Uu. Follow the majority view, it is recommended:

Proposal 2[17/18]:  SL-specific drx-InactivityTimer is not introduced in Uu.

Regarding to the drx-onDurationTimer, it is no doubt that its maintainence should follow legacy Uu. Regarding to the drx-InactivityTimer, no matter which answer is chosen for Question 1-2, new drx-InactivityTimer maintenance mechanism should be considered if taking SL scheduling into account. In [1] and [5], it is proposed that the Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

Question 1-3: For sidelink resource allocation mode 1, regarding to the drx-InactivityTimer maintenance, which options do you prefer? Please give your comments.

- Option 1: Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission;

- Option 2: others (if companies have other options, it can be added here).

| Companies | Option | Comments |
| --- | --- | --- |
| Xiaomi | Option 1 |  |
| Ericsson | Option 1 |  |
| LG | Option 1 |  |
| OPPO | Option 1 | There is no difference between a PDCCH indicating a new SL transmission and DL/UL transmission, therefore the Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission. |
| Lenovo | Option 1 With Comments | In addition to the option 1, RAN2 should also consider to ensure that Tx UE is in ActiveTime for the Uu interface in response to having sent an ACK on PUCCH for a SL data transmission in order to be ready to receive a further SL DCI for a new transmissions. This could be implemented by starting the drx-InactivityTimer. It should be noted that different to Uu, for SL gNB is not really aware of the current BS since UE selects TBS autonomoulsy (in Uu UL the TBS is indicated by DCI). Therefore gNB has for SL less information on UE’s status (buffer status). |
| CATT | Option 1 | Similar as Uu drx-inactivityTimer maintenance for UL/DL can be used. |
| InterDigital | Option 1 | The TX UE restarts the SL inactivity timer if the UE receives SL scheduling, and it restarts the Uu inactivity timer if it receives normal Uu scheduling. |
| Spreadtrum | Option 1 |  |
| Huawei, HiSilicon | Option 1 |  |
| Sharp | Option 1 |  |
| Nokia | Option 1 |  |
| vivo | Option 1 |  |
| ZTE | Option 1 |  |
| Fraunhofer | Option 1 |  |
| Intel | Option 1 |  |
| Apple | Option 1 |  |
| Convida Wireless | Option 1 |  |
| Qualcomm | Option 1 |  |

Rapporteur comment:

Within 18 companies:

- Option 1: All

Proposal 7[18/18]:  For Tx UE configured with sidelink resource allocation mode 1, it should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

## Impact on the drx-HARQ-RTT-Timer and drx-RetransmissionTimer in Uu

According to TS38.321, two drx-HARQ-RTT-Timers are defined as below, one is for UL and another is for DL.

- drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

In there is SL PUCCH configuration in Uu, according to the analysis in [1][2][3][4][5][6], SL-specific drx-HARQ-RTT-Timer is suggested to be introduced in Uu.

Question 2-1: Whether to introduce SL HARQ process related DRX timers, which including drx-HARQ-RTT-Timer and drx-RetransmissionTimer, in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | These timers could reduce UE’s monitoring time and bring power saving gain. |
| Ericsson | Yes | It is beneficial to introduce SL specific HARQ timers, they operate per SL HARQ process. |
| LG | Yes | drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL should be introduced for SL mode 1 UE supporting the DRX. |
| OPPO | Yes | The original RTT and Retx timers are used for UL and DL HARQ process respectively, therefore the corresponding drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL should be defined for SL HARQ process. |
| Lenovo | Yes | For SL Mode 1, if the UE is configured with sl-PUCCH-Config, in order to ensure that the SL Tx UE is ready to receive SL DCI(s) for retransmissions, SL TX UE should transit to ActiveTime on the Uu interface monitoring PDCCH in response to having transmitted a NACK to gNB on PUCCH triggered by the reception of a NACK on PSFCH from a Rx UE. Therefore a SL specific drx-HARQ-RTT-Timer as well as drx-RetransmissionTimer should be introduced. |
| CATT | Yes | SL-specific HARQ related timers can be introduced similar as the HARQ related timers introduced for UL and DL. |
| InterDigital | Yes | HARQ timeline on Uu and SL is different, so different values of timers are required to consider the additional latency over SL for feedback and/or reporting of HARQ feedback to the network. |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes with comments | We agree to define two HARQ related timer, i.e. drx-HARQ-RTT-TimerSL and drx-RetransmssionTimerSL for each SL HARQ process. But we think the corresponding UE behaviour is different under different cases. Case 1: UE is configured with PUCCH, how to start these two timers can follow the legacy Uu behaviour, i.e, - Start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback - Start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL if the data of the corresponding SL HARQ process has not been successfully transmitted via the sidelink Case 2: UE is configured without PUCCH, as blind retransmission is supported, the UE still needs to extend the active time to monitor blind retransmission. Different from case 1, there is no HARQ feedback transmitted and when to schedule the blind retransmission is all up to the NW. Therefore, there is no need to start the drx-HARQ-RTT-TimerSL since there is no need to consider the processing time of HARQ feedback at the network side. The UE only needs to start the drx-RetransmissionTimerSL to monitor the blind retransmission. |
| Sharp | Yes |  |
| Nokia | Yes |  |
| vivo | Yes | When considering the mode-1 scheduling with Uu DRX, the drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL would be affected by the HARQ feedback on PSFCH. The configuration of Uu DRX parameters (e.g. drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL) should consider status of sidelink transmission (e.g. HARQ ACK/NACK on PSFCH) when DCI is used to schedule sidelink mode-1 UE. |
| ZTE | Yes | If a UE is configured with PUCCH resource to carry SL HARQ, a drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL for each sidelink process are necessary for Uu DRX since the UE needs to receive the sidelink retransmission resource allocation information from the gNB. |
| Fraunhofer | Yes | SL specific HARQ related timers should be introduced. |
| Intel | Yes |  |
| Apple | Yes |  |
| Convida Wireless | Yes |  |
| Qualcomm | Yes | A new set of HARQ timers for SL. |

Rapporteur comment:

Within 18 companies

- Yes: All

Proposal 8[18/18]:  SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be introduced in Uu, which are maintained based on sidelink process.

If the answer of Question 2-1 is Yes, the pre-condition for supporting such timers should be further discussed. According to the current NR Uu specification, drx-HARQ-RTT-Timer and drx-RetransmissionTimer for each downlink process are configured for Uu DRX since the UE needs to receive the retransmission resource allocation information from the gNB. In the rapporteur’s understanding, the similar principle is also applied for NR sidelink. For NR sidelink, if PUCCH for SL HARQ FB is not configured, SL retransmission will not happen, these timers does not need to be maintained; on the contrary, SL retransmission is possible and it should main these SL-specific HARQ retransmission related timers in Uu.

Question 2-2: Do companies agree that the SL HARQ process related timers (drx-HARQ-RTT-Timer and drx-RetransmissionTimer) only need to be supported when there is PUCCH configuration for SL HARQ FB in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | Agree with rapp that retransmission scheduled by gNB is only possible when PUCCH is configured for feedback. |
| Ericsson | No | Similar as discussions for SL DRX HARQ timers, we think at least retransmissionTimer needs to be supported in case of PUCCH config is absent. Regarding RTT timer, we don’t have strong views, we can follow the majority view. |
| LG | No | We prefer to use SL HARQ feedback such as Uu DRX design regardless of PUCCH resource. |
| OPPO | No | Even if without PUCCH configuration, there may be PSFCH feedback, and thus both Tx/Rx UE has to respect RTT interval, which means the RTT and Retx timers are essential for all the cases. As for the cases without PUCCH and PSFCH configuration, the start point of RTT timer may be different: - If PUCCH is configured, RTT timer is started after PUCCH transmission. - If PUCCH is not configured, but if PSFCH is configured, considering RAN1 has agreed on the PSFCH-to-PSSCH latency requirement, it seems to start RTT timer since PSFCH is reasonable. - If when neither PUCCH nor PSFCH is not configured, start RTT/Re-transmission timer after PSSCH could be feasible. The retransmission timer can be started upon the expiry of SL RTT timer. |
| Lenovo | No | Similar to the case were UE is configured with PUCCH configuration for SL HARQ FB, it should be also for Blind retransmissions ensured that UE extends its ActiveTime in order to receive SL DCI for SL retransmission resources. Otherwise the gNB needs to schedule the SL retransmission resource on the next OnDuration time. Therefore, UE should also a SL specific drx-RetransmissionTimer to extend the active time to monitor PDCCH for retransmission in case PUCCH is not configured. HARQRTTTimer wouldn’t be required though. |
| CATT | Yes | Regarding to the combinations of PSFCH and PUCCH configuration, there are four combinations: Case; PUCCH configuration; PSFCH configuration; Necessity for maintaining SL-specific RTT/Retx timer in Uu. 1; Yes; Yes; Yes. 2; No; Yes; ?. 3; No; No; ?. 4; Yes; No; Rel-16 V2X has excluded this case. In our understanding: - For case 1, it certainly needs to support SL-specific RTT and retx timer in Uu; - For case 2, this combination is agreed by RAN1 and it is mainly used for the case that SL blind retx is used. That is one scheduling inform in Uu can allocate more than one SL resources, e.g., one for initial tx and the others used for retx). Considering blind retx and HARQ-based retx was not supported till now, in this case, once the gNB sends the scheduling information to UE, it will not schedule the retransmission for the same sidelink HARQ process. Hence, there is no need to maintain the SL-specific RTT and retx timer in Uu. - For case 3, if there is neither PSFCH nor PUCCH, only blind retx is needed. In Uu, only one scheduling information for SL is enough and the scheduling information may include the blind retransmission resources for SL. Similar as case 2, it also does not need to maintain the SL-specific RTT and retx timer in Uu. |
| InterDigital | No | We can define similar HARQ RTT (for microsleep) and retransmission timer (for monitoring PDCCH) regardless of whether PUCCH resources are defined. |
| Spreadtrum | No | Both RTT timer and retransmission timer are needed even if PUCCH is absent. |
| Huawei, HiSilicon | Yes for HARQ RTT timer No for retransmission timer | See our reply above. Even the UE is configured without PUCCH, since blind retransmission is supported, the UE still needs to extend the active to monitor the blind retransmission, otherwise the gNB can only schedule the SL retransmission resource on the next OnDuration time, which is less-efficient. But as there is no HARQ feedback on PUCCH, when UE is configured without PUCCH, there is no need to start the HARQ RTT timer which is to consider the processing time of HARQ feedback at the network side. The UE only needs to start the retransmission timer at some point to monitor the blind retransmission. |
| Sharp | Yes | We agree the timers are maintained only if PUCCH for SL HARQ FB is configured. |
| Nokia | No |  |
| vivo | No | We think even in case of PUCCH for SL HARQ FB not configured, SL retransmission can happen. Therefore, prefer to keep drx-HARQ-RTT-Timer and drx-RetransmissionTimer to have a unified design. |
| ZTE | Yes | Agree with rapporteur, For NR sidelink, if PUCCH for SL HARQ FB is not configured, SL retransmission will not happen, these timers does not need to be maintained. |
| Fraunhofer | No | HARQ RTT and retransmission timer should be supported even if PUCCH is not present |
| Intel | Yes for Retransmission timer | At least retransmission timer needs to be supported as mentioned by the rapporteur. We have no strong view on RTT timer, but it would be good to support both for common design |
| Apple | Yes | The timer is only needed if PUCCH for SL HARQ is configured. |
| Convida Wireless | No | The purpose of SL drx-HARQ-RTT-Timer, if introduced, will be to allow micro-sleep before PDCCH monitoring for a SL retransmission assignment and the purpose of SL drx-RetransmissionTimer, if introduced, will be to allow the monitoring of PDCCH for a maximum duration until a grant for SL retransmission is received. These behaviours are still necessary regardless of whether PUCCH resources are configured, for so long as the possibility for blind retransmission or SL HARQ feedback based retransmission is assumed. |
| Qualcomm | Yes, comment | Yes, if retransmission grant from gNB is based on NACK on PUCCH. FFS: other cases |

Rapporteur comment:

Regarding to the issue whether PUCCH configuration is the pre-condition of maintaining the SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer in Uu, the views of 18 companies are summarized as below:

- For SL-specific drx-HARQ-RTT-Timer:

  - Yes:  7

  - No: 10

- For SL-specific drx-RetransmissionTimer:

  - Yes: 7

  - No: 11

Based on the comments in the above table, majority companies (16 /18) agrees that at least SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained if the sl-PUCCH-Config is configured, and the main divergence is whether SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained when sl-PUCCH-Config is not configured.

Proposal 9 [16/18]:  When sl-PUCCH-Config is configured, SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained for UE configured with sidelink resource allocation mode 1.

Proposal 10: when sl-PUCCH-Config is not configured, RAN2 should further discuss whether SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained.

If the answer of Question 2-1 is Yes, it should also discuss how to define the SL-specific drx-HARQ-RTT-Timer and  drx-RetransmissionTimer. In Uu, the legacy corresponding descriptions are as below:

- drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received; - drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received; - drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

According to [2], [3] and [5], the similar definitions as Uu are suggested.

Proposals in [2]: - drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

Proposals in [2]: - drx-RetransmissionTimerSLmode1 (per SL HARQ process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSLmode1 (per SL HARQ process): the minimum duration before a SL HARQ retransmission grant is expected by the MAC entity.

Proposals in [5]: Proposal 2: For the case when UE is configured with sl-PUCCH-Config, define two HARQ related timer, i.e. drx-HARQ-RTT-TimerSL and drx-RetransmssionTimerSL for each SL HARQ process. The values of these two timers are configured by gNB. - drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL HARQ retransmission grant is expected by the MAC entity.

Question 2-3: Whether the following definitions for SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer can be agreed (note: please ignore the detailed name of the SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer, which will be discussed separately in section 2.3)? Please give your comments.

- drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | Follow Uu design |
| Ericsson | Yes | Same as Uu design |
| LG | Yes | Same as Uu design |
| OPPO | Yes |  |
| Lenovo | Yes | Uu DRX principles should be followed. |
| CATT | Yes |  |
| InterDigital | Yes | Conceptually these are the same for SL process |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Sharp | Yes |  |
| Nokia | Yes |  |
| vivo | Yes |  |
| ZTE | Yes |  |
| Fraunhofer | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Convida Wireless | Yes |  |
| Qualcomm | Yes |  |

Rapporteur comment:

Within 18 companies

- Yes: All

Proposal 11 [18/18]:  Adopt the following definitions of SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer (the detailed name of the timers can be further discussed):

- drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

If the answer of Question 2-1 is Yes, how to maintain the SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer should also be further discussed.  Some similar proposals are suggested in the corresponding contributions [1] [4] [5], listed below:

Proposals in [1]: Proposal 5: For NR sidelink, if mode 1 resource allocation is used, UE should start the drx-HARQ-RTT-TimerUuSL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback in Uu. Proposal 6: For NR sidelink, if mode 1 resource allocation is used, UE should start the drx-RetransmissionTimerULSL if drx-HARQ-RTT-TimerUuSL expiries and the data of the corresponding SL HARQ process is not successfully decoded.

Proposal in [4]: - if a SL HARQ feedback is received, start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback. - if the data of the corresponding SL HARQ process was not successfully decoded, start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL.

Proposal in [5]: Proposal 3: If the UE needs to send the SL HARQ feedback via the PUCCH to the gNB, the UE shall: - Start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback; - Stop the drx-RetransmissionTimerSL for the corresponding HARQ process. Proposal 4: Upon the expiry of a drx-HARQ-RTT-TimerSL, and the data of the corresponding SL HARQ process has not been successfully transmitted via the sidelink, the UE shall: - Start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL

Question 2-4: Whether companies can agree the following timer maintenance mechanism for SL-specific drx-HARQ-RTT-Timer? Please give your comments.

- UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| LG | No | We prefer to use SL HARQ feedback such as Uu DRX design regardless of PUCCH resource. We suggests following modification. UE starts the drx-HARQ-RTT-TimerSL for the corresponding HARQ process ID in the first symbol after the end of the corresponding PSFCH reception carrying the SL HARQ feedback, regardless of whether PUCCH is configured or not. If SL HARQ feedback is disabled, UE starts the drx-HARQ-RTT-TimerSL for the corresponding HARQ process ID in the first symbol after the end of the first repetition of the corresponding PSSCH transmission, regardless of whether PUCCH is configured or not. |
| OPPO | See comments | As our comments to Q2-2, the start point of the RTT timer may be different, can be either PUCCH, PSFCH or PSSCH. |
| Lenovo | Yes |  |
| CATT | Yes |  |
| InterDigital | No | This can work only if PUCCH is configured. We can use this definition for that case (same as Uu). - When PUCCH is not configured start the timer upon reception of PSFCH When PUCCH is not configured and PSFCH is not configured (or transmission is HARQ disabled), start timer upon SCI transmission |
| Spreadtrum | No | - RTT timer should start upon the corresponding PSFCH transmission if SL HARQ feedback is enabled. - RTT timer should start upon the corresponding PSSCH transmission if SL HARQ feedback is disabled and PUCCH presents. - RTT timer is not need if SL HARQ feedback is disable and PUCCH is absent. |
| Huawei, HiSilicon | See comments | As we replied above, we think the maintenance of SL-specific HARQ RTT timer only applies to the case when PUCCH is configured to the UE. Therefore, to make it more clear, we prefer to reword as below: If the UE is configured with sl-PUCCH-Config, UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback |
| Sharp | Yes |  |
| Nokia | Yes |  |
| vivo | Yes with comments | If PUCCH is not configured, it should be further discussion e.g. HARQ RTT timer may be started on reception of PSFCH, or no HARQ RTT timer but only retransmission timer. Considering the discussion can be complicated and hard to converge, we are also wondering whether the issue can be properly handled by implementation, i.e., the NW always configure PUCCH carrying SL HARQ FB and SL-specific drx-HARQ-RTT-Timer together at this release. |
| ZTE | Yes |  |
| Fraunhofer | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Convida Wireless | No | We can discuss this further once agreement is made for question Q2-2. |
| Qualcomm | Yes, comment | For PUCCH configured. |

Rapporteur comment:

According to the summary of Question 2-2, it is still FFS whether the SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained in case of there is no Sl-PUCCH-Config. Hence here, we only summarized the companies view for the case when Sl-PUCCH-Config is configured. When Sl-PUCCH-Config is configured, regarding to the proposed SL-specific drx-HARQ-RTT-Timer maintenance, the companies’ view are summarized as below:

Within 18 companies

- Yes: 15 (12 companies answers Yes plus 3 companies answers no or with comment but indicates it can support the proposed timer maintenance in case of Sl-PUCCH-Config is configured, which including  Huawei, OPPO and InterDigital)

- No: 3

Proposal 12[15/18]:  When sl-PUCCH-Config is configured, the UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback.

Question 2-5: Whether companies can agree the following time maintenance mechanism for SL-specific drx-RetransmissionTimer? Please give your comments.

- If the data of the corresponding HARQ process was not successfully decoded in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes with comment | Unlike Uu, the data decoding and feedback transmission in PUCCH is done at RX UE and Tx UE respectively. Therefore, we suggest rewording such as, If the feedback of the corresponding HARQ process was NACK, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |
| Ericsson | Yes and No with comments | For PUCCH config present case, agree with Xiaomi’s suggested update. For PUCCH config absent case, if RTT timer is not supported, UE starts the retransmission timer in the first symbol after the last PSSCH transmission using the scheduled grant for the corresponding SL HARQ process |
| LG | Yes with comment | Agree with Xiaomi’s modification for HARQ Feedback enabled case. For HARQ feedback disabled case, reference time for starting the HARQ RTT timer should be decided. For example, in the HARQ feedback disabled case, UE starts the drx-HARQ-RTT-TimerSL for the corresponding HARQ process ID in the first symbol after the end of the first repetition of the corresponding PSSCH transmission. |
| OPPO | No | In sidelink, whether the corresponding HARQ process was successfully decoded is judged by the Rx UE, and there may be no HARQ FB to the Tx UE. However, whether a SL grant for retransmission is needed is judged by the Tx UE based on the FB from the Rx UE or other reasons. Therefore, suggest to change the expression as follows: If retransmission of the data of the corresponding HARQ process is needed in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |
| Lenovo | Yes with comments | For BL case also the SL specific drx-RetransmissionTimer should be started. |
| CATT | Yes with comment | We agree with the modification of Xiaomi, If there is PUCCH configuration in Uu and there must be PSFCH configuration in SL, hence whether the data can be decoded successfully can be judged based on the PSFCH feedback. |
| InterDigital | No | Agree with the wording from OPPO – the need for retransmission does not depend on decoding, and may not depend on HARQ feedback if no HARQ feedback is expected. |
| Spreadtrum | No | If RTT timer is used, agree the wording from OPPO. If RTT timer not used, the retransmission timer should start upon the transmission of the corresponding PSSCH. |
| Huawei, HiSilicon | See comments | As we replied above, we think the described maintenance of SL-specific retransmission timer above only applies to the case when PUCCH is configured to the UE. When PUCCH is not configured, there is no need to start the SL-specific HARQ RTT timer and when to start the retransmission timer for blind retransmission monitoring needs FFS. Also from the TX UE’s perspective, it can only judge whether the data is successfully transmitted or not while from the RX UE’s side, the RX UE determines whether the data is successfully decoded or not. Therefore, to make it more clear, we prefer to reword as below: If the UE is configured with sl-PUCCH-Config, if the data of the corresponding HARQ process was not successfully decoded transmitted in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |
| Sharp | Yes with comments | Agree with Xiaomi’s modification. |
| Nokia | No | Agree with OPPO’s view and modification |
| vivo | Yes | OK with Xiaomi’s suggestion. |
| ZTE | Yes with comments | Agree with the modification of Xiaomi |
| Fraunhofer | No | Agree with OPPO’s view and modification. |
| Intel | Yes with comment | If the timer is supported/configured, we are fine to use the proposed wording |
| Apple | See comment | The wording is wrong from TX UE perspective, the ReTx-timer is also only needed if PUCCH is configured and started after the HARQ-RTT timer expires. |
| Convida Wireless | Yes with comments | We do have sympathy for companies who answered ‘no’ and the clarification from OPPO. We have no strong view but one possibility is to keep the wording as suggested in the question so as to be aligned with the wording of the legacy handling of drx-HARQ-RTT-TimerDL, but to clarify that from TX perspective, the “not successfully decoded in sidelink” means that retransmission of the data of the corresponding HARQ process is needed. |
| Qualcomm | Yes, comment | Yes, if for PUCCH configured |

Rapporteur comment:

According to the summary of Question 2-2, it is still FFS whether the SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained in case of there is no Sl-PUCCH-Config. Hence here, we only summarized the companies view for the case when the Sl-PUCCH-Config is configured.

When there is Sl-PUCCH-Config, regarding to the proposed SL-specific drx-RetransmissionTimer maintenance, the main divergence is whether the need for retransmission depends on the SL PSFCH feedback or not. Some companies considered the judgement for retransmission should be based on SL PSFCH feedback, but OPPO suggested that it should also consider the case there is no PSFCH feedback.

Within 18 companies, the judgement for retransmission should be based on:

- SL PSFCH:  7

- No SL PSFCH should also be considered:  5

In the rapporteur’s understanding, OPPO’s concern is reasonable, no SL PSFCH case should also be considered since PSFCH may be dropped due to some reasons. But since there is divergence, we had better recommend RAN2 to further discuss this issue.

Proposal 13:  When sl-PUCCH-Config is configured, if the retransmission of the corresponding sidelink process is necessary, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. RAN2 should further discuss on how to judge whether the retransmission of the corresponding sidelink process is necessary, based on PSFCH feedback or not.

According to the proposal in [5], it also mentioned that if the UE needs to send the SL HARQ feedback via the PUCCH to the gNB, the UE shall start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback. And in the meanwhile, it should stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

Question 2-6: Whether companies can agree to stop the SL-specific drx-RetransmissionTimerSL in the meanwhile of the start of the drx-HARQ-RTT-TimerSL? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Comment | We think the proposal should be modified as following, since there are RTT and RTX timer in both Uu and SL. It’s not clear which timer is in current wording. Stop the SL-specific drx-RetransmissionTimer in Uu in the meanwhile of the start of the drx-HARQ-RTT-TimerSL in SL |
| Ericsson | Yes | Same as Uu DRX |
| LG | Yes | Same as UU DRX design as following; “stop the drx-RetransmissionTimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ ACK feedback.” |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| CATT | Yes | Similar as Uu DRX |
| InterDigital | Yes | Same as Uu |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes | Similar as in Uu. |
| Sharp | Yes |  |
| Nokia | Yes |  |
| vivo | Yes |  |
| ZTE | Yes |  |
| Fraunhofer | Yes |  |
| Intel | Yes | Ok to follow Uu like functionality |
| Apple | Yes |  |
| Convida Wireless | Yes | In cases that the HARQ RTT is used |
| Qualcomm | Yes |  |

Rapporteur comment:

Within 18 companies

- Yes: All

Proposal 14[18/18]:  In Uu, the SL-specific drx-RetransmissionTimer should be stopped accompany with the start of the SL-specific drx-HARQ-RTT-Timer corresponding to the same sidelink process.

## Name of the SL-specific DRX timers in Uu

How to define the name of the SL-specific DRX timers in Uu has been discussed in [1][2][3][5]5]:

- drx-HARQ-RTT-TimerUuSL and drx-RetransmissionTimerUuSL in  [1];

- drx-HARQ-RTT-TimerSLmode1 and drx-RetransmissionTimerSLmode1 in  [3][1];

- drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL in  [2][1].

Based on the rapporteur’s understanding, using suffix –SL may cause confusion with the SL DRX timers used in PC5 interface. In order to facilitate the consecutive discussion, it had better make clear how to define the SL-specific DRX related timers in Uu.

Question 3-1: Regarding to the name of the SL-specific DRX related timers in Uu, which suffix should be used:

- Option 1: UuSL;

- Option 2: SLmode1;

- Option 3: SL;

- Option 4: others (if company has different views, please add it here).

| Companies | Option | Comments |
| --- | --- | --- |
| Xiaomi | Too early to decide | We think this is stage 3 discussion. |
| Ericsson | Option 2 |  |
| LG | Option 3 |  |
| OPPO | Option 3 |  |
| Lenovo |  | We think it might be too early for this. However currently for RRC signalling, similar things happen that SL related signalling can be transmitted on both Uu and SL, maybe we could use it as a reference 1. for SL associated signalling transmitted on Uu: use “SL-“ prefix, e.g. SL-ConfigDedicatedNR, SL-ConfiguredGrantConfig etc. 2. For SL signalling transmitted on PC5 (PC5-RRC signaling): use “-Sidelink” suffix, e.g. RRCReconfigurationSidelink, UECapabilityEnquirySidelink etc. |
| CATT | Option 1 or Option 2 | It had better distinguish the SL-specific DRX timer used in Uu from the SL-specific DRX timers used in PC5 since different value may be needed. |
| InterDigital | Option 3 | No strong view, but we think option 3 is sufficient |
| Spreadtrum | No strong views |  |
| Huawei, HiSilicon | Option 3 | We prefer to use drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL |
| Sharp | No strong views |  |
| Nokia | Option 2 or 3 | No strong view |
| vivo |  | No strong view. Can be up to RRC rapporteur. |
| ZTE | Option1 | We think option3 “SL-“ prefix may be used for sidelink DRX related timer, “UuSL-” seems more clearer. |
| Fraunhofer |  | No strong view. |
| Intel |  | This can be discussed in stage 3, but we are partial to option 1 or 3. |
| Apple | NO strong view. Slightly prefer Option 3 |  |
| Convida Wireless | No strong view | Will go with majority |
| Qualcomm | Option 1 or 2 | No strong view, as long as it can be differentiated from the SL DRX on PC5 and the Uu DRX on DL/UL. |

Rapporteur comment:

Within 18 companies

- Option 1:  3

- Option 2: 4

- Option 3: 5

- Too early to decided:  2

- No strong view:  4

Proposal 15:  RAN2 can further discuss how to define the name of the SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer in Uu.

Others

Regarding the issue that Uu DRX timer impact to support SL, besides the issues listed in the above sections, if companies still have other issues, please listed in the table below:

| Companies | Issue description |
| --- | --- |
| Huawei, HiSilicon | If PUCCH is not configured, when to start the SL-specific retransmission for blind retransmission monitoring considering a single DCI can schedule up to 3 retransmission opportunities? |
| Convida Wireless | 38.321 says that the “Active Time includes the time while a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4)”. Should we clarify that this also includes the sidelink Scheduling Request (as described in clause 5.22.1.5). |

Rapporteur comment:

Regarding to the Huawei and Convida’s concern, the rapporteur thinks both should be considered. But considering the limited discussion time in this meeting, it had better left to further email discussion or left to the next meeting.

# Conclusion

[Easy agreements]:

Proposal 1[18/18]:  SL-specific drx-onDurationTimer is not introduced in Uu.

Proposal 2[17/18]:  SL-specific drx-InactivityTimer is not introduced in Uu.

Proposal 7[18/18]:  For Tx UE configured with sidelink resource allocation mode 1, it should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

Proposal 8[18/18]:  SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be introduced in Uu, which are maintained based on sidelink process.

Proposal 9 [16/18]:  When sl-PUCCH-Config is configured, SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained for UE configured with sidelink resource allocation mode 1.

Proposal 11 [18/18]:  Adopt the following definitions of SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer (the detailed name of the timers can be further discussed):

- drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

Proposal 12[15/18]:  When sl-PUCCH-Config is configured, the UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback.

Proposal 14[18/18]:  In Uu, the SL-specific drx-RetransmissionTimer should be stopped accompany with the start of the SL-specific drx-HARQ-RTT-Timer corresponding to the same sidelink process.

[Need discussion]:

Proposal 16: when sl-PUCCH-Config is not configured, RAN2 should further discuss whether SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained.

Proposal 13:  When sl-PUCCH-Config is configured, if the retransmission of the corresponding sidelink process is necessary, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. RAN2 should further discuss on how to judge whether the retransmission of the corresponding sidelink process is necessary, based on PSFCH feedback or not.

Proposal 15:  RAN2 can further discuss how to define the name of the SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer in Uu.

# Reference

- R2-2102690	DRX Active Time Alignment between Uu and SL  CATT

- R2-2102771	Further discussion on Sidelink DRX   LG

- R2-2103003	General aspects of SL DRX	  Ericsson,Qualcomm Incorporated

- R2-2102979	Discussion on  Coordination between Uu DRX and SL DRX	ZTE

- R2-2104113	Discussion on SL communication impact on Uu DRX	Huawei, HiSilicon

- R2-2103470	Coordination between Uu DRX and SL DRX 	 Lenovo, Motorola Mobility