3GPP TSG RAN WG2#113bis-e	R2-2102609

e-Meeting, 12th - 20th April, 2021

3GPP TSG RAN WG1 #104-e	R1-2101924

e-Meeting, January 25th – February 5th, 2021

Title:	Reply LS on multi-TRP description in Stage-2

Response to:	R1-2100018/R2-2010804

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	RAN1

To:	RAN4, RAN2

Contact Person:

Name:	Karri Ranta-aho

E-mail Address:	Karri.Ranta-aho@nokia.com

Attachments: N/A

1. Overall Description:

RAN1 would like to thank RAN2 for the LS in R1-2100018/R2-2010804 on multi-TRP description in Stage-2.

As requested, RAN1 has reviewed the CR to TS38.300 in R2-2010803 and would like to bring to RAN2’s attention that two concerns were raised on the notion that “operation is done by both physical layer and MAC”. One concern was related to the sentence giving the impression that both L1 and MAC are always involved in each operation, and another concern was about ignoring the role of the RRC layer.

Based on these concerns and discussions in RAN1#104, RAN1 would like to suggest the following update to the stage2 text.

… TRP Transmit/Receive Point … Transmit/Receive Point: Part of the gNB transmitting and receiving radio signals to/from UE according to physical layer properties and parameters inherent to that element. … ## 6.X Multiple Transmit/Receive Point Operation In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule UE from two TRPs, providing better PDSCH coverage, reliability and/or data rates. There are two different operation modes for multi-TRP: single-DCI and multi-DCI. For both modes, control of uplink and downlink operation is can be done by both physical layer and MAC layer, within the configuration provided by the RRC layer. In single-DCI mode, UE is scheduled by the same DCI for both TRPs and in multi-DCI mode, UE is scheduled by independent DCIs from each TRP. …

Further, RAN1 would like to suggest making the Multiple Transmit/Receive Point Operation a subclause of “5 Physical layer” instead of a subclause of “6 Layer 2”.

2. Actions:

To RAN2 and RAN4

ACTION: 	RAN1 respectfully ask RAN2 to take the above into account and consider revising the mTRP stage 2 description accordingly.

3. Date of Next TSG-RAN1 Meetings:

3GPPRAN1#104bis-e	12 - 20 April 2021	E-meeting

3GPPRAN1#105-e	19 - 27 May 2021	E-meeting