3GPP TSG RAN WG2#113bis-e	R2-2102623

e-Meeting, 12th - 20th April, 2021

3GPP TSG RAN WG1 Meeting #104	R1-2102152

e-Meeting, January 25th – February 5th, 2021

Title:	LS on numerology for active DL and UL BWPs

Response to:	N/A

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Pei-Kai Liao

#### E-mail Address:	pk.liao@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 has identified that there is inconsistency between the description of FG6-4 BWP adaptation with different numerology in TR38.822 and the definition of UE capability parameter bwp-DiffNumerology in TS38.306. It triggers the discussion for the following issue in RAN1.

- Whether a UE can assume the same SCS and CP length for its active DL BWP and active UL BWP in a serving cell except for SUL at a given time

After discussion, RAN1 achieves the following conclusion.

Conclusion - It’s RAN1’s understanding that for both paired spectrum and unpaired spectrum UE may assume the same SCS and CP length for its active DL BWP and active UL BWP in a serving cell except for SUL at a given time - No RAN1 CR is needed

2. Actions:

To RAN2:

RAN1 kindly asks RAN2 to take the above information into considerations for Rel-15/16 maintenance works and it’s up to RAN2 to decide whether/how to clarify the definition of UE capability parameter bwp-DiffNumerology in Rel-15 & Rel-16 based on the above conclusion.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 104bis 	12 – 20 April 2021	e-Meeting

TSG-RAN WG1 Meeting 105	19 – 27 May 2021 	e-Meeting