3GPP TSG RAN WG2#113bis-e	R2-2102628

e-Meeting, 12th - 20th April, 2021

3GPP TSG-RAN WG1 Meeting #104-e	R1-2102249

e-Meeting, January 25th – February 5th, 2021

Title:	LS on UL skipping for PUSCH in Rel-16

Release:	Rel-16

Work Item:	NR_newRAT-Core, TEI16

Source:	RAN1

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:	                Xiaohang Chen

E-mail Address: chenxiaohang@vivo.com

# 1. Overall Description:

In RAN1 #104-e meeting, RAN1 discussed the behavior of PUSCH skipping with UCI for Case 1-6 considering both dynamic grant and configured grant in Rel-16. RAN1 made the following agreement.

Agreement: For the case (Case 1-6) when DG PUSCH and CG PUSCH are overlapping on a serving cell and CG PUSCH is overlapping with PUCCH, and DG PUSCH is non-overlapping with the PUCCH - In Rel-16, when timeline condition is met, for Case 1-6 in non-CA and CA cases, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied, - When one or more CG PUSCH(s) overlap with a PUCCH on a same or different serving cell, a DG PUSCH overlaps with the one or more CG PUSCH(s) on one serving cell and the DG PUSCH does not overlap with the PUCCH, and there is no remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH, the UCI is transmitted on the PUCCH. - This is for case 1-6a and 1-6b in Figure 1. - MAC does not generate PDU for the one or more CG PUSCH(s) - If there is data for the DG PUSCH, MAC generates PDU for the DG PUSCH. If there is no data for the DG PUSCH, MAC does not generate PDU for the DG PUSCH - When one or more CG PUSCH(s) overlap with a PUCCH on a same or different serving cell, a DG PUSCH overlaps with the one or more CG PUSCH(s) on one serving cell and the DG PUSCH does not overlap with the PUCCH, and there is remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH, the PUSCH from the remaining PUSCH(s) for UCI multiplexing is determined following the existing UCI multiplexing rules, MAC generates MAC PDU for the PUSCH and delivers the MAC PDU to PHY and the UCI is multiplexed on the PUSCH. - Note the remaining CG PUSCH(s) are not overlapping with any DG PUSCH on the same serving cell - This is for case 1-6c in Figure 1. - MAC does not generate PDU for the one or more CG PUSCH(s) - If there is data for the DG PUSCH, MAC generates PDU for the DG PUSCH. If there is no data for the DG PUSCH, MAC does not generate PDU for the DG PUSCH

| Case 1-6a | Case 1-6b |
| --- | --- |
| Case 1-6c |  |

Figure 1: Case 1-6a/1-6b/1-6c for non-CA and CA case

# 2. Actions:

RAN1 respectfully asks RAN2 to

- Take into account the above agreements and update Rel-16 RAN2 specifications (if needed) to support Rel-16 PUSCH skipping

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #104bis-e	12th April – 20th April 2021	E-meeting.

TSG-RAN WG1 Meeting #105-e	19th May – 27th May 2021	E-meeting.