3GPP TSG RAN WG2#113bis-e	R2-2102642

e-Meeting, 12th - 20th April, 2021

3GPP TSG RAN WG3 #111-e			R3-211338

Online, January 25th – February 4th 2021

Title:	Reply LS on Conditional PSCell Addition/Change agreements

Response to:	R2-2010850

Release:	Release 17

Work Item:	LTE_NR_DC_enh-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Yan Wang

Wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description

RAN3 thanks RAN2 for the LS R2-2010850 on Conditional PSCell Addition/Change agreements.

RAN3 discussed how to support CPAC and would like to consider following information as relevant for RAN2.

- Relevant RAN3 agreements and working assumptions:

  - Target SN to make the decision on the prepared PSCells.

  - WA: Prepare multiple PSCells in one CPAC procedure.

  - In CPA and MN initiated inter-SN CPC, MN does not send execution condition(s) to the Target SN, Target SN provides the prepared PSCell id(s) and the corresponding RRC container(s) (RRCReconfiguration) to the MN, and then the MN generates and transmits the conditional configuration message to the UE.

  - WA: In SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.

- About the inter-node RRC container design

  - In case multiple PSCells are prepared in one CPAC procedure, RAN3 would like to ask RAN2 to feedback on the inter-node RRC container design: will one RRC container for one PSCell be used, or one RRC container for multiple PSCells?

- About the SN initiated inter-SN CPC, RAN3 would like to ask RAN2 to feedback on the following two alternatives:

  - Alternative 1: MN performs the association between the execution condition received from the source SN and the RRC configuration of the candidate PSCell received from the candidate SN.

  - Alternative 2: MN forwards the execution condition received from the source SN to the candidate SN. The candidate SN sends the execution condition and the RRC configuration of the candidate PSCell to the MN.

- About the question asked by RAN2:

Furthermore, Rel-17 CPAC is expected to support the preparation and configuration of multiple PSCell candidate cells. RAN3 is therefore asked to check whether the legacy XnAP/ X2AP signalling is sufficient or whether it shall be extended in Rel-17.

  - Based on the RAN3 progress above, the legacy XnAP/X2AP signalling has to be extended.

# 2. Actions

To RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take the above information into account and to provide feedback on the inter-node RRC container design, and the two alternatives for the SN initiated inter-SN CPC handling.

# 3. Dates of Next TSG-RAN3 Meetings

3GPP RAN3#112 -e 		17 May – 28 May 2021			Electronic Meeting

3GPP RAN3#113 		23 August – 27 August 2021			Toulouse, FR