3GPP TSG-RAN WG2 Meeting #115 electronic	                      	R2-2107030
Online, August, 2021

Source:	OPPO

Title:	Support UE onboarding and provisioning for NPN

Agenda Item:	8.16.3

Document for:	Discussion and Decision

- Introduction

For onboarding, the following WID scope was agreed [1]:

- Support UE onboarding and provisioning for NPN including:

  - The UE onboarding relevant parameter broadcast from SIB [RAN2]

  - The associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3]

  - The necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

During RAN2#114_e meeting, the following agreements were made [2]:

[029] No additional information except for the already agreed broadcast parameters is needed, unless requested by other WG.

[029] There is no need to introduce the 1-bit onboarding indication in SIB1 and optional GINs for PLMNs acting as onboarding networks.

[029] Toggling the 1-bit onboarding indication in SIB1 allows to control congestion due to onboarding request.

[029] RAN2 confirms that onboarding does not impact the cell reselection procedure.

[029] For AMF routing, no extra information is needed in addition to the already agreed onboarding request indication in RRCSetupComplete, unless explicitly requested by other WGs.

[029] Any limitation to a selected set of UEs using uSIM tags is out of RAN2 scope.

In this contribution, we’d like to discuss the leftover issues for onboarding.

- Enhancement on support UE onboarding and provisioning for NPN

From signaling perspective, the same GIN can be associated to more than one SNPN ID, but this configuration is up to network implementation, signaling optimization is not essential.

Proposal 1: From signaling perspective, it’s possible that the same GIN is associated to more than one SNPN ID, but it’s up to network implementation to have this kind of configuration.

Onboarding is a one-shot procedure, once onboarding registration completed, UE will detach from the network selected during on boarding registration procedure. On top of this, in RAN2#114_e meeting, RAN2 confirms that onboarding will not impact cell reselection [2]. It’s still FFS whether any impact is addressed for cell selection for onboarding.

In the SA2 reply LS, SA2 thinks the ‘onboardingEnable’ bit can be set/enabled per cell [3]. In other words, the ‘onboardingEnable’ bit is not set uniformly per SNPN.

Observation 1: The ‘onboardingEnable’ bit is not set uniformly per SNPN based on SA2 clarification.

If only selected SNPN ID is considered in the suitable cell criteria during onboarding cell selection procedure, i.e. follow the legacy mechanism for cell selection, UE may camp on a cell which does not support onboarding.

Observation 2: Reusing the legacy suitable cell criteria may cause onboarding UE to camp on a cell which does not support onboarding.

To solve this problem, suitable cell criteria should be enhanced for onboarding. Once UE NAS informs AS to do onboarding registration, UE AS should find a cell which supports onboarding, i.e. the ‘onboardingEnable’ bit associated to the selected SNPN should be set. Based on the analysis above, we think the suitable cell criteria for onboarding should be enhanced like the following:

For UE initiating onboarding, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

-    The ‘onboardingEnable’ bit associated to the selected SNPN or the registered SNPN of the UE is set;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-    The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to either the selected SNPN or the registered SNPN of the UE.

Proposal 2: For onboarding, the suitable cell criteria should be enhanced like the following:

For UE initiating onboarding, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

-     The ‘onboardingEnable’ bit associated to the selected SNPN or the registered SNPN of the UE is set;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-   The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to either the selected SNPN or the registered SNPN of the UE.

During UE NAS SNPN selection procedure for onboarding, the GIN list configured in UE NAS can also be used for SNPN selection, but the outcome is a selected SNPN ID, which is associated to the selected GIN, so we think there is no need to inform UE AS which GIN is selected, the selected SNPN ID is sufficient.

Proposal 3: During onboarding procedure, the GIN associated to the selected SNPN is invisible to UE AS.

On the other hand, UE AS will have no idea whether the selected SNPN is used for normal registration or onboarding registration if UE NAS only sends the selected SNPN to UE AS. Based on our view in proposal1, we think the UE NAS should also send the onboarding indication along with the selected SNPN for onboarding cell selection.

Proposal 4: During onboarding procedure, UE NAS should send the onboarding indication to UE AS for cell selection.

In SA2 reply LS, load control was also mentioned [3]:

when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs.

RAN2 agreed that Onboarding indicator is configured per SNPN ID, if RAN2 uses the same indicator for cell load control purpose, the cell load is controlled per SNPN. But actually, when the one SNPN network is overload for onboarding, the same situation for the other SNPN networks as all the SNPN networks in one cell share the same resources of that cell. It’s better to have a per cell indicator to bar all UEs for Onboarding access.

Proposal 5: RAN2 to discuss whether to introduce a per cell barring indicator for cell access control for onboarding.

Some companies would like to discuss UAC enhancement for onboarding, we think coordination with SA1/CT1 is needed before discussing the details in RAN2.

Proposal 6: RAN2 to discuss whether to enhance the UAC mechanism for onboarding.

- Conclusion

In conclusion, we propose the followings:

Proposal 1: From signaling perspective, it’s possible that the same GIN is associated to more than one SNPN ID, but it’s up to network implementation to have this kind of configuration.

Observation 1: The ‘onboardingEnable’ bit is not set uniformly per SNPN based on SA2 clarification.

Observation 2: Reusing the legacy suitable cell criteria may cause onboarding UE to camp on a cell which does not support onboarding.

Proposal 2: For onboarding, the suitable cell criteria should be enhanced like the following:

For UE initiating onboarding, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

-     The ‘onboardingEnable’ bit associated to the selected SNPN or the registered SNPN of the UE is set;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-   The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to either the selected SNPN or the registered SNPN of the UE.

Proposal 3: During onboarding procedure, the GIN associated to the selected SNPN is invisible to UE AS.

Proposal 4: During onboarding procedure, UE NAS should send the onboarding indication to UE AS for cell selection.

Proposal 5: RAN2 to discuss whether to introduce a per cell barring indicator for cell access control for onboarding.

Proposal 6: RAN2 to discuss whether to enhance the UAC mechanism for onboarding.

- References

[1] 3GPP RP-202363 revision of ePRN WID

[2] 3GPP RAN2#114_e Chairman Notes

[3] S2-2101076 Reply LS on clarification request for eNPN feature