3GPP TSG-RAN WG2 #115-e	R2-2107042

E-meeting, August 2021

Agenda Item:	8.15.3

Source:	OPPO

Title:	Discussion on resource allocation enhancement

Document for:	Discussion, Decision

Introduction

The resource allocation enhancement includes the following two parts:

- Specify resource allocation to reduce power consumption of the UEs by introducing random resource selection and partial sensing;

- Enhance reliability and reduce latency in mode 2 by introducing inter-UE coordination mechanism.

This paper will have some high-level discussion on inter-UE coordination and partial/random resource selection.

Discussion

Random selection and partial sensing

For random resource selection, RAN1 has working on it and made some conclusions, and they are studying the impact and potential solutions of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes.

For partial sensing resource selection, mechanism for both periodic and aperiodic traffic are designed, i.e. the periodic-based partial sensing and contiguous partial sensing considering the interaction with DRX.

RAN1 is working on random and partial sensing resource selection including the specific mechanism, the resource collision impact and the interaction with DRX.

RAN2 wait for RAN1 progress on the relationship between resource selection and SL DRX.

Inter-UE coordination

RAN1#104b-e has made the following progress on inter-UE coordination:

- Two schemes are concluded:

Scheme-1: the set of resources preferred and/or non-preferred for UE-B’s transmission;

Scheme-2: the presence of expected/potential/detected resource conflict on the resources indicated by UE-B’s SCI

- How for UE-B to take the inter-UE coordination information into account in the resource (re)-selection for its own transmission for each scheme:

- For scheme 1:

- Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

- Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

- Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

- Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

- For scheme 2:

- Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

- Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

Besides, they are also studying the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination. However, there are also some FFS points need further study, so RAN2 impact can only be identified after RAN1 make clear decision on the key points above.

From RAN2 perspective, if L2/RRC signalling is used to carry such “a set of resource” message, our effort may be needed to design the solution for

- Message delivery over PC5, if the “set of resources” are to be delivered to UE-B using RRC/MAC-CE;

- Message delivery over Uu, if the “set of resources” are to be delivered to network (serving UE-B) via RRC/MAC-CE;

If L2/RRC signalling is used, message delivery over both PC5 and Uu needed to be designed by RAN2.

For detailed inter-UE coordination design, RAN2 wait for progress in RAN1 first.

Impact on Uu interface

According to the objective of this WID, the solution should be able to operate in-coverage, partial coverage, and out-of-coverage.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

In R16, RRC_CONNECTED UE may report sidelink UE information to the network for updating sidelink communication status. And based on that, network would allocate resource (mode-1) or resource pool (mode-2) to UEs, together with other Tx related parameters.

Network is in charge of Tx resource (pool) configuration for the in-coverage UE in R16.

If UE-B receives the “set of resource” message from UE-A and may take this message account when deciding on Tx resource configuration, it should report the “set of resource” related messages to its serving cell.

The detailed mechanism of this report can be further studied.

RRC_CONNECTED UE-B reports the received “A set of resources” from UE-A to network.

Impact on PC5 interface

In this paper, the feasibility/benefit for inter-UE coordination in different cast types and the initiation mechanism are discussed.

Inter-UE coordination for different cast type

The inter-UE coordination procedure is introduced for enhanced reliability and reducing latency in mode 2, where consecutive packet loss and resource collision issues due to hidden node, exposed-node and half duplex exist. These issues are valid for all cast types, and need to be solved for enhanced reliability and reduced latency.

However, the procedures to exchange information and signalling are different in unicast, groupcast and broadcast.

- In unicast, it’s easy for the pair of UEs to coordinate and exchange signalling since there is a 1:1 relationship and PC5-RRC connection between them.

- In groupcast, in case the connection/topology is stable/static, it’s possible for signaling exchange between UEs. On the other hand, there is no PC5-RRC connection defined, and so far only PHY layer “connection”, exists (e.g., PSFCH), so there seems some uncertainty whether the PHY layer “connection” is capable to carry the inter-UE coordination information.

- In broadcast, it’s hard for inter-UE coordination since the resource set from UE-A can only be sent out in a broadcast manner and it’s almost impossible for UE-B to take all the “resource set messages” from UEs in the proximity into account.

So in general, there is the least / most difficulty to exchange signaling for inter-UE coordination for unicast/broadcast, while there is some FFS point for group-cast.

The feasibility/complexity of supporting inter-UE coordination is different for different cast type.

Inter-UE coordination is supported at least for unicast, FFS on groupcast, but not for broadcast.

Initiation condition of inter-UE coordination

The inter-UE coordination procedure may be initiated at UE-A or UE-B, which can also be categorized as

1. Stand-alone based procedure, or

2. Request-respond based procedure.

In the request-respond based mechanism as follows, an efficient on-demand inter-UE coordination can be realised.

Figure 1: Req-Res based Inter-UE coordination initiation

- Here “on-demand” means UE-B sends an inter-UE coordination request to UE-A when the reliability and latency needed to be improved, for example serious packet loss is detected.

- And some assistance information from UE-B can be included in the inter-UE coordination request message. Then UE-A can decide the “set of resource”, tailored based on UE-B’s requirement.

In the stand-alone mechanism as follows,

Figure 2: Stand-alone Inter-UE coordination initiation

UE-A autonomous initiates the inter-UE coordination procedure if some events occurred, i.e. serious collision was detected on some resource. It should be noted that the “set of resource” for UE-B is derived by UE-A based on UE-B’s previous transmission behaviours, i.e., cannot be adjusted by UE-B’s specific requirement as in Req-Res procedure.

The following table gives the comparison between these two different ways:

Table 1 Comparison between UE-A initiated and UE-B initiated case

|  | Trigger event | Pros | Cons |
| --- | --- | --- | --- |
| Req-Res type | Initiated by UE-B (can be up to UE-B implementation) | 1. UE-B can include detailed requirement information in the request message 2.No overhead issue 3. UE-B can acquire the ”set of resource” from UE-A before the first transmission | Delay issue |
| Standalone type | Initiated by UE-A | Less delay | 1.Overhead issue 2. No “set of resource” from UE-A at the beginning of UE-B’s transmission |

As shown in Table 1, there are some pros and cons for both ways

- For UE-B initiated solution, the coordination maybe more effective since

- UE-B can include specific configuration in the request message and UE-A can decide the “set of resource” based on UE-B’s configuration.

- In addition, UE-B can send the request message to UE-A before the first resource selection and acquire the “set of resource” at the beginning of transmission.

- However, the delay issue exists since the signalling exchange takes two steps.

- For UE-A initiated solution,

  - UE-A can only decide the “set of resource” based on pervious observation by its own, i.e., UE-A’s reception, hidden node, collision and so on.

  - In this case, the “set of resource” may be helpful for UE-B or maybe not,

  - And the “set of resource” can only be decided after receiving messages from UE-B.

  - Besides, overhead issue needs to be considered especially in broadcast and groupcast.

In general, the comparison is between delay issue and resource efficiency issue

- The delay in UE-B initiating is not critical for periodic transmission, because in periodic transmission, inter-UE coordination can enhance future transmission in mode 2 based on historical information. And in dynamic transmission, the delay can be mitigated by some specific design, such as using the physical layer signalling.

- Regarding the resource efficiency issue in UE-A initiating, as discussed before, the “set of resource” may be not useful for UE-B which means a waste of resource and may bring more collision. And it seems no effective solutions for the no assistant information at first issue.

In general, the request-based way is more reasonable and beneficial.

UE-A delivery of “a set of resource” information based on request from UE-B.

Conclusion

We have the following observations:

Observation 1	RAN1 is working on random and partial sensing resource selection including the specific mechanism, the resource collision impact and the interaction with DRX.

Observation 2	If L2/RRC signalling is used, message delivery over both PC5 and Uu needed to be designed by RAN2.

Observation 3	Network is in charge of Tx resource (pool) configuration for the in-coverage UE in R16.

Observation 4	The feasibility/complexity of supporting inter-UE coordination is different for different cast type.

We have the following proposals:

Proposal 1	RAN2 wait for RAN1 progress on the relationship between resource selection and SL DRX.

Proposal 2	For detailed inter-UE coordination design, RAN2 wait for progress in RAN1 first.

Proposal 3	RRC_CONNECTED UE-B reports the received “A set of resources” from UE-A to network.

Proposal 4	Inter-UE coordination is supported at least for unicast, FFS on groupcast, but not for broadcast.

Proposal 5	UE-A delivery of “a set of resource” information based on request from UE-B.

Reference

- RAN1 Chairman’s Notes for 3GPP TSG RAN WG1 Meeting #104b-e

- RAN1 Chairman’s Notes for 3GPP TSG RAN WG1 Meeting #105-e